| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-N0169 |
| --- | --- |

| Title: | Template-based palette prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenjing Zhu Haitao Yang Huawei Industrial Base, Bantian Longgang District, Shenzhen China 518129. | Tel: Email: | wenjing.zhu@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes a template-based palette prediction method. In contrast with existing palette coding scheme where all palette elements of the current block are directly coded, an element in the palette of the current CU can be predicted from an element in the palette template with the proposed palette prediction method. The palette template is initialized in the beginning of a slice, and is updated every time when a CU with palette coding mode is processed. The proposed method is compared against the existing palette coding scheme MBCIM in the performance evaluation. Simulation results show that compared to RCE3.2, on average 1.3%, 2.6% and 2.2% bit-saving are achieved for class F, SC RGB 444 and SC YUV 444 sequences in the All Intra HE Main-tier case; 1.1%, 2.7% and 1.8% bit-saving are achieved for class F, SC RGB 444 and SC YUV 444 sequences in the Random Access HE Main-tier case; 0.8%, 2.6% and 2.0% bit-saving are achieved for class F, SC RGB 444 and SC YUV 444 sequences in the Low Delay HE Main-tier case.

# Introduction

The coding of screen contents plays an important role in applications like remote desktop control and on-line collaboration. Screen contents refer to images/videos generated/rendered by computers or some other electronic devices, e.g., mobile phone. Screen contents tend to have sharp edge, large region of background and colourful foreground. A good way to compress the screen content is palette plus index map. For palette-coding CU, a palette is first transmitted, then for each sample, an index instead of signal intensity is transmitted. The index is used to find the representative intensity value in the palette. For screen contents, a whole picture may share the same or similar background colors; neighbouring CUs may have similar foreground colors and background colors, so palette prediction can be a way to save bits.

In this contribution, a template-based palette prediction method is proposed; the template can extract similarity colors in terms of temporal and spatial dimension.

# Technical Detail

## Palette template

Palette template is a palette for palette prediction. Every color component has a palette template. The elements of the palette template are most frequently used intensity values for all the previously coded blocks with palette coding mode within a slice. The template size for luma is 8, and the template size for chroma is 2 for 4:2:0, 4 for 4:2:2 and 8 for 4:4:4.

For each element in the palette template, a counter is used to record the number of times it appeared in the palette of previously coded blocks. When a slice starts, all elements in the palette template are initialized with -1 and the counters are initialized with 0. The palette template and the counters are updated every time a palette coded block is processed.

The update mechanism for the palette template is as follows: 1) counters of all template elements is decreased by 1, to eliminate elements that haven’t been used recently; 2) the counter of a template element is increased by 2 if the element can find a matching element in the palette of the current block, an element in the current block palette is consider as the matching element of a template element if the difference of the two is less than 4; 3) insert the elements of the current palette into the palette template one by one in case there is a template element whose counter is less than 2, the counter of the newly added template element is set to 2.

## Template-based palette prediction and coding

For every element in current palette, check every element in the template to find its matching element. Once a matching element is found, the element in the current palette is not directly coded, but predicted by this matching element. In other word, the matching element is used as the predictor of the element in the current palette. The difference between the element and its predictor is signalled to the decoder.

Figure 1, Palette prediciton

Suppose the palette of current CU has 6 elements, the 6 elements are (40,80,125,134,154,206). Suppose the 8 elements of current palette template are (62,89,110,128,154,189,205,240), in the palette, according to the matching rule, 125,154 and 206 can find their matching elements in the template, so for current template, the element prediction flags is (0,0,0,1,1,0,1,0), the prediction difference is -3, 0 and 1.

For every palette coded CU, there is a palette prediction flag indicating whether this CU utilizes the proposed palette prediction method or not. If the proposed palette prediction method is used, for every element in the palette template, an element prediction flag is signalled to the decoder to indicate whether the element is predicted from the palette template or the element is directly coded. When an element in the current template is predicted, the prediction difference is transmitted. The prediction difference has two parts, absolute value and sign. Because there is a restriction that the absolute value is less than or equal to 3, so the TU code with max symbol equal to 3 is used for coding this absolute value. The sign is coded when the absolute value is not zero. The elements not predictive coded are directly coded.

# Experiment Results

Table 1 Result compared with RCE3.2 under lossy configuration

|  |  | All Intra HE Main-tier |  |  |  |  |  | All Intra HE High-tier |  |  |  |  |  |  |  |  |  | All Intra HE Super-High-tier |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U |  | V |  |  | Y |  | U |  |  | V |  |  |  |  | Y |  | U |  | V |  |  |  |
| Class F |  | -1.3% | -1.3% |  | -1.3% |  |  | -1.4% |  | -1.2% |  |  | -1.2% |  |  |  |  | -1.5% |  | -1.1% |  | -1.1% |  |  |  |
| Class B |  | 0.0% | 0.0% |  | 0.0% |  |  | 0.0% |  | 0.0% |  |  | 0.0% |  |  |  |  | 0.0% |  | 0.0% |  | 0.0% |  |  |  |
| SC RGB 444 |  | -2.6% | -2.4% |  | -2.4% |  |  | -2.9% |  | -2.8% |  |  | -2.8% |  |  |  |  | -3.4% |  | -3.2% |  | -3.2% |  |  |  |
| SC YUV 444 |  | -2.2% | -2.1% |  | -2.2% |  |  | -2.5% |  | -2.4% |  |  | -2.3% |  |  |  |  | -2.8% |  | -2.5% |  | -2.5% |  |  |  |
| RangeExt |  | 0.0% | 0.0% |  | 0.0% |  |  | 0.0% |  | 0.0% |  |  | 0.0% |  |  |  |  | 0.0% |  | 0.0% |  | 0.0% |  |  |  |
| Enc Time[%] |  | 100% |  |  |  |  |  | 100% |  |  |  |  |  |  |  |  |  | 101% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 99% |  |  |  |  |  | 99% |  |  |  |  |  |  |  |  |  | 100% |  |  |  |  |  |  |  |
|  |  | Random Access HE Main-tier |  |  |  |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | U |  | V |  |  | Y |  | U |  |  | V |  |  |  |  |  |  |  |  |  |  |  |  |
| Class F |  | -1.1% | -1.2% |  | -1.1% |  |  | -1.1% |  | -1.1% |  |  | -1.0% |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.0% | 0.1% |  | 0.1% |  |  | 0.0% |  | 0.0% |  |  | 0.0% |  |  |  |  |  |  |  |  |  |  |  |  |
| SC RGB 444 |  | -2.7% | -2.6% |  | -2.5% |  |  | -3.0% |  | -2.7% |  |  | -2.7% |  |  |  |  |  |  |  |  |  |  |  |  |
| SC YUV 444 |  | -1.8% | -1.8% |  | -1.7% |  |  | -2.1% |  | -2.0% |  |  | -1.9% |  |  |  |  |  |  |  |  |  |  |  |  |
| RangeExt |  | 0.0% | 0.0% |  | 0.0% |  |  | 0.0% |  | 0.0% |  |  | 0.0% |  |  |  |  |  |  |  |  |  |  |  |  |
| Enc Time[%] |  | 100% |  |  |  |  |  | 100% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  | 100% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Low delay B HE Main-tier |  |  |  |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | U |  | V |  |  | Y |  | U |  |  | V |  |  |  |  |  |  |  |  |  |  |  |  |
| Class F |  | -0.8% | -0.7% |  | -0.5% |  |  | -0.8% |  | -0.7% |  |  | -0.7% |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.0% | -0.1% |  | -0.1% |  |  | 0.0% |  | 0.0% |  |  | -0.1% |  |  |  |  |  |  |  |  |  |  |  |  |
| SC RGB 444 |  | -2.6% | -2.4% |  | -2.5% |  |  | -3.1% |  | -2.8% |  |  | -2.9% |  |  |  |  |  |  |  |  |  |  |  |  |
| SC YUV 444 |  | -2.0% | -1.9% |  | -1.8% |  |  | -2.2% |  | -2.1% |  |  | -2.1% |  |  |  |  |  |  |  |  |  |  |  |  |
| RangeExt |  | 0.0% | 0.1% |  | 0.1% |  |  | 0.0% |  | 0.0% |  |  | 0.0% |  |  |  |  |  |  |  |  |  |  |  |  |
| Enc Time[%] |  | 100% |  |  |  |  |  | 100% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  | 100% |  |  |  |  |  | 100% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Refer | Tested |  | Refer | Tested |  | Refer | Tested |  |
| Class F | 5.3 | 5.3 | -0.1% | 31.7 | 31.7 | 0.0% | 49.8 | 49.8 | 0.0% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB | 18.6 | 19.6 | -2.7% | 189.8 | 201.6 | -2.9% | 747.3 | 801.0 | -3.8% |
| SC YUV | 15.0 | 15.7 | -2.7% | 169.6 | 178.3 | -2.7% | 466.8 | 497.8 | -2.9% |
| RangEx | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| EncT[%] | 97% |  |  | 100% |  |  | 99% |  |  |
| DecT[%] | 100% |  |  | 99% |  |  | 100% |  |  |

# Draft Texts

Text changes are based on text provided in RCE3. All changes are highlighted in yellow.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| MBCIM_mode_flag[x0][y0] | ae(v) |
| if (mbcim_mode_flag) { |  |
| mbcim_coding ( x0, y0, nCbS, nCbS, 0) |  |
| mbcim_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC , 1) |  |
| mbcim_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC, 2) |  |
| } |  |
| else{ |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| …… |  |

| mbcim_coding ( x0, y0, nPbW, nPbH , cIdx){ |  |
| --- | --- |
| color_size_minus1[x0][y0] | ae(v) |
| palette_predict_flag[x0][y0] | ae(v) |
| If(paletter_predict_flag[x0][y0]){ |  |
| for( i = 0; i <palette_template_size[cIdx]; i ++ ) |  |
| palette_predict_indication[x0][y0][i] | u(1) |
| If(palette_predict_indication[x0][y0][i]) |  |
| palette_predict_difference_abs[x0][y0][i] | ae(v) |
| If(palette_predict_difference_abs[x0][y0][i]) |  |
| palette_predict_difference_sign[x0][y0][i] | u(1) |
| } |  |
| for( i = 0; i <=color_size_minus1[x0][y0]-prediction_number; i ++ ) |  |
| color_table[x0][y0][i+prediction_number] | ae(v) |
| for( j = 0; j < nPbW; j = j + 1) |  |
| for( i = 0; i < nPbH ; i = i + 1) |  |
| prev_index_first_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nPbW; j = j + 1) |  |
| for( i = 0; i < nPbH ; i = i + 1) |  |
| if(prev_index_first_pred_flag [ x0 + i ][ y0 + j ] && color_size_minus1 [x0][y0] > 1) |  |
| prev_index_second_pred_flag [ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nPbW; j = j + 1) |  |
| for( i = 0; i < nPbH ; i = i + 1) |  |
| if ( prev_index_first_pred_flag [ x0 + i ][ y0 + j ] && prev_index_second_pred_flag [ x0 + i ][ y0 + j ] && color_size_minus1 [x0][y0] > 2 ) |  |
| rem_index [ x0 + i ][ y0 + j ] | ae(v) |

## Semantics

MBCIM_mode_flag[x0][y0] equal to 1 specifies that the current coding unit is encoded using BCIM mode. palette_mode_flag  equal to 0 specifies the current coding unti is encoded using a mode other than BCIM mode.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

color_size_minus1[x0][y0] plus 1 specifies the number of elements in the color table. palette_size_minus1[x0][y0] shall be the range of 0 to 15.

When palette_size_minus1 [x0][y0] is not present, it is inferred to be equal to 0.

palette_predict_flag specifies that current MBCIM CU utilizes palette predict mode, When equal to 1 , palette_predict_indication[x0][y0][i], i = 0..palette_template_size[cIdx] need to be parsed.

When palette_predict_flag is not present, it is inferred to be equal to 0.

palette_predict_indication[x0][y0][i] specifies that the i-th element of palette template is used to prediction one element of current palette, When equal to 1, palette_predict_difference_abs[x0][y0][i] needs to be parsed.

When palette_predict_indication is not present, it is inferred to be equal to 0.

palette_predict_difference_abs[x0][y0][i] specifies the absolute value  of palette prediction difference of the i-th element of palette template, When larger than 0, palette_predict_difference_sign[x0][y0][i] needs to be parsed.

palette_predict_difference_sign[x0][y0][i] specifies the sign of palette prediction difference of the i-th element of palette template

color_table [x0][y0][i] specifies the i-th element in the color table.

prev_index_first_pred_flag[ x0 + i ][ y0 + j ] equal to 1 specifies that the current index value x is equal to the first prediction index and the decoded pixel value equals to color_table [x0][y0][x].

prev_index_second_pred_flag[ x0 + i ][ y0 + j ]  equal to 1 specifies that the current index value x is equal to the second prediction index and the decoded pixel value equals to color_table [x0][y0][x].

rem_index [ x0 + i ][ y0 + j ]  equals to the actual index value x when the first and second prediction indexes are incorrect,  and the decoded pixel value equals to color_table [x0][y0][x].

# Conclusions

This contribution proposes a template-based palette prediction method.

# Reference

- C. Lan, X. Peng,  J. Xu, F. Wu, “Intra and inter coding tools for screen contents”, JCTVC-E0145, Geneva, Switzerland, Mar. 2011.

- L. Guo, M. Karczewicz, J. Sole, “Palette Mode for Screen Content Coding”, JCTVC-M0323, Incheon, Korea, Apr. 2013.

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE 3): Intra Coding Methods for Screen Content”, JCTVC-M1123, Incheon, Korea, Apr. 2013.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).