| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N173r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0088 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On Layers Present SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung Won Kang, Jinho Lee, Hahyun Lee, Jin Soo Choi 218, Gajeong-no, Yuseong, Daejeon, Korea Truong Cong Thang Tsuruga, Ikki-Machi, Aizu-Wakamatsu, Japan 965-8580 | Tel: Email: | {jungwon, jinosoul, hanilee, jschoi}@ etri.re.kr thang@u-aizu.ac.jp |
| Source: | ETRI, University of Aizu |  |  |

_____________________________

# Abstract

Currently the layer present SEI message provides the information of non-present layers after some modification. The current solution just focuses on the layers identified by the nuh_layer_id parameter. In practice, a CVS could be modified in temporal dimension as well. This contribution proposes to describe the (temporal) sub-layers which are not present in a CVS.

(This revision just added the number for JCT-3V)

# Introduction

Currently the layer present SEI message provides the information of non-present layers after some modification. The current solution just focuses on the layers identified by the nuh_layer_id parameter. In practice, a CVS could be modified in temporal dimension as well. So, we propose to describe the temporal layers which are not present in a CVS.

# Proposal

In the revised layers present SEI message, the new value of vps_max_sub_layers_minus1 is signaled when needed. Also, another flag is used to enable/disable the layer present information in the message. The proposed syntax and semantics are as follows.

| layers_present( payloadSize ) { | Descriptor |
| --- | --- |
| lp_sei_active_vps_id | u(4) |
| changed_layers_flag | u(1) |
| changed_max_sub_layers_flag | u(1)` |
| if( changed_layers_flag) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) |  |
| layer_present_flag[ i ] | u(1) |
| if (changed_max_sub_layers_flag) |  |
| new_vps_max_sub_layers_minus1 | u(3) |
| } |  |

- changed_layers_flag equal to 1 indicates that the presence of at least one layer has been changed. This flag equal to 0 indicates that no layers specified in the related VPS are absent.

- changed_max_sub_layers_flag equal to 1 indicates that the maximum number of sub layers has been changed. This flag equal to 0 indicates that the maximum number of sub layers is unchanged.

- new_vps_max_sub_layers_minus1 plus 1 specifies new maximum number of sub layers of the CVS.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Aizu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).