| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0175 |
| --- | --- |

| Title: | RCE2 Test B.3: Mirroring of Coefficients and significance map context |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajitha Weerakkody, Marta Mrak 56 Wood Lane, London, W12 7SB, UK | Tel: Email: | rajitha.weerakkody@bbc.co.uk,marta.mrak@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

Within RCE2 this proposal presents the method of mirroring the residual for non-transformed coefficients (as proposed in JCTVC K0294) and the associated results. The proposed method is applied on blocks of which the transform is not applied in lossy coding and all blocks in lossless coding. Further, results are presented for the cases where in lossless coding, the mirroring is restricted to upto 4×4 and upto 8×8 blocks. Also included is the usage of a constant context for the significance map (from RCE2 Test B.1).

# Introduction

In this document, a mirroring of coefficients in residual blocks [1] is proposed for cases where transform is not applied, either through transform skip in lossy coding, or in general in lossless coding. At the encoder, after prediction and before entropy coding the method rearranges the coefficients in the block. The method chooses between two distinct rearrangements, depending on the scanning mode for the block. Same rearrangements are applied at the decoder before the predictor is applied during reconstruction.

In HEVC, the scanning mode for the non-transformed blocks is chosen as either diagonal, horizontal or vertical depending on the prediction type and mode (if intra). In the proposed method when the diagonal scan is used the mirroring across the antidiagonal is applied, otherwise mirroring across the main diagonal (transpose) is applied. For Inter coded blocks always the mirroring across anti-diagonal applies.

In lossy coding, the mirroring block size is inherently limited to 4×4 through transform skip. For lossless coding, the performance is evaluated for mirroring all block sizes, and then restricted to 4×4 and 8×8 blocks.

In addition, the method of using a constant context for the significance map (as proposed in RCE2 Test B.1) [2] is combined with the above, and the results are presented.

# Implementation

The proposed method is implemented in HM-10.1-rext3.0 by rearranging the samples during the scaling related to transform skip (for lossy coding). Depending on whether the scan for given block is diagonal (diag) or horizontal or vertical, the mirroring diagonal is selected:

- In the xTransformSkip function, the scaling:

psCoeff[j*height + k] = piBlkResi[j * uiStride + k] << transformSkipShift;

To be changed to:

if (diag)

psCoeff[(width*width)-1-k*width-j]=piBlkResi[j*uiStride+k] <<transformSkipShift;

else

psCoeff[k*width + j] = piBlkResi[j * uiStride + k] << transformSkipShift;

- Similarly, in the xITransformSkip function, the inverse scaling:

pResidual[j * uiStride + k] =  (plCoef[j*width+k] + offset) >> transformSkipShift;

To be changed to:

if (diag)

pResidual[j*uiStride+k]=(plCoef[(width*width)-1-width*k-j]+offset)>> transformSkipShift;

else

pResidual[j*uiStride+k]= (plCoef[k*width + j] + offset) >> transformSkipShift;

For lossless coding:

- in the transformNxN function:

rpcCoeff[k*uiWidth+j]= pcResidual[k*uiStride+j];

To be changed to

if (uiHeight <=TS_COEFF_FLIP_LOSSLESS_MAX)

if (diag)

rpcCoeff[(uiWidth*uiHeight) -1 - j*uiWidth -k] = pcResidual[(k * uiStride) + j];

else

rpcCoeff[j*uiWidth + k] = pcResidual[(k * uiStride) + j];

else

rpcCoeff[k*uiWidth+j]= pcResidual[k*uiStride+j];

where TS_COEFF_FLIP_LOSSLESS_MAX is set to the maximum size (4 for 4×4, 8 for 8×8, 32 for All) of the block to be mirrored.

- Similarly in the invTransformNxN function the inverse mapping:

rpcResidual[k*uiStride+j] = pcCoeff[k*uiWidth+j];

To be changed to

if (uiHeight <=TS_COEFF_FLIP_LOSSLESS_MAX)

if (diag)

rpcResidual[(k * uiStride) + j] =  pcCoeff[(uiWidth*uiHeight) - 1 - (uiWidth*j) - k] ;

else

rpcResidual[(k * uiStride) + j] =  pcCoeff[(j * uiWidth) + k] ;

else

rpcResidual[k*uiStride+j] = pcCoeff[k*uiWidth+j];

The implementation of the new context is identical to that in RCE2- Test B.1.

# Results

The methods proposed in this document are implemented in HM10.1-rext3.0 and simulated under the RCE2 common test conditions [3].

## Results for the lossy coding

Table 1. Mirroring for lossy

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.6% | -1.7% | -1.9% | -1.6% | -1.7% | -1.9% | -1.5% | -1.6% | -1.7% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.0% | -1.9% | -1.7% | -1.9% | -1.7% | -1.6% | -1.7% | -1.4% | -1.3% |
| SC YUV 444 | -1.9% | -1.9% | -1.8% | -1.9% | -2.0% | -1.9% | -1.8% | -1.9% | -1.7% |
| RangeExt | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.1% | -1.3% | -1.5% | -1.1% | -1.3% | -1.3% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -1.5% | -1.4% | -1.3% | -1.5% | -1.4% | -1.3% |  |  |  |
| SC YUV 444 | -1.4% | -1.3% | -1.1% | -1.2% | -1.3% | -1.1% |  |  |  |
| RangeExt | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.8% | -0.6% | -1.2% | -0.7% | -0.6% | -1.1% |  |  |  |
| Class B | 0.0% | 0.2% | -0.2% | 0.0% | 0.1% | -0.1% |  |  |  |
| SC RGB 444 | -0.6% | -0.4% | -0.4% | -0.7% | -0.5% | -0.4% |  |  |  |
| SC YUV 444 | -0.8% | -0.6% | -0.5% | -0.7% | -0.7% | -0.7% |  |  |  |
| RangeExt | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 2. Mirroring with Constant Context for lossy

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.1% | -1.9% | -2.2% | -2.1% | -2.1% | -2.3% | -2.1% | -2.1% | -2.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.2% | 0.1% |
| SC RGB 444 | -2.6% | -2.4% | -2.1% | -2.5% | -2.2% | -2.0% | -2.3% | -2.0% | -1.9% |
| SC YUV 444 | -2.2% | -2.1% | -1.9% | -2.2% | -2.2% | -2.1% | -2.2% | -2.1% | -2.0% |
| RangeExt | -0.2% | -0.1% | -0.3% | -0.2% | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.4% | -1.3% | -1.5% | -1.5% | -1.3% | -1.5% |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| SC RGB 444 | -1.9% | -1.6% | -1.5% | -1.9% | -1.5% | -1.4% |  |  |  |
| SC YUV 444 | -1.9% | -1.7% | -1.5% | -1.7% | -1.6% | -1.4% |  |  |  |
| RangeExt | -0.1% | -0.1% | -0.1% | -0.1% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -0.5% | -0.6% | -0.9% | -0.7% | -1.1% |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.3% | -1.1% | -1.0% | -1.6% | -1.2% | -1.1% |  |  |  |
| SC YUV 444 | -0.8% | -0.6% | -0.6% | -0.9% | -0.8% | -0.8% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.1% | 0.2% | 0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

## Results for the lossless coding

Table 3. Mirroring for lossless (all block sizes)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.9% | 31.7 | 31.9 | -0.4% | 49.8 | 50.0 | -0.3% |
| Class B | 2.2 | 2.3 | -0.4% | 2.6 | 2.6 | -0.1% | 2.6 | 2.6 | -0.1% |
| SC RGB 444 | 10.1 | 10.1 | -0.4% | 100.4 | 100.6 | -0.2% | 381.6 | 380.3 | 0.0% |
| SC YUV 444 | 11.4 | 11.4 | -0.6% | 128.9 | 129.3 | -0.2% | 325.6 | 326.8 | -0.3% |
| RangeExt | 2.4 | 2.4 | -0.4% | 2.5 | 2.5 | -0.2% | 2.5 | 2.5 | -0.1% |
| Enc Time[%] | 100% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 95% |  |  | 97% |  |  | 96% |  |  |

Table 4. Mirroring for lossless (4×4 blocks only)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.8% | 31.7 | 31.8 | -0.4% | 49.8 | 50.0 | -0.4% |
| Class B | 2.2 | 2.3 | -0.4% | 2.6 | 2.6 | -0.3% | 2.6 | 2.6 | -0.3% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.5 | -0.1% | 381.6 | 381.6 | -0.2% |
| SC YUV 444 | 11.4 | 11.4 | -0.5% | 128.9 | 129.2 | -0.2% | 325.6 | 327.1 | -0.5% |
| RangeExt | 2.4 | 2.4 | -0.4% | 2.5 | 2.5 | -0.3% | 2.5 | 2.5 | -0.3% |
| Enc Time[%] | 102% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  | 98% |  |  |

Table 5. Mirroring for lossless (up to 8×8 blocks)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.8% | 31.7 | 31.9 | -0.4% | 49.8 | 50.0 | -0.3% |
| Class B | 2.2 | 2.3 | -0.4% | 2.6 | 2.6 | -0.2% | 2.6 | 2.6 | -0.2% |
| SC RGB 444 | 10.1 | 10.1 | -0.3% | 100.4 | 100.6 | -0.2% | 381.6 | 381.6 | -0.1% |
| SC YUV 444 | 11.4 | 11.4 | -0.5% | 128.9 | 129.3 | -0.2% | 325.6 | 326.9 | -0.4% |
| RangeExt | 2.4 | 2.4 | -0.4% | 2.5 | 2.5 | -0.2% | 2.5 | 2.5 | -0.2% |
| Enc Time[%] | 102% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  | 96% |  |  |

Table 6. Mirroring with Constant Context for lossless (all block sizes)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.5% | 31.7 | 32.0 | -0.9% | 49.8 | 50.0 | -0.6% |
| Class B | 2.2 | 2.3 | -0.6% | 2.6 | 2.6 | -0.2% | 2.6 | 2.6 | -0.2% |
| SC RGB 444 | 10.1 | 10.1 | -0.3% | 100.4 | 100.5 | -0.2% | 381.6 | 378.1 | 0.1% |
| SC YUV 444 | 11.4 | 11.4 | -0.8% | 128.9 | 129.4 | -0.5% | 325.6 | 325.7 | -0.5% |
| RangeExt | 2.4 | 2.4 | -0.5% | 2.5 | 2.5 | -0.3% | 2.5 | 2.5 | -0.2% |
| Enc Time[%] | 96% |  |  | 98% |  |  | 96% |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  | 91% |  |  |

Table 7. Mirroring with Constant Context for lossless (4×4 blocks only)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.4% | 31.7 | 32.0 | -1.0% | 49.8 | 50.1 | -0.8% |
| Class B | 2.2 | 2.3 | -0.6% | 2.6 | 2.6 | -0.5% | 2.6 | 2.6 | -0.5% |
| SC RGB 444 | 10.1 | 10.1 | -0.3% | 100.4 | 100.4 | -0.3% | 381.6 | 380.4 | -0.3% |
| SC YUV 444 | 11.4 | 11.4 | -0.7% | 128.9 | 129.6 | -0.7% | 325.6 | 325.3 | -0.6% |
| RangeExt | 2.4 | 2.4 | -0.5% | 2.5 | 2.5 | -0.5% | 2.5 | 2.5 | -0.5% |
| Enc Time[%] | 96% |  |  | 101% |  |  | 97% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  | 94% |  |  |

Table 8. Mirroring with Constant Context for lossless (up to 8×8 blocks)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.4% | 31.7 | 32.0 | -0.9% | 49.8 | 50.1 | -0.7% |
| Class B | 2.2 | 2.3 | -0.5% | 2.6 | 2.6 | -0.3% | 2.6 | 2.6 | -0.3% |
| SC RGB 444 | 10.1 | 10.1 | -0.3% | 100.4 | 100.4 | -0.1% | 381.6 | 380.5 | -0.2% |
| SC YUV 444 | 11.4 | 11.4 | -0.7% | 128.9 | 129.4 | -0.5% | 325.6 | 326.1 | -0.6% |
| RangeExt | 2.4 | 2.4 | -0.5% | 2.5 | 2.5 | -0.3% | 2.5 | 2.5 | -0.3% |
| Enc Time[%] | 97% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  | 96% |  |  |

# Conclusions

When a transform is skipped, simple rearrangements of coefficients can lead to improved compression. The rearrangements proposed in this document rely on mirroring across a diagonal (either the main diagonal or the antidiagonal, depending on the scan index). By rearranging in this way the positions of significant coefficients are more similar to those of transformed blocks. Therefore the compression efficiency improves.

Further, the above method of mirroring is combined with a constant context for non-transformed residual. The gains from the use of constant context are additive to the above.

# References

- R. Weerakkody and M.Mrak, “Mirroring of Coefficients for Transform Skipping”, JCTVC-K0294, Shanghai, CN, October 2012.

- T. Tsukuba and T. Yamamoto, “Constant coefficient context for intra transform skipping”, JCTVC-J0069, Stockholm, SE, July 2012.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, JCTVC-M1122, Incheon, KR, April 2013.

# Working Draft Text

The change to intra transform skipping in the HEVC working draft is highlighted below.

## Mirroring of the residue block

- 8.6.2	Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Qp′Y			(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb			(8-2)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr			(8-3)

The (nTbS)x(nTbS) array of transform coefficient levels TransCoeffLevel[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS – 1 is modified as follows:

	If CuPredMode[ xTbY ][ yTbY ][ cIdx ] is equal to MODE_INTRA,

	If scanIdx is equal to 0,

TransCoeffLevel[ x ][ y ] = TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ nTbS-1-y ][ nTbS-1-x ]

	Otherwise (if scanIdx is equal to 1 or 2),

TransCoeffLevel[ x ][ y ] = TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ y ][ x ]

	Otherwise (If CuPredMode[ xTbY ][ yTbY ][ cIdx ] is equal to MODE_INTER)

TransCoeffLevel[ x ][ y ] = TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ nTbS-1-y ][ nTbS-1-x ]

The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nTbS)x(nTbS) array r is set equal to the (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ x ][ y ]  <<  7 )	(8-4)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

- The variable bdShift is derived as follows:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-5)

- The residual sample values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-6)

## Constrained mirroring of the residue blocks (4×4 blocks only)

- 8.6.2	Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Qp′Y			(8-7)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb		(8-8)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr			(8-9)

The (nTbS)x(nTbS) array of transform coefficient levels TransCoeffLevel[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS – 1 is modified as follows:

	If nTbS is equal to 4,

	If CuPredMode[ xTbY ][ yTbY ][ cIdx ] is equal to MODE_INTRA,

	If scanIdx is equal to 0,

TransCoeffLevel[ x ][ y ] = TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ nTbS-1-y ][ nTbS-1-x ]

	Otherwise (if scanIdx is equal to 1 or 2),

TransCoeffLevel[ x ][ y ] = TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ y ][ x ]

	Otherwise (If CuPredMode[ xTbY ][ yTbY ][ cIdx ] is equal to MODE_INTER)

TransCoeffLevel[ x ][ y ] = TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ nTbS-1-y ][ nTbS-1-x ]

	Otherwise (if nTbS is more than 4),

TransCoeffLevel[ x ][ y ] = TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][x ][ y ]

The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nTbS)x(nTbS) array r is set equal to the (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ x ][ y ]  <<  7 )	(8-10)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

- The variable bdShift is derived as follows:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-11)

- The residual sample values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-12)

## Significance map context

Refer to RCE2 – B.1 for the draft changes for the new context.

# Patent rights declaration(s)

British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).