| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0178 |
| --- | --- |

| Title: | HEVC profiles for medical imaging applications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Andreas Hutter Siemens Corporate Technology Imaging and Computer Vision Munich, Germany Uwe-Erik Martin, Nikolaus Wirsz Siemens Healthcare Imaging and Therapy Systems Erlangen, Germany | Tel: Email: | +49 89 636 54642 p.amon@siemens.com |
| Source: | Siemens |  |  |

_____________________________

# Abstract

This document proposes HEVC profiles for medical imaging applications in the context of the Range Extensions currently standardized in the JCT-VC. The main requirements addressed by theses proposed profiles are high bit-depth compression, lossless compression and signed coding. This results in the four proposed profiles High 12 Intra 4:0:0, High 12 4:0:0, High 16 Intra 4:0:0, and High 16 4:0:0.

# Introduction

In document JCTVC-M0094 [1], several profiles for professional applications supporting higher bit depth and alternative chroma formats are proposed. These seem to be a good starting point for the discussion on HEVC High profiles. The profiles proposed in this contribution are additions to this initial draft.

In document JCTVC-M0190 [2], requirements for medical imaging applications are derived from typical use cases in the healthcare domain. This document proposes profiles for HEVC Range Extensions based on these requirements.

# Proposed Profiles

Instead of defining super sets of profiles, a more detailed specification of conformance points is preferred here (as of option 1 in [1]). From [2], these conformance points are compression of 12-bit and 16-bit continuous-tone images in a lossless fashion. Note that a necessity of specific conformance points for 14-bit compression is not seen, since for the envisioned applications, a common 16-bit profile is regarded as sufficient. Furthermore, lossless coding of the entire set of images should be allowed. This also means that no specific rate constraints should be made on the bases of single pictures, i.e., no minimal compression ratio should be defined. Since there is no notion of time for the compression of the regarded 3-D image volumes, limitations on the rate in terms of bits per second can be set without prohibiting the envisioned applications.

The proposed profiles derived from these conformance points are listed in Table 1.

Table 1.  Proposed profiles for medical imaging applications

| Profile | Chroma Formats |  |  |  | Maximum | Prediction |  | Lossless | Signed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4:0:0 | 4:2:0 | 4:2:2 | 4:4:4 | bit depth | Intra | Inter | coding | coding |
| High 12 Intra 4:0:0 | Yes | No | No | No | 12 | Yes | No | Yes | Yes |
| High 12 4:0:0 | Yes | No | No | No | 12 | Yes | Yes | Yes | Yes |
| High 16 Intra 4:0:0 | Yes | No | No | No | 16 | Yes | No | Yes | Yes |
| High 16 4:0:0 | Yes | No | No | No | 16 | Yes | Yes | Yes | Yes |

Note that the proposed profiles High 16 Intra 4:0:0 and High 16 4:0:0 are already proposed in JCTVC-M0094 [1], which seems to match the definition in this document. While JCTVC-M0094 mentions the generic support for Range Extension coding tools for High 16 (and also High 12) profiles, this contribution mentions lossless coding and signed coding as specific tools in this context. However, this does not mean that other Range Extension coding tools should not be supported by High profiles.

Lossless coding is necessary to allow accurate medical diagnosis [2]. Signed coding is necessary, since medical image data is stored in unsigned or signed (two’s complement) format, depending on the system. Note that, e.g., JPEG 2000 supports the signalling of unsigned/signed representation.

JCTVC-M0190 [2] also defines a requirement for compression of 8-bit RGB continuous-tone and mixed content. This requirement is shared with the application domain of screen content coding and is therefore not specifically addressed in this document.

# Conclusion

This contribution discussed profiles for medical imaging applications, i.e., High 12 and High 16 profiles. These profiles are required for compression of medical image data, e.g., computed tomography (CT) and magnetic resonance (MR) data. Therefore, they should be considered for the definition of HEVC Range Extension profiles.

# References

- T. Suzuki, K. Sharman, N. Saunder ,“ Proposal on the profile definition of Range extensions”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M0094, Incheon, Korea, April 2013.

- P. Amon, A., Hutter, U.-E. Martin, N. Wirsz, A. Klingler, “Requirements for medical imaging applications”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M0190, Incheon, Korea, April 2013.

# Acknowledgement

The research leading to these results has received funding from the European Union’s Seventh Framework Programme ([FP7/2007-2013]) under grant agreement n◦ 288502 (CONCERTO) and from EIT ICT Labs’ Action Line “Networking Solutions for Future Media” under activity n◦ 13090.

# Patent rights declaration(s)

Siemens may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).