| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0179 |
| --- | --- |

| Title: | Selected medical imaging sequences for HEVC development |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Andreas Hutter Siemens Corporate Technology Imaging and Computer Vision Otto-Hahn-Ring 6 81739 Munich, Germany Uwe-Erik Martin Siemens Healthcare Imaging and Therapy Systems Hartmannstr. 16 91052 Erlangen, Germany Benno Heigl Siemens Healthcare Imaging and Therapy Systems Siemensstr. 1 91301 Forchheim, Germany | Tel: Email: | +49 89 636 54642 p.amon@siemens.com |
| Source: | Siemens |  |  |

_____________________________

# Abstract

This contribution proposes new test sequences from the medical domain for the development of HEVC, especially for the specification of range extensions. The proposed set contains 8-bit, 12-bit, and 16-bit monochrome image data as well as 8-bit RGB content.

# Introduction

The current test set for the development of HEVC (including the development of range extensions) includes mainly natural image data captured by a camera and screen content captured from a computer (e.g., capturing the DVI output). Image content from the medical domain is not contained in the current test set. Therefore, and based on discussions in the BoG on range extensions at the January 2013 JCT-VC meeting [1], new test sequences containing ultrasound, computed tomography, magnetic resonance, and mixed image content (including screen content) are proposed.

# Sequence description

The sequences stem from different medical imaging modalities:

- Ultrasound (US) image data: 8-bit RGB, continuous-tone 2D+t image sequences

  - Sequence Doppler (see Figure 1)

    - Resolution 800x600 pixels

    - Frame rate: 12 fps

    - Number of frames: 12

Figure 1.  Example frame from sequence Doppler

  - Sequence FetalHeart (see Figure 2)

    - Resolution: 800x600 pixels

    - Frame rate: 40 fps

    - Number of frames: 90

Figure 2.  Example frame from sequence FetalHeart

- Computed tomography (CT) image data: 12-bit monochrome, continuous-tone 3D image volumes

  - Sequence Cardiac (see Figure 3)

    - Resolution: 512x512 pixels

    - Frame rate: N/A

    - Value range: 0..2056

    - Number of frames: 281

Figure 3.  Example frame from sequence Cardiac

  - Sequence Longrun (see Figure 4), parameters:

    - Resolution 512x512 pixels

    - Frame rate: N/A

    - Number of frames: 1461

    - Value range: 0..2854

Figure 4.  Example frame from sequence Longrun

- Magnetic resonance (MR) image data: 16-bit monochrome, continuous-tone 3D image volumes.

  - Sequence MR_Axial1 (see Figure 5)

    - Resolution: 256x256 pixels

    - Frame rate: N/A

    - Number of frames: 42

    - Value range: 0..1555

Figure 5.  Example frame from sequence MR_Axial1

  - Sequence MR_Axial2 (see Figure 6)

    - Resolution: 512x512 pixels

    - Frame rate: N/A

    - Number of frames: 13

    - Value range: 0..12005

Figure 6.  Example frame from sequence MR_Axial2

  - Sequence MR_Coronal: (see Figure 7)

    - Resolution: 448x512 pixels

    - Frame rate: N/A

    - Number of frames: 45

    - Value range: 0..723

Figure 7.  Example frame from sequence MR_Coronal

- Mixed content (ultrasound image data and screen content): 8-bit RGB image sequences

  - Sequence 19Segments (see Figure 8)

    - Resolution: 800x600 pixels

    - Frame rate: 30 fps

    - Number of frames: 90

Figure 8.  Example frame from sequence 19Segments

- Angiography (AX) image data: 16-bit monochrome, continuous-tone 2D X-ray projections

  - Sequence AX_Head (see Figure 9)

    - Resolution: 1240x960 pixels

    - Frame rate: N/A

    - Number of frames: 496

    - Value range: 400..59905

Figure 9.  Example frame from sequence AX_Head

# Copyright information

Siemens AG remains the owner of the test sequence and any and all intellectual property rights and/or copyrights therein. The test sequence may only be used for the purposes of development of the standards currently contemplated by this Joint Collaborative Team. The test sequence may not be copied or distributed against payment of royalties. Siemens AG‘s liability with respect to the test sequences is excluded to the widest possible extent. This includes but is not limited to any liability for Siemens AG’s ownership of the test sequences and/or any and all intellectual property rights and/or copyrights, as well as any liability for the infringement of third party intellectual property rights and/or copyrights. Furthermore, Siemens AG makes no warranties whatsoever with regard to the test sequences’ fitness for any purpose.

# Recommendation

The presented sequences provide unique characteristics that are common for medical imaging applications. We recommend the adoption of this material as test sequences for the further development of HEVC range extensions.

# Acknowledgement

The research leading to these results has received funding from the European Union’s Seventh Framework Programme ([FP7/2007-2013]) under grant agreement n◦ 288502 (CONCERTO).

# References

- D. Flynn, “BoG on range extensions”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, document JCTVC-L0445/M28323, Geneva, Switzerland, January 2013.

# Patent rights declaration(s)

Siemens does not have any current or pending patent rights relating to the technology described in this contribution.