| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0180 |
| --- | --- |

| Title: | Colour Mapping SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Pierre Andrivon, Franck Hiron 975 avenue des champs blancs CS 17616, 35576, Cesson-Sévigné Cedex, France | Tel: Email: | +33-2-99-27-32-42 philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes a colour mapping SEI message to support colour mapping post-processing on reconstructed frames. The colour mapping function is coded using a 3D Colour LUT.

# Introduction

The rendering of the reconstructed images onto the end-device display is of key importance to insure the end-to-end service quality. However, the variability of colour formats, capture capability and display characteristics tend to increase, while the number of applications and services using colour video is not limited to the traditional but fully controlled TV market.

Recently, a new and wider colour space format has been proposed by ITU (Rec.2020 [2]) and the compatibility with legacy devices may have to be considered.

Figure 1: Pointer’s colors and primaries of different video systems (courtesy from NHK).

All the rendering devices may not have the capability to adapt to any colour space nor have the required knowledge to perform the optimal colour conversion. The determination of the colour conversion function is not straightforward because the content creation workflows may include deterministic processing (Colour Space 1 toward Colour Space 2 conversion) but also non-deterministic operations such as colour grading. If two targeted displays with different characteristics are used (e.g. UHDTV display and Rec.709 HDTVdisplay), the artistic intent may be different and the colour grading may be different too.

Figure 2: Example of colour conversion from Colour-space-1 to Colour-space-2 with clipping (a) or gamut compression (b).

The colour grading depends both on the content characteristics and the reference display: rather than clipping colours one may prefer graceful colour shading for example (Figure 2).

The embedding of 3D Colour LUT in bit-streams, transmitted out-of band eventually, can provide the necessary flexibility and additional features to applications and services on top of HEVC and SHVC.

In the proposed syntax, the 3D Colour LUT is to be applied on the reconstructed pictures of the same layer as SEI nal unit header (nuh_layer_id).

# Colour LUT coding

The principle of the 3D LUT is depicted in Figure 3: the 3D LUT can be considered as a sub-sampling of the 3D color space 1, where each vertex is associated with a color triplet corresponding to the color space 2 values.

Figure 3: Principle of the 3D Color Look-Up Table (LUT).

The 3D Colour LUTs is a common model to represent colour mapping functions and are used by many image processing applications.

In order to encode the 3D LUT data efficiently, each colour component of a vertex is encoded with previously encoded colour components of neighboring vertices. We propose also to use an octree based description of the 3D LUT in such a way the unused (or less used) 3D colour space regions are encoded with coarsely lattice size as depicted in Figure 4Error: Reference source not found.

At the decoder side, all the not coded vertices inside one octant are interpolated to reconstruct the full definition 3D LUT.

Figure 4: Octree based 3D LUT: each octant is encoded with 8 vertices at most.

The size of the 3D LUT (number of vertices in one direction), is a parameter read in the bit-stream. In that way, one may choose the colour mapping function accuracy.

# Example of use of the Colour Mapping SEI message

In order to demonstrate the usage of the Colour Mapping SEI message, we provide an example of software application based on a modified SHM2.0 encoder and decoder.

## Encoder

The encoder allow to load one 3D colour LUT per layer and to encode one Colour Mapping SEI message per layer. The additional parameters are:

SeiColorLut0 : 1

SeiColorLutFile0 : lutFileBL.txt

SeiColorLut1 : 1

SeiColorLutFile1 : lutFileEL.txt

The corresponding command line parameters allow to set the 3D LUT to be applied to the reconstructed frames for each layer separately:

TAppEncoderStatic –c parameters.cfg –seiColorLut0 1 –SeiColorLutFile0 lutFileBL.txt –seiColorLut1 1 –SeiColorLutFile1 lutFileEL.txt

## Decoder

The Decoder receiving a Colour Mapping SEI message will post-process the output reconstructed frames (output file) of the corresponding layer with the corresponding 3D colour LUT. The reference frames in the DPB remain unchanged.

# Proposed syntax

| colour_mapping_info( payloadSize ) { | Descriptor |
| --- | --- |
| colour_map_id | ue(v) |
| colour_map_cancel_flag | u(1) |
| if( !colour_map_cancel_flag ) { |  |
| colour_map_repetition_period | ue(v) |
| colour_map_video_signal_type_present_flag | u(1) |
| if(colour_map_video_signal_type_present_flag ) { |  |
| colour_map_video_format | u(3) |
| colour_map_video_full_range_flag | u(1) |
| colour_map_description_present_flag | u(1) |
| if (colour_map_description_present_flag) { |  |
| colour_map_primaries | u(8) |
| colour_map_transfer_characteristics | u(8) |
| colour_map_matrix_coeffs | u(8) |
| } |  |
| } |  |
| 3D_ LUT_ colour_data ( ) |  |
| } |  |
| } |  |

Table 1: coding of the 3D LUT colour data.

| 3D_ LUT_ colour_data ( ) { | Descriptor |
| --- | --- |
| bit_depth_lut_minus8 | u(4) |
| nbp_code | u(3) |
| coding_octant(0, 0, 0, 0) |  |
| } |  |

Table 2: Interpretation of nbp_code.

| nbp_code | 3D LUT size |
| --- | --- |
| 0 | reserved |
| 1 | 2 |
| 2 | 3 |
| 3 | 5 |
| 4 | 9 |
| 5 | 17 |
| 6 | 33 |

Table 3: syntax elements for coding_octant().

| coding_octant ( layer_id, y,u,v) { | Descriptor |
| --- | --- |
| for( i = 0; i < 8 ; i++ ) { |  |
| encoded_vertex_flag[i] | u(1) |
| if ( encoded_vertex_flag[i] ) { |  |
| resY[i] | ue(v) |
| resU[i] | ue(v) |
| resV[i] | ue(v) |
| } |  |
| } |  |
| split_octant_flag | u(1) |
| if ( split_octant_flag ) { |  |
| for( i = 0; i < 8 ; i++ ) { |  |
| coding_octant ( layer_id+1, y+dy[i],u+du[i],v+dv[i]) |  |
| } |  |
| } |  |
| } |  |

Figure 5: Relative positions of the 8 vertices in one octant of layer_id=layer (bit_depth_lut_minus8=0).

Figure 6: One octant (layer_id) in red, and itsparent octant in black (layer_id – 1).

|  |  | - - |
| --- | --- | --- |
|  |  | - |
|  |  | - - |
|  |  | - -- |
|  |  | - -- - -- - - - - |
|  |  | - -- - - - - |
|  |  | - - |

  - Colour mapping information SEI message semantics

This SEI message provides information to enable remapping of the colour samples of the output decoded pictures for customization to particular applications or display environments. The remapped samples are expressed either in the luma and chroma or RGB colour space domain, and should be applied to the reconstructed luma and chroma, or the reconstructed RGB samples.

colour_map

_id, colour_map_cancel_flag, colour_map_repetition_period semantic is same as tone_map_id, tone_map_cancel_flag and tone_map_repetition_period defined in Tone mapping information SEI message (specified in Annex D).

colour_map_video_signal_type_present_flag, colour_map_video_format, colour_map_video_full_range_flag, colour_map_description_present_flag, colour_map_primaries, colour_map_transfer_characteristics, colour_map_matrix_coeffs semantic is the same as the semantic of the syntax elements video_signal_type_present_flag, video_format, video_full_range_flag, colour_description_present_flag, colour_primaries, transfer_characteristics, matrix_coeffs in VUI (specified in Annex E) respectively. These syntax elements describe the remapped output decoded pictures video signal characteristics.

bit_depth_lut_minus8 + 8 specifies the bit depth of the 3D LUT samples.

nbp_code indicates the 3D LUT size nbp as listed in Table 2 for the given value of nbp_code (nbp=2(nbp_code-1)+1 ).

The ouput of the 3D LUT decoding is a 3 dimension array lut of size nbp x nbp x nbp. Each lut array element is here below called a vertex and is associated with 3 reconstructed sample values (recSamplesY= lutY[i][j][k], recSamplesU=lutU[i][j][k], recSamplesV=lutV[i][j][k]) of bit depth equal to (lut_bit_depth_minus8+8). A vertex (i,j,k) is said to belonging to layer layer_id if the values of i%(nbp>>layer_id), j%(nbp>>layer_id), k%(nbp>>layer_id) are all equal to zero. One vertex may belong to several layers. An octant of layer layer_id is composed of 8 neighboring vertices belonging to layer_id (Figure 5).

The decoding of the octant( layer_id, y,u,v) is a recursive function:

split_octant_flag specifies whether an octant is split into octants with half horizontal and vertical size. The values (y,u,v) specify the location of the first vertex (Figure 5) in the 3D LUT.

Each octant is composed of 8 vertices (i=0,…7) associated with a flag (encoded_vertex_flag[i]) indicating whether the residual components values (resY[i],resU[i], resV[i]) are encoded or all inferred to be zero. The component values are reconstructed by adding the residuals to the prediction of the components values. The prediction of the components values is computed using tri-linear interpolation of the 8 neighboring vertices of layer_id-1 (Figure 6), except the first layer (layer_id=0) using prediction values shown in Table 4. Once reconstructed a vertex is marked as reconstructed.

Table 4: prediction values used for the 3 components of the 8 vertices belonging to the first layer (layer_id=0)(max=(1<< (bit_depth_lut_minus8+8)-1)).

| i | predSamplesY[i] | predSamplesU[i] | predSamplesV[i] |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | max | 0 | max |
| 2 | 0 | max | 0 |
| 3 | 0 | max | max |
| 4 | max | 0 | 0 |
| 5 | max | 0 | max |
| 6 | max | max | 0 |
| 7 | max | max | max |

If (y+dy[i]), (u+du[i]) and (v+dv[i]) are the 8 children octants coordinates (coordinates of the first 3D colour vertex) of the current octant (having (y,u,v) as first vertex (i=0) coordinates), the values dy[i],du[i] and dv[i] for a given layer are depicted in Table 5:

Table 5: values dy[i],du[i] and dv[i] in function of index I, for vertices belonging to layer = layer_id.

| i | dylayer_id[i] | dulayer_id[i] | dvlayer_id[i] |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | (nbp-1) >> layer_id |
| 2 | 0 | (nbp-1) >> layer_id | 0 |
| 3 | 0 | (nbp-1) >> layer_id | (nbp-1) >> layer_id |
| 4 | (nbp-1) >> layer_id | 0 | 0 |
| 5 | (nbp-1) >> layer_id | 0 | (nbp-1) >> layer_id |
| 6 | (nbp-1) >> layer_id | (nbp-1) >> layer_id | 0 |
| 7 | (nbp-1) >> layer_id | (nbp-1) >> layer_id | (nbp-1) >> layer_id |

The reconstructed 3D colour LUT samples (recSamplesY[i], recSamplesU[i], recSamplesV[i]) for the vertice ( recyi=(y+dy layer_id[i]), recui=(u+du layer_id[i]), recvi=(v+dv layer_id[i]) ) belonging to an octant of the layer=layer_id is given by:

recSamplesY[i] = resY[i] + predSamplesY[i]						(1)

where the values of (predSampleY[i], predSampleU[i], predSampleV[i]) are derived using tri-linear interpolation interpol(recY, recU, recV, LUTY, LUTU, LUTV) with the 8 vertices (yr+dy layer_id-1[k], ur+du layer_id-1[k], vr+dv layer_id-1[k] ) of the octant of layer = layer_id-1 that contains the current octant (eq. (2),(3),(4)):

predSamplesY[i] = ( Ai,0 + Ai,1 + Ai,2 + Ai,3 + Ai,4 + Ai,5 + Ai,6 + Ai,7 + (1<<(shift3-1)) )  >>  shift3    (2)

predSamplesU[i] = ( Bi,0 + Bi,1 + Bi,2 + Bi,3 + Bi,4 + Bi,5 + Bi,6 + Bi,7   + (1<<(shift3-1)) )  >>  shift3    (3)

predSamplesV[i] = ( Ci,0 + Ci,1 + Ci,2 + Ci,3 + Ci,4 + Ci,5 + Ci,6 + Ci,7   + (1<<(shift3-1)) )  >>  shift3    (4)

where the values of (Ai,k , Bi,k , Ci,k )i=0,7 k=0,7 and shift3 are derived as follows:

shift3 = 3 * (nbp_code  – layer_id)

Ai,k = wi,k * LUTY[ yr+dylayer_id-1[k] ][ ur+dulayer_id-1[k] ][ vr+dvlayer_id-1[k] ]

Bi,k = wi,k * LUTU[ yr+dylayer_id-1[k] ][ ur+dulayer_id-1[k] ][ vr+dvlayer_id-1[k] ]

Ci,k = wi,k * LUTV[ yr+dylayer_id-1[k] ][ ur+dulayer_id-1[k] ][ vr+dvlayer_id-1[k] ]

yr = y % ( (nbp-1) >> (layer_id-1) )

ur = u % ( (nbp-1) >> (layer_id-1) )

vr = v % ( (nbp-1) >> (layer_id-1) )

wi,k  = sy(i,k) * su(i,k) * sv(i,k)

sy(i,k) = ((nbp-1) >> (layer_id-1)) - ABS( (yr+dylayer_id-1[k]) – (y+dylayer_id[i]) )

su(i,k) = ((nbp-1) >> (layer_id-1)) - ABS( (ur+dulayer_id-1[k]) – (u+dulayer_id[i]) )

sv(i,k) = ((nbp-1) >> (layer_id-1)) - ABS( (vr+dvlayer_id-1[k]) – (v+dvlayer_id[i]) )

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- T.Suzuki and G.Sullivan, “Text of ISO/IEC 14496-10:2012/DAM 4 Additional Colour Space and Tone Mapping Descriptors.

- ITU-R Recommendation BT.2020  “Parameter values for UHDTV systems for production and international programme exchange” April 2012.

GV LUTher 3D LUT, lustre (.lut) ; Autodesk Lustre 3D LUT file ; quantel 3d (.txt) Quantel 3D LUT file ; scratch (.3dl) ASSIMILATE Scratch 3D LUT file ; davinci (.davlut) 3D LUT file ; http://cinespace.thx.com/bin/cineSpace_LUT_format.zip; http://www.cinematography.net/edited-pages/3D_LUT_Standardisation.htm