| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0181 |
| --- | --- |

| Title: | Non-RCE2: Rice Parameter Initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Joel Sole, Rajan Joshi 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents an initialization process for the Rice parameter of the syntax element coeff_abs_level_remaining to reportedly improve coding efficiency for lossless coding and lossy screen content coding. The proposed modification allows the Rice parameter at the beginning of each 4×4 sub-block to be set to a value different from zero. The method is also combined with the update process of JCTVC-N0101. Results are obtained for RCE2 common test conditions. For lossy configuration, the average coding gains are up to 8.6% for screen content RGB444, 3.0% for YUV444 and 1.6% for class F. For lossless, coding gains are

|  | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -2.1% | -0.1% | -12.6% | -8.6% | -3.6% |
| RA | -1.1% | 0.0% | -10.5% | -7.8% | -2.8% |
| LB | -0.6% | 0.0% | -9.9% | -8.2% | -2.8% |

# Introduction

In HEVC, the binarization of the syntax element coeff_abs_level_remaining depends on the Rice parameter. The parameter (cRiceParam) is set to 0 at the beginning of each 4×4 sub-block of transform coefficients. Then, for each coded coeff_abs_level_remaining, the parameter is conditionally updated depending on its previous value and the absolute coefficient level:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )

The proposed method initializes the Rice parameter for each 4×4 sub-block to a value that depends on the previously coded coefficients, the component index and the 4×4 sub-block position within the transform block.

# Technical Description

Sub-blocks are divided in 4 different types depending on whether the block is luma or chroma and whether the 4×4 sub-block is the top-left one:

sbType  = 2*isLuma + (iSubSet > 0);

Stats are maintained for each type of sub-block depending on the coefficient value (uiLevel):

statCoeff[ sbType ] += (uiLevel == statCoeff[ sbType ]) ? 0 : ( ( uiLevel < statCoeff[ sbType ] ) ? -1 : 1)

This variable is updated at most once per 4×4 sub-block using the value of the first coded coeff_abs_level_remaining of the sub-block. The entries of statCoeff are 8-bit and they are reset to 0 at the beginning of the slice (like the CABAC contexts).

The value of statCoeff is used to initialize the Rice parameter at the beginning of each 4×4 sub-block as

cRiceParam = g_golombTab[ min( statCoeff[ sbType ], 31 ) ];

Where g_golombTab is a table with 32 entries:

g_golombTab[32] = {0, 0, 0, 1, 1, 1, 2, 2, 2, 2, 3, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4, 4, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5}

Maximum Rice equal to 4 and 5 is tested.

## Variants

This initialization is also combined with the update process in JCTVC-N0101 method 3 [1]:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 1 + ( 1  <<  cLastRiceParam ) ) ), 5 )	(9-1)

For the lossy case, two variants are tested. In the first variant, the method is applied to transform as well as transform skip-blocks. In the second variant, the proposed initialization is applied only to transform-skip blocks.

# Results

The proposed methods are implemented in HM10.1_Rext3.0 and simulated under the RCE2 common test conditions Error: Reference source not found. The performance is compared to the anchor in terms of BD-rate.

## Results for the lossless case

Table 1. Lossless results for the proposed initialization with the HEVC update

| Max Rice 4 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -1.9% | -0.1% | -10.4% | -7.7% | -3.1% |
| RA | -1.0% | 0.0% | -8.9% | -7.1% | -2.4% |
| LB | -0.6% | 0.0% | -8.1% | -7.3% | -2.4% |

| Max Rice 5 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -2.1% | -0.1% | -12.5% | -8.5% | -3.4% |
| RA | -1.1% | 0.0% | -10.6% | -7.6% | -2.5% |
| LB | -0.7% | 0.0% | -9.7% | -8.1% | -2.5% |

Table 2. Lossless results for the proposed initialization with the JCTVC-N0101 update

| Max Rice 4 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -1.8% | -0.2% | -10.2% | -7.6% | -3.4% |
| RA | -1.0% | 0.0% | -8.9% | -6.9% | -2.7% |
| LB | -0.6% | 0.0% | -8.0% | -7.3% | -2.7% |

| Max Rice 5 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -2.1% | -0.1% | -12.6% | -8.6% | -3.6% |
| RA | -1.1% | 0.0% | -10.5% | -7.8% | -2.8% |
| LB | -0.6% | 0.0% | -9.9% | -8.2% | -2.8% |

## Results for the lossy case (proposed initialization applied to all blocks)

Table 3. Lossy results for the proposed initialization with the HEVC update and max Rice 5

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.6% | -0.8% | -0.7% | -1.0% | -1.1% | -1.1% | -1.4% | -1.5% | -1.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| SC RGB 444 | -3.5% | -3.4% | -3.5% | -5.7% | -5.7% | -5.7% | -8.3% | -8.7% | -8.7% |
| SC YUV 444 | -0.7% | -0.7% | -0.6% | -1.5% | -1.5% | -1.4% | -2.9% | -2.9% | -3.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.6% | -0.6% | -0.7% | -0.8% | -0.8% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -3.0% | -3.0% | -3.0% | -5.1% | -4.9% | -5.0% |  |  |  |
| SC YUV 444 | -0.4% | -0.4% | -0.3% | -1.2% | -1.2% | -1.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 98% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.2% | 0.3% | -0.5% | -0.4% | -0.4% |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -2.1% | -2.0% | -2.1% | -4.7% | -4.7% | -4.8% |  |  |  |
| SC YUV 444 | -0.3% | -0.3% | -0.3% | -1.2% | -1.2% | -1.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 4. Lossy results for the proposed initialization with the JCTVC-N0101 update and max Rice 5

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.7% | -0.9% | -0.8% | -1.1% | -1.2% | -1.2% | -1.6% | -1.6% | -1.6% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -3.6% | -3.5% | -3.6% | -5.9% | -5.8% | -5.8% | -8.4% | -8.8% | -8.8% |
| SC YUV 444 | -0.7% | -0.8% | -0.7% | -1.6% | -1.6% | -1.5% | -3.0% | -3.1% | -3.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -0.5% | -0.6% | -0.8% | -0.8% | -0.8% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC RGB 444 | -3.1% | -3.1% | -3.1% | -5.0% | -4.9% | -5.0% |  |  |  |
| SC YUV 444 | -0.7% | -0.7% | -0.6% | -1.2% | -1.2% | -1.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 98% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 94% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | 0.0% | -0.6% | -0.6% | -0.4% | -0.8% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -2.8% | -2.8% | -2.8% | -5.1% | -5.0% | -5.2% |  |  |  |
| SC YUV 444 | -0.5% | -0.4% | -0.4% | -1.2% | -1.2% | -1.2% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 98% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |  |  |  |

## Results for the lossy case (proposed initialization applied only to transform-skip blocks)

Table 5. Lossy results for the proposed initialization with the HEVC update and max Rice 5

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.7% | -0.9% | -0.9% | -1.2% | -1.3% | -1.3% | -1.7% | -1.8% | -1.8% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -3.6% | -3.5% | -3.6% | -5.8% | -5.8% | -5.8% | -8.3% | -8.8% | -8.7% |
| SC YUV 444 | -0.7% | -0.7% | -0.7% | -1.6% | -1.5% | -1.5% | -3.0% | -3.0% | -3.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  | 94% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -0.6% | -0.7% | -0.8% | -0.9% | -1.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC RGB 444 | -3.0% | -3.0% | -3.0% | -4.9% | -4.9% | -4.9% |  |  |  |
| SC YUV 444 | -0.4% | -0.4% | -0.3% | -1.0% | -1.1% | -1.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.5% | -0.4% | -0.5% | -0.6% | -0.6% |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -2.8% | -2.7% | -2.8% | -4.8% | -4.7% | -4.9% |  |  |  |
| SC YUV 444 | 0.0% | 0.2% | 0.0% | -0.7% | -0.6% | -0.8% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 97% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 6. Lossy results for the proposed initialization with the JCTVC-N0101 update and max Rice 5

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.8% | -1.0% | -1.0% | -1.3% | -1.4% | -1.4% | -1.8% | -1.9% | -1.9% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -3.7% | -3.7% | -3.7% | -6.0% | -6.0% | -5.9% | -8.6% | -9.0% | -8.9% |
| SC YUV 444 | -0.7% | -0.8% | -0.7% | -1.7% | -1.6% | -1.6% | -3.1% | -3.1% | -3.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -0.7% | -0.7% | -0.9% | -1.0% | -1.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -3.1% | -3.1% | -3.1% | -5.1% | -5.0% | -5.1% |  |  |  |
| SC YUV 444 | -0.5% | -0.5% | -0.5% | -1.3% | -1.2% | -1.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -0.6% | -0.9% | -0.7% | -0.7% | -1.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -2.6% | -2.6% | -2.6% | -5.0% | -4.9% | -5.1% |  |  |  |
| SC YUV 444 | -0.6% | -0.6% | -0.6% | -1.2% | -1.2% | -1.2% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 105% |  |  | 105% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Conclusions

A method for initialization of Rice parameter for each 4×4 sub-block is proposed for HEVC Range Extensions. The method provides very good coding gains for lossless and screen content.

# References

- J. Sole, M. Karczewicz, “RCE2 Test A.3: Simplified update of the coefficient level Rice parameter”, JCTVC-N0101, Vienna, AT, July 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, JCTVC-M1122, Incheon, KR, April 2013.

# Working Draft Text

#### 9.3.3.9 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ], the current sub-block scan index i, a variable cIdx specifying the colour component of the current block, the array statCoeff, and baseLevel.

Output of this process is the binarization of the syntax element.

The variable sbType is derived as follows:

sbType = 2*( cIdx == 0 ? 1 : 0 ) + (i > 0 ? 1 : 0 )	(9-2)

The variable riceStat is derived as follows:

riceStat = min( statCoeff[ sbType], 31 )	(9-3)

Table 9-x specifices the mapping between riceStat and initRiceValue.

The variable statCoeff[sbType] for sbType in the range 0 to 3, inclusive, is initialized as follows:

  - If the coding tree unit is the first coding tree unit in a tile, the initialization process for context variables is invoked as specified in subclause Error: Reference source not found.

  - Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrInRs % PicWidthInCtbsY is equal to 0, the following applies:

  - The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Error: Reference source not found) is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding tree block as follows:

( xNbT, yNbT ) = ( x0 + CtbSizeY, y0 − CtbSizeY )	(9-4)

  - The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ) as inputs, and the output is assigned to availableFlagT.

  - The synchronization process for context variables is invoked as follows:

  - If availableFlagT is equal to 1, the synchronization process for context variables as specified in subclause Error: Reference source not found is invoked with TableStateIdxWpp and TableMpsValWpp as inputs.

  - Otherwise, the initialization process for context variables is invoked as specified in subclause Error: Reference source not found.

  - Otherwise, if CtbAddrInRs is equal to slice_segment_address and dependent_slice_segment_flag is equal to 1, the synchronization process for context variables as specified in subclause Error: Reference source not found is invoked with TableStateIdxDs and TableMpsValDs as inputs.

  - Otherwise, the initialization process for context variables is invoked as specified in subclause Error: Reference source not found.

The variables cLastAbsLevel and cLastRiceParam are derived as follows:

- If this process is invoked for the first time for the current sub-block scan index i, cLastAbsLevel and cLastRiceParam are set equal to 0initRiceValue.

- Otherwise (this process is not invoked for the first time for the current sub-block scan index i), cLastAbsLevel and cLastRiceParam are set equal to the values of cAbsLevel and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element coeff_abs_level_remaining as specified in this subclause.

The variable cAbsLevel is set equal to baseLevel + coeff_abs_level_remaining[ n ].

- If this process is invoked for the first time for the current sub-block scan index i statCoeff[ sbType ] is set to

statCoeff[ sbType] += ( coeff_abs_level_remaining[ n ] == statCoeff[ sbType] ) ? 0 : ( ( coeff_abs_level_remaining[ n ] < statCoeff[ sbType] ) ? -1 : 1 )	(9-5)

statCoeff[ sbType] = Clip3( 0, 255, statCoeff[ sbType ] )	(9-6)

The variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 5 )	(9-7)

The variable cMax is derived from cRiceParam as:

cMax = 4  <<  cRiceParam	(9-8)

The binarization of the syntax element coeff_abs_level_remaining[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of cu_qp_delta_abs, prefixVal, is derived as follows:

prefixVal = Min( cMax, coeff_abs_level_remaining[ n ] )	(9-9)

- The prefix bin string is specified by invoking the TR binarization process as specified in subclause Error: Reference source not found for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 4 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of cu_qp_delta_abs, suffixVal, is derived as follows:

suffixVal = coeff_abs_level_remaining[ n ] − cMax	(9-10)

- The suffix bin string is specified by invoking the EGk binarization process as specified in subclause Error: Reference source not found for suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.

Table 9-x – Specification of initRiceValue

| riceStat | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| initRiceValue | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 4 |
| riceStat | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initRiceValue | 4 | 4 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |

## 9.3 CABAC parsing process for slice segment data

- 9.3.1 General

…

For each requested value of a syntax element a binarization is derived as specified in subclause Error: Reference source not found.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause Error: Reference source not found.

In case the request for a value of a syntax element is processed for the syntax element pcm_flag and the decoded value of pcm_flag is equal to 1, the decoding engine is initialized after the decoding of any pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause Error: Reference source not found.

The storage process for context variables is applied as follows:

  - When ending the parsing of the coding tree unit syntax in subclause Error: Reference source not found, entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrInRs % PicWidthInCtbsY is equal to 1, the storage process for context variables as specified in subclause Error: Reference source not found is invoked with TableStateIdxWpp and TableMpsValWpp as outputs. Similarly, the entries of the Rice parameter initialization counter statCoeff[k], k in the range 0 to 3, inclusive, are stored in TableStatCoeffWpp.

  - When ending the parsing of the general slice segment data syntax in subclause Error: Reference source not found, dependent_slice_segments_enabled_flag is equal to 1 and end_of_slice_segment_flag is equal to 1, the storage process for context variables as specified in subclause Error: Reference source not found is invoked with TableStateIdxDs and TableMpsValDs as outputs. Similarly, the entries Rice parameter initialization counter statCoeff[k], k in the range 0 to 3, inclusive, are stored in TableStatCoeffDs.

…

### 9.3.2 Initialization process

#### 9.3.2.1	General

Outputs of this process are initialized CABAC internal variables and Rice parameter initialization counter statCoeff.

The context variables of the arithmetic decoding engine are initialized as follows:

  - If the coding tree unit is the first coding tree unit in a tile, the initialization process for context variables is invoked as specified in subclause Error: Reference source not found and statCoeff[k] is set to 0 for k in the range 0 to 3, inclusive.

  - Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrInRs % PicWidthInCtbsY is equal to 0, the following applies:

  - The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Error: Reference source not found) is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding tree block as follows:

( xNbT, yNbT ) = ( x0 + CtbSizeY, y0 − CtbSizeY )	(9-11)

  - The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ) as inputs, and the output is assigned to availableFlagT.

  - The synchronization process for context variables is invoked as follows:

  - If availableFlagT is equal to 1, the synchronization process for context variables as specified in subclause Error: Reference source not found is invoked with TableStateIdxWpp and TableMpsValWpp as inputs. The entries in the Rice parameter initialization counter statCoeff[k], k in the range 0 to 3, inclusive, are set to those from the table TableStatCoeffWpp.

  - Otherwise, the initialization process for context variables is invoked as specified in subclause Error: Reference source not found and statCoeff[k] is set to 0 for k in the range 0 to 3, inclusive.

  - Otherwise, if CtbAddrInRs is equal to slice_segment_address and dependent_slice_segment_flag is equal to 1, the synchronization process for context variables as specified in subclause Error: Reference source not found is invoked with TableStateIdxDs and TableMpsValDs as inputs. The entries in the Rice parameter initialization counter statCoeff[k], k in the range 0 to 3, inclusive, are set to those from the table TableStatCoeffDs.

  - Otherwise, the initialization process for context variables is invoked as specified in subclause Error: Reference source not found and statCoeff[k] is set to 0 for k in the range 0 to 3, inclusive.

The initialization process for the arithmetic decoding engine is invoked as specified in subclause Error: Reference source not found.

…

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).