| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0182 |
| --- | --- |

| Title: | SCE1: Results of test 1.1 and 1.2 on sampling offset signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye, Yuwen He X. Li, J. Chen, L. Guo, M. Karczewicz K. Ugur and J. Lainema | Email: | jie.dong@interdigital.com lxiang@qti.qualcomm.com kemal.ugur@nokia.com |
| Source: | InterDigital Communications, Inc. Qualcomm Technologies, Inc. Nokia |  |  |

_____________________________

# Abstract

This contribution presents an upsampling scheme achieving aligned sampling grids between the EL and the upsampled BL pictures, even when the input BL and EL sequences have non-zero phase shift. The proposed upsampling scheme includes signalling the sampling grid offsets between layers in the bitstream and incorporating the signaled offsets into phase filter selection. Simulation results show that, when BL and EL sequences have non-zero phase shift, the proposed scheme outperforms SHM-2.0 significantly. The average {Y, U, V} gains are {-8.5%, -10.2%, -10.2%}, {-6.1%, -4.4%, -4.1%}, {-5.2%, -3.5%, -3.3%}, and {-6.1%, -4.1%, -3.6%} for AI, RA, LDB, and LDP, respectively. At the same time, 3% reduction on average memory access is observed.

# Introduction

In the current SHVC working draft [1], upsampling for inter layer prediction is design based on an assumption that the sampling grids of BL and EL pictures have no phase shift, as shown in Figure 1. To satisfy this assumption, JCT-VC provides the downsampler in [2], which downsamples EL sequences and generates the zero-phase shifted BL sequences. However, the assumption doesn’t always hold, because the downsampler in [2] is not mandated, and an SHVC codec is expected to be able to take input BL and EL sequences with non-zero phase shift. When this is the case, using, the sampling grids between the EL and the upsampled BL pictures are misaligned when the current upsampling process defined in [1] is used, which causes inaccurate inter layer prediction and thus loss of scalable coding performance.

(a)                                                  (b)

Figure 1 Overlaid sampling grids of BL (blue circles) and EL (white squares) pictures. (a) 2x spatial scalability; (b) 1.5x spatial scalability

Signaling phase offset in upsampling process and signaling chroma sampling location were proposed at the last meeting [3]. It achieves aligned sampling grids between the EL and the upsampled BL pictures, even when the input BL and EL sequences have non-zero phase shift, and includes signalling the sampling grid offsets between layers in the bitstream and incorporating the signaled offsets into phase filter selection. The proposal was included in SCE1 “support for additional resampling phase shifts” [4] for a thorough testing and rigorous evaluation.

This document reports the experimental results of SCE1 Test 1.1 and 1.2 under the test conditions specified by SCE1 [4], including the coding efficiency test and complexity assessment under [5].

# Proposed sampling offset signaling and application

## Sampling grid information signaling

Figure 2 gives an example of overlaid luma sampling grids of BL picture (blue circle) and EL picture (white square) with non-zero phase shift in 2x spatial scalability. The precision of the inter layer phase shift is 1/16-pixel in terms of BL sampling grid. The relative positions of the upper-left pixels in BL and EL pictures are examined, and the sampling grid offsets in the horizontal and vertical directions are determined by (1) and (2), respectively,

OffsetHori  =  PositionHori,BL - PositionHori,EL                                               (1)

OffsetVert  =  PositionVert,BL - PositionVert,EL                                                (2)

where PositionHori,BL and PositionHori,EL represent the horizontal positions of the upper-left pixels in the BL and EL pictures, respectively, and PositionVert,BL and PositionVert,EL represent the vertical positions of the upper-left pixels in the BL and EL pictures, respectively. In the example of Figure 2, the sampling grid offsets OffsetHori and OffsetVert are both equal to 4. If the BL and EL sampling grids have zero-phase shift, as shown in Figure 1, the sampling grid offsets have the default values 0.

Figure 2 Overlaid luma sampling grids of BL picture (blue circle) and EL picture (white square) with non-zero phase shift in 2x spatial scalability.

In Table 1, we propose the syntax of the sampling grid information, followed by the semantics.

Table 1 Syntax of sampling grid information

| sampling_grid_information( ) { | Descriptor |
| --- | --- |
| phase_offset_present_flag | u(1) |
| if ( sampling_grid_info_present_flag ) { |  |
| horizontal_phase_offset16 | ue(v) |
| vertical_phase_offset16 | ue(v) |
| chroma_phase_x_flag | u(1) |
| chroma_phase_y | u(2) |
| } |  |

phase_offset_present_flag equal to 1 specifies horizontal_phase_offset16, vertical_phase_offset16, chroma_phase_x_flag, and chroma_phase_y are presented in sampling_grid_information(), sampling_grid_info_present_flag equal to 0 specifies horizontal_phase_offset16, vertical_phase_offset16, chroma_phase_x_flag, and chroma_phase_y are not presented in sampling_grid_information(). When not present, sampling_grid_info_present_flag is inferred to be equal to 0.

horizontal_phase_offset16 specifies the horizontal phase offset of the samples in the current layer with respect to lower layer in 1/16-th pixel units and it is used to calculate the reference layer sample locations used in reseampling. The value of horizontal_phase_offset16 should be in the range 0 to 7 inclusive. When horizontal_phase_offset16 is not present, the value of horizontal_phase_offset16 is inferred to be zero.

vertical_phase_offset16 specifies the vertical phase offset of the samples in the current layer with respect to lower layer in 1/16-th pixel units and it is used to calculate the reference layer sample locations used in reseampling. The value of vertical_phase_offset16 should be in the range 0 to 7 inclusive. When vertical_phase_offset16 is not present, the value of vertical_phase_offset16 is inferred to be zero.

chroma_phase_x _flag specifies the horizontal phase shift of the chroma components in units of half luma samples of a picture or layer picture in the CVS. When chroma_phase_x_flag is not present, it shall be inferred to be equal to 0. The phase shift refers to the spatial displacement between the top left chroma sample and the top left luma sample.

chroma_phase_y specifies the vertical phase shift of the chroma components in units of half luma samples of a picture or layer picture in the CVS. When chroma_phase_y is not present, it shall be inferred to be equal to 0. The value of chroma_phase_y shall be in the range of 0 to 2, inclusive. The phase shift refers to the spatial displacement between the top left chroma sample and the top left luma sample.

sampling_grid_information( ) is proposed to be signaled as part of SPS extension, as shown in Table 2.

Table 2 Signaling sampling grid information in SPS extension

| sps_extension ( ) { | Descriptor |
| --- | --- |
| ...... |  |
| sampling_grid_information() |  |
| } |  |

## Upsampling using signaled sampling grid offsets

The variables phaseXC and phaseYC are derived as follows:

phaseXC = chroma_phase_x_flag 	
phaseYC = chroma_phase_y

The variable xRef16 is derived as follows:

–	If cIdx is equal to 0, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX =  1 << ( iShiftX - 5 );

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) – horizontal_phase_offset16;

–	Otherwise, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX = ( ( (PicWRL* phaseXC) << (iShiftX - 2) ) +(ScaledW >> 1) ) / ScaledW+ (1 << ( iShiftX - 5));

iDeltaX= 4* phaseXC;

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) – iDeltaX –(horizontal_phase_offset16>>1);

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY =  1 << ( iShiftY - 5 );

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16  = ( (yP*iScaleY + iAddY) >> iShiftYM4) – vertical_phase_offset16;

–	Otherwise, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY  = ( ( (PicHRL* phaseYC) << (iShiftY - 2) ) +(ScaledH >> 1) ) / ScaledH+ (1 << ( iShiftY - 5 ));

iDeltaY= 4*phaseYC;

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16 = ((yP*iScaleY + iAddY) >> iShiftYM4) – iDeltaY – (vertical_phase_offset16>>1);

# Experimental Results

## Test 1.1

In Test 1.1, the proposed technique with default parameter values (i.e., horizontal_phase_offset16=0, vertical_phase_offset16=0, chroma_phase_x_flag=0, and chroma_phase_y=1) is applied on CTC sequences. Without transmitting the parameters in SPS, this technique provides bit-exact results of SHM-2.0 CTC anchor.

## Test 1.2

In these tests, coding efficiency of signaling phase offset is analyzed when the phase of base and enhancement layer pictures are different.

As shown in Table 3 and Table 9, the performance gap between 1/16- and 1/4-pixel precisions for 1.5x spatial scalability is large, about 2.0% to 3.3% (for luma) bit-rate change. However, as shown in Table 3 and Table 6, the gap between 1/16- and 1/8-pixel precisions is relatively small, i.e., less than 1.0% (for luma). The reason is explained in the following.

According to SCE1 description [4], Test 1.2 uses misaligned BL and EL sequences. The offset between BL and EL in terms of BL sampling grid can be calculated as in (3),

offset = ( 1.0 – 1/ratio ) / 2                                                           (3)

where ratio is equal to 2.0 and 1.5 for 2x and 1.5x spatial scalabilities, respectively. Test 1.2 is focused on 1.5x case, so the offset between BL and EL is equal to 1/6-pixel in terms of BL sampling grid (i.e., 2.67/16-pixel). With 1/16-pixel precision, where the offset 3/16-pixel is signaled, the offset error is 1/3-pixel; with 1/8- pixel precision, where the offset 2/16-pixel is signaled, the offset error is 2/3-pixel. That means in the particular case of Test 1.2, 1/16-pixel precision is only 1/3-pixel more accurate than 1/8-pixel precision.

### Phase offset with 1/16-pixel precision

Table 3 BD-rate reduction with 1/16-pixel precision compared with SHM-2.0

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -13.8% | -15.9% | -16.5% |  |  |  |
| Class B | -7.6% | -8.8% | -8.7% | -7.7% | -9.5% | -9.5% |
| Overall (Test vs Ref) | -9.4% | -10.9% | -10.9% | -7.7% | -9.5% | -9.5% |
| Overall (Test vs single layer) | 13.2% | 15.1% | 14.7% | 11.6% | 10.8% | 10.4% |
| Overall (Ref vs single layer) | 25.1% | 29.3% | 28.9% | 21.2% | 22.5% | 22.0% |
| EL only (Test vs Ref) | -17.0% | -18.4% | -18.5% | -21.2% | -23.0% | -23.1% |
| Enc Time[%] | 138.7% |  |  | 137.9% |  |  |
| Dec Time[%] | 109.6% |  |  | 111.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -10.0% | -5.5% | -6.8% |  |  |  |
| Class B | -5.2% | -3.5% | -2.8% | -5.6% | -4.7% | -4.3% |
| Overall (Test vs Ref) | -6.6% | -4.1% | -3.9% | -5.6% | -4.7% | -4.3% |
| Overall (Test vs single layer) | 19.6% | 33.6% | 32.2% | 17.1% | 29.7% | 29.6% |
| Overall (Ref vs single layer) | 28.3% | 39.3% | 37.8% | 24.1% | 36.1% | 35.5% |
| EL only (Test vs Ref) | -12.1% | -9.6% | -9.4% | -14.9% | -13.9% | -13.5% |
| Enc Time[%] | 117.8% |  |  | 127.9% |  |  |
| Dec Time[%] | 102.6% |  |  | 103.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -7.5% | -2.7% | -3.8% |  |  |  |
| Class B | -3.8% | -2.7% | -2.0% | -5.6% | -4.4% | -4.1% |
| Overall (Test vs Ref) | -4.8% | -2.7% | -2.5% | -5.6% | -4.4% | -4.1% |
| Overall (Test vs single layer) | 28.9% | 39.3% | 40.3% | 25.5% | 33.6% | 36.2% |
| Overall (Ref vs single layer) | 35.7% | 43.2% | 44.2% | 33.0% | 39.8% | 42.0% |
| EL only (Test vs Ref) | -8.8% | -6.7% | -6.6% | -14.2% | -13.0% | -12.7% |
| Enc Time[%] | 119.4% |  |  | 101.7% |  |  |
| Dec Time[%] | 101.0% |  |  | 90.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -8.8% | -3.1% | -4.6% |  |  |  |
| Class B | -4.6% | -3.3% | -2.4% | -6.3% | -4.9% | -4.2% |
| Overall (Test vs Ref) | -5.8% | -3.2% | -3.1% | -6.3% | -4.9% | -4.2% |
| Overall (Test vs single layer) | 27.1% | 38.2% | 39.6% | 23.5% | 33.1% | 36.2% |
| Overall (Ref vs single layer) | 35.2% | 43.0% | 44.2% | 31.9% | 39.9% | 42.2% |
| EL only (Test vs Ref) | -10.1% | -7.6% | -7.5% | -15.6% | -14.2% | -13.5% |
| Enc Time[%] | 118.5% |  |  | 115.5% |  |  |
| Dec Time[%] | 104.7% |  |  | 99.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 4 Average memory access with 1/16-pixel precision with PU-based implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 101% | 95% | 96% |  |  |  |
| Class B | 103% | 101% | 102% | 100% | 98% | 98% |
| Overall | 102% | 98% | 99% | 100% | 98% | 98% |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 96% | 95% | 95% |  |  |  |
| Class B | 99% | 98% | 98% | 98% | 97% | 97% |
| Overall | 97% | 97% | 97% | 98% | 97% | 97% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 96% | 95% | 95% |  |  |  |
| Class B | 99% | 98% | 98% | 98% | 97% | 97% |
| Overall | 97% | 96% | 96% | 98% | 97% | 97% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 94% | 93% | 93% |  |  |  |
| Class B | 98% | 98% | 98% | 97% | 96% | 96% |
| Overall | 96% | 96% | 95% | 97% | 96% | 96% |

Table 5 Average memory access with 1/16-pixel precision with Picture-based implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 99% | 95% | 96% |  |  |  |
| Class B | 100% | 98% | 99% | 100% | 98% | 99% |
| Overall | 99% | 97% | 97% | 100% | 98% | 99% |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 96% | 95% | 95% |  |  |  |
| Class B | 99% | 98% | 98% | 98% | 97% | 97% |
| Overall | 97% | 97% | 97% | 98% | 97% | 97% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 95% | 94% | 94% |  |  |  |
| Class B | 98% | 98% | 98% | 98% | 97% | 97% |
| Overall | 97% | 96% | 96% | 98% | 97% | 97% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 94% | 94% | 94% |  |  |  |
| Class B | 98% | 98% | 98% | 97% | 97% | 97% |
| Overall | 96% | 96% | 96% | 97% | 97% | 97% |

### Phase offset with 1/8-pixel precision

Table 6 BD-rate reduction with 1/8-pixel precision compared with SHM-2.0

|  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -7.0% | -8.6% | -8.6% |
| Overall (Test vs Ref) | -7.0% | -8.6% | -8.6% |
| Overall (Test vs single layer) | 12.5% | 12.0% | 11.5% |
| Overall (Ref vs single layer) | 21.2% | 22.5% | 22.0% |
| EL only (Test vs Ref) | -19.3% | -20.8% | -20.9% |
| Enc Time[%] | 123.3% |  |  |
| Dec Time[%] | 103.2% |  |  |
| BL Match | Matched |  |  |
|  | RA HEVC 1.5x |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -5.1% | -4.2% | -3.8% |
| Overall (Test vs Ref) | -5.1% | -4.2% | -3.8% |
| Overall (Test vs single layer) | 17.7% | 30.3% | 30.3% |
| Overall (Ref vs single layer) | 24.1% | 36.1% | 35.5% |
| EL only (Test vs Ref) | -13.5% | -12.5% | -12.1% |
| Enc Time[%] | 97.3% |  |  |
| Dec Time[%] | 94.0% |  |  |
| BL Match | Matched |  |  |
|  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -5.0% | -3.9% | -3.5% |
| Overall (Test vs Ref) | -5.0% | -3.9% | -3.5% |
| Overall (Test vs single layer) | 26.3% | 34.2% | 36.9% |
| Overall (Ref vs single layer) | 33.0% | 39.8% | 42.0% |
| EL only (Test vs Ref) | -12.7% | -11.6% | -11.3% |
| Enc Time[%] | 103.6% |  |  |
| Dec Time[%] | 97.6% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -5.7% | -4.4% | -3.9% |
| Overall (Test vs Ref) | -5.7% | -4.4% | -3.9% |
| Overall (Test vs single layer) | 24.3% | 33.7% | 36.7% |
| Overall (Ref vs single layer) | 31.9% | 39.9% | 42.2% |
| EL only (Test vs Ref) | -14.1% | -12.8% | -12.3% |
| Enc Time[%] | 111.8% |  |  |
| Dec Time[%] | 99.6% |  |  |
| Enc Mem[%] | 100.0% |  |  |
| BL Match | Matched |  |  |

Table 7 Average memory access with 1/8-pixel precision with PU-based implementation

|  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 100% | 98% | 98% |
| Overall | 100% | 98% | 98% |
|  | RA HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 98% | 97% | 97% |
| Overall | 98% | 97% | 97% |
|  | LD-P HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 98% | 97% | 97% |
| Overall | 98% | 97% | 97% |
|  | LD-B HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 97% | 97% | 96% |
| Overall | 97% | 97% | 96% |

Table 8 Average memory access with 1/8-pixel precision with Picture-based implementation

|  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 100% | 99% | 99% |
| Overall | 100% | 99% | 99% |
|  | RA HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 98% | 98% | 98% |
| Overall | 98% | 98% | 98% |
|  | LD-P HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 98% | 97% | 97% |
| Overall | 98% | 97% | 97% |
|  | LD-B HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 98% | 97% | 97% |
| Overall | 98% | 97% | 97% |

### Phase offset with 1/4-pixel precision

Table 9 BD-rate reduction with 1/4-pixel precision compared with SHM-2.0

|  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -4.4% | -5.6% | -5.6% |
| Overall (Test vs Ref) | -4.4% | -5.6% | -5.6% |
| Overall (Test vs single layer) | 15.8% | 15.6% | 15.2% |
| Overall (Ref vs single layer) | 21.2% | 22.5% | 22.0% |
| EL only (Test vs Ref) | -12.3% | -13.6% | -13.6% |
| Enc Time[%] | 114.1% |  |  |
| Dec Time[%] | 101.7% |  |  |
| BL Match | Matched |  |  |
|  | RA HEVC 1.5x |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -3.4% | -2.8% | -2.5% |
| Overall (Test vs Ref) | -3.4% | -2.8% | -2.5% |
| Overall (Test vs single layer) | 19.8% | 32.2% | 32.0% |
| Overall (Ref vs single layer) | 24.1% | 36.1% | 35.5% |
| EL only (Test vs Ref) | -9.2% | -8.5% | -8.2% |
| Enc Time[%] | 97.7% |  |  |
| Dec Time[%] | 92.5% |  |  |
| BL Match | Matched |  |  |
|  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -3.6% | -3.0% | -2.7% |
| Overall (Test vs Ref) | -3.6% | -3.0% | -2.7% |
| Overall (Test vs single layer) | 28.2% | 35.6% | 38.2% |
| Overall (Ref vs single layer) | 33.0% | 39.8% | 42.0% |
| EL only (Test vs Ref) | -9.2% | -8.5% | -8.2% |
| Enc Time[%] | 96.0% |  |  |
| Dec Time[%] | 93.3% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -4.0% | -3.4% | -3.0% |
| Overall (Test vs Ref) | -4.0% | -3.4% | -3.0% |
| Overall (Test vs single layer) | 26.5% | 35.2% | 37.9% |
| Overall (Ref vs single layer) | 31.9% | 39.9% | 42.2% |
| EL only (Test vs Ref) | -10.1% | -9.4% | -9.0% |
| Enc Time[%] | 100.8% |  |  |
| Dec Time[%] | 96.8% |  |  |
| BL Match | Matched |  |  |

Table 10 Average memory access with 1/4-pixel precision with PU-based implementation

|  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 100% | 100% | 100% |
| Overall | 100% | 100% | 100% |
|  | RA HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 99% | 98% | 98% |
| Overall | 99% | 98% | 98% |
|  | LD-P HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 99% | 98% | 98% |
| Overall | 99% | 98% | 98% |
|  | LD-B HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 98% | 98% | 98% |
| Overall | 98% | 98% | 98% |

Table 11 Average memory access with 1/4-pixel precision with Picture-based implementation

|  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 100% | 99% | 99% |
| Overall | 100% | 99% | 99% |
|  | RA HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 99% | 98% | 98% |
| Overall | 99% | 98% | 98% |
|  | LD-P HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 99% | 98% | 98% |
| Overall | 99% | 98% | 98% |
|  | LD-B HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 |
| Class A |  |  |  |
| Class B | 98% | 98% | 98% |
| Overall | 98% | 98% | 98% |

# Conclusions

This contribution presents an upsampling scheme achieving aligned sampling grids between the EL and the upsampled BL pictures, even when the input BL and EL sequences have non-zero phase shift. The proposed upsampling scheme includes signalling the sampling grid offsets between layers in the bitstream and incorporating the signaled offsets into phase filter selection. Simulation results show that, when BL and EL sequences have non-zero phase shift, the proposed scheme outperforms SHM-2.0 significantly. The average {Y, U, V} gains are {-8.5%, -10.2%, -10.2%}, {-6.1%, -4.4%, -4.1%}, {-5.2%, -3.5%, -3.3%}, and {-6.1%, -4.1%, -3.6%} for AI, RA, LDB, and LDP, respectively. At the same time, 3% reduction on average memory access is observed. It is further observed that there is no complexity increase in upsampling when higher precision filters are utilized, only complexity increase being the support for additional filters. Considering its high efficiency and complexity reduction, we suggest adopting the proposed technique into WD.

# Reference

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Working Draft 2,” JCT-VC document, JCTVC-M1008, Incheon, South Korea, April 2013.

- J. Dong, Y. He, Y. Ye, "Downsampling filter for anchor generation for scalable extensions of HEVC," m24499, 100th MPEG meeting, Geneva, Switzerland, Apr. 30-May 4, 2012.

- L. Guo, J. Chen, M. Karczewicz (Qualcomm), J. Dong, Y. Ye, Y. He (InterDigital), K. Ugur, J. Lainema (Nokia), “Signaling of Phase Offset in Up-sampling Process and Chroma Sampling Location,” JCT-VC document, JCTVC-M0465, Incheon, South Korea, April 2013.

- E. Alshina, J. Dong, L. Guo, “HEVC Scalable Extensions Core Experiment 1 (SCE1): Support for additional resampling phase shifts,” JCT-VC document, JCTVC-M1101, Incheon, South Korea, April 2013.

- E. Alshina, E. François, “BoG Report on SHVC complexity assessment,” JCTVC-M0455, Incheon, South Korea, April 2013.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).