| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0184_r1 |
| --- | --- |

| Title: | SCE3: Results of test 3.4.1 on chroma enhancement for inter layer prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye, Yuwen He 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858.210.4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to enhance the chroma planes for better inter layer prediction, which is achieved by using the corresponding information from the luma plane. Specifically, for each chroma pixel, an appropriate offset calculated based on the values of surrounding 3×4 luma pixels is added. The average {Y, U, V} gains are {-0.8%, -7.5%, -9.1%}, {-0.4%, -9.2%, -9.5%}, {-0.2%, -6.4%, -7.2%}, and {-0.2%, -7.0%, -7.8%} for AI, RA, LDP, and LDB, respectively. Compared with simulcast, the proposed chroma enhancement brings down the gap between luma gain and chroma gain significantly, which is large by only using SHM-2.0. The average memory access increase is from 0% to 2% for RA/LDP/LDB, and 0% to 4% AI.

# Introduction

Chroma enhancement for interlayer prediction was proposed at the last meeting [1]. It enhances the chroma planes by using the corresponding information from the luma plane. Specifically, for each chroma pixel, an appropriate offset calculated based on the values of surrounding 3×4 luma pixels is added. It is included in SCE3 inter-layer filtering [2] for a thorough testing and rigorous evaluation.

This document reports the experimental results by testing the chroma enhancement technique under the test conditions specified by SCE3 [2], including the coding efficiency test under SHVC CTC [3] and complexity assessment under [4].

# Proposed Chroma Enhancement for Inter-layer prediction

Fig. 1 shows the relative luma and chroma pixel positions in 4:2:0 color format. Each chroma pixel (red dot) in an ILR picture is enhanced by adding an appropriate offset, where the offset is the result of filtering the surrounding 3×4 luma pixels (white squares) by a high-pass filter. By doing this, the chroma edges lost during base layer compression are well restored using the high frequency components from the corresponding luma plane.

## High-Pass Filter Design

On the encoder side, for each enhancement layer slice, two high-pass filters are designed for Cb and Cr planes of the ILR picture, respectively, using Least Minimum Mean Squared Error (LMMSE) estimator. The objective, formulated by (1) and (2) below, is to find the optimal high-pass filters hCb,opt and hCr,opt, such that MSE between the original enhancement layer chroma plane and the enhanced chroma plane in the ILR picture is minimized.

For Cb: hCb,opt=argminhCbEj=−21i=−11hCb(i,j)Y(2x−i,2y−j)+Cb(x,y)−SCb(x,y)2          (1)

For Cr:  hCr,opt=argminhCrEj=−21i=−11hCr(i,j)Y(2x−i,2y−j)+Cr(x,y)−SCr(x,y)2          (2)

In (1) and (2), Y, Cb, and Cr are the three planes in the given ILR picture; SCb and SCr are the original enhancement layer Cb and Cr planes to be coded.

Obviously, hCb,opt and hCr,opt have real-valued coefficients, and therefore need quantizing before transmission.

Fig. 1 Relative Luma and Chroma pixel positions in 4:2:0 color subsampling format.

## Quantizing and Signaling the High-Pass Filter Coefficients

Without loss of generality, here we only explain how to quantize and signal hCb,opt. Each coefficient hCb,opt(i,j) is quantized by a 16-level uniform quantizer, of which the quantization step size, controlled by two parameters QCb and NCb, is equal to QCb/2NCb. Then the integer high-pass filter, denoted as fCb, is obtained, of which the dynamic range is from -8 to 7 (4-bit representation). So the original real-valued high-pass filter hCb,opt is approximated in the following way.

hCb,opt(i,j)=fCb(i,j)×QCb2NCb                                                       (3)

As an approximation to the optimal high-pass filter,  also has the zero-summation constraint. Therefore, only 11 filter coefficients are explicitly coded, and the remaining one can be implicitly derived.

All the filter coefficients and parameters are transmitted in the slice header, including a binary flag indicating whether this tool is used for each chroma plane. Altogether, for each of the two chroma planes, 61 bits are needed to signal the side information when chroma enhancement is enabled:

- 1-bit flag

- 11 filter coefficients, each of which with 4-bit precision

- QCb represented by 11 bits (10 bits for magnitude and 1 bit for sign)

- NCb represented by 5 bits

For higher level syntax, we added one binary flag to SPS to indicate whether the proposed tool is used in the current coded video sequence.

## Process of Chroma Enhancement

Like Section 2.1, only the enhancement for Cb plane of the ILR picture is introduced here. To enhance a Cb pixel with position (x,y) (take the red circle in Fig. 1 as an example), high-pass filter fCb is first applied to the surrounding 3×4 luma pixels to generate the intermediate result z(x,y), as shown in (4).

z(x,y)=j=−21i=−11fCb(i,j)Y(2x−i,2y−j)                                            (4)

Then, z(x,y) is scaled by QCb/2NCb to the normal range, denoted as o(x,y). The integer realization of this scaling step is shown as below.

o(x,y)=SignQCbz(x,y)AbsQCbz(x,y)+2NCb−1>>NCb                                (5)

The enhanced Cb pixel, denoted as Cbenh(x,y), is obtained by Cbenh(x,y)=Cb(x,y)+o(x,y).

## Picture-Level On/Off Decision

As introduced in Section 2.2, a binary flag is used to indicate whether chroma enhancement for each chroma plane is applied. This means an encoder has the freedom to enable or disable the tool for either chroma plane in a certain picture. In this proposal, the encoder decision is made by predicting chroma enhancement’s performance on ILR blocks. In the following, we explain the method on Cb plane and it can be applied to Cr plane in the same way.

The Cb planes before and after chroma enhancement, denoted as Cb and Cbenh, are compared with the original EL Cb plane, SCb. The measure for such comparison is called weighted MSE (WMSE), as shown in (6) and (7).

before chroma enhancement:	WMSEbefore=meanx,y∈CbPlaneILPMap(x2,y2)(Cb(x,y)−SCb(x,y))2                 (6)

after chroma enhancement:	WMSEafter=meanx,y∈CbPlaneILPMap(x2,y2)(Cbenh(x,y)−SCb(x,y))2                 (7)

In (6) and (7), ILP map is a 2-D array, in which each entry represents the likelihood that a 4×4 block in the luma plane (or a 2×2 block in the chroma plane) is used in inter layer prediction in enhancement layer coding. The possible values of each entry range from 0 to 128, where 128 represents the maximum likelihood and 0 represents the minimum likelihood. The squared error (Cb(x,y)−SCb(x,y))2 or (Cbenh(x,y)−SCb(x,y))2 for each pixel is weighted by the corresponding factor in the ILP map, before accumulated into WMSEbefore or WMSEafter. This means the distortion on the pixels more likely to be used for ILP has higher weight in WMSE. Then, WMSEbefore and  are compared. If  is smaller than , which means the distortion is reduced especially for the potential ILP blocks, chroma enhancement is enabled for this Cb plane; otherwise, chroma enhancement is disabled. The ILP map is obtained by the following steps.

- ILPMap is initialized by setting all the entries to 128.

- Make the On/Off decision for the current EL picture to be coded.

- Encode the current EL picture based on the decision.

- Record the locations of all the ILP blocks (in the unit of 4×4) used to code the current EL picture into another ILP map, denoted as CurrILPMap. CurrILPMap has the same dimensions as ILPMap. In CurrILPMap, the entries representing blocks used in ILP are set to 128, and the entries representing blocks not used in ILP are set to 0.

- Update ILPMap by (8) as below.

ILPMap(x,y)=( ILPMap(x,y) + CurrILPMap(x,y) )>>1                                                 (8)

- The updated ILPMap from step 5 is used to code the next EL picture.

Fig. 2(b) shows an exemplary ILP map, which predicts the likelihood that each 4×4 block is used for ILP and is used in calculating WMSE in (6) and (7). It is very close to the ground truth (see Fig. 2(c)), which is the actual ILP map (CurrILPMap) recorded after that EL picture has been coded. That is because the moving objects in pictures are more likely to be coded by the ILP picture, and the ILP maps of successive pictures are highly correlated, except for a small displacement due to the different time instances of the pictures.

The aforementioned method is applied to the pictures in the higher layers of the hierarchical coding structure. For the pictures with higher coding quality, i.e., with POC number multiples of 4, the conventional MSE without weighting is used as the measure for comparison.

(a)

(b)

(c)

Fig. 2 Examples of (a) original picture, (b) predicted ILP map, (c) current accurate ILP map

# Complexity Assessment

## Multiplications and Additions

To enhance a chroma pixel, the following operations are needed.

- 12 multiplications and 11 additions used for 3×4 non-separable filtering as shown in (4).

- 1 multiplication to multiply the scaling factor QCb or QCr

- 1 addition to add the offset o(x,y) to the chroma pixel to be enhanced.

In total, 13 multiplications and 12 additions are required to enhance one chroma pixel. Given a block with the size Width×Height, as shown in Fig. 3(a), the total multiplications and additions required for two chroma components are 2×(Width/2)×(Height/2)×13 and 2×(Width/2)×(Height/2)×12, respectively.

To enhance the chroma components of a Width×Height ILP block, a (Width+1)×(Height+2) luma block should be obtained (see Fig. 3(b)), because the supporting luma pixels may be outside the ILP block boundary according to Fig. 1.  In spatial scalability, a (Width+1)×(Height+2) luma block needs to be generated by upsampling, when the ILP block size is Width×Height.

Fig. 3 Illustration of (a) an ILP block used in SHM and (b) corresponding block used to support chroma enhancement filtering.

## Memory Access

To enhance the chroma components of a Width×Height ILP block, a (Width+1)×(Height+2) luma block should be obtained (see Fig. 3(b)). In complexity analysis [4], the memory access of generating a (Width+1)×(Height+2) luma block is calculated, in comparison of generating a Width×Height luma block in anchor.

The memory access for chroma components is exactly the same as that for SHM-2.0.

# Experimental Results

The proposed chroma enhancement algorithm is integrated into the modified SHM-2.0 generated by applying the complexity assessment patch file [4] on the original SHM-2.0. The R-D performance is tested under the common SHVC test conditions [3]. The complexity assessment modules in the source code and the worst case calculation in the Excel template are modified specially for the proposed algorithm, according to Sections 3.1 and 3.2.

## Coding Efficiency

TABLE I Average BD-rate reduction compared with SHM-2.0

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.9% | -7.8% | -6.2% |  |  |  |  |  |  |
| Class B | -0.8% | -6.4% | -8.4% | -0.7% | -8.2% | -10.4% |  |  |  |
| Overall (Test vs Ref) | -0.9% | -6.8% | -7.8% | -0.7% | -8.2% | -10.4% |  |  |  |
| Overall (Test vs single layer) | 11.8% | 7.3% | 6.1% | 9.7% | 1.1% | -1.1% |  |  |  |
| Overall (Ref vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| EL only (Test vs Ref) | -1.6% | -8.0% | -8.9% | -2.2% | -11.2% | -13.6% |  |  |  |
| Enc Time[%] | 110.9% |  |  | 103.0% |  |  |  |  |  |
| Dec Time[%] | 108.8% |  |  | 103.8% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.5% | -11.3% | -7.7% |  |  |  | -0.4% | -9.0% | -5.2% |
| Class B | -0.4% | -8.1% | -9.7% | -0.3% | -10.4% | -11.9% | -0.3% | -7.6% | -8.2% |
| Overall (Test vs Ref) | -0.4% | -9.1% | -9.1% | -0.3% | -10.4% | -11.9% | -0.4% | -8.0% | -7.4% |
| Overall (Test vs single layer) | 18.7% | 20.9% | 20.4% | 15.8% | 15.7% | 14.5% | 14.0% | 21.4% | 24.6% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.7% | -9.7% | -9.7% | -0.8% | -11.9% | -13.4% | -0.6% | -8.9% | -8.1% |
| Enc Time[%] | 111.4% |  |  | 116.6% |  |  | 120.2% |  |  |
| Dec Time[%] | 114.8% |  |  | 120.7% |  |  | 124.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | -9.9% | -5.9% |  |  |  | -0.3% | -8.0% | -5.0% |
| Class B | -0.2% | -5.1% | -6.6% | -0.2% | -8.1% | -10.8% | -0.2% | -5.6% | -6.8% |
| Overall (Test vs Ref) | -0.2% | -6.5% | -6.4% | -0.2% | -8.1% | -10.8% | -0.3% | -6.3% | -6.3% |
| Overall (Test vs single layer) | 28.2% | 29.4% | 30.8% | 24.5% | 22.4% | 21.7% | 24.0% | 26.1% | 30.7% |
| Overall (Ref vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| EL only (Test vs Ref) | -0.4% | -6.8% | -6.6% | -0.5% | -8.8% | -11.4% | -0.4% | -6.8% | -6.7% |
| Enc Time[%] | 97.7% |  |  | 100.3% |  |  | 100.5% |  |  |
| Dec Time[%] | 101.9% |  |  | 108.0% |  |  | 113.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | -9.4% | -5.3% |  |  |  | -0.3% | -7.3% | -4.4% |
| Class B | -0.2% | -4.6% | -6.3% | -0.2% | -7.6% | -10.0% | -0.2% | -5.1% | -6.1% |
| Overall (Test vs Ref) | -0.2% | -6.0% | -6.0% | -0.2% | -7.6% | -10.0% | -0.2% | -5.8% | -5.6% |
| Overall (Test vs single layer) | 26.3% | 29.4% | 30.9% | 22.5% | 22.9% | 22.5% | 23.1% | 26.7% | 31.6% |
| Overall (Ref vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | -0.4% | -6.3% | -6.2% | -0.6% | -8.5% | -10.7% | -0.4% | -6.3% | -6.0% |
| Enc Time[%] | 99.0% |  |  | 100.9% |  |  | 101.4% |  |  |
| Dec Time[%] | 104.0% |  |  | 106.8% |  |  | 117.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

## Average Complexity Increase

TABLE II Average increase of memory bandwidth compared with SHM-2.0 picture-based implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |  |  |  |
| Class A | 100% | 100% | 100% |  |  |  |  |  |  |
| Class B | 100% | 100% | 100% | 101% | 103% | 100% |  |  |  |
| Overall | 100% | 100% | 100% | 101% | 103% | 100% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 100% | 100% | 100% |
| Class B | 100% | 100% | 100% | 101% | 102% | 100% | 100% | 100% | 100% |
| Overall | 100% | 100% | 100% | 101% | 102% | 100% | 100% | 100% | 100% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 100% | 100% | 100% |
| Class B | 100% | 100% | 100% | 101% | 102% | 100% | 100% | 100% | 100% |
| Overall | 100% | 100% | 100% | 101% | 102% | 100% | 100% | 100% | 100% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 100% | 100% | 100% |
| Class B | 100% | 100% | 100% | 101% | 102% | 100% | 100% | 100% | 100% |
| Overall | 100% | 100% | 100% | 101% | 102% | 100% | 100% | 100% | 100% |

TABLE III Average increase of memory bandwidth compared with SHM-2.0 PU-based implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |  |  |  |
| Class A | 102% | 100% | 100% |  |  |  |  |  |  |
| Class B | 102% | 100% | 100% | 104% | 101% | 101% |  |  |  |
| Overall | 102% | 100% | 100% | 104% | 101% | 101% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 101% | 102% | 101% |
| Class B | 100% | 100% | 100% | 101% | 100% | 100% | 101% | 101% | 101% |
| Overall | 100% | 100% | 100% | 101% | 100% | 100% | 101% | 101% | 101% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 101% | 102% | 102% |
| Class B | 100% | 100% | 100% | 101% | 100% | 100% | 101% | 101% | 101% |
| Overall | 100% | 100% | 100% | 101% | 100% | 100% | 101% | 102% | 101% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 101% | 102% | 102% |
| Class B | 100% | 100% | 100% | 101% | 100% | 100% | 101% | 101% | 101% |
| Overall | 100% | 100% | 100% | 101% | 100% | 100% | 101% | 101% | 101% |

## Chroma Performance Improvement Compared with Simulcast

The chroma performances without and with the proposed chroma enhancement are compared with simulcast, respectively. It is noticed that without chroma enhancement (i.e., using original SHM-2.0), the chroma gain significantly lags the luma gain (see the left half of TABLE IV) in RA, LDP, and LDB cases. By enabling the proposed chroma enhancement for SHM-2.0, the performance gap between luma and chroma is brought down (see the right half of TABLE IV), thus inter layer prediction provides comparable benefits for luma and chroma coding.

TABLE IV Chroma performance w/o and w/ chroma enhancement compared with simulcast

|  | SHM-2.0 |  |  | SHM-2.0 + Chroma Enhancement |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| AI | -26.9% | -26.5% | -26.8% | -27.4% | -31.6% | -32.8% |
| RA | -21.4% | -11.5% | -11.3% | -21.6% | -19.4% | -19.1% |
| LDB | -14.7% | -8.1% | -6.3% | -14.9% | -14.3% | -13.2% |
| LDP | -15.4% | -8.1% | -6.2% | -15.6% | -13.8% | -12.5% |

# Conclusion

This contribution proposes to enhance the chroma planes for inter layer prediction using the corresponding information from the luma plane. Specifically, for each chroma pixel, an appropriate offset calculated based on the values of surrounding 3×4 luma pixels is added. The average {Y, U, V} gains are {-0.8%, -7.5%, -9.1%}, {-0.4%, -9.2%, -9.5%}, {-0.2%, -6.4%, -7.2%}, and {-0.2%, -7.0%, -7.8%} for AI, RA, LDP, and LDB, respectively. Compared with simulcast, the proposed chroma enhancement brings down the gap between luma gain and chroma gain significantly, which is large by only using SHM-2.0. The average memory access increase is from 0% to 2% for RA/LDP/LDB, and 0% to 4% AI.  Considering its high efficiency and moderate complexity increase, we suggest adopting the proposed technique into WD.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Reference

- J. Dong, Y. Ye, Y. He, “Chroma enhancement for ILR picture,” JCTVC-M0183, Incheon, South Korea, April 2013.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, “Description of HEVC Scalable Extension Core Experiment SCE3: Inter-layer filtering,” JCTVC-M1103, Incheon, South Korea, April 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” JCTVC-M1009, Incheon, South Korea, April 2013.

- E. Alshina, E. François, “BoG Report on SHVC complexity assessment,” JCTVC-M0455, Incheon, South Korea, April 2013.

# Specification Text

## Sequence parameter set extension syntax and semantics

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_extension_vui_parameters( ) |  |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_left_offset[ i ] | se(v) |
| scaled_ref_layer_top_offset[ i ] | se(v) |
| scaled_ref_layer_right_offset[ i ] | se(v) |
| scaled_ref_layer_bottom_offset[ i ] | se(v) |
| } |  |
| sps_inter_layer_chroma_enhancement_enable_flag | u(1) |
| } |  |

sps_inter_layer_chroma_enhancement_enable_flag equal to 1 specifies that chroma enhancement is enabled. sps_inter_layer_chroma_enhancement_enable_flag equal to 0 specifies that chroma enhancement is disabled. When not present, sps_inter_layer_chroma_enhancement_enable_flag is inferred to be equal to 0.

## General slice segment header syntax and semantics

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| ...... |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if ( sps_inter_layer_chroma_enhancement_enable_flag ) { |  |
| cb_enhancement_flag | u(1) |
| if (cb_enhancement_flag) { |  |
| for ( i=0; i<11; i++ ) |  |
| cb_filter_coeff_ plus8 [i] | u(4) |
| cb_scaling_factor_abs_minus1 | u(10) |
| cb_scaling_factor_sign | u(1) |
| cb_bit_shifting | u(5) |
| } |  |
| cr_enhancement_flag | u(1) |
| if ( cr_enhancement_flag ) { |  |
| for ( i=0; i<11; i++ ) |  |
| cr_filter_coeff_ plus8 [i] | u(4) |
| cr_scaling_factor_abs_minus1 | u(10) |
| cr_scaling_factor_sign | u(1) |
| cr_bit_shifting | u(5) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

cb_enhancement_flag equal to 1 specifies that cb_filter_coeff_ plus8 [i], cb_scaling_factor_abs, cb_scaling_factor_sign, and cb_bit_shifting are present in the slice header. cb_enhancement_flag equal to 0 specifies that cb_filter_coeff_ plus8 [i], cb_scaling_factor_abs, cb_scaling_factor_sign, and cb_bit_shifting are not present in the slice header. When not present, cb_enhancement_flag is inferred to be equal to 0.

cb_filter_coeff_ plus8[i] minus 8 specifies the i-th filter coefficient of the high pass filter for the chroma component Cb. The value of cb_filter_coeff_ plus8 [i] shall be in the range of 0 to 15, inclusive.

cb_filter_coeff_ plus8 [i] (i=0, 1, …10) is signaled in slice_segment_header( ), and cb_filter_coeff_ plus8 [11] is obtained as follows:

cb_filter_coeff_ plus8 [11] = −i=010cb_filter_coeff_ plus8 [i]

cb_scaling_factor_abs_minus1 plus 1 and cb_scaling_factor_sign together specify the value of variable CbScalingFactor as follows:

CbScalingFactor = (1 – 2 * cb_scaling_factor_sign) * (cb_scaling_factor_abs_minus1+1)

The value of cb_scaling_factor_abs shall be in the range of 1 to 1023, inclusive.

cb_bit_shifting specifies the number of bits to be right shifted after the scaling process to enhance the chroma component Cb. The value of cb_bit_shifting shall be in the range of 0 to 31, inclusive.

cr_enhancement_flag equal to 1 specifies that cr_filter_coeff_ plus8 [i], cr_scaling_factor_abs, cr_scaling_factor_sign, and cr_bit_shifting are present in the slice header. cr_enhancement_flag equal to 0 specifies that cr_filter_coeff_ plus8 [i], cr_scaling_factor_abs, cr_scaling_factor_sign, and cr_bit_shifting are not present in the slice header. When not present, cr_enhancement_flag is inferred to be equal to 0.

cr_filter_coeff_ plus8[i] minus 8 specifies the i-th filter coefficient of the high pass filter for the chroma component Cr. The value of cr_filter_coeff_ plus8 [i] shall be in the range of 0 to 15, inclusive.

cr_filter_coeff_ plus8 [i] (i=0, 1, …10) is signaled in slice_segment_header( ), and cr_filter_coeff_ plus8 [11] is obtained as follows:

cr_filter_coeff_ plus8 [11] = −i=010cr_filter_coeff_ plus8 [i]

cr_scaling_factor_abs_minus1 plus 1 and cr_scaling_factor_sign together specify the value of variable CrScalingFactor as follows:

CrScalingFactor = (1 – 2 * cr_scaling_factor_sign) * (cr_scaling_factor_abs_minus1+1)

The value of cr_scaling_factor_abs shall be in the range of 1 to 1023, inclusive.

cr_bit_shifting specifies the number of bits to be right shifted after the scaling process to enhance the chroma component Cr. The value of cr_bit_shifting shall be in the range of 0 to 31, inclusive.

## Decoding Process

G.8.1.4 Resampling Derivation process for inter layer reference pictures

……

The following steps are applied to derive the resampled inter layer reference picture rsPic.

–	if PicWidthInSamplesL is equal to RefLayerPicWidthInSamplesL and PicHeightInSamplesL is equal to RefLayerPicHeightInSamplesL and the values of ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are all equal to 0

  - rsPicSample is set equal to rlPicSample,

  - If cb_enhancement_flag is equal to 1 or cr_enhancement_flag is equal to 1, the chroma enhancement filtering process as specified in subclause G.8.1.4.3 is invoked with the sample values of rsPicSample as input, and with the modifed chroma sample values of rsPicSample as output.

  - When alt_collocated_indication_flag is equal to 1, rsPicMotion is set equal to rlPicMotion.

–	otherwise, rsPic is derived as follows:

  - The picture sample resampling process as specified in subclause G.8.1.4.1 is invoked with the sample values of rlPicSample as input, and with the resampled sample values of  rsPicSample as output.

  - If cb_enhancement_flag is equal to 1 or cr_enhancement_flag is equal to 1, the chroma enhancement filtering process as specified in subclause G.8.1.4.3 is invoked with the sample values of rsPicSample as input, and with the modifed chroma sample values of rsPicSample as output.

  - When alt_collocated_indication_flag is equal to 1, the picture motion field resampling process as specified in subclause G.8.1.4.2 is invoked with rlPicMotion as input, and with the  resampled motion field of rsPicMotion as output.

……

G.8.1.4.3 Chroma enhancement filtering process for inter layer reference pictures

Input to this process is:

–  a (PicWidthInSamplesL)x(PicHeightInSamplesL) array rsPicSampleL of luma samples

–  a (PicWidthInSamplesC)x(PicHeightInSamplesC) array rsPicSampleCb of chroma samples of the component Cb

–  a (PicWidthInSamplesC)x(PicHeightInSamplesC) array rsPicSampleCr of chroma samples of the component Cr

Outputs of this process are:

–	a modified (PicWidthInSamplesC)x(PicHeightInSamplesC) array rsPicSampleCb of chroma samples of the component Cb,

–	a modified (PicWidthInSamplesC)x(PicHeightInSamplesC ) array rsPicSampleCr of chroma samples of the component Cr.

If cb_enhancement_flag is equal to 1, the modified array rsPicSampleCb of the samples of the chroma component Cb is derived by invoking the enhancing process of chroma sample values specified in subclause G.8.1.4.3.1 with the (PicWidthInSamplesL)x(PicHeightInSamplesL) array rsPicSampleL, (PicWidthInSamplesC)x(PicHeightInSamplesC) array rsPicSampleCb, cb_filter_coeff_ plus8, CbScalingFactor, and cb_bit_shifting given as inputs, and with the modified (PicWidthInSamplesC)x(PicHeightInSamplesC) array rsPicSampleCb as output. If cb_enhancement_flag is equal to 0, the array rsPicSampleCb of the samples of the chroma component Cb is not modified.

If cr_enhancement_flag is equal to 1, the modified array rsPicSampleCr of the samples of the chroma component Cr is derived by invoking the enhancing process of chroma sample values specified in subclause G.8.1.4.3.1 with the (PicWidthInSamplesL)x(PicHeightInSamplesL) array rsPicSampleL, (PicWidthInSamplesC)x(PicHeightInSamplesC ) array rsPicSampleCr, cr_filter_coeff_ plus8, CrScalingFactor, and cr_bit_shifting given as inputs, and with the modified (PicWidthInSamplesC)x(PicHeightInSamplesC) array rsPicSampleCr as output. If cr_enhancement_flag is equal to 0, the array rsPicSampleCr of the samples of the chroma component Cr is not modified.

G.8.1.4.3.1 Enhancing process of chroma sample values

Inputs to this process are:

–	a (PicWidthInSamplesL)x(PicHeightInSamplesL) array rsPicSampleL,

–	a (PicWidthInSamplesC)x(PicHeightInSamplesC) array rsPicSampleC,

–	a variable array filter_coeff_ plus8[i] ( i = 0…11 ),

–	a variable ScalingFactor,

–	a variable bit_shifting.

Outputs of this process is:

–	a modified (PicWidthInSamplesC)x(PicHeightInSamplesC) array rsPicSampleC.

The modified chroma samples rsPicSampleC[ xPC ][ yPC ] with ( xPC = 0.. PicWidthInSamplesC – 1, yPC = 0.. PicHeightInSamplesC – 1) are derived by applying the following ordered steps:

- The variable Temp is derived as follows:

Temp=( filter_coeff_ plus8[0] - 8 ) * (rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC-1), Clip3(0, PicHeightInSamplesL-1, 2*yPC-1)] )
+( filter_coeff_ plus8[1] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC    ), Clip3(0, PicHeightInSamplesL-1, 2*yPC-1)] )
+ ( filter_coeff_ plus8[2] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC+1), Clip3(0, PicHeightInSamplesL-1, 2*yPC-1)] )
+ ( filter_coeff_ plus8[3] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC-1), Clip3(0, PicHeightInSamplesL-1, 2*yPC     )] )
+ ( filter_coeff_ plus8[4] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC    ), Clip3(0, PicHeightInSamplesL-1, 2*yPC     )] )
+ ( filter_coeff_ plus8[5] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC+1), Clip3(0, PicHeightInSamplesL-1, 2*yPC     )] )
+ ( filter_coeff_ plus8[6] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC-1), Clip3(0, PicHeightInSamplesL-1, 2*yPC+1)] )
+ ( filter_coeff_ plus8[7] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2* xPC    ), Clip3(0, PicHeightInSamplesL-1, 2*yPC+1)] )
+ ( filter_coeff_ plus8[8] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC+1), Clip3(0, PicHeightInSamplesL-1, 2*yPC+1)] )
+ ( filter_coeff_ plus8[9] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC-1), Clip3(0, PicHeightInSamplesL-1, 2*yPC+2)] )
+ ( filter_coeff_ plus8[10] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC    ), Clip3(0, PicHeightInSamplesL-1, 2*yPC+2)] )
+ ( filter_coeff_ plus8[11] - 8 ) * ( rsPicSampleL[ Clip3(0, PicWidthInSamplesL-1, 2*xPC+1), Clip3(0, PicHeightInSamplesL-1, 2*yPC+2)] )

- The variable ScaledTemp is derived as follows:

ScaledTemp=Sign(Temp * ScalingFactor) * ((Abs(Temp * ScalingFactor) + (1 << (bit_shifting-1))) >> bit_shifting)

- The modified rsPicSampleC[ xPC ][ yPC ] is derived as follows:

rsPicSampleC[ xPC ][ yPC ] = Clip3( 0, ( 1 << BitDepthC) – 1, rsPicSampleC[ xPC ][ yPC ]+ScaledTemp).