| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 August 2013 | Document: JCTVC-N0188 |
| --- | --- |

| Title: | AHG 5 and 18: Internal Precision for High Bit Depths |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

The document presents the current capabilities of HEVC Range Extensions when operating at low (including negative) QPs, and proposes that such low QPs will be needed if the codec is to truly support high bit depths, as required by the mandate of the Range Extensions amendment. Possible sources of errors that may be caused by internal accuracy limitations currently present in HEVC are explored. It is claimed that some changes to those accuracies can mitigate the errors and thereby extend the operating range of HEVC.

# Introduction

The current HEVC international standard describes an 8 and 10 bit codec; this is to be extended to include 12 and 14 bit operation under the remit of Range Extensions. Although the software and standard have been written to allow input bit depths up to 14, the capability of the codec to actually code 14-bit video has not previously been studied.

Normally, and especially for the 8 and 10 bit codec, QPs in the positive range are studied. However, for each additional bit on the input over 8 bits, the minimum QP can be reduced by 6, i.e.

minQP = -6*(bitDepth-8)

As can be seen later in Figure 4, the PSNR curves peak at around 90dB, although the excessive flattening of the curve is not ideal, and the system does not function as expected when coding 16-bit input.

# PSNR and MSE

Higher PSNRs are required in order to make higher bit depths meaningful.

PSNR is defined as a function of mean-square error (MSE) and bit depth (internal bit depth using HEVC software terminology):

PSNR=10 * log10(((2bitDepth)-1)2 / MSE)

Using this equation, the following table of PSNRs for given bit depths and MSEs can be derived:

Figure 1 - Table of PSNRs for a Range of Bit Depths and MSEs

If (for example) a 14-bit codec is only able to achieve 72.2 dB, then each output value is only accurate to within the range of ±4 of the corresponding original value. The two least significant bits are therefore effectively noise, so the codec is really a 12-bit codec with two additional random bits added at the output. (It is important to note that this is based upon averages, and that actually in some parts of the picture, better quality may be achieved.)

Extending this argument, when comparing PSNRs in this purely numerical fashion, it could be thought that the best system would therefore be an 8-bit system with lossless encoding, achieving an infinite PSNR (MSE=0). However, this does not take into account the loss in rounding or truncating the video from n bits to 8 bits.

If a lossless (n - 1)-bit system were available to encode the n-bit data, then at the output, the n-bit MSE observed would be (0+12) / 2 = 0.5.

If a lossless (n - 2)-bit system were available to encode the n-bit data, then at the output, the n-bit MSE observed would be (0+12+22+12) / 4 = 1.5.

If a lossless (n - 3)-bit system were available to encode the n-bit data, then at the output, the n-bit MSE observed would be (0+12+22+32+42+32+22+12) / 8 = 5.5.

If a lossless (n - 4)-bit system were available to encode the n-bit data, then at the output, the n-bit MSE observed would be (0+12+22+32+42+52+62+72+82+72+62+52+42+32+22+12) / 16 = 21.5.

Therefore, if the 14-bit system does not achieve an MSE of 21.5 (or less), which is equivalent to 71.0dB, and if the bit rate of a lossless 10-bit system were similar, then numerically speaking, only 10 bits are effectively being coded.

Consider a lossy, low bit depth (n - r)-bit system with MSE of m. If this system is used to code higher bit depth n-bit data, its MSE will therefore be given by (2r)2m.

e.g.	for a lossy (n-1)-bit system, MSE in an n-bit system would be 4m,
		for a lossy (n-2)-bit system, MSE in an n-bit system would be 16m,
		for a lossy (n-3)-bit system, MSE in an n-bit system would be 64m,
		for a lossy (n-4)-bit system, MSE in an n-bit system would be 256m.

Therefore for the case where lossy lower bit depth systems encode higher n-bit depth data, their loss is generally the main contributor for the MSE observed in the n-bit domain – simple PSNRs can be used as straight comparisons of quality.

The current Range Extensions software (HM_10.1_RExt3.0 with some bug-fixes) peaks at 90dB at QP=-26 (as shown in Figure 4); this is certainly suitable for encoding 14-bit data, but if the standard were extended beyond 14-bit (e.g. for medical imaging), then the current settings are not suitable. However, at this operating point, more efficiency can be gained, as investigated in this proposal.

# Sources of Error

The core HEVC system has been designed for 8 and 10 bit operation. As the number of bits increases, the accuracies of parts of the system should be reconsidered. A simplified flow of data through the HEVC encoder is shown in Figure 2.

Figure 2 - Diagram showing magnitude bit depths in HEVC encoding path

The main sources of calculation noise (i.e. ignoring noise shaping caused by the various predictions and the RQT and RDOQ decision processes) in HEVC are:

- Transform matrix coefficient values.
Ideally, the inverse transform applied to transformed coefficients will reproduce the original input values. However, this is limited by the integer nature of the calculations. In HEVC, the transform matrix coefficients have 6 fractional bits (i.e. they have an inherent left-shift of 6).

- Shifting results to MAX_TR_DYNAMIC_RANGE after each stage of the transform.
The forward transform will (after compensation in the quantiser) result in values that are bitDepth + log2(size) bits in size. After the first stage of the transform, the coefficients’ width in bits should be at least bitDepth + log2(size) (though additional bits will help maintain more accuracy). However, in HEVC, these intermediates are shifted so that they never exceed MAX_TR_DYNAMIC_RANGE; similarly for the second stage.
In the inverse transform, the values into the first stage are MAX_TR_DYNAMIC_RANGE in size, although the value equivalent to ‘1’ is log2(size) smaller. The inverse transform does not shift by log2(size) after each stage; instead clipping to MAX_TR_DYNAMIC_RANGE is applied.

- Noise added during quantisation.
Although the quantiser and inverse quantiser will add noise when quantising, additional noise may be inadvertently added when the scaling lists are applied, and because the quantisation coefficients defined in the arrays ‘quantScales’ and ‘invQuantScales’ are not perfect reciprocals. These results are not studied here, although patterns with a QP periodicity of 6 can be seen in some data.

- Clipping at the output of the quantiser.
Following quantisation, the coefficients are clipped to limit the range of values that are required to be supported by the entropy coder. If this “ENTROPY_CODING_DYNAMIC_RANGE” (which is actually MAX_TR_DYNAMIC_RANGE in HEVC, but split out in this report for the purpose of investigation) is too small, the quantised coefficients for low QPs can saturate it and become distorted as a result. The same clipping is applied at the input to the dequantiser.

In HEVC, ENTROPY_CODING_DYNAMIC_RANGE of 15 is used for up to 10 bit operation, although coefficients in 32x32 blocks may be clipped for QP < -8.

The effects of transform matrix precision, MAX_TR_DYNAMIC_RANGE and ENTROPY_CODING_DYNAMIC_RANGE are explored in Section 5 on the study of the higher bit-depths.

# Source Data and Test Conditions

Five of the six sequences studied are from the SVT test set (1920x1080 50p at 16bit, scaled down from 4K video). The sixth sequence is Traffic_RGB (2560x1600 30p at 12 bit), as described for the standard Range Extension test conditions. Of these sequences, only the first 10 frames have been used, since numerous configurations have been tested (and are presented in the accompanying Excel spreadsheet). All frames have been studied for some tests and the trends are the same as those for 10 frames. First-frame thumbnails of these sequences can be seen in Figure 3.

Figure 3 - First frames of high bit depth test material

The HM10.1_RExt3.0 software has been modified to allow larger internal accuracies to be used.

Only the intra_main profile has been studied (at different bit depths).

The “FULL_NBIT” setting that enables the encoder to evaluate distortion using the full bit depth of the sample data when making decisions has been set to 1.

As defined in the standard Range Extensions test conditions, if the file bit depth is less than the internal bit depth (the codec’s input bit depth), then the samples are padded (with the LSBs set to 0); if the file bit depth is more than the internal bit depth, the samples are scaled and rounded.

Below, bitDepth is used to describe the internal bit depth; the file’s bit depth is not mentioned.

Systems with internal bit depth (bitDepth) up to 16 are considered. Although 14 is the limit for the Range Extensions amendment, studying the function of the system beyond the limit gives reassurance that the system functions within the limits. In addition, if 16 bits are later required by the standard (e.g. for medical imaging), then the required configuration is also known.

Only twelve consecutive even QPs at each bitDepth are considered, in each case, the minimum QP being -6 (bitDepth-8).

As the effects of transform matrix precision are considered, transform-skip has been disabled for some simulations (for example, see ). It has been seen that, if the transform matrices are unsuitable, the encoder (given the option) elects to use transform-skip for all TUs where possible, hiding the effect of the matrix precision. Similarly, IPCM has not been enabled for the majority of simulations, because this will be selected for many of the high bit simulations and prevent analysis of the response of the transforms.

Due to the high level of noise in the source sequences, it is observed that the encoder selects almost exclusively 4x4 TUs for the base HEVC configuration. To address this bias in the results, an option has been added to the software to apply a [1,2,1] or [1,4,6,4,1] 2-dimensional filter to the input images. Results are presented for DucksTakeOff with the [1,2,1] filter applied and InToTree with the [1,4,6,4,1] filter applied. In these sequences, it is observed that all available TU sizes are used, even with transform-skip enabled.

Where lossless coding has been enabled, both transquant-bypass and IPCM have been made available to the encoder, which selects whether to apply lossless or lossy coding on a per-CU basis. In addition, in these cases, the sequence PSNR has been derived from the average of the MSEs of each frame, rather than the standard linear average of the per-frame PSNRs. This is because losslessly coded PSNRs are technically infinite, and therefore cannot be averaged (although HM uses the value of 99.99dB as an approximation).

PSNRs have been evaluated using both the internal bit depth (bitDepth) and at the source bit depth. The accompanying Excel spreadsheets allow either set of PSNRs to be plotted.

Note that this document gives only an overview of the results, and accompanying Excel spreadsheets allow exploration of many combinations of settings.

## Lambda Parameters

In the HM encoder, costs for decision-making are evaluated using Lagrangian optimisation, where the two parameters to be minimised (bit count and distortion) are combined into one cost by multiplying the bit count by a conversion factor lambda. Since lambda is intended to serve as an exchange rate between bits and distortion, it should be equal to the derivative of the bit rate/distortion curve (which will vary depending on the choice of distortion metric) at the desired operating point. It is deemed too costly however, to evaluate multiple points on this curve for each picture, so lambda is approximated by the following equation for intra slices (with additional multiplicative factors for other slice types):

lambda=0.57×2trueQp−123

where trueQp is the QP at the point of quantisation, including the offset due to bitDepth.

This formula dates back to the AVC JM model and was originally optimised for operating points much lower than those being investigated, so there was a need to determine if a new formula is needed for high operating points. Through experimental investigation, it was determined that although modifying the coefficients of this equation does yield some benefit, the complexity of implementation is too high (each sequence and bitDepth has a different optimum) and the savings are not large enough (less than 1%) to warrant a change from HM.

# Adjusting the Internal Accuracies

## Base HEVC Configuration

Figure 4 illustrates the performance of HEVC at a number of internal bit depths. The graph contains traces for 8-bit (QP 22 to 0), 10-bit (QP 10 to -12), 12-bit (QP -2 to -24), 14-bit (QP -14 to -36) and 16-bit (QP -26 to -48) processing. As can be seen, the system reaches a PSNR maximum at approximately 90dB, and becomes more inefficient as QP becomes more negative (as bit rate continues to increase).

Figure 4 - Bit rate vs. green channel PSNR for OldTownCross at QP range 22 to -48 with 6-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 15 and ENTROPY_CODING_DYNAMIC_RANGE = 15 (RDOQ disabled, transform-skip enabled)

Figure 5 - Bit rate vs. green channel PSNR for OldTownCross at QP range 22 to -48 with 6-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 15 and ENTROPY_CODING_DYNAMIC_RANGE = 15 (RDOQ disabled, transform-skip disabled)

As observed in Figure 4, the PSNR for bitDepth = 8 and bitDepth = 10 increases sharply at QPs of 4 (-8 for 10-bit) and below (rightmost three data points on each trace). At QP 4, the quantisation divisor for 8-bit is 1 (QP -8 for 10-bit), leaving the mismatch between the forward and inverse transforms and between the quantisation and dequantisation coefficients as the only sources of error. As the system tends towards lossless processing, the MSE approaches zero and the PSNR spikes upward.

Although these spikes would appear to indicate that a lower bit depth system is more efficient than a higher bit depth system, the cost of converting the higher bit depth (n+2)-bit video to n-bit video must also be considered as described in Section 2. This conversion will add a MSE of 1.5 to the almost lossless PSNR of the lower-accuracy system, resulting in a lower PSNR. An (n+2)-bit system operating at the same QP will have errors mainly in the two least significant bits, meaning it also approaches lossless processing when considering only n-bit accuracy.

In systems that are not limited by internal accuracy to a maximum PSNR, this “upward spike” pattern can be seen for each bitDepth as QP drops below (4 - (6 * (bitDepth - 8))). This is shown later in Figure 6.

The 16-bit system is affected by the clipping to 15-bit (MAX_TR_DYNAMIC_RANGE) in the last stage of the inverse transform (which affects all QPs, but does not affect transform-skip mode), and the clipping to 15-bit (ENTROPY_CODING_DYNAMIC_RANGE) before and after the entropy coder (which affects the lowest QPs).

## Increasing the Internal Accuracies

Experiments (see accompanying Excel spreadsheets) have determined that:

- The transform matrix precision should be at least bitDepth - 2.

- MAX_TR_DYNAMIC_RANGE and ENTROPY_CODING_DYNAMIC_RANGE should be at least 6 more than the bitDepth (1 for the “quantisation” factor applied by QPs less than (4 - (6 * (bitDepth - 8))) plus 5 for the maximum value of log2(size)). Additional accuracy has been shown to further improve coding efficiency at lower QPs.

- QUANT_SHIFT seems to be suitable at the standard value of 14 for up to the 16-bit system.

For the 16-bit system, the transform matrix precision should be set to 14, emulating the 8-bit base HEVC system (where the precision is 6) and MAX_TR_DYNAMIC_RANGE should be set to 22. Since having more internal accuracy is rarely harmful, these parameters have also been tested at different bitDepths, producing the results illustrated in Figure 6. It can be seen that, for the same number of bits, significantly higher SNRs are achievable, and that the increased-accuracy system has PSNR/MSE operating points that are suitable for bitDepths of up to 16.

Note that with transform-skip enabled with these increased internal accuracies, similar curves are observed, although the ‘spikes’ are slightly less regular.

Figure 6 – Bit rate vs. green channel PSNR for OldTownCross at QP range 22 to -48 with 14-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 22 and ENTROPY_CODING_DYNAMIC_RANGE = 22 (RDOQ disabled, transform-skip disabled)

If HEVC Range Extensions is intended to produce a single new profile for all bit depths, then the system described above is suitable.

However, if different profiles are to be described for different maximum bitDepths, then having different parameter values may be useful for reducing hardware complexity in systems that do not require the highest profiles. This will require multiple sets of transform matrices (Appendix A describes a set of matrices for each transform matrix precision) if the transform matrix precision is to be varied. A set of possible profiles is proposed in Section Error: Reference source not found, based upon the following table relating the bitDepth to the various minimum values:

*These values are limited to 15 as used in HEVC.

†Although this value is recommended from these studies at these low QPs, HEVC uses 15, which may result in clipping of coefficients in 32x32 TUs at QPs less than -8.

‡Although this value is recommended from these studies at these low QPs, HEVC uses 6.

Table 1 - Recommended internal accuracies.

If different profiles are to be used, then it is recommended that these specifications be used as minima (noting that the HEVC 10-bit system does not quite meet these targets). For a 14-bit system, Figure 7 shows that when the transform matrix precision is set to 2 less than the recommended values from the above table, the PSNR is reduced, especially at the lowest QPs for the bit depth of 14. There is some benefit in increasing the parameters further, especially for matrix precision at very low QPs. Although the return for this added complexity is diminished, it does provide a reliable monotonic response suitable for rate control. The system responds similarly at the other bit depths studied.

The benefit to having MAX_TR_DYNAMIC_RANGE set to bitDepth + 6 lies in rate control. This is illustrated in Figure 8, where the bit rate/PSNR curve exhibits non-monotonic behaviour when MAX_TR_DYNAMIC_RANGE is set to less than the ideal value, making it difficult to automatically adjust encoding parameters. The reason for this is that additional accuracy is needed at the input of the quantiser. The graph shows that the “kink” in the curve occurs 6 QPs (i.e. one bit) lower when MAX_TR_DYNAMIC_RANGE is set to 18 compared to MAX_TR_DYNAMIC_RANGE = 19. The curve remains smooth throughout its range when MAX_TR_DYNAMIC_RANGE = 20.

These cases can be explored in the accompanying Excel spreadsheets.

Figure 7 – Bit rate vs. green channel PSNR for InToTree smoothed with a [1 4 6 4 1] filter at 14-bit QP range -14 to -36 with various-precision transform matrices, MAX_TR_DYNAMIC_RANGE = 20 and ENTROPY_CODING_DYNAMIC_RANGE = 22 (RDOQ disabled, transform-skip disabled)

Figure 8 – Bit rate vs. green channel PSNR for InToTree smoothed with a [1 4 6 4 1] filter at 14-bit QP range -14 to -36 with 12-bit transform matrices, various values of MAX_TR_DYNAMIC_RANGE and ENTROPY_CODING_DYNAMIC_RANGE = 22 (RDOQ disabled, transform-skip disabled)

## Lossless Coding

Under normal operating conditions, lossless coding can be used as a fall-back position if the bit rate produced by conventional rate-controlled lossy coding becomes too large. By increasing the efficiency of the HEVC lossy path, the encoder can be given the option to reach a high target PSNR without using lossless coding and hence at a lower bit rate.

The HEVC standard describes two tools for lossless coding: transquant-bypass and IPCM, where the latter is used to limit the maximum bit rate. These methods are explored in the following sections.

### HM Cost Evaluation for Transquant-Bypass

The conventional cost function in HM evaluates cost as:

cost=distortion+bits×lambda

This equation works for lossy coding, but in transquant-bypass mode the distortion is always zero, reducing the equation to:

cost=bits×lambda

Since lambda is derived from the QP, this means that the cost evaluation for lossless coding is affected by QP, even though no quantisation is applied. Alternatively, the cost can be derived using:

cost=distortionlambda+bits

This is equivalent to dividing through the HM equation by lambda. Though the costs will be different, they are only used in comparison with each other so the decisions will remain valid. With zero distortion (i.e. when using lossless coding), the cost becomes equal to the number of bits and is QP independent.

When evaluating mixed lossless and lossy encoding costs, this particular cost metric was used instead.

### Effects of Increased Internal Accuracy where Lossless Coding is Enabled

One method to increase the effective operating range of a less accurate system is to rely upon the non-transform paths through the coding process, such as transform skip, IPCM and transquant-bypass. This situation is highlighted in Figure 9 and Figure 10 for two different 14-bit sequences. For noisier sequences (such as OldTownCross), there seems to be little benefit in increasing the internal accuracies when mixed lossy/lossless coding is available, with the mixed traces of Figure 9 showing similar behaviour no matter whether higher internal accuracies are used or not. However, for easier sequences (such as the filtered versions of DucksTakeOff), such as in Figure 10, the high accuracy system is only slightly affected by the availability of lossless coding tools, indicating that in this case, the increased accuracy is a significant benefit.

In Figure 9 the PSNR of the mixed trace of the base HEVC system (red trace) shows a similar trend to the mixed trace of the high bit depth system (purple trace). However, the gains observed are not because the PSNR of all CUs is improving as QP is reduced, but because an ever increasing number of CUs are switching to lossless coding.

In professional applications, it is generally required that not only should the PSNRs increase as the QP is reduced, but also that the degree of noise in each CU should be similar. For that reason, lossless coding tools are a useful tool when all of the CUs can be losslessly coded, or when a particular CU is practically lossless and losslessly coding a CU would free-up bits that could be used to improve other CUs. However, producing a video image consisting of a mix of lossless CUs and relatively noisy lossy CUs is not so useful.

In addition, Figure 9 and Figure 10 both have transform skip enabled, which can be seen from Figure 4 and  to give a significant benefit to the base-HEVC-accuracy system by removing the insufficiently accurate transform from the encoding path, and thereby reducing the observed losses. However, this tool and the lossless coding tools would prevent the use of scaling lists and also remove the general noise-hiding advantages of using a frequency-based transform in the processing path.

It is therefore important in these applications to ensure the performance of the system extends over the largest operating range as possible to handle different video types.

In JCTVC-N0189, the entropy coder has been modified to increase efficiency for the higher operating points. This further highlights the benefits of increased internal accuracy for sequences that are difficult to compress.

Figure 9 – Bit rate vs. green channel PSNR for OldTownCross at 14-bit QP range -14 to -36 with lossless and lossy coding enabled. Base HEVC configuration (as in Section 5.1) vs. increased internal accuracy configuration (as in Table 1) (RDOQ disabled, transform-skip enabled)

Figure 10 – Bit rate vs. green channel PSNR for DucksTakeOff smoothed with a [1 2 1] filter at 14-bit QP range -14 to -36 with lossless and lossy coding enabled. Base HEVC configuration (as in Section 5.1) vs. increased internal accuracy configuration (as in Table 1) (RDOQ disabled, transform-skip enabled)

## Recommendations

In summary, the recommendations are:

- Use at least one additional transform matrix precision for higher accuracy.

  - The transform matrix precision is recommended to be at least bitDepth - 2.

  - It is preferable to have a single set for all higher accuracies, to simplify multi-profile encoders/decoders.

  - Proposed transform matrices are provided for matrix coefficient precisions 7 to 14 – see Appendix A.

  - It is suggested to use the 14-bit accuracy transform matrices as the coefficients will fit within 16-bit data types for software, and will provide sufficient accuracy to allow future extension to 16-bit video.

  - The choice of transform matrix precision could be configured by the bit depth of the input data and the profile, or alternately determined by parameters specified at the sequence, picture or slice level.

- Change MAX_TR_DYNAMIC_RANGE for higher accuracy.

  - MAX_TR_DYNAMIC_RANGE is recommended to be at least bitDepth + 6.

  - Multiple values of MAX_TR_DYNAMIC_RANGE should not present a problem for multi-profile decoders.

  - As with the transform matrix precision, the choice of MAX_TR_DYNAMIC_RANGE could be configured by the bit depth of the input data and the profile.

    - It is suggested to derive MAX_TR_DYNAMIC_RANGE = bitDepth + 6 (with a minimum of 15), to simplify encoder design.

- Change ENTROPY_CODING_DYNAMIC_RANGE for higher accuracy.

  - ENTROPY_CODING_DYNAMIC_RANGE is recommended to be at least bitDepth + 6.

  - Multiple values of ENTROPY_CODING_DYNAMIC_RANGE should not present a problem for multi-profile decoders.

# Conclusion

The base HEVC Range Extensions system that is limited to about 90dB has been extended to 120dB. The various sources of error have been studied and recommendations have been made. An increase in internal accuracy has been shown to benefit HEVC coding efficiency, including in situations where lossless coding is available.

It is proposed that internal accuracies be increased according to the recommendations made in Section 5.4.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Increased-Precision Transform Matrices

This appendix details the transform matrices at various levels of precision as referred to in Section 5.2.

## 4x4 DST

The transform matrix is of the form:

where the values in the grid are defined by the matrix precision according to the following table (6-bit HEVC standard matrix values included for comparison):

|  | 6-bit | 7-bit | 8-bit | 9-bit | 10-bit | 11-bit | 12-bit | 13-bit | 14-bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a | 29 | 58 | 117 | 233 | 467 | 934 | 1868 | 3736 | 7472 |
| b | 55 | 110 | 219 | 439 | 878 | 1755 | 3510 | 7021 | 14042 |
| c | 74 | 148 | 296 | 591 | 1182 | 2365 | 4730 | 9459 | 18919 |
| d | 84 | 168 | 336 | 672 | 1345 | 2689 | 5378 | 10757 | 21513 |
| e | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

## Combined DCT Matrix

For ease of implementation, a single 32x32 DCT matrix M32 can be described, from which each smaller NxN DCT matrix MN is derived through subsampling according to the following:

MN[x][y] = M32[x][(2(5 – log2(N)))y]      for x, y = 0..(N – 1).

The combined matrix M32 is of the form:

with the values in the grid defined by the matrix precision according to the following table (6-bit HEVC standard matrix values included for comparison):

|  | 6-bit | 7-bit | 8-bit | 9-bit | 10-bit | 11-bit | 12-bit | 13-bit | 14-bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a | 64 | 128 | 256 | 512 | 1024 | 2048 | 4096 | 8192 | 16384 |
| b | 83 | 167 | 334 | 669 | 1338 | 2676 | 5352 | 10703 | 21407 |
| c | 36 | 69 | 139 | 277 | 554 | 1108 | 2217 | 4433 | 8867 |
| d | 89 | 178 | 355 | 710 | 1420 | 2841 | 5681 | 11363 | 22725 |
| e | 75 | 151 | 301 | 602 | 1204 | 2408 | 4816 | 9633 | 19266 |
| f | 50 | 101 | 201 | 402 | 805 | 1609 | 3218 | 6436 | 12873 |
| g | 18 | 35 | 71 | 141 | 283 | 565 | 1130 | 2260 | 4520 |
| h | 90 | 180 | 360 | 721 | 1441 | 2882 | 5765 | 11529 | 23059 |
| i | 87 | 173 | 346 | 693 | 1386 | 2772 | 5543 | 11086 | 22173 |
| j | 80 | 160 | 319 | 639 | 1277 | 2554 | 5109 | 10217 | 20435 |
| k | 70 | 140 | 280 | 560 | 1119 | 2239 | 4478 | 8956 | 17911 |
| l | 57 | 115 | 230 | 459 | 919 | 1837 | 3675 | 7350 | 14699 |
| m | 43 | 85 | 171 | 341 | 683 | 1365 | 2731 | 5461 | 10922 |
| n | 25 | 53 | 105 | 210 | 420 | 841 | 1682 | 3363 | 6726 |
| o | 9 | 18 | 35 | 71 | 142 | 284 | 568 | 1136 | 2271 |
| p | 90 | 181 | 362 | 723 | 1446 | 2893 | 5786 | 11571 | 23143 |
| q | 90 | 179 | 358 | 716 | 1432 | 2865 | 5730 | 11460 | 22920 |
| r | 88 | 176 | 351 | 702 | 1405 | 2810 | 5619 | 11238 | 22476 |
| s | 85 | 170 | 341 | 682 | 1364 | 2727 | 5454 | 10908 | 21816 |
| t | 82 | 164 | 327 | 655 | 1309 | 2618 | 5236 | 10473 | 20946 |
| u | 78 | 155 | 311 | 621 | 1242 | 2484 | 4968 | 9937 | 19874 |
| v | 73 | 145 | 291 | 582 | 1163 | 2326 | 4653 | 9305 | 18611 |
| w | 67 | 134 | 268 | 537 | 1073 | 2146 | 4292 | 8584 | 17168 |
| x | 61 | 122 | 243 | 486 | 973 | 1945 | 3890 | 7780 | 15560 |
| y | 54 | 108 | 216 | 431 | 863 | 1725 | 3451 | 6901 | 13803 |
| z | 46 | 93 | 186 | 372 | 745 | 1489 | 2978 | 5956 | 11912 |
| A | 38 | 77 | 155 | 310 | 619 | 1238 | 2477 | 4953 | 9907 |
| B | 31 | 61 | 122 | 244 | 488 | 976 | 1951 | 3903 | 7806 |
| C | 22 | 44 | 88 | 176 | 352 | 704 | 1407 | 2815 | 5630 |
| D | 13 | 27 | 53 | 106 | 212 | 425 | 850 | 1700 | 3400 |
| E | 4 | 9 | 18 | 36 | 71 | 142 | 284 | 568 | 1137 |

For information, the smaller DCT matrices derived from the 32x32 matrix are presented here. The values in each grid are defined by the matrix coefficient precision according to the above table.

## 4x4 DCT

The matrix M4 is defined as the first 4 coefficients of every 8th row of the combined matrix M32.

## 8x8 DCT

The matrix M8 is defined as the first 8 coefficients of every 4th row of the combined matrix M32.

## 16x16 DCT

The matrix M16 is defined as the first 16 coefficients of every even row of the combined matrix M32.

# Appendix B: Working Draft Text

Changes relative to M1005-v2.

7.4.3.2	Sequence parameter set RBSP semantics

sps_video_parameter_set_id specifies the value of the vps_video_parameter_set_id of the active VPS.

…

bit_depth_luma_minus8 specifies the bit depth of the samples of the luma array BitDepthY and , the value of the luma quantization parameter range offset QpBdOffsetY, the value of the luma transform precision TransformRangeY and the value of the luma entropy coding precision EntropyRangeY as follows:

BitDepthY      = 8 + bit_depth_luma_minus8	(7 4)

QpBdOffsetY = 6 * bit_depth_luma_minus8	(7 5)

When general_profile_compatibility_flag[ 1 ] is equal to 1 or general_profile_compatibility_flag[ 2 ] is equal to 1 or general_profile_compatibility_flag[ 3 ] is equal to 1, then

TransformRangeY = 15	(7 5)

EntropyRangeY = 15	(7 5)

Otherwise (general_profile_compatibility_flag[ 1 ] is not equal to 1 and general_profile_compatibility_flag[ 2 ] is not equal to 1 and general_profile_compatibility_flag[ 3 ] is not equal to 1) then

TransformRangeY = max(15, 6 + BitDepthY)	(7 5)

EntropyRangeY = max(15, 6 + BitDepthY)	(7 5)

bit_depth_luma_minus8 shall be in the range of 0 to 68, inclusive.

bit_depth_chroma_minus8 specifies the bit depth of the samples of the chroma arrays BitDepthC and, the value of the chroma quantization parameter range offset QpBdOffsetC, the value of the chroma transform precision TransformRangeC and the value of the chroma entropy coding precision EntropyRangeC as follows:

BitDepthC      = 8 + bit_depth_chroma_minus8	(7 6)

QpBdOffsetC = 6 * bit_depth_chroma_minus8	(7 7)

When general_profile_compatibility_flag[ 1 ] is equal to 1 or general_profile_compatibility_flag[ 2 ] is equal to 1 or general_profile_compatibility_flag[ 3 ] is equal to 1, then

TransformRangeC = 15	(7 5)

EntropyRangeC = 15	(7 5)

Otherwise (general_profile_compatibility_flag[ 1 ] is not equal to 1 and general_profile_compatibility_flag[ 2 ] is not equal to 1 and general_profile_compatibility_flag[ 3 ] is not equal to 1) then

TransformRangeC = max(15, 6 + BitDepthC)	(7 5)

EntropyRangeC = max(15, 6 + BitDepthC)	(7 5)

bit_depth_chroma_minus8 shall be in the range of 0 to 68 , inclusive.

7.4.4	Profile, tier and level semantics

…

general_profile_compatibility_flag[ j ] equal to 1, when general_profile_space is equal to 0, indicates that the CVS conforms to the profile indicated by general_profile_idc equal to i as specified in Annex A. When general_profile_space is equal to 0, general_profile_compatibility_flag[ general_profile_idc ] shall be equal to 1. The value of general_profile_compatibility_flag[ j ] shall be equal to 0 for any value of j that is not specified as an allowed value of general_profile_idc in Annex A.

The value MatrixPrecision is determined according to general_profile_compatibility_flag[j], as specified in Annex A.

general_progressive_source_flag and …

7.4.9.11	Residual coding semantics

…

It is a requirement of bitstream conformance that the value of coeff_abs_level_remaining[ n ] shall be constrained such that the corresponding value of TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] is in the  range of (–(1<<EntropyRangeY)) to ((1<<EntropyRangeY)–1) inclusive when cIdx is equal to 0, and in the range of (–(1<<EntropyRangeC)) to ((1<<EntropyRangeC) –1) inclusive when cIdx is not equal to 0−32768 to 32767, inclusive.

8.6.2	Scaling and transformation process

…

The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nTbS)x(nTbS) array r is set equal to the (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause  is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ x ][ y ]  <<  (1+MatrixPrecision) 7 )	(8-1)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause  is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

- The variable bdShift is derived as follows:

bdShift = ( cIdx  = =  0 ) ? (TransformRangeY + MatrixPrecision – 1)20 − BitDepthY
 	                                                        : (TransformRangeC + MatrixPrecision – 1)20 − BitDepthC	 (8-2)

- The residual sample values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-3)

8.6.3	Scaling process for transform coefficients

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable nTbS specifying the size of the current transform block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is the (nTbS)x(nTbS) array d of scaled transform coefficients with elements d[ x ][ y ].

The variables bdShift, transformMin and transformMax areis derived as follows:

–	If cIdx is equal to 0,

bdShift = BitDepthY + Log2( nTbS ) + 10 − 5TransformRangeY	(8-4)

transformMin = – ( 1 << TransformRangeY ) 		(8-5)

transformMax =  ( 1 << TransformRangeY ) – 1		(8-6)

–	Otherwise,

bdShift = BitDepthC + Log2( nTbS ) + 10 − 5TransformRangeC	(8-7)

transformMin = – ( 1 << TransformRangeC ) 		(8-8)

transformMax =  ( 1 << TransformRangeC ) – 1		(8-9)

The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1, the following applies:

–	The scaling factor m[ x ][ y ] is derived as follows:

–	If scaling_list_enabled_flag is equal to 0,

m[ x ][ y ] = 16		(8-10)

–	Otherwise (scaling_list_enabled_flag is equal to 1),

m[ x ][ y ] = ScalingFactor[ sizeId ][ matrixId ][ x ][ y ]	(8-11)

Where sizeId is specified in Error: Reference source not found for the size of the quantization matrix equal to (nTbS)x(nTbS) and matrixId is specified in Error: Reference source not found for sizeId, CuPredMode[ xTbY ][ yTbY ], and cIdx, respectively.

–	The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( transformMin-32768, transformMax32767, ( ( TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * m[ x ][ y ] * 
			levelScale[ qP%6 ]  <<  (qP / 6 ) ) + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift )	(8-12)

### 8.6.4	Transformation process for scaled transform coefficients

8.6.4.1 General

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable nTbS specifying the size of the current transform block,

–	a variable cIdx specifying the colour component of the current block,

–	an (nTbS)x(nTbS) array d of scaled transform coefficients with elements d[ x ][ y ].

Output of this process is the (nTbS)x(nTbS) array r of residual samples with elements r[ x ][ y ].

Depending on the values of CuPredMode[ xTbY ][ yTbY ], nTbS, and cIdx, the variable trType is derived as follows:

–	If CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, nTbS is equal to 4, and cIdx is equal to 0, trType is set equal to 1.

–	Otherwise, trType is set equal to 0.

The variables transformMin and transformMax are derived as follows:

–	If cIdx is equal to 0,

transformMin = – ( 1 << TransformRangeY ) 		(8-13)

transformMax =  ( 1 << TransformRangeY ) – 1		(8-14)

–	Otherwise,

transformMin = – ( 1 << TransformRangeC ) 		(8-15)

transformMax =  ( 1 << TransformRangeC ) – 1		(8-16)

The (nTbS)x(nTbS) array r of residual samples is derived as follows:

- Each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 is transformed to e[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 by invoking the one-dimensional transformation process as specified in subclause  for each column x = 0..nTbS − 1 with the size of the transform block nTbS, the list d[ x ][ y ] with y = 0..nTbS − 1, and the transform type variable trType as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbS − 1.

- The intermediate sample values g[ x ][ y ] with  x = 0..nTbS − 1, y = 0..nTbS − 1  are derived as follows:

g[ x ][ y ] = Clip3(transformMin-32768, transformMax32767,
		( e[ x ][ y ] + (1<<TransformPrecision) )  >>  (TransformPrecision+1) )	(8-17)

- Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 is transformed to r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 by invoking the one-dimensional transformation process as specified in subclause  for each row y = 0..nTbS − 1 with the size of the transform block nTbS, the list g[ x ][ y ] with x = 0..nTbS − 1, and the transform type variable trType as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbS − 1.

8.6.4.2 Transformation process

Inputs to this process are:

–	a variable nTbS specifying the sample size of scaled transform coefficients,

–	a list of scaled transform coefficients x with elements x[ j ], with j = 0..nTbS − 1.

–	a transform type variable trType

Output of this process is the list of transformed samples y with elements y[ i ], with i = 0..nTbS − 1.

Depending on the value of trType, the following applies:

–	If trType is equal to 1, the following transform matrix multiplication applies:

y[ i ] = j=0nTbS−1transMatrix[i][j]∗x[j] with i = 0..nTbS − 1	(8-18)

where the transform coefficient array transMatrix is specified as follows:

transMatrix =	(8-19)

{

{29  55  74  84}

{74  74   0 −74}

{84 −29 −74  55}

{55 −84  74 −29}

}

{

{F  G  H  I}

{H  H  J −H}

{I −F −H  G}

{G −I  H −F}

}

where the values F, G, H, I and J in the above transMatrix are defined in Table X-X.

–	Otherwise (trType is equal to 0), the following transform matrix multiplication applies:

y[ i ] = j=0nTbS−1transMatrix[i][j∗25−Log2(nTbS)]∗x[j] with i = 0..nTbS− 1,	(8-20)

where the transform coefficient array transMatrix is specified as follows:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0...31	(8-21)

transMatrixCol0to15 =	(8-22)

{

{64  64  64  64  64  64  64  64  64  64  64  64  64  64  64  64}

{90  90  88  85  82  78  73  67  61  54  46  38  31  22  13   4}

{90  87  80  70  57  43  25   9  −9 −25 −43 −57 −70 −80 −87 −90}

{90  82  67  46  22  −4 −31 −54 −73 −85 −90 −88 −78 −61 −38 −13}

{89  75  50  18 −18 −50 −75 −89 −89 −75 −50 −18  18  50  75  89}

{88  67  31 −13 −54 −82 −90 −78 −46  −4  38  73  90  85  61  22}

{87  57   9 −43 −80 −90 −70 −25  25  70  90  80  43  −9 −57 −87}

{85  46 −13 −67 −90 −73 −22  38  82  88  54  −4 −61 −90 −78 −31}

{83  36 −36 −83 −83 −36  36  83  83  36 −36 −83 −83 −36  36  83}

{82  22 −54 −90 −61  13  78  85  31 −46 −90 −67   4  73  88  38}

{80   9 −70 −87 −25  57  90  43 −43 −90 −57  25  87  70  −9 −80}

{78  −4 −82 −73  13  85  67 −22 −88 −61  31  90  54 −38 −90 −46}

{75 −18 −89 −50  50  89  18 −75 −75  18  89  50 −50 −89 −18  75}

{73 −31 −90 −22  78  67 −38 −90 −13  82  61 −46 −88  −4  85  54}

{70 −43 −87   9  90  25 −80 −57  57  80 −25 −90  −9  87  43 −70}

{67 −54 −78  38  85 −22 −90   4  90  13 −88 −31  82  46 −73 −61}

{64 −64 −64  64  64 −64 −64  64  64 −64 −64  64  64 −64 −64  64}

{61 −73 −46  82  31 −88 −13  90  −4 −90  22  85 −38 −78  54  67}

{57 −80 −25  90  −9 −87  43  70 −70 −43  87   9 −90  25  80 −57}

{54 −85  −4  88 −46 −61  82  13 −90  38  67 −78 −22  90 −31 −73}

{50 −89  18  75 −75 −18  89 −50 −50  89 −18 −75  75  18 −89  50}

{46 −90  38  54 −90  31  61 −88  22  67 −85  13  73 −82   4  78}

{43 −90  57  25 −87  70   9 −80  80  −9 −70  87 −25 −57  90 −43}

{38 −88  73  −4 −67  90 −46 −31  85 −78  13  61 −90  54  22 −82}

{36 −83  83 −36 −36  83 −83  36  36 −83  83 −36 −36  83 −83  36}

{31 −78  90 −61   4  54 −88  82 −38 −22  73 −90  67 −13 −46  85}

{25 −70  90 −80  43   9 −57  87 −87  57  −9 −43  80 −90  70 −25}

{22 −61  85 −90  73 −38  −4  46 −78  90 −82  54 −13 −31  67 −88}

{18 −50  75 −89  89 −75  50 −18 −18  50 −75  89 −89  75 −50  18}

{13 −38  61 −78  88 −90  85 −73  54 −31   4  22 −46  67 −82  90}

{ 9 −25  43 −57  70 −80  87 −90  90 −87  80 −70  57 −43  25  −9}

{ 4 −13  22 −31  38 −46  54 −61  67 −73  78 −82  85 −88  90 −90}

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31,	(8-23)

transMatrixCol16to31 = 	(8-24)

{

{ 64  64  64  64  64  64  64  64  64  64  64  64  64  64  64  64}

{ −4 −13 −22 −31 −38 −46 −54 −61 −67 −73 −78 −82 −85 −88 −90 −90}

{−90 −87 −80 −70 −57 −43 −25  −9   9  25  43  57  70  80  87  90}

{ 13  38  61  78  88  90  85  73  54  31   4 −22 −46 −67 −82 −90}

{ 89  75  50  18 −18 −50 −75 −89 −89 −75 −50 −18  18  50  75  89}

{−22 −61 −85 −90 −73 −38   4  46  78  90  82  54  13 −31 −67 −88}

{−87 −57  −9  43  80  90  70  25 −25 −70 −90 −80 −43   9  57  87}

{ 31  78  90  61   4 −54 −88 −82 −38  22  73  90  67  13 −46 −85}

{ 83  36 −36 −83 −83 −36  36  83  83  36 −36 −83 −83 −36  36  83}

{−38 −88 −73  −4  67  90  46 −31 −85 −78 −13  61  90  54 −22 −82}

{−80  −9  70  87  25 −57 −90 −43  43  90  57 −25 −87 −70   9  80}

{ 46  90  38 −54 −90 −31  61  88  22 −67 −85 −13  73  82   4 −78}

{ 75 −18 −89 −50  50  89  18 −75 −75  18  89  50 −50 −89 −18  75}

{−54 −85   4  88  46 −61 −82  13  90  38 −67 −78  22  90  31 −73}

{−70  43  87  −9 −90 −25  80  57 −57 −80  25  90   9 −87 −43  70}

{ 61  73 −46 −82  31  88 −13 −90  −4  90  22 −85 −38  78  54 −67}

{ 64 −64 −64  64  64 −64 −64  64  64 −64 −64  64  64 −64 −64  64}

{−67 −54  78  38 −85 −22  90   4 −90  13  88 −31 −82  46  73 −61}

{−57  80  25 −90   9  87 −43 −70  70  43 −87  −9  90 −25 −80  57}

{ 73  31 −90  22  78 −67 −38  90 −13 −82  61  46 −88   4  85 −54}

{ 50 −89  18  75 −75 −18  89 −50 −50  89 −18 −75  75  18 −89  50}

{−78  −4  82 −73 −13  85 −67 −22  88 −61 −31  90 −54 −38  90 −46}

{−43  90 −57 −25  87 −70  −9  80 −80   9  70 −87  25  57 −90  43}

{ 82 −22 −54  90 −61 −13  78 −85  31  46 −90  67   4 −73  88 −38}

{ 36 −83  83 −36 −36  83 −83  36  36 −83  83 −36 −36  83 −83  36}

{−85  46  13 −67  90 −73  22  38 −82  88 −54  −4  61 −90  78 −31}

{−25  70 −90  80 −43  −9  57 −87  87 −57   9  43 −80  90 −70  25}

{ 88 −67  31  13 −54  82 −90  78 −46   4  38 −73  90 −85  61 −22}

{ 18 −50  75 −89  89 −75  50 −18 −18  50 −75  89 −89  75 −50  18}

{−90  82 −67  46 −22  −4  31 −54  73 −85  90 −88  78 −61  38 −13}

{ −9  25 −43  57 −70  80 −87  90 −90  87 −80  70 −57  43 −25   9}

{ 90 −90  88 −85  82 −78  73 −67  61 −54  46 −38  31 −22  13  −4}

}

transMatrix[ ][ ] =	(8-25)

{

{a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a  a}

{p  q  r  s  t  u  v  w  x  y  z  A  B  C  D  E -E -D -C -B -A -z -y -x -w -v -u -t -s -r -q -p}

{h  i  j  k  l  m  n  o -o -n -m -l -k -j -i -h -h -i -j -k -l -m -n -o  o  n  m  l  k  j  i  h}

{q  t  w  z  C -E -B -y -v -s -p -r -u -x -A -D  D  A  x  u  r  p  s  v  y  B  E -C -z -w -t -q}

{d  e  f  g -g -f -e -d -d -e -f -g  g  f  e  d  d  e  f  g -g -f -e -d -d -e -f -g  g  f  e  d}

{r  w  B -D -y -t -p -u -z -E  A  v  q  s  x  C -C -x -s -q -v -A  E  z  u  p  t  y  D -B -w -r}

{i  l  o -m -j -h -k -n  n  k  h  j  m -o -l -i -i -l -o  m  j  h  k  n -n -k -h -j -m  o  l  i}

{s  z -D -w -p -v -C  A  t  r  y -E -x -q -u -B  B  u  q  x  E -y -r -t -A  C  v  p  w  D -z -s}

{b  c -c -b -b -c  c  b  b  c -c -b -b -c  c  b  b  c -c -b -b -c  c  b  b  c -c -b -b -c  c  b}

{t  C -y -p -x  D  u  s  B -z -q -w  E  v  r  A -A -r -v -E  w  q  z -B -s -u -D  x  p  y -C -t}

{j  o -k -i -n  l  h  m -m -h -l  n  i  k -o -j -j -o  k  i  n -l -h -m  m  h  l -n -i -k  o  j}

{u -E -t -v  D  s  w -C -r -x  B  q  y -A -p -z  z  p  A -y -q -B  x  r  C -w -s -D  v  t  E -u}

{e -g -d -f  f  d  g -e -e  g  d  f -f -d -g  e  e -g -d -f  f  d  g -e -e  g  d  f -f -d -g  e}

{v -B -p -C  u  w -A -q -D  t  x -z -r -E  s  y -y -s  E  r  z -x -t  D  q  A -w -u  C  p  B -v}

{k -m -i  o  h  n -j -l  l  j -n -h -o  i  m -k -k  m  i -o -h -n  j  l -l -j  n  h  o -i -m  k}

{w -y -u  A  s -C -q  E  p  D -r -B  t  z -v -x  x  v -z -t  B  r -D -p -E  q  C -s -A  u  y -w}

{a -a -a  a  a -a -a  a  a -a -a  a  a -a -a  a  a -a -a  a  a -a -a  a  a -a -a  a  a -a -a  a}

{x -v -z  t  B -r -D  p -E -q  C  s -A -u  y  w -w -y  u  A -s -C  q  E -p  D  r -B -t  z  v -x}

{l -j -n  h -o -i  m  k -k -m  i  o -h  n  j -l -l  j  n -h  o  i -m -k  k  m -i -o  h -n -j  l}

{y -s -E  r -z -x  t  D -q  A  w -u -C  p -B -v  v  B -p  C  u -w -A  q -D -t  x  z -r  E  s -y}

{f -d  g  e -e -g  d -f -f  d -g -e  e  g -d  f  f -d  g  e -e -g  d -f -f  d -g -e  e  g -d  f}

{z -p  A  y -q  B  x -r  C  w -s  D  v -t  E  u -u -E  t -v -D  s -w -C  r -x -B  q -y -A  p -z}

{m -h  l  n -i  k  o -j  j -o -k  i -n -l  h -m -m  h -l -n  i -k -o  j -j  o  k -i  n  l -h  m}

{A -r  v -E -w  q -z -B  s -u  D  x -p  y  C -t  t -C -y  p -x -D  u -s  B  z -q  w  E -v  r -A}

{c -b  b -c -c  b -b  c  c -b  b -c -c  b -b  c  c -b  b -c -c  b -b  c  c -b  b -c -c  b -b  c}

{B -u  q -x  E  y -r  t -A -C  v -p  w -D -z  s -s  z  D -w  p -v  C  A -t  r -y -E  x -q  u -B}

{n -k  h -j  m  o -l  i -i  l -o -m  j -h  k -n -n  k -h  j -m -o  l -i  i -l  o  m -j  h -k  n}

{C -x  s -q  v -A -E  z -u  p -t  y -D -B  w -r  r -w  B  D -y  t -p  u -z  E  A -v  q -s  x -C}

{g -f  e -d  d -e  f -g -g  f -e  d -d  e -f  g  g -f  e -d  d -e  f -g -g  f -e  d -d  e -f  g}

{D -A  x -u  r -p  s -v  y -B  E  C -z  w -t  q -q  t -w  z -C -E  B -y  v -s  p -r  u -x  A -D}

{o -n  m -l  k -j  i -h  h -i  j -k  l -m  n -o -o  n -m  l -k  j -i  h -h  i -j  k -l  m -n  o}

{E -D  C -B  A -z  y -x  w -v  u -t  s -r  q -p  p -q  r -s  t -u  v -w  x -y  z -A  B -C  D -E}

}

where the values a,b,c,..x,y,z,A,B,C,D and E in the above transMatrix are defined in Table X-X.

Table X-X – transform coefficient matrix values for a given MatrixPrecision

|  | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a | 64 | 128 | 256 | 512 | 1024 | 2048 | 4096 | 8192 | 16384 |
| b | 83 | 167 | 334 | 669 | 1338 | 2676 | 5352 | 10703 | 21407 |
| c | 36 | 69 | 139 | 277 | 554 | 1108 | 2217 | 4433 | 8867 |
| d | 89 | 178 | 355 | 710 | 1420 | 2841 | 5681 | 11363 | 22725 |
| e | 75 | 151 | 301 | 602 | 1204 | 2408 | 4816 | 9633 | 19266 |
| f | 50 | 101 | 201 | 402 | 805 | 1609 | 3218 | 6436 | 12873 |
| g | 18 | 35 | 71 | 141 | 283 | 565 | 1130 | 2260 | 4520 |
| h | 90 | 180 | 360 | 721 | 1441 | 2882 | 5765 | 11529 | 23059 |
| i | 87 | 173 | 346 | 693 | 1386 | 2772 | 5543 | 11086 | 22173 |
| j | 80 | 160 | 319 | 639 | 1277 | 2554 | 5109 | 10217 | 20435 |
| k | 70 | 140 | 280 | 560 | 1119 | 2239 | 4478 | 8956 | 17911 |
| l | 57 | 115 | 230 | 459 | 919 | 1837 | 3675 | 7350 | 14699 |
| m | 43 | 85 | 171 | 341 | 683 | 1365 | 2731 | 5461 | 10922 |
| n | 25 | 53 | 105 | 210 | 420 | 841 | 1682 | 3363 | 6726 |
| o | 9 | 18 | 35 | 71 | 142 | 284 | 568 | 1136 | 2271 |
| p | 90 | 181 | 362 | 723 | 1446 | 2893 | 5786 | 11571 | 23143 |
| q | 90 | 179 | 358 | 716 | 1432 | 2865 | 5730 | 11460 | 22920 |
| r | 88 | 176 | 351 | 702 | 1405 | 2810 | 5619 | 11238 | 22476 |
| s | 85 | 170 | 341 | 682 | 1364 | 2727 | 5454 | 10908 | 21816 |
| t | 82 | 164 | 327 | 655 | 1309 | 2618 | 5236 | 10473 | 20946 |
| u | 78 | 155 | 311 | 621 | 1242 | 2484 | 4968 | 9937 | 19874 |
| v | 73 | 145 | 291 | 582 | 1163 | 2326 | 4653 | 9305 | 18611 |
| w | 67 | 134 | 268 | 537 | 1073 | 2146 | 4292 | 8584 | 17168 |
| x | 61 | 122 | 243 | 486 | 973 | 1945 | 3890 | 7780 | 15560 |
| y | 54 | 108 | 216 | 431 | 863 | 1725 | 3451 | 6901 | 13803 |
| z | 46 | 93 | 186 | 372 | 745 | 1489 | 2978 | 5956 | 11912 |
| A | 38 | 77 | 155 | 310 | 619 | 1238 | 2477 | 4953 | 9907 |
| B | 31 | 61 | 122 | 244 | 488 | 976 | 1951 | 3903 | 7806 |
| C | 22 | 44 | 88 | 176 | 352 | 704 | 1407 | 2815 | 5630 |
| D | 13 | 27 | 53 | 106 | 212 | 425 | 850 | 1700 | 3400 |
| E | 4 | 9 | 18 | 36 | 71 | 142 | 284 | 568 | 1137 |
| F | 29 | 58 | 117 | 233 | 467 | 934 | 1868 | 3736 | 7472 |
| G | 55 | 110 | 219 | 439 | 878 | 1755 | 3510 | 7021 | 14042 |
| H | 74 | 148 | 296 | 591 | 1182 | 2365 | 4730 | 9459 | 18919 |
| I | 84 | 168 | 336 | 672 | 1345 | 2689 | 5378 | 10757 | 21513 |
| J | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

A.3.2 Main profile

…

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

- The value of the variable MatrixPrecision is equal to 6.

...

A.3.3 Main 10 profile

…

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

- The value of the variable MatrixPrecision is equal to 6.

...

A.3.4 Main Still Picture profile

…

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

- The value of the variable MatrixPrecision is equal to 6.

...

Example High Bit Accuracy Profile (Changes shown relative to Main 10 Profile):

A.3.5 High 16 4:4:4 profile

Bitstreams conforming to the Main 10High 16 4:4:4 profile shall obey the following constraints:

–	SPSs shall have chroma_format_idc equal to 1 only.

–	SPSs shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

–	SPSs shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

–	CtbLog2SizeY shall be in the range of 4 to 6, inclusive.

–	When a PPS has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

–	When a PPS has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

–	The number of times read_bits( 1 ) is called in subclauses 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

–	The level constraints specified for the Main 10High 16 4:4:4 profile in subclause A.4 shall be fulfilled.

–	The value of the variable MatrixPrecision is equal to 14.

Conformance of a bitstream to the Main 10High 16 4:4:4 profile is indicated by general_profile_idc being equal to 24 or general_profile_compatibility_flag[24 ] being equal to 1.

Decoders conforming to the Main 10High 16 4:4:4 profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which all of the following conditions apply:

–	general_profile_compatibility_flag[ 1 ] is equal to 1 or general_profile_compatibility_flag[ 2 ] is equal to 1 or general_profile_compatibility_flag[ 4 ] is equal to 1.

–	general_level_idc represents a level lower than or equal to the specified level.

–	general_tier_flag represents a tier lower than or equal to the specified tier.