| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 August 2013 | Document: JCTVC-N0189 |
| --- | --- |

| Title: | AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

Four systems are presented that are described as requiring only small changes to the current entropy coding scheme, while reportedly yielding a substantial BD-rate saving at 12-bit and higher.

Out of the four systems, the proposed method is Golomb-Rice Parameter modification with auto-adaptation, which is indicated to give efficiency improvements of -30.3% at 96dB (16-bit operating point), -21.5% at 84dB (14-bit operating point), -8.3% at 72dB (12-bit operating point), -0.7% at 60dB (10-bit operating point) and no significant change at 48dB (8-bit operating point) relative to the proposed higher internal accuracy system described in JCTVC-N0188. Improvements are not observed for positive QPs, however, this is not the intended operating range for this high bit rate/high bit depth tool.

In addition, the system has been trialled under AHG8 lossless test conditions, with reported intra BD-rate changes of -1.6%, 0.0%, -16.4%, -8.3%, -2.4% for Classes F, B, ScreenContent RGB, ScreenContent 4:4:4 and a limited subset of the Range Extensions sequences respectively; inter results are similar.

# Introduction

As shown in JCTVC-N0188, the coding efficiency (bit rate vs. PSNR) of HEVC at high bit depths can be substantially increased relative to HEVC international standard by increasing the internal precision of the transform and quantisation path. Although this removes the inconsistencies in the response curve, there are still further performance improvements to be obtained.

The HEVC entropy coder was designed for efficiently coding 15-bit (magnitude) coefficients that correspond to residual samples of 8- to 10-bit precision input samples. In order to efficiently code 22-bit (magnitude) coefficients associated with 16-bit input samples, modifications are needed.

In this report, the test conditions and sequences are those specified in JCTVC-N0188 (an extension of those proposed in JCTVC-M0178), being substantially different to the standard Range Extension test conditions.

# Improving Compression Efficiency

The current HEVC CABAC entropy coder codes syntax elements using the following processes:

- The location of the last significant coefficient (in scan order) in the TU is coded.

- For each 4x4 coefficient group (groups are processed in reverse scan order), a significant-coefficient-group flag is coded, indicating whether or not the group contains non-zero coefficients. This is not required for the group containing the last significant coefficient and is assumed to be 1 for the top-left group (containing the DC coefficient). If the flag is 1, then the following syntax elements pertaining to the group are coded immediately following it:

  - Significance map:
For each coefficient in the group, a flag is coded indicating whether or not the coefficient is significant (has a non-zero value). No flag is necessary for the coefficient indicated by the last-significant position.

  - Greater-than-one map:
For up to eight coefficients with significance map value 1 (counted backwards from the end of the group), this indicates whether the magnitude is greater than 1.

  - Greater-than-two flag:
For up to one coefficient with greater-than-one map value 1 (the one nearest the end of the group), this indicates whether the magnitude is greater than 2.

  - Sign bits:
For all non-zero coefficients, sign bits are coded as equiprobable CABAC bins, with the last sign bit (in reverse scan order) possibly being instead inferred from parity when sign bit hiding is used.

  - Escape codes:
For any coefficient whose magnitude was not completely described by an earlier syntax element, the remainder is coded as an escape code.

For a 16-bit, 14-bit or even 12-bit system at the operating point where the MSE is less than 1 (typically at QPs -34, -22 and -10 respectively), the system typically yields very little compression (for 16-bit, it actually inflates the source data). The coefficients are generally large numbers, and therefore are almost always escape-coded.

For that reason, two alternative changes to the entropy coder are proposed to allow for higher bit depths, as described in the following two sections.

## Direct Coding of Fixed Bits

To increase efficiency at higher bit depths, the process of the HEVC entropy coder can be changed to the following scheme, where a number of fixed bits BF for each significant coefficient are bypass-coded. This means that the CABAC syntax elements following the significance map, apply to only the most significant part of each coefficient, and therefore are still useful at the higher operating points (lower QPs). In essence, the CABAC engine that was developed for 8 and 10-bit operation can be used as it was originally intended.

This is similar to the algorithm previously presented in JCTVC-M0178 Section 6.1.

The changes to the current HEVC process (see above) are indicated below in bold:

- The location of the last significant coefficient (in scan order) in the TU is coded.

- For each 4x4 coefficient group (groups are processed in reverse scan order), a significant-coefficient-group flag is coded, indicating whether or not the group contains non-zero coefficients. This is not required for the group containing the last significant coefficient and is assumed to be 1 for the top-left group (containing the DC coefficient). If the flag is 1, then each significant coefficient C is split into a most-significant part CMSB and a least-significant part CLSB, where
CMSB = (abs(C) – 1) >> BF
and
CLSB = (abs(C) – 1) – (CMSB << BF)
and the following syntax elements pertaining to the group are coded immediately following it:

  - Significance map:
For each coefficient in the group, a flag is coded indicating whether or not the coefficient C is significant (has a non-zero value). No flag is necessary for the coefficient indicated by the last-significant position.

  - Greater-than-one map:
For up to eight coefficients with significance map value 1 (counted backwards from the end of the group), this indicates whether CMSB is greater than or equal to 1.

  - Greater-than-two flag:
For up to one coefficient with greater-than-one map value 1 (the one nearest the end of the group), this indicates whether CMSB is greater than or equal to 2.

  - Sign bits:
For all non-zero coefficients, sign bits are coded as equiprobable CABAC bins, with the last sign bit (in reverse scan order) possibly being instead inferred from parity when sign bit hiding is used.

  - For each non-zero coefficient:

    - Escape codes:
If the magnitude of CMSB was not completely described by an earlier syntax element, the remainder is coded as an escape code.

    - Fixed bits:
If BF is greater than 0, the value of CLSB is coded.

The value of BF would be signalled by the encoder in the slice header (with the sequence parameter set (SPS) indicating that the feature is enabled), since the best value is source dependent and some encoders may wish to adaptively control this function. However, a simple derivation for BF based on QP is as follows:

BF = max(0, int(QP / -6))

The results using this simple derivation for the fixed-bit scheme are shown in Figure 1.

Figure 1 - Bit rate vs. green channel PSNR for OldTownCross with 14-bit transform matrices, MAX_TR_DYNAMIC_RANGE = max(15, (bitDepth + 6)) and
ENTROPY_CODING_DYNAMIC_RANGE = max(15, (bitDepth + 6))
(RDOQ disabled, transform-skip enabled) with proposed scheme.

The saving in bit rate for the system with fixed bits (relative to the system without fixed bits) varies from 4% at QP -6 (10-bit) up to 35% at QP -48 (16-bit).

Enabling transform-skip produces similar, though slightly more irregular, bit rate/PSNR curves. Other settings can be explored in the accompanying Excel spreadsheet.

## Golomb-Rice Parameter Modification

The method presented above in Section 2.1 introduces an additional stage in the entropy coder wherein the fixed bits are split from the coefficient and coded separately. This could instead be combined into the existing Golomb-Rice/exponential-Golomb escape coding mechanism.

Currently, when coding an escape code, the value to be coded is split into a prefix and a suffix using either a Golomb-Rice or exponential-Golomb-order-k scheme, with the suffix including a number cRiceParam of least-significant bits of the coded value. For example, to code the value 13 (0b1101) with cRiceParam = 2, the LSBs of the suffix would be equal to the 2 LSBs 0b01 of the coded value. The value of cRiceParam is maintained by the entropy coder; being reset to 0 at the start of each coefficient group and conditionally incremented after each escape coding operation.

By applying an offset to cRiceParam equal to BF, the fixed bits can be extracted from the coefficient magnitude as part of the normal escape coding operation. This is a simpler method than the fixed bits scheme presented in Section 2.1, but at the cost of reduced coding efficiency since the fixed bits are now split from the coefficient after coding greater-than-one and greater-than-two flags rather than before, reducing the relevance of those flags.

Results comparing this scheme with that presented in Section 2.1 are shown in Figure 2. The loss in coding efficiency (BD-rate change) when compared to direct coding of fixed bits was found to be 0.6%.

Figure 2 - Bit rate vs. green channel PSNR for 16-bit OldTownCross with 14-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 22 and ENTROPY_CODING_DYNAMIC_RANGE = 22 (RDOQ disabled, transform-skip enabled) Golomb-Rice parameter modification vs. direct coding of fix.

## Automatic Adaptation of the Number of Fixed Bits BF

In the results presented in Sections 2.1 and 2.2, the proposed scheme used a fixed value of BF, signalling the value in the slice header. Having BF be configurable by the encoder in this way allows it to select a preferred value according to the properties of the source video, but cannot account for variation within a slice.

An algorithm is proposed to allow the value of BF to adaptively change over the course of a slice; conceptually similar to the context variables that are used to adapt CABAC. The steps of this algorithm differ slightly depending on which method of coding fixed bits is in use.

Again, the Fixed Bit scheme is enabled with a flag in the SPS; no additional signalling is required.

### Automatic Adaptation of BF for Direct Coding of Fixed Bits (see Section 2.1)

- The value of BF is initialised to 0 at the start of each slice.

- For each TU encoded/decoded:

  - The variables undershootCount, overshootCount and totalCount are each initialised to 0.

  - For each significant coefficient C, the variables coefficientMagnitude and baseLevel are defined by the following logic (as in HEVC version 1):

coefficientMagnitude = abs(C)

If the coefficient had a greater-than-two flag coded for it, 		baseLevel = 3
otherwise if the coefficient had a greater-than-one flag coded for it, 	baseLevel = 2
otherwise, 								baseLevel = 1

then, steps are taken as defined by the following pseudo-code:

totalCount++

remainingMagnitude = (coefficientMagnitude - 1) >> BF

if ((remainingMagnitude >> 1) >= baseLevel)
{
  undershootCount++
}
else if ((BF > 0) && ((remainingMagnitude << 1) <= 0))
{
  overshootCount++
}

  - At the end of encoding/decoding the TU, BF is adjusted according to the following pseudo-code:

if       ((undershootCount * 4) > totalCount)              BF++	[Note ]
else if (((overshootCount  * 2) > totalCount) && (BF > 0)) BF--

Conceptually, the adaptation algorithm determines if the current value of BF has undershot or overshot the ideal value for each coefficient. To determine this ideal value, it is asserted that coding CABAC syntax elements (greater-than-one flag, greater-than-two flag) is efficient, whereas coding escape codes is not. Hence, the ideal number of fixed bits is such that remainingMagnitude lies in the range (0 to baseLevel) exclusive. An undershoot or overshoot is only considered significant if it is by at least two bits, therefore the increment conditions check (remainingMagnitude >> 1) and (remainingMagnitude << 1) respectively.

The end-of-TU update condition causes an increment if more than 25% of coefficients undershot the ideal value of BF and decrement if more than 50% of coefficients overshot the ideal value of BF. The reason for this asymmetry is that undershooting causes more inefficiency than overshooting.

### Automatic Adaptation of BF for Golomb-Rice Parameter Modification (see Section 2.2)

- The value of BF is initialised to 0 at the start of each slice.

- For each TU encoded/decoded:

  - The variables undershootCount, overshootCount and totalCount are each initialised to 0.

  - For each significant coefficient, if an escape code escapeCode is coded for that coefficient, steps are taken as defined by the following pseudo-code:

totalCount++

if (escapeCode > (3 << cRiceParam))
{
  undershootCount++
}
else if ((escapeCode * 3) < (1 << cRiceParam))
{
  overshootCount++
}

  - At the end of encoding/decoding the TU, BF is adjusted according to the following pseudo-code:

if       ((undershootCount * 4) > totalCount)              BF++	[Note ]
else if (((overshootCount  * 2) > totalCount) && (BF > 0)) BF--

Apart from the conditions used to define undershoots and overshoots, this adaptation method is the same as that presented in Section 2.3.1. The under/overshoot conditions are kept the same as existing comparisons in HEVC version 1 for ease of implementation. Likewise, the end-of-TU update condition prioritises prevention of undershoots as these cause more inefficiency.

### Results of Automatic Adaptation of BF

The difference made to the bit rate/PSNR curve when using automatic adaptation is illustrated in Figure 3 for direct coding of fixed bits (as presented in Section 2.1) and Figure 4 for Golomb-Rice parameter modification (as presented in Section 2.2). The BD-rate changes between the two curves are -2.1% and -2.3% respectively favouring automatic adaptation in both cases.

Figure 3 - Bit rate vs. green channel PSNR for 16-bit OldTownCross with 14-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 22 and ENTROPY_CODING_DYNAMIC_RANGE = 22 (RDOQ disabled, transform-skip enabled).

Figure 4 - Bit rate vs. green channel PSNR for 16-bit OldTownCross with 14-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 22 and ENTROPY_CODING_DYNAMIC_RANGE = 22 (RDOQ disabled, transform-skip enabled).

# Results

## Lossy Coding

As illustrated in the accompanying Excel spreadsheets and in the above graphs, coding of fixed bits yields the following BD-rate changes:

Table 1 – BD-rate changes using green channel PSNR for OldTownCross with 14-bit transform matrices, MAX_TR_DYNAMIC_RANGE = max(15, (bitDepth + 6)) and ENTROPY_CODING_DYNAMIC_RANGE = max(15, (bitDepth + 6)) 
(RDOQ disabled, transform-skip enabled, lossless coding disabled).

As can be seen, the savings are substantial at high bit depths and operating points. In addition, when using the standard RExt test conditions (being in the range of 36 to 53 dB) the BD-rate changes in Table 2 are observed.

Table 2 – BD-rate changes under RExt intra test conditions using the Golomb-Rice parameter modification with auto-adaptation

Since the proposed scheme is selectable by the encoder at the sequence parameter set level, the losses at lower operating points (higher QPs) can be disregarded as the encoder would simply opt not to use the scheme where it provides no or little benefit (i.e. at positive QPs when using lossy coding).

## Mixed Lossy and Lossless Coding

As illustrated in the accompanying Excel spreadsheets and in Figure 5, coding of fixed bits while using mixed lossy and lossless coding yields the following BD-rate changes (note the interpretation of PSNRs is different, as specified in JCTVC-N0188):

Table 3 – BD-rate changes using green channel PSNR for OldTownCross with 14-bit transform matrices, MAX_TR_DYNAMIC_RANGE = max(15, (bitDepth + 6)) and ENTROPY_CODING_DYNAMIC_RANGE = max(15, (bitDepth + 6)) 
(RDOQ disabled, transform-skip enabled, lossless coding enabled).

Figure 5 shows the benefit given by Golomb-Rice parameter modification with auto-adaptation. Unlike similar results previously presented in JCTVC-M0178, the proposed system now reaches an asymptotic point. This was not observed before and is because in these new simulations, transquant-bypass mode is enabled in addition to IPCM. CUs coded using transquant-bypass mode still pass through the entropy coder (unlike when using IPCM) and hence benefit from the Golomb-Rice parameter modification. As a result, the bit-rate at which the encoder selects lossless mode for every CU is lowered.

Of course, for simpler sequences, the asymptote is not observed and the system is rate-controllable with a monotonic relationship between bits and SNR over the full QP range.

Figure 5 - Bit rate vs. green channel PSNR for 16-bit OldTownCross with 14-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 20 and ENTROPY_CODING_DYNAMIC_RANGE = 20 (RDOQ disabled, transform-skip enabled).

## Lossless Coding

Since the described system is able to move the asymptote for high bit depths under mixed lossless conditions, the system has also been trialled using the lossless test conditions defined by AHG8, giving the bit-rate changes shown in Table 4. The results indicate a substantial saving for the screen content sequences, while giving no loss for other types of source material.

Table 4 – Bit rate changes under AHG8 lossless test conditions

# Conclusion

Four systems have been presented that, while requiring only small changes to the current entropy coding scheme, yield a substantial BD-rate saving at 12-bit and higher in a lossy system.

The recommended method is Golomb-Rice Parameter modification with auto-adaptation, giving efficiency improvements of -30.3% at 96dB (16-bit operating point), -21.5% at 84dB (14-bit operating point), -8.3% at 72dB (12-bit operating point), -0.7% at 60dB (10-bit operating point) and no significant change at 48dB (8-bit operating point) relative to the proposed higher internal accuracy system described in JCTVC-N0188. There are no significant changes at the RExt intra operating conditions.

In addition, the system has been tested using the AHG8 lossless test conditions, with intra BD-rate changes of -1.6%, 0.0%, -16.4%, -8.3%, -2.4% for Classes F, B, ScreenContent RGB, ScreenContent 4:4:4, Range Extension sequences, and similar savings for the inter test conditions.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Working Draft Text

Changes required for base system and for adaptive system are shown in yellow.

Additional changes required for adaptive system only are shown in green.

Additional changes required for fixed golomb-rice system are shown in cyan.

#### 7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| … |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| adaptive_golomb_rice_parameter_offset_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( adaptive_golomb_rice_parameter_offset_flag ) { |  |
| golomb_rice_parameter_offset | ue(v) |
| } |  |
| … |  |
| } |  |

#### 7.3.8.11	Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize = = 2 ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| golombRiceUndershoots = 0 |  |
| golombRiceOvershoots = 0 |  |
| golombRiceTotal = 0 |  |
| last_sig_coeff_x_prefix | ae(v) |
| … |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| … |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = lastGreater1ScanPos) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| if ( adaptive_golomb_rice_parameter_offset_flag ) { |  |
| golombRiceTotal++ |  |
| if (coeff_abs_level_remaining[ n ]> ( 3 << cRiceParam ) ) |  |
| golombRiceUndershoots++ |  |
| else if ( (coeff_abs_level_remaining[ n ]* 3 ) < ( 1 << cRiceParam ) ) |  |
| golombRiceOvershoots++ |  |
| } |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = (coeff_abs_level_remaining[ n ] + baseLevel) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_enabled_flag && signHidden ) { |  |
| sumAbsLevel += coeff_abs_level_remaining[ n ] + baseLevel |  |
| if( ( n = = firstSigScanPos ) && ( ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| numSigCoeff++ |  |
| } |  |
| } |  |
| } |  |
| if ( adaptive_golomb_rice_parameter_offset_flag ) { |  |
| if ( ( golombRiceUndershoots * 4 ) > golombRiceTotal ) |  |
| cRiceParamOffset + + |  |
| if ( ( ( golombRiceOvershoots * 2 ) > golombRiceTotal ) && ( golombRiceOffset > 0 ) ) |  |
| cRiceParamOffset − − |  |
| } |  |
| } |  |

#### 7.4.3.2	Sequence parameter set RBSP semantics

bit_depth_chroma_minus8 specifies the bit depth of the samples of the chroma arrays BitDepthC and the value of the chroma quantization parameter range offset QpBdOffsetC as follows:

BitDepthC      = 8 + bit_depth_chroma_minus8		(7-1)

QpBdOffsetC = 6 * bit_depth_chroma_minus8		(7-2)

bit_depth_chroma_minus8 shall be in the range of 0 to 6, inclusive

adaptive_golomb_rice_parameter_offset_flag equal to 1 indicates that the adaptive Golomb-Rice parameter offset system is in use. adaptive_golomb_rice_parameter_offset_flag equal to 0 indicates that the Golomb-Rice parameter offset is always equal to 0. When not present, the value of adaptive_golomb_rice_parameter_offset_flag is inferred to be equal to 0.

log2_max_pic_order_cnt_lsb_minus4 specifies the value of the variable MaxPicOrderCntLsb that is used in the decoding process for picture order count as follows:

MaxPicOrderCntLsb = 2( log2_max_pic_order_cnt_lsb_minus4 + 4 )	(7-3)

The value of log2_max_pic_order_cnt_lsb_minus4 shall be in the range of 0 to 12, inclusive.

[ Note that existing profiles would be amended to include the clause “The syntax element adaptive_golomb_rice_parameter_offset_flag shall not be present in the bit stream.” ]

#### 7.4.7.1	General slice segment header semantics

slice_cr_qp_offset specifies a difference to be added to the value of pps_cr_qp_offset when determining the value of the Qp′Cr quantization parameter. The value of slice_cr_qp_offset shall be in the range of −12 to +12, inclusive. When slice_cr_qp_offset is not present, it is inferred to be equal to 0. The value of pps_cr_qp_offset + slice_cr_qp_offset shall be in the range of −12 to +12, inclusive.

golomb_rice_parameter_offset specifies the number of fixed bits to be added as an offset to the Golomb Rice Parameter used during the entropy coding process. When not present, the value of golomb_rice_parameter_offset is inferred to be equal to 0.

deblocking_filter_override_flag equal to 1 specifies that deblocking parameters are present in the slice header. deblocking_filter_override_flag equal to 0 specifies that deblocking parameters are not present in the slice header. When not present, the value of deblocking_filter_override_flag is inferred to be equal to 0.

#### 9.3.2.1	General

Outputs of this process are initialized CABAC internal variables.

The context variables of the arithmetic decoding engine and the variable cRiceParamOffset are initialized as follows:

  - If the coding tree unit is the first coding tree unit in a tile, the initialization process for context variables and cRiceParamOffset is invoked as specified in subclause .

  - Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrInRs % PicWidthInCtbsY is equal to 0, the following applies:

  - The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Error: Reference source not found) is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding tree block as follows:

( xNbT, yNbT ) = ( x0 + CtbSizeY, y0 − CtbSizeY )	(9-4)

  - The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ) as inputs, and the output is assigned to availableFlagT.

  - The synchronization process for context variables is invoked as follows:

  - If availableFlagT is equal to 1, the synchronization process for context variables and cRiceParamOffset as specified in subclause Error: Reference source not found is invoked with TableStateIdxWpp and TableMpsValWpp and cRiceParamOffsetSync and as inputs.

  - Otherwise, the initialization process for context variables and cRiceParamOffset is invoked as specified in subclause .

  - Otherwise, if CtbAddrInRs is equal to slice_segment_address and dependent_slice_segment_flag is equal to 1, the synchronization process for context variables and cRiceParamOffset as specified in subclause Error: Reference source not found is invoked with TableStateIdxDs and TableMpsValDs and cRiceParamOffsetSync as inputs.

  - Otherwise, the initialization process for context variables and cRiceParamOffset is invoked as specified in subclause .

The initialization process for the arithmetic decoding engine is invoked as specified in subclause Error: Reference source not found.

The whole initialization process for a syntax element synEl is illustrated in the flowchart of Error: Reference source not found.

#### 9.3.2.2	Initialization process for context variables and cRiceParamOffset

Outputs of this process are the initialized CABAC context variables indexed by ctxTable and ctxIdx and the variable cRiceParamOffset.

The variable cRiceParamOffset is set equal to 0.

…

#### 9.3.2.3	Storage process for context variables and cRiceParamOffset

Inputs to this process are the CABAC context variables indexed by ctxTable and ctxIdx and the variable cRiceParamOffset.

Outputs of this process are variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMps used in the initialization process of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found through , except end_of_slice_segment_flag, end_of_sub_stream_one_bit, and pcm_flag and cRiceParamOffsetSync containing the value of cRiceParamOffset.

For each context variable, the corresponding entries pStateIdx and valMps of tables tableStateSync and tableMPSSync are initialized to the corresponding pStateIdx and valMps.

The value cRiceParamOffsetSync is set equal to the value cRiceParamOffset.

…

#### 9.3.2.4	Synchronization process for context variables and cRiceParamOffset

Inputs to this process are variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMps used in the storage process of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found through , except end_of_slice_segment_flag, end_of_sub_stream_one_bit, and pcm_flag and the variable cRiceParamOffsetSync containing the value of cRiceParamOffset.

Outputs of this process are the initialized CABAC context variables indexed by ctxTable and ctxIdx and the variable cRiceParamOffset.

For each context variable, the corresponding context variables pStateIdx and valMps are initialized to the corresponding entries pStateIdx and valMps of tables tableStateSync and tableMPSSync.

The variable cRiceParamOffset is set equal to the value cRiceParamOffsetSync.

#### 9.3.3.9	Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ], the current sub-block scan index i, the Golomb-Rice parameter offset cRiceParamOffset and baseLevel.

Output of this process is the binarization of the syntax element.

The variable cRiceParamOffset is set equal to golomb_rice_parameter_offset.

The variables cLastAbsLevel and cLastRiceParam are derived as follows:

- If this process is invoked for the first time for the current sub-block scan index i, cLastAbsLevel and cLastRiceParam are is set equal to 0 and cLastRiceParam is set equal to cRiceParamOffset.

- Otherwise (this process is not invoked for the first time for the current sub-block scan index i), cLastAbsLevel and cLastRiceParam are set equal to the values of cAbsLevel and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element coeff_abs_level_remaining as specified in this subclause.

The variable cAbsLevel is set equal to baseLevel + coeff_abs_level_remaining[ n ].

The variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), (cRiceParamOffset + 4) )	(9-5)

The variable cMax is derived from cRiceParam as:

cMax = 4  <<  cRiceParam	(9-6)

The binarization of the syntax element coeff_abs_level_remaining[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of cu_qp_delta_abs, prefixVal, is derived as follows:

prefixVal = Min( cMax, coeff_abs_level_remaining[ n ] )	(9-7)

- The prefix bin string is specified by invoking the TR binarization process as specified in subclause Error: Reference source not found for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 4 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of cu_qp_delta_abs, suffixVal, is derived as follows:

suffixVal = coeff_abs_level_remaining[ n ] − cMax	(9-8)

- The suffix bin string is specified by invoking the eGk binarization process as specified in subclause Error: Reference source not found for suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.

BF is limited to ENTROPY_CODING_DYNAMIC_RANGE (as defined in JCTVC-N0188)

BF is limited to ENTROPY_CODING_DYNAMIC_RANGE (as defined in JCTVC-N0188)