| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 August 2013 | Document: JCTVC-N0191 |
| --- | --- |

| Title: | AHG 5 and 18: Profiles for Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
|  | T. Suzuki 2-10-1, Osaki, Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
|  | A. Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | +1-408-352-4725 ali.tabatabai@am.sony.com |
| Source: | Sony Europe Ltd and Sony Corporation |  |  |

_____________________________

# Abstract

A set of profiles are presented that propose to address the uses envisioned for HEVC Range Extensions. These proposed profiles are further refined, with the view to simplify encoder implementation and increase interoperability. This contribution proposes the following.

- JCT-VC (with its parent bodies) should specify all conformance points required by industry to avoid de-facto profiles defined outside this standardization body.

- Two categories of profiles should be defined.

  - Main xxx profiles: No new RExt tools are included (intended for consumer applications)

  - High xxx profiles: Include new RExt tools and higher input bit-depths

- Consideration of tools for non-camera captured content

- Requirement for a 4:2:0 12 bit profile

# Introduction

The initial idea of HEVC RExt profiles was proposed in JCTVC-M0094 and m29109 for Incheon meeting. To avoid de-facto profiles, JCT-VC and its parent bodies should consider specifying conformance points required by industry. Reducing the number of profiles is important, however, a balance between the number of profiles and desired conformance points should be considered. Two options were suggested in JCTVC-M0094, and the direction agreed during the Incheon meeting was Option 1 as shown in below.

Option 1: To specify required conformance points directly by JCT-VC (with its parent body). The required conformance points should be clarified first, and then specify the profiles for all required conformance points.

The HEVC international standard defines three profiles for coding up to 10-bit data. As the remit of Range Extensions includes providing support for higher bit depths and non-4:2:0 chroma formats, it necessarily follows that new profiles must be defined.

Following on from the recommendations in JCTVC-N0188, a set of profiles is proposed that is intended to suit many of the expected applications of HEVC. It is proposed that these profiles be discussed so that the community agrees on a final profile set for use in HEVC version 2.

# Profiles

It is preferable from an interoperability standpoint that, rather than define a small number of vague profiles and allow implementers to decide which features to support, all the expected use cases are covered by profiles defined in the standard. However, it would be cumbersome and counterproductive to define too many profiles as it would reduce interoperability through fragmentation.

As a starting point for discussion regarding profiles to be used in Range Extensions, the following options in Table 1 are proposed:

Table 1 : Initial proposed profiles for RExt

“Range Extensions Coding Tools” may include the entropy coding modifications described in JCTVC-N0189 and JCTVC-N0190, as well as the Sample-Adaptive Intra Prediction (JCTVC-M0056) introduced to the Range Extension tool set during the previous meeting.

The points of the above new profiles are,

- Main 10 4:0:0, Main 10 4:2:2 and Main 10 4:4:4 are for the chroma format extension of the current HEVC version. To simplify the implementation, no new tools are included (no LM chroma mode, additional lossless coding tools, etc.), and only changes associated with supporting the additional chroma sampling formats are included. These profiles are assumed to be used for consumer applications.
If the committee consider adding some or all of the new “Range Extensions Coding Tools” to these “main” profiles over those defined in Version 1, it is recommended that these new profiles be renamed to “Extended 10 4:x:x” – for the purposes of reducing fragmentation, it would be undesirable for there to be both main and extended sets of new profiles.

- High profiles support greater input bit depths, and would give the opportunity for new tools to be defined (which may improve coding efficiency or throughput or reduce complexity). New tools and higher internal precisions would be included in these profiles.

- Additional still picture profiles have not been defined at this time.

Note that, with the profiles proposed above, there are two methods of encoding 12-bit data (i.e. using High 12 and High 16 parameters). Unless there is a significant advantage to coding the 12-bit data with a High 16 encoder, it would be better for interoperability if the 12-bit data was coded in a manner that could be decoded by the High 12 decoder (i.e. in High 12 profile).

No significant advantage is observed when encoding 12-bit data using a 16-bit (higher internal precision) encoder, as shown in Figures 7 and 8 of JCTVC-N0188. Considering this, it may be better to either have the 16-bit profiles define a minimum bitDepth of 13, or to adjust the definition of the profiles as shown in Table 2 so that the internal accuracies are defined as dynamic variables based on bitDepth. Either method would result in fewer conformance points.

Table 2: Refined proposed profiles for RExt. Lower bit-depth video encoded in higher profiles is interoperable with lower profile decoders. Only one additional transform matrix precision is required, and encoders only need to support the minimum desired accuracy for their application.

The result of these alternative definitions is that 12-bit bit streams are identical whether they were encoded in High 12 profile or High 16 profile.

These definitions would also aid implementation of encoders wishing to implement (for example) just 14 bits, since they do not require the additional internal accuracy for the 16-bit operation. To keep decoder overheads low, only one transform matrix precision over that defined in the HEVC standard for the Main profiles is considered in this table; there should be little overhead in decoder implementation of using different MAX_TR_DYNAMIC_RANGEs and ENTROPY_CODING_DYNAMIC_RANGEs.

The proposed 14-bit transform matrix coefficients are specified in JCTVC-N0188.

## Tools for non-camera captured content

Tools for non-camera captured content (e.g. screen content) are under investigation by the JCT-VC. Those tools are designed to compress computer graphics, captions, etc, efficiently. However, they are not useful for camera applications. It is recommended that the complexity of such tools should be evaluated, and only if complexity is not high, such tools could be integrated into the profiles proposed in Table 2. If complexity is high, it is recommended that another profile is created for non-camera captured content. In the recent wireless transmission environments, natural video and computer generated contents are transmitted separately. In such environments, to create separate profiles for non-camera captured content could be a reasonable approach.

More detail should be discussed during the Vienna meeting.

## Requirement for 4:2:0 12 bit

Main10 profile in HEVC v.1 was defined for high end consumer applications, e.g. 4K video. Wide gamut colour space, e.g. ITU-R BT.2020, will be used for such application. To support wide gamut colour space, higher bit depth is necessary to reduce banding artefacts. For example, ITU-R BT.2020 supports up to 12 bit. There will be a demand to support 4:2:0 12 bit.

Therefore, this contribution proposes to define a 4:2:0 12 bit profile. One solution to define a 4:2:0 12 bit profile to be consistent within Table 2 is to define it as a High 4:2:0 12 bit profile.

JCT-VC (with its parent bodies) should consider this requirement and define appropriate profiles for 4:2:0 12 bit in the AMD for RExt.

## Minimal set of profiles

If a decoder is not obliged to decode every bit stream that it should be capable of decoding, then the following table of profiles may be applicable.

In this, the Extended 12 profile has been created which utilises the same transform as the Main, although the internal accuracy of the processing has been increased, to make the 32x32 and 16x16 point transforms useful. It is important to note that the Extended 12 system scales its internal accuracy depending on the bit depth, and therefore for systems that only wish to support 4:4:4 at 8 or 10 bit, the hardware can be reduced.

This extended profile could contain tools that are not in Main.

In addition, High 16 profiles have been created, which would have increased accuracy transforms and general processing. It could have new tools added to address issues regarding the compression of 14 and 16 bit data.

# Level definition

The maximum bitrate and CPB size should be higher than Main and Main 10 to support 4:2:2 and 4:4:4. These values are derived from the current level tables (Table A-1 and Table A-2) by multiplying the scale factors defined in the following table.

| Profile | CpbBrVclFactor | CpbBrNalFactor |
| --- | --- | --- |
| Main Main10 | 1 000 | 1 100 |
| Main 10 4:0:0 | 500 | 550 |
| Main 10 4:2:2 High 12 4:2:2 | 1 500 | 1 800 |
| Main 10 4:4:4 High 12 4:4:4 High 16 4:4:4 | 2 000 | 2 400 |
| High 16 4:0:0 | 700 | 800 |
| High 16 Intra 4:0:0 | 1 500 | 1 800 |
| High 12 Intra 4:4:4 High 16 Intra 4:4:4 | 4 000 | 4 800 |

Table 3: Scaling factors for RExt profiles

In the case of AVC, all intra coding was supposed to be included in High 4:2:2 and High 4:4:4 profiles. The maximum bitrate of the High 4:2:2 profile was inadvertently larger than that of the High 4:4:4 profile. However, this bitrate is too high for inter-frame coding (and all intra profiles were created later). The above table is to promote discussion of intra only profiles from the outset.

# Conclusion

A set of profiles has been presented that encompasses most if not all of the uses envisioned for HEVC Range Extensions. These proposed profiles have been further refined to simplify encoder implementation and increase interoperability.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Proposed changed to Draft Amendment Text

A.3.2 Main profile

…

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

- The value of the variable MatrixPrecision is equal to 6.

...

A.3.3 Main 10 profile

…

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

- The value of the variable MatrixPrecision is equal to 6.

...

A.3.4 Main Still Picture profile

…

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

- The value of the variable MatrixPrecision is equal to 6.

...

Example High Bit Accuracy Profile (Changes shown relative to Main 10 Profile):

A.3.5 High 16 4:4:4 profile

Bitstreams conforming to the Main 10High 16 4:4:4 profile shall obey the following constraints:

–	SPSs shall have chroma_format_idc equal to 1 only.

–	SPSs shall have bit_depth_luma_minus8 in the range of 0 to 2 0 to 8, inclusive.

–	SPSs shall have bit_depth_chroma_minus8 in the range of 0 to 2 0 to 8, inclusive.

–	CtbLog2SizeY shall be in the range of 4 to 6, inclusive.

–	When a PPS has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

–	When a PPS has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

–	The number of times read_bits( 1 ) is called in subclauses 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

–	The level constraints specified for the Main 10High 16 4:4:4 profile in subclause A.4 shall be fulfilled.

–	The value of the variable MatrixPrecision is equal to 14.

Conformance of a bitstream to the Main 10High 16 4:4:4 profile is indicated by general_profile_idc being equal to 24 or general_profile_compatibility_flag[24 ] being equal to 1.

Decoders conforming to the Main 10High 16 4:4:4 profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which all of the following conditions apply:

–	general_profile_compatibility_flag[ 1 ] is equal to 1 or general_profile_compatibility_flag[ 2 ] is equal to 1 or general_profile_compatibility_flag[ 4 ] is equal to 1.

–	general_level_idc represents a level lower than or equal to the specified level.

–	general_tier_flag represents a tier lower than or equal to the specified tier.

- A.4.1	General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag equal to 0 is considered to be a lower tier than the tier with general_tier_flag equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this annex:

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0 (i.e. the 0-th access unit).

- Let picture n be the coded picture or the corresponding decoded picture of access unit n.

- Let the variable CpbBrVclFactor be equal to 1000.

- Let the variable CpbBrNalFactor be equal to 1100.

The values of CpbBrVclFactor and CpbBrNalFactor are derived from the Table A-XX.

Table A-4: Specification of CpbBrVclFactor and CpbBrNalFactor

| Profile | CpbBrVclFactor | CpbBrNalFactor |
| --- | --- | --- |
| Main Main10 | 1 000 | 1 100 |
| Main 10 4:0:0 | 500 | 550 |
| Main 10 4:2:2 High 12 4:2:2 | 1 500 | 1 800 |
| Main 10 4:4:4 High 12 4:4:4 High 16 4:4:4 | 2 000 | 2 400 |
| High 16 4:0:0 | 700 | 800 |
| High 16 Intra 4:0:0 | 1 500 | 1 800 |
| High 12 Intra 4:4:4 High 16 Intra 4:4:4 | 4 000 | 4 800 |

Bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

- A.4.2  Profile-specific level limits for the Main, Main 10, High 12 422, High 16 444 and High 16 Intra 444 profiles

- …