| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 August 2013 | Document: JCTVC-N0192 |
| --- | --- |

| Title: | AHG 5: 32x32 Scaling List Derivation for Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

The document details the method currently used in HEVC Range Extensions to derive 32x32 scaling lists for chroma. An alternative derivation is presented using source scaling lists that are expected to be more closely correlated.

# Introduction

In the current HEVC international standard, scaling lists are defined for each combination of channel (Y/G, Cb/B, Cr/R), prediction type (intra, inter) and TU size (4x4, 8x8, 16x16, 32x32). Though there are 24 possible combinations, only 20 scaling lists are actually used as there are no 32x32 chroma TUs in 4:2:0-only coding.

For the 4:4:4 format, 32x32 chroma TUs may be used, necessitating that scaling lists be defined for them. Currently, to maintain compatibility with the HEVC standard header format, the 32x32 scaling lists for the chroma channels are each copied from the luma 32x32 scaling lists for the same prediction type. This is only the case in HM_RExt, whereas there are no provisions for the 32x32 chroma scaling lists in the current working draft.

Though this may give a usable approximation, there is little correlation between 32x32 luma TUs and 32x32 chroma TUs, especially in the YCbCr colour space where most of the information has been concentrated in the luma channel.

# Current Scaling List Derivation

In the bit stream, scaling lists for 8x8, 16x16 and 32x32 TUs are defined using 64 coefficients in diagonal scan order. Simple position-repeat is used to interpolate the 16x16 and 32x32 scaling lists to their defined size. 4x4 scaling lists are defined using 16 coefficients in diagonal scan order. All coefficients are contained within the scaling_list_data syntax element, which is attached to the picture parameter set.

For the 4:4:4 format, scaling lists for 32x32 chroma are derived as copies of equivalent 32x32 luma scaling lists (i.e. Cb/B intra and Cr/R intra copy Y/G intra, Cb/B inter and Cr/R inter copy Y/G inter). This derivation is shown in Figure 1.

Figure 1 - Scaling list set in RExt3.0 showing derivation of 32x32 chroma scaling lists from 32x32 luma scaling lists

# Alternative Scaling List Derivations

Ideally, the additional chroma scaling lists would be sent in the stream along with the rest of the scaling lists whenever the 4:4:4 chroma format is used. However, scaling lists are attached to the picture parameter set, which is not allowed to be dependent on any information in the sequence parameter set (which contains the chroma format). Additional syntax could be added via the PPS extension data mechanism to signal that extra scaling lists are included, but it is not clear if these scaling lists yield sufficient benefit to justify this overhead.

Alternatively, derived scaling lists could be used as currently, but derived from the equivalent 16x16 chroma scaling lists rather than luma 32x32 lists. Different-sized TUs on the same channel are expected to be more closely correlated than different-channel TUs of the same size. This alternative derivation is shown in Figure 2.

Figure 2 - Proposed scaling list set showing derivation of 32x32 chroma scaling lists from 16x16 chroma scaling lists

# Conclusion

32x32 chroma scaling lists are used in RExt3.0, but the current derivation of them may not be optimal. Alternative methods of defining 32x32 chroma scaling lists have been presented, and it is proposed that the committee consider which of the options should be supported.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Working Draft Text

Note that the current Working Draft Text does not implement any handling method for Chroma 32x32 scaling lists.

7.4.5	Scaling list data semantics

scaling_list_pred_mode_flag[ sizeId ][ matrixId ] equal to 0 specifies that the values of the scaling list are the same as the values of a reference scaling list. The reference scaling list is specified by scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ]. scaling_list_pred_mode_flag[ sizeId ][ matrixId ] equal to 1 specifies that the values of the scaling list are explicitly signalled.

scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] specifies the reference scaling list used to derive ScalingList[ sizeId ][ matrixId ] as follows:

–	If scaling_list_pred_matrix_id_delta is equal to 0, the scaling list is inferred from the default scaling list ScalingList[ sizeId ][ matrixId ][ i ] as specified in Table 7-3 and Table 7-4 for i = 0..Min( 64, ( 1  <<  ( 4 + ( sizeId  <<  1 ) ) ) ).

–	Otherwise, the scaling list is inferred from the reference scaling list as follows:

refMatrixId = matrixId − scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ]	(7-1)

ScalingList[ sizeId ][ matrixId ][ i ] = ScalingList[ sizeId ][ refMatrixId ][ i ]
with i =0..Min( 64, ( 1  <<  ( 4 + ( sizeId  <<  1 ) ) ) )	(7-2)

The value of scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] shall be in the range of 0 to matrixId, inclusive.

Table 7-1 – Specification of sizeId

| - Size of quantization matrix | - sizeId |
| --- | --- |
| - 4x4 | - 0 |
| - 8x8 | - 1 |
| - 16x16 | - 2 |
| - 32x32 | - 3 |

Table 7-2 – Specification of matrixId according to sizeId, prediction mode and colour component

| - sizeId | - CuPredMode | - cIdx (colour component) | - matrixId |
| --- | --- | --- | --- |
| - 0, 1, 2, 3 | - MODE_INTRA | - 0 (Y) | - 0 |
| - 0, 1, 2, 3 | - MODE_INTRA | - 1 (Cb) | - 1 |
| - 0, 1, 2, 3 | - MODE_INTRA | - 2 (Cr) | - 2 |
| - 0, 1, 2, 3 | - MODE_INTER | - 0 (Y) | - 3 |
| - 0, 1, 2, 3 | - MODE_INTER | - 1 (Cb) | - 4 |
| - 0, 1, 2, 3 | - MODE_INTER | - 2 (Cr) | - 5 |
| - 3 | - MODE_INTRA | - 0 (Y) | - 0 |
| - 3 | - MODE_INTER | - 0 (Y) | - 1 |

scaling_list_dc_coef_minus8[ sizeId − 2 ][ matrixId ] plus 8 specifies the DC value of the scaling list for 16x16 size when sizeId is equal to 2 and specifies the DC value of the scaling list for 32x32 size when sizeId is equal to 3. The value of scaling_list_dc_coef_minus8[ sizeId − 2 ][ matrixId ] shall be in the range of −7 to 247, inclusive. When scaling_list_dc_coef_minus8 is not present, it is inferred to be equal to 8.

scaling_list_delta_coef specifies the difference between the current matrix coefficient ScalingList[ sizeId ][ matrixId ][ i ] and the previous matrix coefficient ScalingList[ sizeId ][ matrixId ][ i − 1 ], when scaling_list_pred_mode_flag[ sizeId ][ matrixId ] is equal to 1. The value of scaling_list_delta_coef shall be in the range of −128 to 127, inclusive. The value of ScalingList[ sizeId ][ matrixId ][ i ] shall be greater than 0.

Table 7-3 – Specification of default values of ScalingList[ 0 ][ matrixId ][ i ] with i = 0..15

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[ 0 ][ 0..5 ][ i ] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 |

Table 7-4 – Specification of default values of ScalingList[ 1..3 ][ matrixId ][ i ] with i = 0..63

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[ 1..23 ][ 0..2 ][ i ] - ScalingList[ 3 ][ 0 ][ i ] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 17 | - 16 | - 17 | - 16 | - 17 | - 18 |
| - ScalingList[ 1.. 23 ][ 3..5 ][ i ] - ScalingList[ 3 ][ 1 ][ i ] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 17 | - 17 | - 18 |
| - i – 16 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[ 1.. 23 ][ 0..2 ][ i ] - ScalingList[ 3 ][ 0 ][ i ] | - 17 | - 18 | - 18 | - 17 | - 18 | - 21 | - 19 | - 20 | - 21 | - 20 | - 19 | - 21 | - 24 | - 22 | - 22 | - 24 |
| - ScalingList[ 1.. 23 ][ 3..5 ][ i ] - ScalingList[ 3 ][ 1 ][ i ] | - 18 | - 18 | - 18 | - 18 | - 18 | - 20 | - 20 | - 20 | - 20 | - 20 | - 20 | - 20 | - 24 | - 24 | - 24 | - 24 |
| - i − 32 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[ 1.. 23 ][ 0..2 ][ i ] - ScalingList[ 3 ][ 0 ][ i ] | - 24 | - 22 | - 22 | - 24 | - 25 | - 25 | - 27 | - 30 | - 27 | - 25 | - 25 | - 29 | - 31 | - 35 | - 35 | - 31 |
| - ScalingList[ 1.. 23 ][ 3..5 ][ i ] - ScalingList[ 3 ][ 1 ][ i ] | - 24 | - 24 | - 24 | - 24 | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 | - 28 | - 28 | - 28 | - 28 | - 28 |
| - i − 48 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[ 1.. 23 ][ 0..2 ][ i ] - ScalingList[ 3 ][ 0 ][ i ] | - 29 | - 36 | - 41 | - 44 | - 41 | - 36 | - 47 | - 54 | - 54 | - 47 | - 65 | - 70 | - 65 | - 88 | - 88 | - 115 |
| - ScalingList[ 1.. 23 ][ 3..5 ][ i ] - ScalingList[ 3 ][ 1 ][ i ] | - 28 | - 33 | - 33 | - 33 | - 33 | - 33 | - 41 | - 41 | - 41 | - 41 | - 54 | - 54 | - 54 | - 71 | - 71 | - 91 |

Table 7-X – Specification of sourceMatrixId and sourceSizeId according to matrixId for sizeId=3

| - matrixId | - sourceMatrixId | - sourceSizeId |
| --- | --- | --- |
| - 0 | - 0 | - 3 |
| - 1 | - 1 | - 2 |
| - 2 | - 2 | - 2 |
| - 3 | - 1 | - 3 |
| - 4 | - 4 | - 2 |
| - 5 | - 5 | - 2 |

The four-dimensional array ScalingFactor[ sizeId ][ matrixId ][ x ][ y ], with x, y = 0..( 1  <<  ( 2 + sizeId ) ) − 1, specifies the array of scaling factors according to the variables sizeId specified in Table 7-1 and matrixId specified in Table 7-2.

The elements of the quantization matrix of size 4x4, ScalingFactor[ 0 ][ matrixId ][ ][ ], are derived as follows:

ScalingFactor[ 0 ][ matrixId ][ x ][ y ] = ScalingList[ 0 ][ matrixId ][ i ]	(7-3)
with i = 0..15, matrixId = 0..5, x = ScanOrder[ 2 ][ 0 ][ i ][ 0 ], and y = ScanOrder[ 2 ][ 0 ][ i ][ 1 ]

The elements of the quantization matrix of size 8x8, ScalingFactor[ 1 ][ matrixId ][ ][ ], are derived as follows:

ScalingFactor[ 1 ][ matrixId ][ x ][ y ] = ScalingList[ 1 ][ matrixId ][ i ]	(7-4)
with i = 0..63, matrixId = 0..5, x = ScanOrder[ 3 ][ 0 ][ i ][ 0 ], and y = ScanOrder[ 3 ][ 0 ][ i ][ 1 ]

The elements of the quantization matrix of size 16x16, ScalingFactor[ 2 ][ matrixId ][ ][ ], are derived as follows:

ScalingFactor[ 2 ][ matrixId ][ x * 2 + k ][ y * 2 + j ] = ScalingList[ 2 ][ matrixId ][ i ]	(7-5)
with i = 0..63, j = 0..1, k = 0..1, matrixId = 0..5, x = ScanOrder[ 3 ][ 0 ][ i ][ 0 ],
and y = ScanOrder[ 3 ][ 0 ][ i ][ 1 ]

ScalingFactor[ 2 ][ matrixId ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 0 ][ matrixId ] + 8	(7-6)
with matrixId = 0..5

The elements of the quantization matrix of size 32x32, ScalingFactor[ 3 ][ matrixId ][ ][ ], are derived as follows:

ScalingFactor[ 3 ][ matrixId ][ x * 4 + k ][ y * 4 + j ] = ScalingList[ sourceSizeId3 ][ sourceMmatrixId ][ i ]	(7-7)
with i = 0..63, j = 0..3, k = 0..3, matrixId = 0.. 51, x = ScanOrder[ 3 ][ 0 ][ i ][ 0 ],
and y = ScanOrder[ 3 ][ 0 ][ i ][ 1 ] and with the values of sourceMatrixId and sourceSizeId according to the Table 7-X .

ScalingFactor[ 3 ][ matrixId ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ sourceSizeId-21 ][ sourceMmatrixId ] + 8	(7-8)
with matrixId = 0..51 and with the values of sourceMatrixId and sourceSizeId according to the Table 7-X .