| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0193 |
| --- | --- |

| Title: | SCE1: cross-check for Test 2.2 on phase compensation by signaling filter coefficients at PPS with sample shift |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858.210.4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document is the cross-check report on Arris’ proposal JCTVC-N0078 on phase compensation by signaling filter coefficients at PPS with sample shift. JCTVC-N0078 proposes to use adaptive upsampling filters, which are transmitted in PPS, to compensate the misalignment between BL and EL sequences. The number of adaptive upsampling filters, which represents the number of phases, is also signaled in PPS. The implementations and experimental results provided by the proponent are confirmed in this document.

# Introduction

JCTVC-N0078 proposes to use adaptive upsampling filters, which are transmitted in PPS, to compensate the misalignment between BL and EL sequences. The number of adaptive upsampling filters, which represents the number of phases and doesn’t have to be a power of 2, is also signaled in PPS. Negative phase offset compensation is signaled by a “pixel-shift” flag per filter.

We checked the implementation of the algorithm, the implementation of complexity assessment modules, the R-D performance, and the average complexity results obtained by decoding, and can confirm the implementation in software matches the description in the proposal JCTVC-M263, and implementation of the complexity assessment module correctly reflects the complexity of the proposed method, and the performance and complexity results match what are provided by the proponent.

# Experimental Results

The cross-check experiments were conducted on 64-bit Linux cluster under SHVC CTC and the additional test conditions defined by SCE1. The results match what is provided by the proponent.

TABLE 1 BD-rate reduction compared with SHM-2.0

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -13.8% | -15.9% | -16.5% |  |  |  |
| Class B | -7.6% | -8.8% | -8.7% | -8.4% | -10.2% | -10.2% |
| Overall (Test vs Ref) | -9.4% | -10.9% | -10.9% | -8.4% | -10.2% | -10.2% |
| Overall (Test vs single layer) | 13.2% | 15.1% | 14.7% | 10.8% | 9.9% | 9.5% |
| Overall (Ref vs single layer) | 25.1% | 29.3% | 28.9% | 21.2% | 22.5% | 22.0% |
| EL only (Test vs Ref) | -17.0% | -18.4% | -18.5% | -22.7% | -24.5% | -24.6% |
| Enc Time[%] | 107.8% |  |  | 111.4% |  |  |
| Dec Time[%] | 92.3% |  |  | 94.3% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -10.0% | -5.5% | -6.8% |  |  |  |
| Class B | -5.2% | -3.5% | -2.8% | -6.2% | -5.1% | -4.6% |
| Overall (Test vs Ref) | -6.6% | -4.1% | -3.9% | -6.2% | -5.1% | -4.6% |
| Overall (Test vs single layer) | 19.6% | 33.6% | 32.2% | 16.3% | 29.1% | 29.2% |
| Overall (Ref vs single layer) | 28.3% | 39.3% | 37.8% | 24.1% | 36.1% | 35.5% |
| EL only (Test vs Ref) | -12.1% | -9.6% | -9.4% | -16.0% | -14.9% | -14.4% |
| Enc Time[%] | 84.0% |  |  | 84.3% |  |  |
| Dec Time[%] | 76.9% |  |  | 73.3% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -7.5% | -2.7% | -3.8% |  |  |  |
| Class B | -3.8% | -2.7% | -2.0% | -6.1% | -4.9% | -4.4% |
| Overall (Test vs Ref) | -4.8% | -2.7% | -2.5% | -6.1% | -4.9% | -4.4% |
| Overall (Test vs single layer) | 28.9% | 39.3% | 40.3% | 24.9% | 32.9% | 35.6% |
| Overall (Ref vs single layer) | 35.7% | 43.2% | 44.2% | 33.0% | 39.8% | 42.0% |
| EL only (Test vs Ref) | -8.8% | -6.7% | -6.6% | -15.1% | -14.0% | -13.6% |
| Enc Time[%] | 90.0% |  |  | 91.2% |  |  |
| Dec Time[%] | 85.9% |  |  | 84.1% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -8.8% | -3.1% | -4.6% |  |  |  |
| Class B | -4.6% | -3.3% | -2.4% | -6.9% | -5.5% | -4.7% |
| Overall (Test vs Ref) | -5.8% | -3.2% | -3.1% | -6.9% | -5.5% | -4.7% |
| Overall (Test vs single layer) | 27.1% | 38.2% | 39.6% | 22.7% | 32.3% | 35.4% |
| Overall (Ref vs single layer) | 35.2% | 43.0% | 44.2% | 31.9% | 39.9% | 42.2% |
| EL only (Test vs Ref) | -10.1% | -7.6% | -7.5% | -16.7% | -15.2% | -14.6% |
| Enc Time[%] | 95.5% |  |  | 93.3% |  |  |
| Dec Time[%] | 86.3% |  |  | 83.9% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

TABLE II Average memory access with Picture-based implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 99% | 95% | 96% |  |  |  |
| Class B | 100% | 98% | 99% | 100% | 98% | 98% |
| Overall | 99% | 97% | 97% | 100% | 98% | 98% |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 96% | 95% | 95% |  |  |  |
| Class B | 99% | 98% | 98% | 98% | 97% | 97% |
| Overall | 97% | 97% | 97% | 98% | 97% | 97% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 95% | 94% | 94% |  |  |  |
| Class B | 98% | 98% | 98% | 97% | 97% | 97% |
| Overall | 97% | 96% | 96% | 97% | 97% | 97% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 94% | 94% | 94% |  |  |  |
| Class B | 98% | 98% | 98% | 97% | 97% | 97% |
| Overall | 96% | 96% | 96% | 97% | 97% | 97% |

TABLE III Average memory access with PU-based implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 101% | 95% | 96% |  |  |  |
| Class B | 103% | 101% | 102% | 99% | 97% | 97% |
| Overall | 102% | 98% | 99% | 99% | 97% | 97% |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 96% | 95% | 95% |  |  |  |
| Class B | 99% | 98% | 98% | 97% | 97% | 97% |
| Overall | 97% | 97% | 97% | 97% | 97% | 97% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 96% | 95% | 95% |  |  |  |
| Class B | 99% | 98% | 98% | 98% | 97% | 97% |
| Overall | 97% | 96% | 96% | 98% | 97% | 97% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 94% | 93% | 93% |  |  |  |
| Class B | 98% | 98% | 98% | 96% | 96% | 96% |
| Overall | 96% | 96% | 95% | 96% | 96% | 96% |