| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0194 |
| --- | --- |

| Title: | SCE3: Cross-check of Test 3.1 on switchable alternative inter-layer filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858.210.4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document is the cross-check report on Qualcomm and Samsung’s joint proposal JCTVC-N0151 on switchable alternative inter-layer filter. JCTVC-N0151 proposes to apply a low-pass filter [-1, 3, 12, 3, -1] for upsampling the integer pixels in BL picture, which are directly copied for inter-layer prediction in the current SHVC text. The additional filter can be enabled and disabled at the picture level or the PU level, in the proposal called picture-switchable and PU-switchable methods, respectively. The implementations and experimental results provided by the proponent are confirmed in this document.

# Introduction

JCTVC-N0151 proposes to apply a low-pass filter [-1, 3, 12, 3, -1] for upsampling the integer pixels in BL picture, which are directly copied for inter-layer prediction in the current SHVC text. The additional filter can be enabled and disabled at the picture level or the PU level, in the proposal called picture-switchable and PU-switchable methods, respectively. The PU-switchable method can be realized as a picture-based implementation, with which the conventional and filtered ILR pictures are added into DPB with respective ref_idx and used for inter-layer prediction.

We checked the implementation of the algorithm, the implementation of complexity assessment modules, the R-D performance, and the average complexity results obtained by decoding, and can confirm the implementation in software matches the description in the description, and implementation of the complexity assessment module correctly reflects the complexity of the proposed method, and the performance and complexity results match what are provided by the proponent.

# Experimental Results

The cross-check experiments were conducted on 64-bit Linux cluster under SHVC CTC and the additional test conditions defined by SCE3. JCTVC-N0151 also includes other variants of the aforementioned two methods, and the package includes 15 spreadsheets. We are responsible for cross-checking the following 6 spreadsheets and the results match what are provided by the proponent.

JCTVC-N0151_SCE3_T31_PU.xls

JCTVC-N0151_SCE3_T31_Pic.xls

SCE3_Test31_PU_Complexity_vs_SHM2.0_(Pic_based_processing)_r1.xls

SCE3_Test31_Pic_Complexity_vs_SHM2.0_(PU_based_processing)_r1.xls

SCE3_Test31_Pic_Complexity_vs_SHM2.0_(Pic_based_processing)_r1.xls

SCE3_Test31_PU_Complexity_vs_SHM2.0_(PU_based_processing)_r1.xls

TABLE 1 BD-rate reduction by picture-switchable method compared with SHM-2.0

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs Ref) | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs single layer) | 12.7% | 14.9% | 14.6% | 10.4% | 9.8% | 9.3% |  |  |  |
| Overall (Ref vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| EL only (Test vs Ref) | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 111.1% |  |  | 99.9% |  |  |  |  |  |
| Dec Time[%] | 107.5% |  |  | 98.4% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | -3.6% | -0.8% | -0.8% |
| Class B | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.4% | -0.1% | 0.0% |
| Overall (Test vs Ref) | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -1.3% | -0.3% | -0.2% |
| Overall (Test vs single layer) | 19.1% | 33.3% | 31.9% | 16.1% | 28.8% | 29.1% | 12.9% | 31.7% | 33.8% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -2.0% | -0.9% | -0.8% |
| Enc Time[%] | 110.6% |  |  | 115.3% |  |  | 126.8% |  |  |
| Dec Time[%] | 109.9% |  |  | 115.2% |  |  | 145.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | -3.3% | -0.8% | -0.9% |
| Class B | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -1.1% | -0.2% | -0.3% |
| Overall (Test vs single layer) | 28.4% | 38.9% | 39.6% | 24.6% | 33.0% | 35.9% | 22.9% | 34.4% | 39.1% |
| Overall (Ref vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| EL only (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -1.6% | -0.7% | -0.7% |
| Enc Time[%] | 93.5% |  |  | 104.5% |  |  | 102.8% |  |  |
| Dec Time[%] | 96.1% |  |  | 96.1% |  |  | 123.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | -4.9% | -1.0% | -0.8% |
| Class B | -0.3% | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | -0.9% | 0.0% | 0.1% |
| Overall (Test vs Ref) | -0.3% | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | -2.0% | -0.3% | -0.2% |
| Overall (Test vs single layer) | 26.3% | 38.0% | 39.1% | 22.6% | 32.8% | 35.5% | 20.9% | 34.2% | 39.2% |
| Overall (Ref vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | -0.3% | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -2.8% | -1.0% | -0.9% |
| Enc Time[%] | 98.2% |  |  | 102.5% |  |  | 106.1% |  |  |
| Dec Time[%] | 99.9% |  |  | 101.7% |  |  | 134.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

TABLE II BD-rate reduction by PU-switchable method compared with SHM-2.0

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.1% | 0.2% | 0.2% |  |  |  |  |  |  |
| Class B | -0.2% | 0.2% | 0.2% | -0.1% | 0.1% | 0.2% |  |  |  |
| Overall (Test vs Ref) | -0.2% | 0.2% | 0.2% | -0.1% | 0.1% | 0.2% |  |  |  |
| Overall (Test vs single layer) | 12.5% | 15.1% | 14.8% | 10.4% | 9.9% | 9.5% |  |  |  |
| Overall (Ref vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| EL only (Test vs Ref) | 0.0% | 0.4% | 0.4% | 0.3% | 0.6% | 0.7% |  |  |  |
| Enc Time[%] | 124.6% |  |  | 121.6% |  |  |  |  |  |
| Dec Time[%] | 124.5% |  |  | 124.7% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.2% |  |  |  | -3.7% | -0.7% | -0.6% |
| Class B | -0.2% | 0.2% | 0.2% | -0.1% | 0.2% | 0.2% | -1.0% | 0.1% | 0.4% |
| Overall (Test vs Ref) | -0.2% | 0.2% | 0.2% | -0.1% | 0.2% | 0.2% | -1.7% | -0.1% | 0.1% |
| Overall (Test vs single layer) | 19.0% | 33.5% | 32.3% | 16.1% | 29.1% | 29.5% | 12.4% | 31.9% | 34.2% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.1% | 0.3% | 0.3% | 0.3% | 0.6% | 0.6% | -2.5% | -0.8% | -0.5% |
| Enc Time[%] | 115.8% |  |  | 122.7% |  |  | 127.7% |  |  |
| Dec Time[%] | 139.9% |  |  | 147.4% |  |  | 166.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.1% | 0.1% |  |  |  | -3.2% | -0.8% | -0.8% |
| Class B | -0.2% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | -0.7% | -0.1% | 0.1% |
| Overall (Test vs Ref) | -0.2% | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | -1.4% | -0.3% | -0.2% |
| Overall (Test vs single layer) | 28.3% | 39.1% | 39.8% | 24.7% | 33.2% | 35.9% | 22.5% | 34.3% | 39.2% |
| Overall (Ref vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| EL only (Test vs Ref) | -0.1% | 0.1% | 0.1% | 0.2% | 0.3% | 0.2% | -2.0% | -0.8% | -0.7% |
| Enc Time[%] | 105.9% |  |  | 106.6% |  |  | 107.9% |  |  |
| Dec Time[%] | 129.7% |  |  | 135.4% |  |  | 147.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.0% | 0.1% |  |  |  | -4.8% | -1.1% | -0.8% |
| Class B | -0.4% | 0.0% | -0.2% | -0.3% | 0.0% | 0.1% | -1.4% | 0.1% | 0.4% |
| Overall (Test vs Ref) | -0.4% | 0.0% | -0.1% | -0.3% | 0.0% | 0.1% | -2.4% | -0.2% | 0.0% |
| Overall (Test vs single layer) | 26.1% | 38.0% | 39.0% | 22.5% | 32.8% | 35.7% | 20.4% | 34.3% | 39.5% |
| Overall (Ref vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | -0.4% | 0.0% | -0.1% | -0.1% | 0.2% | 0.2% | -3.3% | -1.1% | -0.8% |
| Enc Time[%] | 111.3% |  |  | 112.3% |  |  | 109.3% |  |  |
| Dec Time[%] | 126.8% |  |  | 128.1% |  |  | 139.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

TABLE III Average memory access of picture-switchable method with Picture-based implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |  |  |  |
| Class A | 100% | 100% | 100% |  |  |  |  |  |  |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% |  |  |  |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 100% | 100% | 100% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 101% | 101% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 100% | 99% | 100% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 100% | 100% | 100% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 101% | 101% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

TABLE IV Average memory access of picture-switchable method with PU-based implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |  |  |  |
| Class A | 100% | 100% | 100% |  |  |  |  |  |  |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% |  |  |  |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 100% | 100% | 100% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 101% | 101% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 100% | 99% | 100% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 100% | 100% | 100% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 101% | 101% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

TABLE V Average memory access of PU-switchable method with Picture-based implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |  |  |  |
| Class A | 150% | 145% | 146% |  |  |  |  |  |  |
| Class B | 150% | 145% | 146% | 151% | 145% | 147% |  |  |  |
| Overall | 150% | 145% | 146% | 151% | 145% | 147% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 130% | 130% | 132% |  |  |  | 135% | 134% | 135% |
| Class B | 129% | 130% | 132% | 132% | 132% | 134% | 135% | 135% | 136% |
| Overall | 129% | 130% | 132% | 132% | 132% | 134% | 135% | 134% | 136% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 136% | 135% | 136% |  |  |  | 141% | 138% | 139% |
| Class B | 135% | 135% | 137% | 139% | 138% | 140% | 141% | 140% | 141% |
| Overall | 136% | 135% | 136% | 139% | 138% | 140% | 141% | 139% | 140% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 132% | 132% | 133% |  |  |  | 136% | 135% | 136% |
| Class B | 130% | 131% | 132% | 134% | 134% | 136% | 136% | 136% | 137% |
| Overall | 131% | 131% | 133% | 134% | 134% | 136% | 136% | 135% | 136% |

TABLE VI Average memory access of PU-switchable method with PU-based implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |  |  |  |
| Class A | 98% | 96% | 97% |  |  |  |  |  |  |
| Class B | 98% | 97% | 97% | 97% | 95% | 96% |  |  |  |
| Overall | 98% | 96% | 97% | 97% | 95% | 96% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 99% | 98% | 98% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 99% | 99% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 99% | 99% | 99% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 99% | 99% | 100% |  |  |  | 101% | 99% | 99% |
| Class B | 99% | 100% | 100% | 99% | 99% | 99% | 101% | 100% | 100% |
| Overall | 99% | 100% | 100% | 99% | 99% | 99% | 101% | 99% | 100% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 98% | 98% | 98% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 99% | 99% | 99% |