Text for N0195 - Proposal 2

The highlighted text below shows syntax and semantics changes for adoption - N0195 Proposal 2 compared to JCTVC-M1008 and JCTVC-D1004.

Text for adoptions – N0195 Proposal 1 and N0195 Proposal 5 are in the contribution document.

VPS Extension Syntax and Semantics :

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| ilp_slice_signaling_enabled_flag | u(1) |
| … |  |

ilp_slice_signaling_enabled_flag equal to 1 specifies that inter_layer_pred_enabled_flag, num_inter_layer_ref_pics_minus1, inter_layer_pred_layer_idc[ i ] are present in the slice segment headers. ilp_slice_signaling_enabled_flag equal to 0 specifies that inter_layer_pred_enabled_flag, num_inter_layer_ref_pics_minus1, inter_layer_pred_layer_idc[ i ] are not present in the slice segment header.

- G.7.3.6 Slice segment header syntax

- G.7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| … |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if(ilp_slice_signaling_enabled_flag) { |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| … |  |
| byte_alignment( ) |  |
| } |  |

…

- G.7.4.7 Slice segment header semantics

G.7.4.7.1 General slice segment header semantics

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture. When not present, if ilp_slice_signaling_enabled_flag is equal to 1 and NumDirectRefLayers[ nuh_layer_id ] is equal to 0, the value of inter_layer_pred_enabled_flag is inferred to be equal to 0 Otherwise the value of inter_layer_pred_enabled_flag is inferred to be equal to 1.

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictures that may be used in decoding of the current picture for inter-layer prediction. If ilp_slice_signaling_enabled_flag is equal to 1 and inter_layer_pred_enabled_flag is inferred to be equal to 1 the value of num_inter_layer_ref_pics_minus1 is inferred to be equal to NumDirectRefLayers[ nuh_layer_id ] -1. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0  | |  !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element inter_layer_pred_layer_idc[ i ] is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.  When not present, if ilp_slice_signaling_enabled_flag is equal to 1 and inter_layer_pred_enabled_flag is inferred to be equal to 1 the value of inter_layer_pred_layer_idc[i] is inferred as follows:

for( i = 0; i < NumDirectRefLayers[ nuh_layer_id ]; i++)

{

inter_layer_pred_layer_idc[ i ]  is inferred to be equal to i

}

Otherwise the value of inter_layer_pred_layer_idc[ i ] is inferred to be equal to 0.

When i is greater than 0, inter_layer_pred_layer_idc[ i ] shall be greater than inter_layer_pred_layer_idc[ i − 1 ]. [Ed. (JB): This restriction was imposed to make behavior match the earlier design of increasing entries, but is an area noted for future study.]

The variables RefPicLayerId[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, NumActiveMotionPredRefLayers, and ActiveMotionPredRefLayerId[ j ] for each value of j in the range of 0 to NumActiveMotionPredRefLayers  − 1, inclusive, are derived as follows:

for( i = 0, j = 0; i < NumActiveRefLayerPics; i++)
	RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]
	if( MotionPredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] )
		ActiveMotionPredRefLayerId[ j++ ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]
}
NumActiveMotionPredRefLayers = j

All slices of a picture shall have the same value of inter_layer_pred_layer_idc[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, either of the following two conditions shall be true:

–	The value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] is greater than TemporalId.

–	The values of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] and TemporalId are both equal to 0 and the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is an IRAP picture.

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, the value of SamplePredEnabledFlag[ nuh_layer_id ] [ RefPicLayerId[ i ] ] or MotionPredEnabledFlag[ nuh_layer_id ] [ RefPicLayerId[ i ] ] shall be equal to 1.