| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0196 WG11 Number: m29827 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0079 |
| --- | --- |

| Title: | On Sub-layer Non-reference Pictures Indication for Inter-layer Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes syntax and semantics in VPS for indicating that sub-layer non-reference pictures belonging to a layer are not used for inter-layer prediction. A bitstream conformance constraint is proposed based on this indication which gets used in the decoding process for inter-layer reference picture set. Additionally a change to the marking process for sub-layer non-reference pictures not needed for inter-layer prediction is proposed.

In r1 revision some changes to the proposed specification text are made with no change to the proposed design.

# Introduction

In SHVC [1] and MV-HEVC[2]working drafts inter-layer prediction indication in the form of max_tid_il_ref_pics_plus1[ i ] is signalled in VPS extension. max_tid_il_ref_pics_plus1[ i ] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction. max_tid_il_ref_pics_plus1[ i ] greater than 0 specifies that within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ] – 1 are not used as reference for inter-layer prediction. When not present, max_tid_il_ref_pics_plus1[ i ] is unspecified.

This document proposes syntax and semantics in VPS for indicating that sub-layer non-reference pictures belonging to a layer are not used for inter-layer prediction.

Two syntax variants are proposed.

- Variant 1: New syntax elements no_sub_layer_non_ref_inter_layer_sample_pred_tid_flag[ i ] are signalled in VPS for each layer indicating if within the CVS sub-layer non-reference pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId equal to max_tid_il_ref_pics_plus1[i] -1 are not used as reference for inter-layer prediction.

- Variant 2: New syntax elements no_sub_layer_non_ref_inter_layer_sample_pred_tid_flag[ i ] [ j ]  are signalled in VPS for each layer for each temporal sub-layer j indicating if within the CVS sub-layer non-reference pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId equal to j are not used as reference for inter-layer prediction.

For each of the syntax variants:

- A bitstream conformance constraint is proposed based on this indication which gets used in the decoding process for inter-layer reference picture set.

- Additionally a change to the marking process for sub-layer non-reference pictures not needed for inter-layer prediction is proposed. It is asserted that this change in the marking process approach has the advantage that additional sub-layer non-reference pictures of target layers could be marked “unused for reference”. Thus DPB size / memory could be reduced.

One of the motivations for the proposed signalling is to enable low complexity decoding for multi-loop decoding.

  - It adds a further indication on top of max_tid_il_ref_pics_plus1[ i ] – 1 related indication by dividing pictures into sub-layer reference pictures and sub-layer non-reference pictures and indicating if sub-layer non-reference pictures are not used for inter-layer prediction. This indication is useful for the highest temporal sub-layer because a sub-layer non-reference picture contains samples that cannot be used for inter prediction in the decoding process of subsequent pictures of the same sub-layer in decoding order. Thus these pictures could be discarded thus helping reduce the complexity of decoding and motion compensation that needs to be preformed for reference layer pictures when decoding a target layer.

  - It is helpful in the case when an encoder is not using nuh_temporal_id_plus1 assignment and thus has assigned a value of nuh_temporal_id_plus1 equal to 1 for all the pictures. In this case this indication can still be used to indicate support for low complexity decoding by providing an indication if sub-layer non-reference pictures belonging to TemporalId 0 are not used for inter-layer prediction.

# Proposal

We propose following 2 alternative variant syntax and semantics for indicating that sub-layer non-reference pictures belonging to a layer are not used for inter-layer prediction. For each variant, syntax and semantics are provided for video parameter set (VPS).

Proposed changes are highlighted below compared to JCTVC-M1008 [1].

## Variant 1

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| … | u(1) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| if(max_tid_il_ref_pics_plus1[ i ]!=0) |  |
| no_sub_layer_non_ref_inter_layer_sample_pred_tid_flag[ i ] | u(1) |
| … |  |

no_sub_layer_non_ref_inter_layer_sample_pred_tid_flag[ i ] equal to 1 specifies that within the CVS sub-layer non-reference pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId equal to max_tid_il_ref_pics_plus1[i]-1 if max_tid_il_ref_pics_plus1[i] is less than 7 or equal to sps_max_sub_layers_minus1 from active SPS of layer with index i otherwise are not used as reference for inter-layer prediction. no_sub_layer_non_ref_inter_layer_sample_pred_flag[ i ] equal to 0 specifies that within the CVS sub-layer non-reference pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId equal to max_tid_il_ref_pics_plus1[i]-1 if max_tid_il_ref_pics_plus1[i] is less than 7 or equal to sps_max_sub_layers_minus1 from active SPS of layer with index i otherwise may be used as reference for inter-layer prediction. When not present, no_sub_layer_non_ref_inter_layer_sample_pred_flag[ i ][ j ] is inferred to be equal to 0.

Note: When max_tid_il_ref_pics_plus1[i] is 0 non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[i] are not used as reference for inter-layer reference prediction. In this case no_sub_layer_non_ref_inter_layer_sample_pred_tid_flag[ i ][ j ] is not signaled and inferred to be 0.

Decoding process Impact:

Following can be added in the semantics of inter_layer_pred_layer_idc[ i ]

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, no_sub_layer_non_ref_inter_layer_sample_pred_flag[ LayerIdxInVps[RefPicLayerId[i]] is not equal to 1 if the picture in the current access unit with nuh_layer_id value equal to RefPicLayerId[i] is a sub-layer non-reference picture and TemporalId of the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is equal to max_tid_il_ref_pics_plus1[ LayerIdxInVps[RefPicLayerId[i]]-1 if max_tid_il_ref_pics_plus1[LayerIdxInVps[RefPicLayerId[i]] is less than 7 or equal to sps_max_sub_layers_minus1 from active SPS of layer of the picture RefPicLayerId[i] otherwise.

- F.8.1.2.1 Marking process for sub-layer non-reference pictures not needed for inter-layer prediction

Following change is proposed to this process.

Input to this process is:

–	a nuh_layer_id value latestDecLayerId

Output of this process is:

–	potentially updated marking as "unused for reference" for some decoded pictures

NOTE – This process marks pictures that are not needed for inter or inter-layer prediction as "unused for reference". When TemporalId is less than HighestTid, the current picture may be used for reference in inter prediction and this process is not invoked.

- The variables numTargetDecLayers, and latestDecIdx are derived as follows:

–	numTargetDecLayers is set equal to the number of entries in TargetDecLayerIdList.

–	latestDecIdx is set equal to the value of i for which TargetDecLayerIdList[ i ] is equal to latestDecLayerId.

- For i in the range of 0 to latestDecIdx, inclusive, the following applies for marking of pictures as "unused for reference":

–	Let currPic be the picture in the current access unit with nuh_layer_id equal to TargetDecLayerIdList[ i ].

–	When currPic is marked as "unused for reference" and is a sub-layer non-reference picture, the following applies:

–	The variable currTid is set equal to the value of TemporalId of currPic.

–	The variable remainingInterLayerReferencesFlag is derived as specified in the following:

remainingInterLayerReferencesFlag = 0
		if (  ( currTid  <  ( max_tid_il_ref_pics_plus1[ LayerIdxInVps[ TargetDecLayerIdList[ i ] ] ] –1 ) )  || ((currTid  == ( max_tid_il_ref_pics_plus1[ LayerIdxInVps[ TargetDecLayerIdList[ i ] ] ] –1 )) && ! no_sub_layer_non_ref_inter_layer_sample_pred_flag[ LayerIdxInVps[ TargetDecLayerIdList[ i ] ] ]) )
			for( j = latestDecIdx + 1; j < numTargetDecLayers; j++ )
				for( k = 0; k < NumDirectRefLayers[ TargetDecLayerIdList[ j ] ]; k++ )
					if( TargetDecLayerIdList[ i ]  = =  RefLayerId[ TargetDecLayerIdList[ j ] ][ k ] )
						remainingInterLayerReferencesFlag = 1

- When remainingInterLayerReferenceFlag is equal to 0, currPic is marked as "unused for reference".

## Variant 2

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| … | u(1) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| for(j=0;j<= ((max_tid_il_ref_pics_plus1[ i ]==7) ? vps_max_sub_layers_minus1: max_tid_il_ref_pics_plus1[ i ]-1;j++) |  |
| no_sub_layer_non_ref_inter_layer_sample_pred_tid_flag[ i ][ j ] | u(1) |
| … |  |

no_sub_layer_non_ref_inter_layer_sample_pred_tid_flag[ i ][ j ] equal to 1 specifies that within the CVS sub-layer non-reference pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId equal to j are not used as reference for inter-layer prediction. no_sub_layer_non_ref_inter_layer_sample_pred_flag[ i ] equal to 0 specifies that within the CVS sub-layer non-reference pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId equal to j may be used as reference for inter-layer prediction. When not present, no_sub_layer_non_ref_inter_layer_sample_pred_flag[ i ][ j ] is inferred to be equal to 0.

Note: When max_tid_il_ref_pics_plus1[i] is 0 non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[i] are not used as reference for inter-layer reference prediction. In this case no_sub_layernon_ref_inter_layer_sample_pred_tid_flag[ i ][ j ] is not signaled and inferred to be 0.

Decoding process Impact:

Following can be added in the semantics of inter_layer_pred_layer_idc[ i ]

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, no_sub_layer_non_ref_inter_layer_sample_pred_flag[ LayerIdxInVps[RefPicLayerId[i]][TemporalIdx] is not equal to 1 if the picture in the current access unit with nuh_layer_id value equal to RefPicLayerId[i] is a sub-layer non-reference picture and TemporalId TemporalIdx of the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is equal to max_tid_il_ref_pics_plus1[ LayerIdxInVps[RefPicLayerId[i]]-1 if max_tid_il_ref_pics_plus1[LayerIdxInVps[RefPicLayerId[i]] is less than 7 or equal to sps_max_sub_layers_minus1 from active SPS of layer of the picture RefPicLayerId[i] otherwise.

- F.8.1.2.1 Marking process for sub-layer non-reference pictures not needed for inter-layer prediction

Following change is proposed to this process.

Input to this process is:

–	a nuh_layer_id value latestDecLayerId

Output of this process is:

–	potentially updated marking as "unused for reference" for some decoded pictures

NOTE – This process marks pictures that are not needed for inter or inter-layer prediction as "unused for reference". When TemporalId is less than HighestTid, the current picture may be used for reference in inter prediction and this process is not invoked.

- The variables numTargetDecLayers, and latestDecIdx are derived as follows:

–	numTargetDecLayers is set equal to the number of entries in TargetDecLayerIdList.

–	latestDecIdx is set equal to the value of i for which TargetDecLayerIdList[ i ] is equal to latestDecLayerId.

- For i in the range of 0 to latestDecIdx, inclusive, the following applies for marking of pictures as "unused for reference":

–	Let currPic be the picture in the current access unit with nuh_layer_id equal to TargetDecLayerIdList[ i ].

–	When currPic is marked as "unused for reference" and is a sub-layer non-reference picture, the following applies:

–	The variable currTid is set equal to the value of TemporalId of currPic.

–	The variable remainingInterLayerReferencesFlag is derived as specified in the following:

remainingInterLayerReferencesFlag = 0
		if ( ( currTid  <  ( max_tid_il_ref_pics_plus1[ LayerIdxInVps[ TargetDecLayerIdList[ i ] ] ] –1 ) )  || ((currTid  == ( max_tid_il_ref_pics_plus1[ LayerIdxInVps[ TargetDecLayerIdList[ i ] ] ] –1 )) && ! no_sub_layer_non_ref_inter_layer_sample_pred_tid_flag[ LayerIdxInVps[ TargetDecLayerIdList[ i ] ] ][currTid] ))
			for( j = latestDecIdx + 1; j < numTargetDecLayers; j++ )
				for( k = 0; k < NumDirectRefLayers[ TargetDecLayerIdList[ j ] ]; k++ )
					if( TargetDecLayerIdList[ i ]  = =  RefLayerId[ TargetDecLayerIdList[ j ] ][ k ] )
						remainingInterLayerReferencesFlag = 1

–	When remainingInterLayerReferenceFlag is equal to 0, currPic is marked as "unused for reference".

# Conclusion

This document proposes syntax and semantics in VPS for indicating that sub-layer non-reference pictures belonging to a layer are not used for inter-layer prediction.  Two variant syntax and semantics are proposed. We propose to adopt one of the two variants in the working draft of SHVC and MV-HEVC. Our preference is for variant 1.

Additionally a bitstream constraint and a change to the marking process for sub-layer non-reference pictures not needed for inter-layer prediction is proposed. We propose to adopt these in the working draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 1,” JCTVC-M1008, Incheon, May 2013

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 4 (ISO/IEC 23008-2:201x/PDAM2),” JCTVC-D1004, Incheon, May 2013.