| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0199 WG11 Number: m29830 |
| --- | --- |

| Title: | On Tile Alignment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Kiran Misra 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes:

- Bitstream conformance constraints on various syntax elements when tile_boundaries_aligned_flag is equal to 1 are proposed.

- Alternatively syntax and semantics for picture parameter set (PPS) is proposed to be modified when nuh_layer_id > 0 and tile_boundaries_aligned_flag is equal to 1 to not signal certain tile related syntax elements whose value could be inferred.

- Signaling tile alignment information regarding layers in video parameter set (VPS).

In the r1 revision of this document additional bitstream constraints and equations for inference of syntax elements not signaled in PPS are added.

# Introduction

In SHVC Draft Text 2 [1] and MV-HEVC Draft Text 4 [2], tile_boundaries_aligned_flag is signalled for nuh_layer_id > 0.

In JCTVC-M0464 [3] the semantics of tile_boundaries_aligned_flag were defined as:

tile_boundaries_aligned_flag equal to 1 indicates that all tile boundaries of the corresponding base layer picture have corresponding tile boundaries in the given enhancement layer and there exist no additional tile boundaries in the enhancement layer. That means, for any two enhancement layer picture samples that lie within the same enhancement layer tile, the collocated base layer samples shall also lie within the same reference layer tile and for any two reference layer picture samples that lie within the same reference layer tile, the collocated enhancement layer samples shall also lie within the same enhancement layer tile. tile_boundaries_aligned_flag equal to 0 indicates that there is no restriction on the tile configuration between corresponding enhancement layer and the given reference layer.

In SHVC Draft Text 2 [1] and MV-HEVC Draft Text 4 [2] the semantics of tile_boundaires_aligned_flag are defined as:

tile_boundaries_aligned_flag equal to 1 indicates that, when any two samples of one picture in an access unit belong to one tile, the collocated samples, if any, in another picture in the same access unit belong to one tile, and when any two samples of one picture in an access unit belong to different tiles, the collocated samples in another picture in the same access unit shall belong to different tiles. tile_boundaries_aligned_flag equal to 0 indicates that such a restriction may or may not apply.

where the definition of “collocated sample” appears to be TBD in M1008_v3.doc as follows:

F.3.4 collocated sample: A sample TBD.

Based on our interpretation of tile_boundaries_aligned_flag we propose following:

- Proposal 1A: Bitstream conformance constraints on various syntax elements when tile_boundaries_aligned_flag is equal to 1 are proposed.

- Proposal 1B: Alternatively syntax and semantics for picture parameter set (PPS) is proposed to be modified when nuh_layer_id > 0 and tile_boundaries_aligned_flag is equal to 1 to not signal certain tile related syntax elements whose value could be inferred.

Proposal 1A and Proposal 1B are alternatives to each other.

- Proposal 2: Signaling tile alignment information regarding layers in video parameter set (VPS).

# Proposal

Proposed changes are highlighted below compared to JCTVC-M1008 [1].

## Proposal 1A:

We propose following constraints to satisfy the tile boundary alignment requirement when tile_boundaries_aligned_flag is equal to 1.

- If tile_boundaries_aligned_flag  is equal to 1 and tiles_fixed_structure_flag is equal to 1 for one picture in an access unit then tiles_fixed_structure_flag shall be equal to 1 in the vui_parameters() of the active SPS of the layer of other picture(s) in the same access unit.

- If tile_boundaries_aligned_flag  is equal to 1 then num_tile_columns_minus1 shall be equal for each PPS that is active for picture(s) in the same access unit.

- If tile_boundaries_aligned_flag  is equal to 1 then num_tile_rows_minus1 shall be equal for each PPS that is active for picture(s) in the same access unit.

- If tile_boundaries_aligned_flag  is equal to 1 and then uniform_spacing_flag shall be equal to uniform_spacing_flag for each PPS that is active for pictures in the same access unit.

- If tile_boundaries_aligned_flag is equal to 1 then for each reference layer with layer identifier RefLayerId[ nuh_layer_id ][ k ] for k ranging from 0 to NumDirectRefLayer[ nuh_layer_id ] − 1:

  - row_height_minus1[0] shall be equal to ( ScaledRefLayerTopOffset + ( ( CtbSizeYRl * (RowHeightMinus1Rl[0] + 1 ) ) ÷ ScaleFactorY ) ) / CtbSizeY −  1 and column_width_minus1[0] shall be equal to ( ScaledRefLayerLeftOffset + ( ( CtbSizeYRl * (ColumnWidthMinus1Rl[0] + 1 ) ) ÷ ScaleFactorX ) ) / CtbSizeY − 1.

  - row_height_minus1[i] shall be equal to (RowHeightMinus1Rl[i] + 1) / ( ScaleFactorY * CtbSizeYRatio ) − 1 for each i from 1 to (num_tile_row_minus1−1) inclusive

  - column_width_minus1[j] shall be equal to (ColumnWidthMinus1Rl[j] + 1 ) / ( ScaleFactorX * CtbSizeYRatio ) − 1 for each i from 1 to (num_tile_columns_minus1−1) inclusive

Where CtbSizeYRl is the array size of each luma coding tree block in both width and height for the corresponding reference layer, CtbSizeYRatio is equal to CtbSizeY ÷ CtbSizeYRl,
RowHeightMinus1Rl[ i ] plus 1 specifies the height of the i-th tile row in units of coding tree blocks for the corresponding reference layer picture,
ColumnWidthMinus1Rl[ j ] plus 1 specifies the width of the j-th tile column in units of coding tree blocks for the corresponding reference layer picture.

## Proposal 1B:

Proposal 1B is an alternative to Proposal 1A.

In this proposal syntax and semantics for picture parameter set (PPS) is proposed to be modified when nuh_layer_id > 0 and tile_boundaries_aligned_flag is equal to 1 to not signal certain tile related syntax elements whose value could be inferred.

## Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| dependent_slice_segments_enabled_flag | u(1) |
| … |  |
| tiles_enabled_flag | u(1) |
| entropy_coding_sync_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| if ( !(nuh_layer_id > 0 && tile_boundaries_aligned_flag) ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| pps_loop_filter_across_slices_enabled_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

When nuh_layer_id >0 && tile_boundaries_aligned_flag for the SPS corresponding to pps_seq_parameter_set_id is 1, num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width_minus1[i], row_height_minus1[i] are not signaled.

The corresponding values from the active PPS of reference layer are used for inference of the values of these PPS syntax elements for the current layer. They are derived as follows:

- The values for num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag for the current layer PPS are respectively set equal to the value for num_tile_columns_minus1 and num_tile_rows_minus1, uniform_spacing_flag  syntax element from the active PPS of the layer with nuh_layer_id equal to 0.

- The values for column_width_minus1[i], row_height_minus1[i] are derived as follows:

ScaledRefLayerLeftOffset, ScaleFactorX, ScaledRefLayerTopOffset, ScaleFactorY are calculated using equations (G-10) to (G-21) using information from picture with nuh_layer_id equal to RefLayerId[currLayerId][0] and index DirectRefLayerIdx[ currLayerId][ RefLayerId[currLayerId][0] ] as input where currLayerId is the nuh_layer_id of the current picture. ColumnWidthMinus1Rl[i] is respectively set equal to column_width_minus1[i] for each i=0,…,num_tile_columns_minus1-1 and RowHeightMinus1Rl[i] is respectively set equal to row_height_minus1[i] for each i=0,…,num_tile_rows_minus1-1 values from active PPS of layer with nuh_layer_id equal to RefLayerId[currLayerId][0]. CtbSizeYRatio is set equal to CtbSizeY ÷ CtbSizeYRl where CtbSizeYRl is the array size of each luma coding tree block in both width and height for the layer with nuh_layer_id equal to RefLayerId[currLayerId][0].

  - The value of column_width_minus1[ 0 ] is set equal to the value ( ScaledRefLayerLeftOffset + ( CtbSizeYRl * (ColumnWidthMinus1Rl[ 0 ] + 1 ) ) ÷ ScaleFactorX ) / CtbSizeY - 1.
The values of column_width_minus1[ i ] for (i=1,…, num_tile_columns_minus1-1) are respectively set equal to the value (ColumnWidthMinus1Rl[ i ] + 1 ) / ( ScaleFactorX * CtbSizeYRatio ) – 1.

  - The value of row_height_minus1 [ 0 ] is set equal to the value ( ScaledRefLayerTopOffset + ( CtbSizeYRl * (RowHeightMinus1Rl[ 0 ] + 1 ) ) ÷ ScaleFactorY ) / CtbSizeY - 1.
The values of row_height_minus1[ i ] for (i=1,…, num_tile_rows_minus1-1) are respectively set equal to the value (RowHeightMinus1Rl[ i ] + 1 ) / ( ScaleFactorY * CtbSizeYRatio ) – 1.

## Proposal 2:

Tile alignment information regarding layers is proposed to be signaled in video parameter set (VPS).

The signalling in VPS regarding tile alignment can be used by a decoder to make decision regarding parallelization strategy that allows parallel decoding of a layer and its reference layers. In some case a decision regarding which layers could be decoded may be based on this information. Thus including this information in VPS may be useful.

Another reason for signalling the information in VPS is that it can allow more flexible signalling regarding tile alignment between each layer with respect to  the layers it directly depends on. For this type of signalling VPS may be a better location compared to SPS.

Two variants are proposed for the VPS signalling of tile alignment indication.

- Variant 1: In this variant only one tile_boundaries_aligned_flag is signaled in VPS to indicate alignment between coded pictures within an access unit.

- Variant 2: In this variant reflayer_tile_boundaries_aligned_flag[ i ][ j ]  is defined to signal tile alignment for a layer with respect to its reference layers. This variant has an advantage that the tile alignment can be declared more flexibly for each layer with respect to the layers it directly depends on.

Variant 1:

In this variant only one tile_boundaries_aligned_flag is signaled in VPS to indicate alignment between coded pictures within an access unit.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| tile_boundaries_aligned_vps_flag | u(1) |
| … |  |

tile_boundaries_aligned_vps_flag equal to 1 indicates that, when any two samples of one picture in an access unit belong to one tile, the collocated samples, if any, in another picture in the same access unit belong to one tile, and when any two samples of one picture in an access unit belong to different tiles, the collocated samples in another picture in the same access unit shall belong to different tiles. tile_boundaries_aligned_flag equal to 0 indicates that such a restriction may or may not apply.

Variant 2:

In this variant reflayer_tile_boundaries_aligned_flag[ i ][ j ]  is defined to signal tile alignment for a layer with respect to its reference layers. This variant has an advantage that the tile alignment can be declared more flexibly for each layer with respect to the layers it directly depends on.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) { |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| if(direct_dependency_flag[ i ][ j ]) |  |
| reflayer_tile_boundaries_aligned_flag[ i ][ j ] | u(1) |
| } |  |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| … |  |

reflayer_tile_boundaries_aligned_vps_flag[ i ][ j ]  equal to 1 indicates that, when any two samples of a picture in an access unit belonging to the layer with index i, belong to one tile, the collocated samples, if any, in another picture in the same access unit belonging to the layer with index j belong to one tile, and when any two samples of a picture belonging to the layer with index i, belong to different tiles, the collocated samples in another picture belonging to the layer with index j shall belong to different tiles. ref_layer_tile_boundaries_aligned_vps_flag[ i ][ j ]  equal to 0 indicates that such a restriction may or may not apply for pictures belonging to layer with index i and layer with index j. When ref_layer_tile_boundaries_aligned_vps_flag[ i ][ j ] is not present it is inferred to be equal to 0.

The proposed tile boundary alignment signalling in VPS can be sent in addition to the current signalling of tile_boundaries_aligned_flag in SPS. In this case the information in SPS can update / over-ride the information signalled in VPS.

# Conclusion

This document proposes bitstream conformance constraints on various syntax elements when tile_boundaries_aligned_flag is equal to 1. Alternatively syntax and semantics for picture parameter set (PPS) is proposed to be modified elements when nuh_layer_id > 0 and tile_boundaries_aligned_flag is equal to 1. We propose to adopt one of these two proposals into SHVC and MV-HEVC drafts.

Additionally we propose signalling tile alignment indication is VPS. We propose to adopt this signalling in SHVC and MV-HEVC drafts.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 1,” JCTVC-M1008, Incheon, May 2013

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 4 (ISO/IEC 23008-2:201x/PDAM2),” JCTVC-D1004, Incheon, May 2013.