| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0200 WG11 Number: m29831 |
| --- | --- |

| Title: | On Scaling List Data Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. Shan Liu, Shawmin Lei MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA Kazushi Sato, Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com +1 4085261899 {shan.liu, shawmin.lei} @mediatek.com +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sharp Labs of America, Inc., MediaTek Inc., Sony Corp. |  |  |

_____________________________

# Abstract

This document proposes syntax and semantics for signaling scaling list data for SHVC layers with nuh_layer_id > 0. Three variant syntax alternatives are proposed. PPS and SPS activation method and bitstream constraint is proposed in relation to the proposed syntax and semantics.

In r1 revision only some editorial changes are made.

# Introduction

In SHVC [1] scaling list data (scaling_list_data()) can be signalled in Sequence Parameter Set (SPS) and/or in Picture Parameter Set (PPS).

A method for signalling scaling list for SHVC layers with nuh_layer_id > 0 was proposed in JCTVC-M0211 [2]. A few missing elements of the method with respect to indicating which reference layer to copy scaling_list_data and regarding PPS and SPS activation were noted.

BoG report on HLS from Incheon meeting [3] concludes that:

“Worst case scaling list matrix is 15kbits per picture.  Some typical cases from earlier core experiment (CE4) in JCTVC-H0230 are closer to 1kbit per picture. 2-16 kbps is typical when not updated for each picture. Scaling list signaling would likely benefit from some type of inter-layer prediction. Further study encouraged.”

This document proposes following improvements to the scaling list data signalling for SHVC addressing noted issues related to JCTVC-M0211:

- Syntax and semantics for signaling the reference layer to copy scaling_list_data() from for PPS and SPS of layers with nuh_layer_id > 0 is proposed. Three different alternative variants for identifying the reference layer are proposed.

- Bitstream constraints are proposed to avoid indirect referencing/ chaining of PPS and/ or SPS activation when efficiently signaling scaling list data using inter-layer parameter prediction.

The proposed approach of applying bitstream constraints for avoiding indirect referencing / chaining of PPS and/ or SPS activation is also applicable and beneficial to other inter-layer parameter set prediction techniques such as those proposed in JCTVC-M0134 [4].

We propose 3 alternative syntax variants for scaling list signalling for SHVC layers with nuh_layer_id > 0.

Each syntax variant includes a flag to indicate if the scaling_list_data() is signalled in SPS/ PPS or if it is inferred from active SPS/ PPS of a reference layer. The variants differ in how the reference layer to infer the data is signalled.

- Variant 1: In this variant new syntax elements scaling_list_sps_pred_layer_idc/ scaling_list_pps_pred_layer_idc specify that the active SPS/ PPS of the reference layer with nuh_layer_id equal to RefLayerId[ nuh_layer_id ][ scaling_list_sps_pred_layer_idc/ scaling_list_sps_pred_layer_idc] is used to obtain the scaling_list_data() for the current SPS/ PPS.

Thus the new syntax elements provide an index into the RefLayerId list of the current layer from whose SPS/ PPS the scaling list data is copied.

- Variant 2: In this variant new syntax elements scaling_list_sps_pred_layer_idx_delta/ scaling_list_pps_pred_layer_idx_delta specify that the active SPS/ PPS of the reference layer with nuh_layer_id equal to RefLayerId[ nuh_layer_id ][ NumDirectRefLayers[ nuh_layer_id ] -scaling_list_sps_pred_layer_idx_delta/ scaling_list_pps_pred_layer_idx_delta -1] is used to obtain the scaling_list_data() for the current SPS/ PPS.

Thus the new syntax elements provide a delta value which is subtracted from the number of direct reference layers for the current layer to obtain an index into the RefLayerId list of the current layer from whose SPS/ PPS the scaling list data is copied.

- Variant 3: In this variant new syntax elements scaling_list_sps_delta_ref_layer_id_minus1/ scaling_list_pps_delta_ref_layer_id_minus1 +1 specify the difference in the value of nuh_layer_id and the value of nuh_layer_id of the reference layer whose active SPS/ PPS is used to obtain the scaling_list_data() for the current SPS / PPS.

Thus the new syntax elements provide a delta value which is subtracted from the current layer’s nuh_layer_id value to obtain the nuh_layer_id of the layer from whose SPS/ PPS the scaling list data is copied.

# Proposal

We propose following 3 alternative variant syntax and semantics for scaling list signalling for SHVC. For each variant, syntax and semantics are provided for Sequence Parameter Set (SPS) and picture parameter set (PPS).

Proposed changes are highlighted below compared to JCTVC-M1008 [1].

## Variant 1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... ... |  |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enaled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| if (nuh_layer_id > 0) { |  |
| sps_base_pred_scaling_list_flag | u(1) |
| if( sps_base_pred_scaling_list_flag ) |  |
| scaling_list_sps_pred_layer_idc | u(v) |
| } else |  |
| scaling_list_data( ) |  |
| } |  |
| ... ... |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- scaling_list_enabled_flag equal to 1 specifies that a scaling list is used for the scaling process for transform coefficients. scaling_list_enabled_flag equal to 0 specifies that scaling list is not used for the scaling process for transform coefficients.

- sps_scaling_list_data_present_flag equal to 1 specifies that scaling list data are present in the SPS. sps_scaling_list_data_present_flag equal to 0 specifies that scaling list data are not present in the SPS. When not present, the value of sps_scaling_list_data_present_flag is inferred to be equal to 0. When scaling_list_enabled_flag is equal to 1 and sps_scaling_list_data_present_flag is equal to 0, the default scaling list data are used to derive the array ScalingFactor as described in the scaling list data semantics specified in subclause Error: Reference source not found.

- sps_base_pred_scaling_list_flag is signalled if sps_scaling_list_data_present_flag is equal to 1. sps_base_pred_scaling_list_flag equal to 1 specifies that the scaling list data is inferred from the scaling list data in the reference layer sequence parameter set. sps_base_pred_scaling_list_flag equal to 0 specifies that scaling list data is present in the sequence parameter set. The value shall be 0 if the reference layer is the base layer and the value avc_base_layer_flag in vps_extension() is 1. When not present, the value is inferred to be 0.

- scaling_list_sps_pred_layer_idc specifies that the active SPS of the reference layer with nuh_layer_id equal to RefLayerId[ nuh_layer_id ][ scaling_list_sps_pred_layer_idc ] is used to obtain the scaling_list_data() for the current SPS. The length of scaling_list_sps_pred_layer_idc is Ceil(Log2(NumDirectRefLayers[ nuh_layer_id ]) bits. The scaling_list_sps_pred_layer_idc shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] -1, inclusive.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... ... |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| if (nuh_layer_id > 0) { |  |
| pps_base_pred_scaling_list_flag | u(1) |
| if( pps_base_pred_scaling_list_flag ) |  |
| scaling_list_pps_pred_layer_idc | u(v) |
| } else |  |
| scaling_list_data( ) |  |
| } |  |
| ... ... |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- pps_scaling_list_data_present_flag equal to 1 specifies that parameters are present in the PPS to modify the scaling lists specified in the active SPS. pps_scaling_list_data_present_flag equal to 0 specifies that the scaling lists used for the pictures referring to the PPS is inferred to be equal to those specified by the active SPS. When scaling_list_enabled_flag is equal to 0, the value of pps_scaling_list_data_present_flag shall be equal to 0. When scaling_list_enabled_flag is equal to 1, sps_scaling_list_data_present_flag is equal to 0, and pps_scaling_list_data_present_flag is equal to 0, the default scaling list data are used to derive the array ScalingFactor as described in the scaling list data semantics Error: Reference source not found.

- pps_base_pred_scaling_list_flag is signalled if pps_scaling_list_data_present_flag is equal to 1. pps_base_pred_scaling_list_flag equal to 1 specifies that the scaling list data is inferred from the scaling list data in the reference layer picture parameter set. pps_base_pred_scaling_list_flag equal to 0 specifies that scaling list data is present in the picture parameter set. The value shall be 0 if the reference layer is the base layer and the value of avc_base_layer_flag in vps_extension() is 1. When not present, the value is inferred to be 0.

- scaling_list_pps_pred_layer_idc specifies that the active PPS of the reference layer with nuh_layer_id equal to RefLayerId[ nuh_layer_id ][ scaling_list_pps_pred_layer_idc ] is used to obtain the scaling_list_data() for the current PPS. The length of scaling_list_sps_pred_layer_idc is Ceil(Log2(NumDirectRefLayers[ nuh_layer_id ]) bits. The scaling_list_sps_pred_layer_idc shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] -1, inclusive.

## Variant 2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... ... |  |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enaled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| if (nuh_layer_id > 0) { |  |
| sps_base_pred_scaling_list_flag | u(1) |
| if( sps_base_pred_scaling_list_flag ) |  |
| scaling_list_sps_pred_layer_idx_delta | ue(v) |
| } else |  |
| scaling_list_data( ) |  |
| } |  |
| ... ... |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- scaling_list_enabled_flag equal to 1 specifies that a scaling list is used for the scaling process for transform coefficients. scaling_list_enabled_flag equal to 0 specifies that scaling list is not used for the scaling process for transform coefficients.

- sps_scaling_list_data_present_flag equal to 1 specifies that scaling list data are present in the SPS. sps_scaling_list_data_present_flag equal to 0 specifies that scaling list data are not present in the SPS. When not present, the value of sps_scaling_list_data_present_flag is inferred to be equal to 0. When scaling_list_enabled_flag is equal to 1 and sps_scaling_list_data_present_flag is equal to 0, the default scaling list data are used to derive the array ScalingFactor as described in the scaling list data semantics specified in subclause Error: Reference source not found.

- sps_base_pred_scaling_list_flag is signalled if sps_scaling_list_data_present_flag is equal to 1. sps_base_pred_scaling_list_flag equal to 1 specifies that the scaling list data is inferred from the scaling list data in the reference layer sequence parameter set. sps_base_pred_scaling_list_flag equal to 0 specifies that scaling list data present in the sequence parameter set. The value shall be 0 if the reference layer is the base layer and the value avc_base_layer_flag in vps_extension() is 1. When not present, the value is inferred to be 0.

- scaling_list_sps_pred_layer_idx_delta specifies that the active SPS of the reference layer with nuh_layer_id equal to RefLayerId[ nuh_layer_id ][ NumDirectRefLayers[ nuh_layer_id ] - scaling_list_sps_pred_layer_idc -1] is used to obtain the scaling_list_data() for the current SPS. The scaling_list_sps_pred_layer_idx_delta shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] -1, inclusive.

Sub-variant:

- scaling_list_sps_pred_layer_idx  could be coded as u(v) instead of as ue(v). Then the length of scaling_list_sps_pred_layer_idx_delta is Ceil(Log2(NumDirectRefLayers[nuh_layer_id ]) bits. And the scaling_list_sps_pred_layer_idx_delta shall be in the range of 0 to NumDirectRefLayers[  nuh_layer_id ] -1, inclusive.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... ... |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| if (nuh_layer_id > 0) { |  |
| pps_base_pred_scaling_list_flag | u(1) |
| if( pps_base_pred_scaling_list_flag ) |  |
| scaling_list_pps_pred_layer_idx_delta | ue(v) |
| } else |  |
| scaling_list_data( ) |  |
| } |  |
| ... ... |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- pps_scaling_list_data_present_flag equal to 1 specifies that parameters are present in the PPS to modify the scaling lists specified in the active SPS. pps_scaling_list_data_present_flag equal to 0 specifies that the scaling lists used for the pictures referring to the PPS is inferred to be equal to those specified by the active SPS. When scaling_list_enabled_flag is equal to 0, the value of pps_scaling_list_data_present_flag shall be equal to 0. When scaling_list_enabled_flag is equal to 1, sps_scaling_list_data_present_flag is equal to 0, and pps_scaling_list_data_present_flag is equal to 0, the default scaling list data are used to derive the array ScalingFactor as described in the scaling list data semantics Error: Reference source not found.

- pps_base_pred_scaling_list_flag is signalled if pps_scaling_list_data_present_flag is equal to 1. pps_base_pred_scaling_list_flag equal to 1 specifies that the scaling list data is inferred from the scaling list data in the reference layer picture parameter set. pps_base_pred_scaling_list_flag equal to 0 specifies that scaling list data present in the sequence parameter set. The value shall be 0 if the reference layer is the base layer and the value of avc_base_layer_flag in vps_extension() is 1. When not present, the value is inferred to be 0.

- scaling_list_pps_pred_layer_idx_delta specifies that the active PPS of the reference layer with nuh_layer_id equal to RefLayerId[ nuh_layer_id ][ NumDirectRefLayers[ nuh_layer_id ] - scaling_list_pps_pred_layer_idc -1 ] is used to obtain the scaling_list_data() for the current PPS. The scaling_list_pps_pred_layer_idx_delta shall be in the range of 0 to NumDirectRefLayers[  nuh_layer_id ] -1, inclusive.

Sub-variant:

- scaling_list_pps_pred_layer_idx  could be coded as u(v) instead of as ue(v). Then the length of scaling_list_pps_pred_layer_idx_delta is Ceil(Log2(NumDirectRefLayers[ nuh_layer_id ]) bits. And the scaling_list_pps_pred_layer_idx_delta shall be in the range of 0 to NumDirectRefLayers[  nuh_layer_id ] -1, inclusive.

## Variant 3

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... ... |  |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enaled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| if (nuh_layer_id > 0) { |  |
| sps_base_pred_scaling_list_flag | u(1) |
| if( sps_base_pred_scaling_list_flag ) |  |
| scaling_list_sps_delta_ref_layer_id_minus1 | ue(v) |
| } else |  |
| scaling_list_data( ) |  |
| } |  |
| ... ... |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- scaling_list_enabled_flag equal to 1 specifies that a scaling list is used for the scaling process for transform coefficients. scaling_list_enabled_flag equal to 0 specifies that scaling list is not used for the scaling process for transform coefficients.

- sps_scaling_list_data_present_flag equal to 1 specifies that scaling list data are present in the SPS. sps_scaling_list_data_present_flag equal to 0 specifies that scaling list data are not present in the SPS. When not present, the value of sps_scaling_list_data_present_flag is inferred to be equal to 0. When scaling_list_enabled_flag is equal to 1 and sps_scaling_list_data_present_flag is equal to 0, the default scaling list data are used to derive the array ScalingFactor as described in the scaling list data semantics specified in subclause Error: Reference source not found.

- sps_base_pred_scaling_list_flag is signalled if sps_scaling_list_data_present_flag is equal to 1. sps_base_pred_scaling_list_flag equal to 1 specifies that the scaling list data is inferred from the scaling list data in the reference layer sequence parameter set. sps_base_pred_scaling_list_flag equal to 0 specifies that scaling list data present in the sequence parameter set. The value shall be 0 if the reference layer is the base layer and the value avc_base_layer_flag in vps_extension() is 1. When not present, the value is inferred to be 0.

- scaling_list_sps_delta_ref_layer_id_minus1 +1 specifies the difference in the value of nuh_layer_id and the value of nuh_layer_id of the reference layer whose active SPS is used to obtain the scaling_list_data() for the current SPS. The scaling_list_sps_delta_ref_layer_id_minus1 shall be in the range of 0 to nuh_layer_id-1, inclusive.

Sub-variant:

- scaling_list_sps_delta_ref_layer_id_minus1  could be coded as u(v) instead of as ue(v).

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... ... |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| if (nuh_layer_id > 0) { |  |
| pps_base_pred_scaling_list_flag | u(1) |
| if( pps_base_pred_scaling_list_flag ) |  |
| scaling_list_pps_delta_ref_layer_id_minus1 | ue(v) |
| } else |  |
| scaling_list_data( ) |  |
| } |  |
| ... ... |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- pps_scaling_list_data_present_flag equal to 1 specifies that parameters are present in the PPS to modify the scaling lists specified in the active SPS. pps_scaling_list_data_present_flag equal to 0 specifies that the scaling lists used for the pictures referring to the PPS is inferred to be equal to those specified by the active SPS. When scaling_list_enabled_flag is equal to 0, the value of pps_scaling_list_data_present_flag shall be equal to 0. When scaling_list_enabled_flag is equal to 1, sps_scaling_list_data_present_flag is equal to 0, and pps_scaling_list_data_present_flag is equal to 0, the default scaling list data are used to derive the array ScalingFactor as described in the scaling list data semantics Error: Reference source not found.

- pps_base_pred_scaling_list_flag is signalled if pps_scaling_list_data_present_flag is equal to 1. pps_base_pred_scaling_list_flag equal to 1 specifies that the scaling list data is inferred from the scaling list data in the reference layer picture parameter set. pps_base_pred_scaling_list_flag equal to 0 specifies that scaling list data present in the sequence parameter set. The value shall be 0 if the reference layer is the base layer and the value of avc_base_layer_flag in vps_extension() is 1. When not present, the value is inferred to be 0.

- scaling_list_pps_delta_ref_layer_id_minus1 +1 specifies the difference in the value of nuh_layer_id and the value of nuh_layer_id of the reference layer whose active PPS is used to obtain the scaling_list_data() for the current PPS. The scaling_list_pps_delta_ref_layer_id_minus1 shall be in the range of 0 to nuh_layer_id-1, inclusive.

Sub-variant:

- scaling_list_pps_delta_ref_layer_id_minus1  could be coded as u(v) instead of as ue(v).

## Parameter Set Activation and Bitstream Constraints

In addition to the three variant syntax and semantics proposed above, bitstream constraints are proposed to avoid indirect referencing/ chaining of PPS and/ or SPS activation when efficiently signaling scaling list data using inter-layer parameter prediction.

Also PPS and SPS activation method is proposed in relation to the proposed variant syntax and semantics above.

## SPS Activation and Bitstream Constraint:

The specifications in subclause F.7.4.2.4.2 apply with the following additions.

- When a SPS RBSP having nuh_layer_id equal to nuhLayerIdA is activated for a layer having nuh_layer_id equal to nuhLayerIdB and scaling_list_sps_pred_layer_idx_delta / scaling_list_sps_pred_layer_idx_delta / scaling_list_sps_delta_ref_layer_id_minus1 is present in the SPS RBSP, the value of scaling_list_data() for the layer having nuh_layer_id equal to RefLayerId[ nuhLayerIdB ][ scaling_list_sps_pred_layer_idc ] /  RefLayerId[ nuhLayerIdB ][ NumDirectRefLayers[ nuhLayerIdB ] - scaling_list_sps_pred_layer_idc -1 ] / nuhLayerIdB – (scaling_list_delta_ref_layer_id_minus1 + 1) shall be considered in effect for the layer having nuh_layer_id equal to nuhLayerIdB.

- When a SPS RBSP having nuh_layer_id equal to nuhLayerIdA is active for the layer having nuh_layer_id equal to nuhLayerIdB and scaling_list_sps_pred_layer_idx_delta / scaling_list_sps_pred_layer_idx_delta / scaling_list__sps_delta_ref_layer_id_minus1 is present in the SPS RBSP and the SPS RBSP active for the layer with nuh_layer_id equal to RefLayerId[ nuhLayerIdB ][ scaling_list_sps_pred_layer_idc ] /  RefLayerId[ nuhLayerIdB ][ NumDirectRefLayers[ nuhLayerIdB ] -scaling_list_sps_pred_layer_idc -1 ] / nuhLayerIdB – (scaling_list_sps_delta_ref_layer_id_minus1 + 1) is deactivated, then the SPS RBSP active for the layer having nuh_layer_id equal to nuhLayerIdB shall be deactivated by activating another SPS RBSP for that layer.

Bitstream Constraint

- It is a requirement of the bitstream conformance that when a SPS RBSP having a nuh_layer_id equal to nuhLayerIdA is activated for a layer having nuh_layer_id equal to nuhLayerIdB and sps_base_pred_scaling_list_flag is equal to 1 in the SPS RBSP then the sps_base_pred_scaling_list_flag  must be 0 (i.e. must not be 1)  for SPS RBSP active for the layer with nuh_layer_id equal to RefLayerId[ nuhLayerIdB ][ scaling_list_sps_pred_layer_idc ] /  RefLayerId[ nuhLayerIdB ][ NumDirectRefLayers[ nuhLayerIdB ] -scaling_list_sps_pred_layer_idc -1 ] / nuhLayerIdB – (scaling_list_sps_delta_ref_layer_id_minus1 + 1).

## PPS Activation and Bitstream Constraint:

The specifications in subclause F.7.4.2.4.2 apply with the following additions.

- When a PPS RBSP having nuh_layer_id equal to nuhLayerIdA is activated for a layer having nuh_layer_id equal to nuhLayerIdB and scaling_list_pps_pred_layer_idx_delta / scaling_list_pps_pred_layer_idx_delta / scaling_list_pps_delta_ref_layer_id_minus1 is present in the PPS RBSP, the values of scaling_list_data() for the layer having nuh_layer_id equal to RefLayerId[ nuhLayerIdB ][ scaling_list_pps_pred_layer_idc ] /  RefLayerId[ nuhLayerIdB ][ NumDirectRefLayers[ nuhLayerIdB ] -scaling_list_pps_pred_layer_idc -1 ] / nuhLayerIdB – (scaling_list_pps_delta_ref_layer_id_minus1 -+ 1) shall be considered in effect for the layer having nuh_layer_id equal to nuhLayerIdB.

- When a PPS RBSP having nuh_layer_id equal to nuhLayerIdA is active for the layer having nuh_layer_id equal to nuhLayerIdB and scaling_list_pps_pred_layer_idx_delta / scaling_list_pps_pred_layer_idx_delta / scaling_list_pps_delta_ref_layer_id_minus1 is present in the SPS RBSP and the SPS RBSP active for the layer with nuh_layer_id equal to RefLayerId[ nuhLayerIdB ][ scaling_list_pps_pred_layer_idc ] /  RefLayerId[ nuhLayerIdB ][ NumDirectRefLayers[ nuhLayerIdB ] -scaling_list_pps_pred_layer_idc -1 ] / nuhLayerIdB – (scaling_list_pps_delta_ref_layer_id_minus1 + 1) is deactivated, then the SPS RBSP active for the layer having nuh_layer_id equal to nuhLayerIdB shall be deactivated by the activating another SPS RBSP for that layer.

Bitstream Constraint

- It is a requirement of the bitstream conformance that when a PPS RBSP having a nuh_layer_id equal to nuhLayerIdA is activated for a layer having nuh_layer_id equal to nuhLayerIdB and pps_base_pred_scaling_list_flag is equal to 1 in the PPS RBSP then the pps_base_pred_scaling_list_flag  must be 0 (i.e. must not be 1)  for PPS RBSP active for the layer with nuh_layer_id equal to RefLayerId[ nuhLayerIdB ][ scaling_list_pps_pred_layer_idc ] /  RefLayerId[ nuhLayerIdB ][ NumDirectRefLayers[ nuhLayerIdB ] -scaling_list_pps_pred_layer_idc -1 ] / nuhLayerIdB – (scaling_list_pps_delta_ref_layer_id_minus1 + 1).

- The concept behind the above proposed bitstream constraint to avoid SPS and PPS indirect referencing/chaining could be also applied to other inter-layer parameter set prediction techniques such as [4].

# Conclusion

This document proposes syntax and semantics for signaling scaling list data for SHVC layers with nuh_layer_id > 0. Three variant syntax alternatives are proposed. We propose to adopt one of the three variants.

PPS and SPS activation method and bitstream constraint is proposed in relation to the proposed syntax and semantics.  We propose to adopt this method and constraint.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 1,” JCTVC-M1008, Incheon, May 2013

[2] S. Liu, X. Zhang, S. Lei, K. Sato, “AHG9: On Scaling List Signaling for SHVC,” JCTVC-M0211, Incheon, April 2013

[3] Y.-K. Wang, J. Boyce, “Report of the Joint BoG on High-Level Syntax,” JCTVC-M0450, Incehon, April 2013

[4] T. Rusert, “AHG9: Inter-layer SPS Prediction for HEVC Extensions,” JCTVC-M0134, Incheon, April 2013