| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0201 |
| --- | --- |

| Title: | SAO extension for higher bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis Apple Inc. 1 Infinite Loop Cupertino CA | Tel: | +1 408 228 7983 |
|  |  | Email: | atourapis@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

The sample adaptive offset (SAO) process was one of the new coding tools introduced in the HEVC/H.265 video coding standard because of its perceived improvements in objective as well as subjective quality. The Edge Offset mode of SAO in particular, tries to establish distortion that may have been introduced during encoding around edge boundaries, and then attempts to correct this distortion through the addition of a “gradient-dependent” offset. However, this process may be significantly impacted when resolution and bit-depth are increased, due to the likely increase of noise in the underlying material. Noisy neighbouring samples may be misclassified as edges and a less than optimal offset may be applied on all the samples. This contribution proposes the use of an optional but simple quantization step during the edge classification process that can provide further flexibility and enhance the performance of the SAO process in the presence of noise.

# Introduction

The next generation High Efficiency Video Coding (HEVC) standard, which was developed jointly by the ITU and ISO MPEG, has introduced several new video coding tools in an effort to improve video coding efficiency versus previous video coding standards and technologies such as MPEG-2, MPEG-4 part2, MPEG-4 AVC/H.264, VC1, and VP8 among others. One of these tools was the Sample Adaptive Offset (SAO). SAO is a mechanism that was added after the in-loop deblocking filter of HEVC that tries to reduce distortion by adding an offset to each sample based on a predefined classification/categorization process.

HEVC supports two different types of classification. A band offset (BO) method, which classifies the samples based on their magnitude, and an edge offset (EO) method, where classification is based on a simple edge analysis technique around each sample. SAO is also applied to different color samples separately. For the BO method in particular, samples are classified in 32 different bins, regardless of the bit-depth of the currently processed color plane, based on their magnitude. The bins are basically derived by appropriately splitting the entire dynamic range of the color sample space, i.e. from 0 to (2BitDepth-1), into equal regions of size 2BitDepth-5. That is, for 8 bit data, each bin would contain 8 samples in sequence, whereas for 10 bit data each bin will contain 10 samples in sequence. Although it would have been possible to signal an offset for each one of these bins, in an attempt to reduce the overhead required for signaling the offsets as well as for unification reasons with the EO method, only 4 values, representing the offsets of 4 contiguous bins out of the 32, are signaled. Only samples belonging to these 4 bins are adjusted using the SAO specified offset values.

Figure 1. The four different edge direction classifiers supported by EO SAO

For the EO method first a direction of the edge classification method is selected and specified within the bit stream. Edge classification can be performed either horizontally, vertically, or using the two diagonals. A sample can be classified as belonging in a flat, monotonic area (no offset), as a local minimum or maximum, or corresponding to an edge sample with a positive or negative gradient depending on its relationships with its adjacent directional neighbors. Except for the monotonic case, an offset is specified in the bit stream for all other classification cases. The relationships with the adjacent directional neighbors are as simple as performing a larger or equal comparison with each neighbor without, though, any consideration of the sample’s bit-depth.

Table 1. Different classifiers for the EO method

| EdgeIdx | Condition | Meaning |
| --- | --- | --- |
| 0 | None of the above | Monotonic Area |
| 1 | p < n0 and p < n1 | Local Min |
| 2 | p < n0 and p = n1 or p < n1 and p = n0 | Edge |
| 3 | p > n0 and p = n1 or p > n1 and p = n0 | Edge |
| 4 | p > n0 and p > n1 | Local Max |

Another characteristic of the specified SAO in HEVC is that the signaling of the magnitude of the offset values is limited to a maximum value of (1 << (Min(bitDepth, 10) – 5)) – 1,  whereas sign values are either signaled in the bit stream (for the BO method), or predefined for each edge category. The final offset value is computed as

SaoOffset = offsetSign * saoOffsetAbs << ( bitDepth − Min( bitDepth, 10 ) ),

where offsetSign is the specified sign for an offset, saoOffsetAbs is the signalled offset magnitude, and bitDepth is the specified bitdepth of the samples processed. In an example, the offset range for 8 bit data is within [-7, 7], for 10 bit data is within [-31,31], whereas for 12 bit data the range is within [-31,31] * 4.

Focusing on the EO method, we believe that the classification process can easily be impacted by noise that may exist in the video signal. That is, small noise variations, which commonly lie in the least significant bits of the video signal, can be missclassified into one of the EO classification bins impacting both the classifications accuracy and the objective and subjective performance of this process.

# Proposed Scheme

To reduce and control the impact of noise on the EO classification process we propose introducing an additional, optional quantization step during the classifier computation process. Quantization can have the effect of masking the noise that may exist in the content, and provide better focus on the edges of the video content.

Quantization can be applied in two different ways. In the first method (Method A) all samples to be processed are first quantized using the same quantizer Q. Then, the decision process is modified as is presented in Table 2 to consider the new quantized samples Q[p], Q[n0], and Q[n1].

Table 2. Different classifiers for the EO method using quantized samples

| EdgeIdx | Condition | Meaning |
| --- | --- | --- |
| 0 | None of the above | Monotonic Area |
| 1 | Q[p] < Q[n0] and Q[p] < Q[n1] | Local Min |
| 2 | Q[p] < Q[n0] and Q[p] = Q[n1] or Q[p] < Q[n1] and Q[p] = Q[n0] | Edge |
| 3 | Q[p] > Q[n0] and Q[p] = Q[n1] or Q[p] > Q[n1] and Q[p] = Q[n0] | Edge |
| 4 | Q[p] > Q[n0] and Q[p] > Q[n1] | Local Max |

In the second method (Method B), one may also see the SAO EO condition decision process as an evaluation of the sign of the difference between p and its neighbors n0 and n1.  That is Table 1 can be converted into:

Table 3. Reimagining of the EO classifier process using differences

| EdgeIdx | Condition | Meaning |
| --- | --- | --- |
| 0 | None of the above | Monotonic Area |
| 1 | (p - n0) < 0 and (p - n1) < 0 | Local Min |
| 2 | ((p - n0) < 0 and (p - n1) = 0) or ((p - n1) < 0 and (p - n0) = 0) | Edge |
| 3 | ((p - n0) > 0 and (p - n1) = 0) or ((p - n1) > 0 and (p - n0) = 0) | Edge |
| 4 | (p - n0) > 0 and (p - n1) > 0 | Local Max |

Therefore, instead of quantizing the actual samples, in this method we can quantize the difference of the neighboring samples. The sign of this value can then be considered as the condition for the EO SAO offset process:

Table 4. Reimagining of the EO classifier process using differences

| EdgeIdx | Condition | Meaning |
| --- | --- | --- |
| 0 | None of the above | Monotonic Area |
| 1 | Q[p - n0] < 0 and Q[p - n1] < 0 | Local Min |
| 2 | (Q[p - n0]) < 0 and Q[p - n1] = 0) or (Q[p - n1] < 0 and Q[p - n0] = 0) | Edge |
| 3 | (Q[p - n0] > 0 and Q[p - n1] = 0) or (Q[p - n1] > 0 and Q[p - n0] = 0) | Edge |
| 4 | Q[p - n0] > 0 and Q[p - n1] > 0 | Local Max |

Assuming that the quantization process is a simple right shift operation by N bits, with or without (in the case of Method A) rounding control, the two processes operate somewhat similarly, with method A having some advantage in operations (all samples can be quantized once, stored in a temporary memory, and their outcome reused for all neighborhood decisions), while method B is of higher precision and is likely preferable by some implementations.

Two different modes for the quantization process were considered. In the first mode the quantization step size is dynamically computed based on the bit-depth of the samples, by always quantizing the samples to its 8 most significant bits. That is, assuming that the bit-depth of the current color sample to be processed is bitDepth, N is computed as:

N = (bitDepth – 8)

and quantization can be performed as:

Method A: Q[x] = x >> N

Method B: if (N > 0) then, Q[x] = (x + (1 << (N – 1)) >> N, else Q[x] = x

In the second mode, which provides considerably more flexibility, N is explicitly signaled for each color component. We have selected signaling this information in the slice header to minimize overhead, although it is also possible to extend the signaling into the sample adaptive offset syntax and adapted for every coded tree unit, or restrict signalling further at the PPS or SPS level.

Since it is currently unclear to us how range extensions will be handled in relationship to the already defined profiles, and to avoid any contamination of the Main and Main-10 profiles, our modifications were conditioned using (ChromaArrayType != 1), which corresponds to a test for 4:2:0 material. In this case, and if no other signaling in the SPS, PPS, or sao syntax is desired, then the only modifications needed are highlighted in the following table:

7.3.6.1 General slice segment header syntax modifications

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| …. |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType > 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| if( ChromaArrayType != 1 ) { |  |
| slice_sao_luma_thres_scale | ue(v) |
| if( ChromaArrayType > 0 ) |  |
| slice_sao_chroma_thres_scale | ue(v) |
| } |  |
| } |  |
| ….. |  |
| } |  |

# Results

The test conditions specified in JCTVC-L1006 [2] were used for this experiment. We have only focused on the evaluation of the second proposed scheme, i.e. EO SAO based on quantized threshold classification, with a quantizer value of 3. No adaptive decision for the SAO EO quantizer was performed, which though would likely improve performance. Furthermore, only a slice based approach was tested, even though, as discussed above, a SAO syntax level method could also be possible, with likely an improved performance, and higher flexibility since in that scenario the quantizers could be changed locally and better adapt to local characteristics.

|  | All Intra Main-tier |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| RGB 4:4:4 |  |  |  | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% |
| YCbCr 4:4:4 | -0.1% | 0.0% | 0.0% |  |  |  | -0.1% |  |  |
| YCbCr 4:2:2 | -0.1% | -0.1% | -0.1% |  |  |  | -0.1% |  |  |
| Overall | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | N/A |  |  |  |  |  |  |  |  |
| Dec Time[%] | N/A |  |  |  |  |  |  |  |  |

|  | All Intra High-tier |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| RGB 4:4:4 |  |  |  | -0.1% | -0.0% | -0.1% |  | -0.1% | -0.1% |
| YCbCr 4:4:4 | -0.1% | 0.0% | 0.0% |  |  |  | -0.1% |  |  |
| YCbCr 4:2:2 | -0.1% | 0.0% | -0.1% |  |  |  | -0.1% |  |  |
| Overall | -0.1% | 0.0% | -0.0% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | N/A |  |  |  |  |  |  |  |  |
| Dec Time[%] | N/A |  |  |  |  |  |  |  |  |

|  | All Intra Super High-tier |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| RGB 4:4:4 |  |  |  | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% |
| YCbCr 4:4:4 | -0.1% | 0.0% | 0.1% |  |  |  | -0.1% |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | -0.1% |  |  |  | 0.0% |  |  |
| Overall | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  |  |  |  |  |  |  |
| Dec Time[%] | N/A |  |  |  |  |  |  |  |  |

|  | Random Access Main-tier |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| RGB 4:4:4 |  |  |  | -0.1% | -0.2% | -0.1% |  | -0.1% | -0.1% |
| YCbCr 4:4:4 | 0.0% | -0.2% | 0.4% |  |  |  | 0.0% |  |  |
| YCbCr 4:2:2 | 0.0% | -0.2% | -0.1% |  |  |  | -0.1% |  |  |
| Overall | 0.0% | -0.2% | 0.1% | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | N/A |  |  |  |  |  |  |  |  |
| Dec Time[%] | N/A |  |  |  |  |  |  |  |  |

|  | Random Access High-tier |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| RGB 4:4:4 |  |  |  | -0.2% | -0.3% | -0.1% |  | -0.2% | -0.2% |
| YCbCr 4:4:4 | 0.0% | 0.3% | 0.9% |  |  |  | 0.1% |  |  |
| YCbCr 4:2:2 | -0.1% | -0.3% | -0.1% |  |  |  | -0.1% |  |  |
| Overall | -0.1% | 0.0% | 0.5% | -0.2% | -0.3% | -0.1% | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | N/A |  |  |  |  |  |  |  |  |
| Dec Time[%] | N/A |  |  |  |  |  |  |  |  |

|  | Low delay B Main-tier |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| RGB 4:4:4 |  |  |  | 0.0% | -0.1% | -0.1% |  | -0.1% | -0.1% |
| YCbCr 4:4:4 | 0.0% | 0.3% | 0.7% |  |  |  | 0.1% |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% |  |  |
| Overall | 0.0% | 0.1% | 0.4% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 119% |  |  |  |  |  |  |  |  |
| Dec Time[%] | 116% |  |  |  |  |  |  |  |  |

|  | Low delay B High-tier |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| RGB 4:4:4 |  |  |  | -0.1% | -0.2% | -0.1% |  | -0.1% | -0.1% |
| YCbCr 4:4:4 | 0.0% | 0.7% | 1.4% |  |  |  | 0.2% |  |  |
| YCbCr 4:2:2 | -0.1% | 0.0% | 0.1% |  |  |  | 0.0% |  |  |
| Overall | 0.0% | 0.3% | 0.8% | -0.1% | -0.2% | -0.1% | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 117% |  |  |  |  |  |  |  |  |
| Dec Time[%] | 117% |  |  |  |  |  |  |  |  |

# Conclusion

An extension mechanism to the Edge Offset mode of the Sample Adaptive Offset (SAO) process is presented. It is suggested that additional performance benefits could be achieved by including a quantization step during the edge classification process, especially in the presence of noise in the video signal.

# References

- D. Flynn, J. Sole, T. Suzuki, “HEVC Range Extensions Draft 2,” JCTVC-L1005, Geneva, Switzerland, January 2013.

- D. Flynn, and K. Sharman, “Common Test Conditions and Software Reference,” JCTVC-L1006, Geneva, Switzerland, January 2013.

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).