| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 Jul. – 2 Aug. 2013 | Document: JCTVC-N0202 |
| --- | --- |

| Title: | [AHG16] Key picture concept and single loop decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Feldmann Fabian Jäger Mathias Wien Institut für Nachrichtentechnik RWTH Aachen University 52056 Aachen, Germany | Email: Email: Email: | feldmann@ient.rwth-aachen.de jaeger@ient.rwth-aachen.de wien@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

In this document a performance analysis of the key picture concept in SHVC is presented. The scheme has similarity with MGS as used in SVC. A multi-loop and a single-loop variant are presented. In the multi-loop variant, the enhancement layer performance is reportedly increased by 3.1% BD rate on average. Due to the induced drift, the quality of the reconstructed base layer is reportedly reduced by 4.5% BD rate.

As a modification of the scheme, a variant is presented that adds restrictions to the encoder in order to allow for single loop decoding of the enhancement layer. The enhancement layer performance is reportedly comparable to the multi-loop approach of SHM while the quality of the base layer is reportedly reduced by 0.42 dB BD PSNR on average.

# Introduction

In this document an implementation of the key picture concept into the SHM 2.0 reference software is presented. This modification of the prediction scheme only requires minimal changes to the reference decoder. The scheme has some similarity with MGS as used in SVC.

Furthermore some additional restrictions for the base layer have been applied in order to allow for single loop decoding of the enhancement layer.

Results are reported for the random access configuration and quality scalability.

# Key picture concept

## Concept

For the random access configuration every first frame of the GOP in the base layer (0, 8, 16…) is a key-picture. These base layer key-pictures only use other base layer key-pictures as reference for inter prediction. All non-key-pictures in the base layer use the reconstructed enhancement layer pictures for prediction. See Table 1 for a list of references used in the base layer.

For all base layer frames, the temporal motion vector predictor candidate is always derived from the base layer motion. This way it can be ensured that the correct motion is decoded when only decoding the base layer.

| POC | Base layer references | Enhancement layer references |
| --- | --- | --- |
| 0 | -8, -16 |  |
| 8 | 0, -8 |  |
| 4 |  | 0, -2, 8 |
| 2 |  | 0, -2, 4, 8 |
| 1 |  | 0, 2, 4 |
| 3 |  | 2, 0, 4, 8 |
| 6 |  | 4, 2, 8 |
| 5 |  | 4, 0, 6, 8 |
| 7 |  | 6, 4, 8 |

Table 1 - Used references in the base layer

## Results

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -3,5% | -10,2% | -10,4% |
| Class B | -2,9% | -8,4% | -10,1% |
| Overall (Test vs Ref) | -3,1% | -8,9% | -10,2% |
| Overall (Test vs single layer) | 10,8% | 19,7% | 19,8% |
| Overall (Ref vs single layer) | 14,4% | 32,1% | 34,1% |
| EL only (Test vs Ref) | -6,0% | -12,1% | -13,5% |

Table 2 - key picture concept both layers

|  | BL Drift |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 4,0% | 2,0% | 1,8% |
| Class B | 4,7% | 2,4% | 2,4% |
| Overall (Test vs Ref) | 4,5% | 2,3% | 2,2% |

Table 3 - key picture concept base layer

|  | Base layer (dB) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0,1725 | -0,0437 | -0,0343 |
| Class B | -0,1167 | -0,0320 | -0,0433 |
| Overall (Test vs Ref) | -0,1326 | -0,0354 | -0,0407 |

Table 4 - BD-PSNR results for the base layer

The reference results were created following the random access configuration and the common test conditions [1]. For the key picture concept the prediction structure was changed as described in (2.1).

Table 2 shows the BD-rate results for the implemented key picture concept compared to the unmodified SHM2.0 reference. Table 3 shows the BD-rate results when decoding only the base layer and Table 4 shows the corresponding delta PSNR results. The full table is available in the accompanying data sheet [JCTVC-N0202-RWTH-KeyPictureConcept.xls]

The results suggest that there is an enhancement layer performance increase when decoding both layers. However, when only decoding the base layer, some drift occurs which diminishes the reconstruction quality of the base layer.

# Single loop decoding

## Concept

In order to allow for single loop decoding, the coding scheme is now further modified:

- The deblocking filter and SAO are deactivated in the base layer.

- Constrained intra prediction is enabled in the base layer

- Only zero motion is allowed for prediction from the base layer reconstruction

These modifications make it possible to decode the enhancement layer without full reconstruction of the base layer. In the base layer, the key-pictures have to be fully reconstructed. In non-key-pictures, only the intra coded CUs have to be reconstructed. Since deblocking and SAO are disabled and constrained intra prediction is used, motion compensation does not need to be performed in the base layer if the full stream is decoded.

In the enhancement layer normal intra or inter prediction can be used as well as inter layer prediction. If the base layer information is used, the enhancement layer decoder has to perform motion compensation with the motion vector and reference from the base layer and reconstruct the residual signal from the base layer. This is possible since base and enhancement layer use the same references for inter prediction. The enhancement layer can then add another residual signal to the reconstruction.

The proposed modifications make the scheme similar to the medium grain SNR scalability approach followed in SVC.

## Results

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 2,1% | -6,8% | -6,9% |
| Class B | 1,4% | -5,4% | -7,6% |
| Overall (Test vs Ref) | 1,6% | -5,8% | -7,4% |
| Overall (Test vs single layer) | 16,2% | 23,5% | 23,2% |
| Overall (Ref vs single layer) | 14,4% | 32,1% | 34,1% |
| EL only (Test vs Ref) | 1,8% | -6,2% | -8,0% |

Table 5 - key picture concept with possibility of single loop decoding, both layers

|  | BL Drift |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 15,3% | 11,2% | 11,3% |
| Class B | 13,3% | 11,7% | 12,0% |
| Overall (Test vs Ref) | 13,9% | 11,5% | 11,8% |

Table 6 - key picture concept with possibility of single loop decoding, base layer only

|  | Base layer (dB) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0,6131 | -0,2314 | -0,2078 |
| Class B | -0,3366 | -0,1467 | -0,1932 |
| Overall (Test vs Ref) | -0,4156 | -0,1709 | -0,1974 |

Table 7 - BD-PSNR results for the base layer

The reference results were created following the random access configuration and the common test conditions [1]. For the key picture concept the prediction structure was changed as described in (2.1).

Table 5 shows the results for the implemented key picture concept with the described restrictions compared to the unmodified SHM2.0 reference. Table 6 shows the results when only decoding the base layer and Table 7 shows the corresponding delta PSNR results. The full table is available in the accompanying data sheet [JCTVC-N0202-RWTH-KeyPictureConceptSingleLoop.xls]

The results suggest that the overall performance when decoding the enhancement layer is still comparable to the performance of the SHM2.0 reference. The results also show that the performance in the base layer is decreased. It should be noted that in the current implementation, the deblocking and SAO filters were completely switched off in the base layer for sake of simplicity. In a next step, the filters are to be activated in case of base layer only reconstruction.

## Results with loop filtering and SAO enabled for BL decoding

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 1,6% | -7,0% | -7,1% |
| Class B | 1,1% | -5,4% | -7,7% |
| Overall (Test vs Ref) | 1,2% | -5,8% | -7,5% |
| Overall (Test vs single layer) | 15,8% | 23,5% | 23,1% |
| Overall (Ref vs single layer) | 14,4% | 32,1% | 34,1% |
| EL only (Test vs Ref) | 1,2% | -6,5% | -8,3% |

Table 8 - key picture concept with possibility of single loop decoding, both layers

|  | BL Drift |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 6,7% | 3,5% | 3,6% |
| Class B | 6,7% | 4,7% | 4,6% |
| Overall (Test vs Ref) | 6,7% | 4,4% | 4,3% |

Table 9 - key picture concept with possibility of single loop decoding, base layer only

|  | Base layer (dB) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0,2838 | -0,0747 | -0,0679 |
| Class B | -0,1673 | -0,0629 | -0,0839 |
| Overall (Test vs Ref) | -0,2006 | -0,0663 | -0,0793 |

Table 10 - BD-PSNR results for the base layer

Table 8 shows the results for the implemented key picture concept with activated loop filtering and SAO in the base layer compared to the unmodified SHM2.0 reference. Table 9 shows the results when only decoding the base layer with loop filtering and SAO. Table 10 shows the corresponding delta PSNR results. The full table is available in the accompanying data sheet [JCTVC-N0202-RWTH-KeyPictureConceptSingleLoop_WithBLFiltering.xls]

Discussion

The suggested concepts only require high level changes to the decoding and encoding process. The only modification to the reference decoder is the modified prediction structure. Except for this modification the conventional multi-loop decoder can be used.

When further restrictions are applied at the encoder side, single loop decoding can be enabled. This way only the base layer key pictures have to be reconstructed completely while for all other base layer frames only the intra coded area requires reconstruction. Since both layers use the same pictures for inter prediction the motion compensation only has to be performed once at the enhancement layer.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCTVC-M1009, “Common SHM test conditions and software reference configurations” Incheon, KR, 18-26 Apr. 2013