| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 Jul. – 2 Aug. 2013 | Document: JCTVC-N0203 |
| --- | --- |

| Title: | [AHG16] HG16: Cross-Check of JCTVC-N0186 Single-loop decoding of SNR scalability for refIdx based SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christian Feldmann Mathias Wien Institut für Nachrichtentechnik RWTH Aachen University 52056 Aachen, Germany | Email: Email: | feldmann@ient.rwth-aachen.de wien@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This document reports the cross-verification results for the input document JCTVC-N0186 on single-loop decoding of SNR scalability for refIdx based SHVC. The software from the proponent was studied and complies with the proposed single-loop decoding techniques described in JCTVC-N0186. The experimental results as well as the decoder complexity results were cross checked and verified.

# Introduction

In JCTVC-N0186 a SNR single-loop decoding scheme is proposed, that does not use the reconstructed and filtered base layer picture as a reference. Instead, for coding the enhancement layer a new alternative ILR is constructed. This alternative ILR is generated in the following way:

- For inter blocks, prediction is done using the base layer motion information and using the enhancement layer reconstruction as reference. In the second step the base layer residual signal is added to this prediction.

- For intra blocks, conventional intra prediction in the base layer is performed and the resulting reconstruction signal is directly used in the enhancement layer.

In order to ensure that the intra blocks can be reconstructed without performing motion compensation in the base layer, constrained intra prediction is enabled in the base layer. Also the unfiltered information (loop filter, SAO) is used to ensure single-loop decodability.

Secondly, JCTVC-N0186 presents results using additional restrictions for the alternative ILR. At the decoder a situation can arise in which three motion compensation operations are required (bi-prediction for alternative ILR, bi-prediction in EL where one reference is the artificial ILR). In order to prevent this, bi-directional prediction using the alternative ILR is not allowed if the alternative ILR already uses bi-prediction.

# Coding performance results

For the coding performance results the two presented schemes are compared to a modified reference that uses constrained intra prediction in the base layer. Otherwise the common test conditions are followed [2].

Encoding and decoding timing could not be verified because a heterogeneous cluster was used for the cross check. The complete crosscheck results can be found in the accompanying files “JCTVC-N0203-SHM-2.0-SLoop-vs-SHM-2.0-Refidx_CrossCheck.xls” and “JCTVC-N0203-SHM-2.0-SLoop-PredCond-vs-SHM-2.0-Refidx_CrossCheck.xls”.

|  | RA HEVC SNR |  |  | LD-B HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.4% | -8.6% | -9.3% | 1.9% | -8.0% | -9.0% |
| Class B | 1.3% | -7.4% | -10.3% | 1.4% | -7.0% | -9.8% |
| Overall (Test vs Ref) | 1.3% | -7.7% | -10.0% | 1.5% | -7.3% | -9.6% |
| Overall (Test vs single layer) | 17.2% | 22.5% | 21.0% | 27.9% | 26.8% | 27.6% |
| Overall (Ref vs single layer) | 15.8% | 33.7% | 35.8% | 26.0% | 37.5% | 42.6% |
| EL only (Test vs Ref) | 2.6% | -7.4% | -9.9% | 3.1% | -6.4% | -8.9% |

Table 1 - BD-rates of single-loop decoding compared with SHM2.0 multi-loop anchor using constrained intra prediction

|  | RA HEVC SNR |  |  | LD-B HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.8% | -8.0% | -8.8% | 3.3% | -6.4% | -7.6% |
| Class B | 2.3% | -6.5% | -9.5% | 3.6% | -4.7% | -7.4% |
| Overall (Test vs Ref) | 2.2% | -7.0% | -9.3% | 3.5% | -5.2% | -7.5% |
| Overall (Test vs single layer) | 18.2% | 23.5% | 22.0% | 30.4% | 29.6% | 30.5% |
| Overall (Ref vs single layer) | 15.8% | 33.7% | 35.8% | 26.0% | 37.5% | 42.6% |
| EL only (Test vs Ref) | 3.6% | -6.4% | -9.0% | 6.1% | -3.4% | -5.9% |

Table 2 - BD-rates of single-loop decoding with EL inter-prediction constraint compared with SHM2.0 multi-loop anchor using constrained intra prediction

# Complexity results

The average memory access results of the proposed methods for 3 different memory architectures are presented in JCTVC-N0186. These results were cross verified and perfectly match the results reported by the proponents.

The complete crosscheck results can be found in the accompanying files “JCTVC-N0203-Complexity-BLK_CrossCheck.xls”, “JCTVC-N0203-Complexity-PIC_CrossCheck.xls”, “JCTVC-N0203-PredCond-Complexity-BLK_CrossCheck.xls” and “JCTVC-N0203-PredCond-Complexity-PIC_CrossCheck.xls”.

| Tested method | Block-based implementation |  |  | Picture-based implementation |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| SHM2.0 single-loop decoding | 64% | 67% | 68% | 69% | 69% | 68% |
| SHM2.0 single-loop decoding with inter-prediction constraint | 60% | 63% | 63% | 69% | 69% | 68% |

Table 3 - Average memory access comparison between single-loop decoding and multi-loop decoding

# Conclusion

In this cross-check document the results for the input document JCTVC-N0186 on single-loop decoding of SNR scalability for refIdx based SHVC have been verified. The software implementation is compliant with the description in the document JCTVC-N0186. All reported results in the document JCTVC-N0186 were cross-verified.

# References

- JCTVC-M1009, “Common SHM test conditions and software reference configurations” Incheon, KR, 18-26 Apr. 2013

- X. Li, J. Boyce, P. Onno and Y. Ye, “Common Test Conditions and Software Reference Configurations for the Scalable Test Model”, JCTVC document JCTVC-M1009, Incheon, Korea, April, 2013.