| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0205 |
| --- | --- |

| Title: | RCE3: Results of test 3.3 on Intra motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-479-1797 d-kwon@ti.com +1-214-567-0630 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Applications such as wireless displays, automotive infotainment, remote desktop, remote gaming, cloud computing etc. are becoming popular. Video in these applications often has mixed content consisting of natural video, text, graphics etc. In text and graphics regions, patterns (e.g. text characters, icons, lines etc.) can repeat within a picture. This contribution proposes a CU-level intra motion compensation tool to remove this redundancy to reportedly achieve coding gain. When intra motion compensation is enabled for a CU, either horizontal motion or vertical motion is signaled. The proposed method is tested with two different search ranges in encoder. In the first test, horizontal and vertical motions are limited to a range of 0 ~ -63. In the second test, the vertical motion is further limited so that displaced block does not go beyond LCU boundary. The following bit rate savings under RCE3 common conditions for lossless coding are reported:

- First test - Class F: (2.5%/1.4%/0.7%), SC RGB 444: (25.2%/19.9%/16.6%), SC YUV 444: (20.7%/18.9%/17.0%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

- Second test - Class F: (2.3%/1.4%/0.6%), SC RGB 444: (21.5%/17.0%/14.8%), SC YUV 444: (18.5%/16.6%/16.1%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

The following average luma BD-Rate savings under RCE3 common conditions for lossy coding are reported:

- First test - AI-MT: 19.0%, AI-HT: 18.4%, AI-SHT: 17.9%, RA-MT: 15.7%, RA-HT: 15.2%,  LDB-MT: 12.0% and LDB-HT: 11.7%.

- Second test - AI-MT: 16.8%, AI-HT: 16.3%, AI-SHT: 15.9%, RA-MT: 13.8%, RA-HT: 13.4%,  LDB-MT: 10.5% and LDB-HT: 10.0%.

# Introduction

In text and graphics regions, patterns (e.g. text characters, icons, lines etc.) can repeat within a picture as highlighted in Figure 1. We propose use of intra picture motion compensation to exploit this redundancy and achieve bit rate savings.

Figure 1: Example video picture with mixed content consisting of natural scene, text, and graphics from sc_map. Red highlights are overlaid on original picture.

JCTVC-L0303 [1] is another recent contribution that exploits this redundancy to reduce bits using a dictionary based coding system. The advantage of our contribution is that it uses NxN blocks (i.e. coding units) and existing tools such as motion compensation and is therefore better tuned to existing video codec architectures and implementations when compared to dictionary coding.

# Proposed technique

A CU level flag is used to signal whether Intra motion compensation is used or not. When Intra motion compensation is used, a motion vector is transmitted. If Intra motion compensation is not used, then normal HEVC prediction is carried out. Motion compensation is currently restricted to blocks displaced exactly to the left or blocks displaced exactly on the top of the current block. So only the x- or the y-motion vector component is signaled. Absolute values of integer-pel luma motion vectors are coded directly without prediction using fixed length coding in bypass mode. For 4:2:0 or 4:2:2 format, chroma motion vector has half-pel precision. The chroma MC interpolation filter is used for chroma Intra MC as well. The Intra motion compensation method is tested with two different search ranges in encoder as described below.

2.1 Horizontal and vertical motion vectors in a range of 0 to -63

Horizontal and vertical motion vectors are limited to 0 to -63 as shown in Figure 2. When LCU size is 64x64, it requires 3 rows of LCU to be stored.

Figure 2. Horizontal and vertical Intra MV search ranges in a range of 0 to -63

2.2 Vertical motion vector within LCU

Horizontal motion vector is limited to 0 to -63 as in Section 2.1 and vertical motion vector is further restricted so that displaced block does not go beyond LCU boundary as shown in Figure 3. Therefore, only two left LCUs need to be stored for motion compensation.

Figure 3. Vertical Intra MV search range within LCU

# Simulation results

Software based on HM RExt-3.0 was used in simulations Based on RCE3 common conditions. Bit-rate savings by the methods in Sections 2.1 and 2.2 are shown in Tables 1 and 2, respectively, for lossless coding and Tables 3 and 4, respectively, for lossy coding, when compared to RCE3 anchor.

Table 1. Bit-rate saving for lossless coding by IntraMC using horizontal and vertical motions in a range of 0 to -63

Table 2. Bit-rate saving for lossless coding by IntraMC using horizontal motion in a range of 0 to -63 and vertical motion within LCU

Table 3. Bit-rate saving for lossy coding by IntraMC using horizontal and vertical motions in a range of 0 to -63

Table 4. Bit-rate saving for lossy coding by IntraMC using horizontal motion in a range of 0 to -63 and vertical motion within LCU

# Draft Text

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| intra_mc_flag[x0][y0] | ae(v) |
| if(intra_mc_flag[x][y0]) { |  |
| hor_motion_flag[x0][y0] | ae(v) |
| intra_motion[x0][y0] | ae(v) |
| } |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if ( !intra_mc_flag[x0[y0] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| …… |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( (CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] )) ¦¦ PredMode[ x0 ][ y0] = = MODE_INTRA_MC ) |  |
| rqt_root_cbf | ae(v) |
| …… |  |
| } |  |
| } |  |
| } |  |

intra_mc_flag [x0][y0] equals to 1 specifies that the current coding unit is encoded using MODE_INTRA_MC. intra_mc_flag [x0][y0] equals to 1 specifies that the coding unit using other intra or inter mode.

hor_motion_flag [x0][y0] eqauls to 1 specifies that the current coding unit has non-zero horizontal intra motion and vertical intra motion is inferred to be equal to 0. hor_mv_flag [x0][y0] eqauls to 0 specifies that the current coding unit has non-zero vertical intra motion and horizontal intra motion is inferred to be equal to 0.

intra_motion [x0][y0] specifies the absoltue value of either horizontal or vertical luma intra motion, which is specifified by hor_motion_flag[x0][y0], in the unit of integer pel.

#### 9.3.2.2 Initialization process for context variables

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | intra_mc_flag | Table 9-xx | 0..2 | 3..5 | 6..8 |

Table 9-xx. Values of variable initValue for ilp_flag ctxIdx

| Initialization variable | intra_mc_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 185 | 185 | 201 | 197 | 197 | 185 | 197 | 197 | 185 |

#### 9.3.3 Binarization

9.3.3.1 General

Table 9-2 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_uint( ) | intra_mc_flag | FL | cMax = 1 |
|  | hor_motion_flag | FL | cMax = 1 |
|  | intra_motion | FL | cMax = 6 |

#### 9.3.4.2 Derivatino process for ctxTable, ctxIdx and bypassFlag

9.3.4.2.1 General

Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_mc_flag | 0,1,2 | na | na | na | na | na |
| hor_motion_flag | bypass | na | na | na | na | na |
| intra_motion | bypass | bypass | bypass | bypass | bypass | bypass |

9.3.4.2.2 Derivation process of ctxInc using left and above syntax elements

Table 9-4 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| intra_mc_flag[ x0 ][ y0 ] | intra_mc_flag [ xNbL ][ yNbL ] | intra_mc_flag [ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |

# References

1. Tao Lin, Shuhui Wang, Peijun Zhang, Kailun Zhou, “AHG8: P2M based dual-coder extension of HEVC,” JCTVC-L0303.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).