| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0206 |
| --- | --- |

| Title: | Non-RCE3: Intra motion compensation with variable length intra MV coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-479-1797 d-kwon@ti.com +1-214-567-0630 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The CU-level intra motion compensation method of RCE 3.3.3 (JCTVC-N0205) uses fixed length coding for intra motion vector. In this contribution, it is proposed to use variable length coding for intra motion vector. When intra motion compensation is enabled for a CU, either horizontal motion or vertical motion is allowed for a CU. The horizontal motion is limited so that a displaced block does not go beyond two previous LCUs and the vertical motion is limited so that a displaced block does not go beyond the current LCU boundary. Therefore, the proposed intra MC method requires only two left LCUs to be stored additionally. Since intra motion vector is variable length coded, it is possible to use larger horizontal search range than fixed length coding. Intra motion is binarized by using the 3rd order Exp-Golomb code and each bin is bypassed coded. The proposed method is compared with HM RExt-3.0 anchor and RCE 3.3.3 Method 2, which requires the same additional memory for intra MC. The proposed method with 1 left LCU reference is also compared with HM-RExt-3.0 anchor. The following bit rate savings under RCE3 common conditions for lossless coding are reported:

- 2 Left LCUs compared to HM RExt-3.0 anchor - Class F: (3.1%/1.9%/0.8%), SC RGB 444: (24.0%/18.9%/16.7%), SC YUV 444: (21.6%/19.6%/19.2%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

- 2 Left LCUs compared to RCE 3.3.3 Method 2 - Class F: (0.9%/0.5%/0.2%), SC RGB 444: (3.4%/2.6%/2.7%), SC YUV 444: (4.4%/4.0%/4.2%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

- 1 Left LCU compared to HM RExt-3.0 anchor - Class F: (2.5%/1.5%/0.7%), SC RGB 444: (22.1%/17.5%/15.3%), SC YUV 444: (19.3%/17.4%/16.8%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

The following average luma BD-Rate savings under RCE3 common conditions for lossy coding are reported:

- 2 Left LCUs compared to HM RExt-3.0 anchor - AI-MT: 19.3%, AI-HT: 18.7%, AI-SHT: 18.2%, RA-MT: 15.8%, RA-HT: 15.3%, LDB-MT: 12.5% and LDB-HT: 12.2%.

- 2 Left LCUs compared to RCE 3.3.3 Method 2 - AI-MT: 3.6%, AI-HT: 3.4%, AI-SHT: 3.3%, RA-MT: 2.8%, RA-HT: 2.7%, LDB-MT: 2.7% and LDB-HT: 2.4%.

- 1 Left LCU compared to HM RExt-3.0 anchor - AI-MT: 17.5%, AI-HT: 17.0%, AI-SHT: 16.5%, RA-MT: 14.3%, RA-HT: 13.9%, LDB-MT: 11.0% and LDB-HT: 10.9%.

# Introduction

In text and graphics regions, patterns (e.g. text characters, icons, lines etc.) can repeat within a picture as highlighted in Figure 1. In JCTVC-N0205 [1], we proposed use of intra picture motion compensation to exploit this redundancy and achieve bit rate savings. While fixed length code is used for intra motion vector coding in [1], intra motion vector is coded using variable length code to improve coding efficiency in this contribution. To reduce memory size, both horizontal and vertical motions are limited so that only two left LCUs need to be stored for intra motion compensation.

Figure 1: Example video picture with mixed content consisting of natural scene, text, and graphics from sc_map. Red highlights are overlaid on original picture.

# Proposed technique

A CU level flag is used to signal whether Intra motion compensation is used or not. When Intra motion compensation is used, a motion vector is transmitted. If Intra motion compensation is not used, then normal HEVC prediction is carried out. Motion compensation is restricted to blocks displaced exactly to the left or blocks displaced exactly on the top of the current block. So only the x- or the y-motion vector component is signaled. To reduce the memory for intra motion compensation, horizontal motion is limited so that a displaced block does not go beyond two previous LCUs and vertical motion is limited so that a displaced block does not go beyond the current LCU boundary. Therefore, only two left LCUs need to be stored for intra motion compensation. For example, Figure 2 shows a search range for a particular CU. Since intra motion vector is variable length coded, search range can vary depending on CU position and size. In most cases, we have larger horizontal search range than fixed length motion vector coding in [1].

Figure 2. Intra motion vector search range with 2 left LCUs as reference.

Note that horizontal and vertical motion starts from –cuWidth and –cuHeight, respectively. So either absolute value of (MVx + cuWidth) or absolute value of (MVy + cuHeight) is signaled without prediction. These values are binarized by the 3rd order Exp-Golomb code and each bin is bypass coded.

# Simulation results

Software based on HM RExt-3.0 was used in simulations using RCE3 common conditions. When compared to HM RExt-3.0 (RCE3) anchor, bit-rate savings by the proposed method with 2 left LCU reference are shown in Tables 1 and 2 for lossless coding and lossy coding, respectively. When compared to RCE3.3.3 Method 2 [1], which requires the same additional memory for intra MC, bit-rate savings by the proposed method are shown in Tables 3 and 4 for lossless coding and lossy coding, respectively.

Table 1. Bit-rate saving by the proposed IntraMC method with 2 left LCU reference over RCE3 anchor for lossless coding

Table 2. Bit-rate saving by the proposed IntraMC method with 2 left LCU reference over RCE3 anchor for lossy coding

Table 3. Bit-rate saving by the proposed IntraMC method with 2 left LCU reference over RCE 3.3.3 Method 2 [1] for lossless coding

Table 4. Bit-rate saving by the proposed IntraMC method with 2 left LCU reference over RCE 3.3.3 Method 2 [1] for lossy coding

The proposed method is also applied with 1 left LCU reference and compared with HM RExt-3.0 (RCE3) anchor. Test results for lossless coding and lossy coding are shown in Tables 5 and 6, respectively.

Table 5. Bit-rate saving by the proposed IntraMC method with 1 left LCU reference over RCE3 anchor for lossless coding

Table 6. Bit-rate saving by the proposed IntraMC method with 1 left LCU reference over RCE3 anchor for lossy coding

# Draft Text

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| intra_mc_flag[x0][y0] | ae(v) |
| if(intra_mc_flag[x][y0]) { |  |
| hor_motion_flag[x0][y0] | ae(v) |
| intra_motion[x0][y0] | ae(v) |
| } |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if ( !intra_mc_flag[x0[y0] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| …… |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( (CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] )) ¦¦ PredMode[ x0 ][ y0] = = MODE_INTRA_MC ) |  |
| rqt_root_cbf | ae(v) |
| …… |  |
| } |  |
| } |  |
| } |  |

intra_mc_flag [x0][y0] equals to 1 specifies that the current coding unit is encoded using MODE_INTRA_MC. intra_mc_flag [x0][y0] equals to 1 specifies that the coding unit using MODE_INTRA or MODE_INTER.

hor_motion_flag [x0][y0] eqauls to 1 specifies that the current coding unit has non-zero horizontal intra motion and vertical intra motion is inferred to be equal to 0. hor_mv_flag [x0][y0] eqauls to 0 specifies that the current coding unit has non-zero vertical intra motion and horizontal intra motion is inferred to be equal to 0.

intra_motion [x0][y0] specifies the absoltue value of either horizontal or vertical luma intra motion, which is specifified by hor_motion_flag[x0][y0], in the unit of integer pel.

8.x Decoding process for coding units coded in intra motion compensation mode

8.x.1 General decoding process for coding units coded in intra motion compensation mode

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

The variable nCbSL is set equal to 1  <<  log2CbSize. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to 1  <<  ( log2CbSize ) / SubWidthC and the variable nCbShC is set equal to ( 1  <<  log2CbSize ) / SubHeightC.

The decoding process for coding units coded in intra motion compensation mode consists of following ordered steps:

- The intra motoin compensation prediction process as specified in subclause  is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the output is the array predSamplesL and when ChromaArrayType is not equal to 0, the arrays predSamplesCb, and predSamplesCr.

- The decoding process for the residual signal of coding units coded in inter prediction mode specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the output is the array resSamplesL and when ChromaArrayType is not equal to 0, the arrays resSamplesCb, and resSamplesCr.

- The reconstructed samples of the current coding unit are derived as follows:

–	The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the variable nCurrSw set equal to nCbSwL, the variable nCurrSh set equal to nCbShL, the variable cIdx set equal to 0, the (nCbSL)x(nCbSL) array predSamples set equal to predSamplesL, and the (nCbSL)x(nCbSL) array resSamples set equal to resSamplesL as inputs.

–	When ChromaArrayType is not equal to 0, the picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the chroma coding block location ( xCb / SubWidthC, yCb / SubHeightC ), the variable nCurrSw set equal to nCbSwC, the variable nCurrSh set equal to nCbShC, the variable cIdx set equal to 1, the (nCbSwC)x(nCbShC) array predSamples set equal to predSamplesCb, and the (nCbSwC)x(nCbShC) array resSamples set equal to resSamplesCb as inputs.

–	When ChromaArrayType is not equal to 0, the picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the chroma coding block location ( xCb / SubWidthC, yCb / SubHeightC ), the variable nCurrSw set equal to nCbSwC, the variable nCurrSh set equal to nCbShC, the variable cIdx set equal to 2, the (nCbSwC)x(nCbShC) array predSamples set equal to predSamplesCr, and the (nCbSwC)x(nCbShC) array resSamples set equal to resSamplesCr as inputs.

8.x.2 Intra motion compensation prediction process

This process is invoked when decoding coding unit whose CuPredMode[ xCb ][ yCb ] is equal to MODE_INTRA_MC.

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCb of chroma prediction samples for the component Cb, where nCbSwC and nCbShC are derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCr of chroma prediction samples for the component Cr, where nCbSwC and nCbShC are derived as specified below.

The variable nCbSL is set equal to 1  <<  log2CbSize. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to nCbSL / SubWidthC and the variable nCbShC is set equal to nCbSL / SubHeightC.

The variable nCbS1L is set equal to nCbSL  >>  1.

The decoding process for prediction units in intra motion compensation mode as specified in subclause  is invoked with the luma location ( xCb, yCb ), the luma location ( xBl, yBl ) set equal to ( 0, 0 ), the size of the luma coding block nCbSL, the width of the luma prediction block nPbW set equal to nCbSL, the height of the luma prediction block nPbH set equal to nCbSL, and a partition index partIdx set equal to 0 as inputs, and the outputs are an (nCbSL)x(nCbSL) array predSamplesL and when ChromaArrayType is not equal to 0, two (nCbSwC)x(nCbShC) arrays predSamplesCb and predSamplesCr.

8.x.3 Decoding process for prediction units in intra motion compensation mode

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable nCbS specifying the size of the current luma coding block,

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCb of chroma prediction samples for the component Cb, where nCbSwC and nCbShC are derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCr of chroma prediction samples for the component Cr, where nCbSwC and nCbShC are derived as specified below.

The variable nCbSL is set equal to nCbS. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to nCbS / SubWidthC and the variable nCbShC is set equal to nCbS / SubHeightC.

The decoding process for prediction units in inter prediction mode consists of the following ordered steps:

- The derivation process for motion vector components in subclause  is invoked with the luma coding block location ( xCb, yCb ), the luma coding block size block nCbS, mvLI, hor_motion_flag, intra_motion, when ChromaArrayType is not equal to 0, the chroma motion vectors mvCLI.

- The decoding process for intra motion compensation sample prediction as specified in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the luma coding block size block nCbS, the luma prediction block width nPbW, the luma prediction block height nPbH, mvLI, when ChromaArrayType is not equal to 0, the chroma motion vectors mvCLI, and the intra motion compensation prediction samples (predSamples) that are an (nCbSL)x(nCbSL) array predSamplesL of prediction luma samples and when ChromaArrayType is not equal to 0, two (nCbSwC)x(nCbShC) arrays predSamplesCr and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, as outputs.

- Derivation process for intra motion vector components

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable hor_motion_flag,

- a variable intra_motion,

- a variable nCbS specifying the size of the current luma coding block,

Outputs of this process are:

- the luma motion vectors mvLI,

- when ChromaArrayType is not equal to 0, the chroma motion vectors mvCLI.

For the derivation of the variable mvLI, the following applies:

- if hor_motion_flag is equal to 1, mvCL[0] = -(intra_motion + nCbSL) * 4; mvCL[1] = 0.

- Otherwise, mvCL[0] = 0; mvCL[1] = -(intra_motion  + nCbSL) * 4.

- When ChromaArrayType is not equal to 0, for the derivation of variable mvCLI, the following applies:

- mvCLI [0] =  mvLI[0] * (3 – SubWidthC); mvCLI[1] = (3 – SubHeightC)

- Error: Reference source not found Derivation process for intra motion compensation prediction samples

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable nCbS specifying the size of the current luma coding block,

–	the luma motion vectors mvLI,

–	when ChromaArrayType is not equal to 0, the chroma motion vectors mvCLI,

- a bit depth of samples, bitDepth.

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array preSamplesCb of chroma prediction samples for the component Cb, where nCbSwC and nCbShC are derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCr of chroma residual samples for the component Cr, where nCbSwC and nCbShC are derived as specified below.

The variable nCbSL is set equal to nCbS. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to nCbS / SubWidthC and the variable nCbShC is set equal to nCbS / SubHeightC. The reference picture consisting of current reconstructed picture.

Variables shift1, shift2, offset1, and offset2 are derived as follows:

–	The variable shift1 is set equal to 14 − bitDepth and the variable shift2 is set equal to 15 − bitDepth.

–	The variable offset1 is derived as follows:

- If shift1 is greater than 0, offset1 is set equal to 1  <<  ( shift1 − 1 ).

- Otherwise (shift1 is equal to 0), offset1 is set equal to 0.

–	The variable offset2 is set equal to 1  <<  ( shift2 − 1 ).

The prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLI[ x ][ y ] + offset1 )  >>  shift1 )

#### 9.3.2.2 Initialization process for context variables

Table 4-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | intra_mc_flag | Table 9-xx | 0..2 | 3..5 | 6..8 |

Table 9-xx. Values of variable initValue for ilp_flag ctxIdx

| Initialization variable | intra_mc_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 185 | 185 | 201 | 197 | 197 | 185 | 197 | 197 | 185 |

#### 9.3.3 Binarization

9.3.3.1 General

Table 4-2 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_uint( ) | intra_mc_flag | FL | cMax = 1 |
|  | hor_motion_flag | FL | cMax = 1 |
|  | intra_motion | EG3 | _ |

#### 9.3.4.2 Derivatino process for ctxTable, ctxIdx and bypassFlag

9.3.4.2.1 General

Table 4-3 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_mc_flag | 0,1,2 | na | na | na | na | na |
| hor_motion_flag | bypass | na | na | na | na | na |
| intra_motion | bypass | bypass | bypass | bypass | bypass | bypass |

9.3.4.2.2 Derivation process of ctxInc using left and above syntax elements

Table 4-4 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| intra_mc_flag[ x0 ][ y0 ] | intra_mc_flag [ xNbL ][ yNbL ] | intra_mc_flag [ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |

# References

1. D.-K. Kwon and M. Budagavi, “RCE3: Results of test 3.3 on intra motion compensation,” JCTVC-N0205.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).