| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0206 |
| --- | --- |

| Title: | Non-RCE3: Intra motion compensation with variable length intra MV coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-479-1797 d-kwon@ti.com +1-214-567-0630 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The CU-level intra motion compensation method of RCE 3.3.3 (JCTVC-N0205) uses fixed length coding for intra motion vector. In this contribution, it is proposed to use variable length coding for intra motion vector. When intra motion compensation is enabled for a CU, either horizontal motion or vertical motion is allowed for a CU. The horizontal motion is limited so that a displaced block does not go beyond two previous LCUs and the vertical motion is limited so that a displaced block does not go beyond the current LCU boundary. Therefore, the proposed intra MC method requires only two left LCUs to be stored additionally. Since intra motion vector is variable length coded, it is possible to use larger horizontal search range than fixed length coding. Intra motion is binarized by using the 3rd order Exp-Golomb code and each bin is bypassed coded. The proposed method is compared with HM RExt-3.0 anchor and RCE 3.3.3 Method 2, which requires the same additional memory for intra MC. The proposed method with 1 left LCU reference is also compared with HM-RExt-3.0 anchor. The following bit rate savings under RCE3 common conditions for lossless coding are reported:

- 2 Left LCUs compared to HM RExt-3.0 anchor - Class F: (3.1%/1.9%/0.8%), SC RGB 444: (24.0%/18.9%/16.7%), SC YUV 444: (21.6%/19.6%/19.2%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

- 2 Left LCUs compared to RCE 3.3.3 Method 2 - Class F: (0.9%/0.5%/0.2%), SC RGB 444: (3.4%/2.6%/2.7%), SC YUV 444: (4.4%/4.0%/4.2%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

- 1 Left LCU compared to HM RExt-3.0 anchor - Class F: (2.5%/1.5%/0.7%), SC RGB 444: (22.1%/17.5%/15.3%), SC YUV 444: (19.3%/17.4%/16.8%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

The following average luma BD-Rate savings under RCE3 common conditions for lossy coding are reported:

- 2 Left LCUs compared to HM RExt-3.0 anchor - AI-MT: 19.3%, AI-HT: 18.7%, AI-SHT: 18.2%, RA-MT: 15.8%, RA-HT: 15.3%, LDB-MT: 12.5% and LDB-HT: 12.2%.

- 2 Left LCUs compared to RCE 3.3.3 Method 2 - AI-MT: 3.6%, AI-HT: 3.4%, AI-SHT: 3.3%, RA-MT: 2.8%, RA-HT: 2.7%, LDB-MT: 2.7% and LDB-HT: 2.4%.

- 1 Left LCU compared to HM RExt-3.0 anchor - AI-MT: 17.5%, AI-HT: 17.0%, AI-SHT: 16.5%, RA-MT: 14.3%, RA-HT: 13.9%, LDB-MT: 11.0% and LDB-HT: 10.9%.

# Introduction

In text and graphics regions, patterns (e.g. text characters, icons, lines etc.) can repeat within a picture as highlighted in Figure 1. In JCTVC-N0205 [1], we proposed use of intra picture motion compensation to exploit this redundancy and achieve bit rate savings. While fixed length code is used for intra motion vector coding in [1], intra motion vector is coded using variable length code to improve coding efficiency in this contribution. To reduce memory size, both horizontal and vertical motions are limited so that only two left LCUs need to be stored for intra motion compensation.

Figure 1: Example video picture with mixed content consisting of natural scene, text, and graphics from sc_map. Red highlights are overlaid on original picture.

# Proposed technique

A CU level flag is used to signal whether Intra motion compensation is used or not. When Intra motion compensation is used, a motion vector is transmitted. If Intra motion compensation is not used, then normal HEVC prediction is carried out. Motion compensation is restricted to blocks displaced exactly to the left or blocks displaced exactly on the top of the current block. So only the x- or the y-motion vector component is signaled. To reduce the memory for intra motion compensation, horizontal motion is limited so that a displaced block does not go beyond two previous LCUs and vertical motion is limited so that a displaced block does not go beyond the current LCU boundary. Therefore, only two left LCUs need to be stored for intra motion compensation. For example, Figure 2 shows a search range for a particular CU. Since intra motion vector is variable length coded, search range can vary depending on CU position and size. In most cases, we have larger horizontal search range than fixed length motion vector coding in [1].

Figure 2. Intra motion vector search range with 2 left LCUs as reference.

Note that horizontal and vertical motion starts from –cuWidth and –cuHeight, respectively. So either absolute value of (MVx + cuWidth) or absolute value of (MVy + cuHeight) is signaled without prediction. These values are binarized by the 3rd order Exp-Golomb code and each bin is bypass coded.

# Simulation results

Software based on HM RExt-3.0 was used in simulations using RCE3 common conditions. When compared to HM RExt-3.0 (RCE3) anchor, bit-rate savings by the proposed method with 2 left LCU reference are shown in Tables 1 and 2 for lossless coding and lossy coding, respectively. When compared to RCE3.3.3 Method 2 [1], which requires the same additional memory for intra MC, bit-rate savings by the proposed method are shown in Tables 3 and 4 for lossless coding and lossy coding, respectively.

Table 1. Bit-rate saving by the proposed IntraMC method with 2 left LCU reference over RCE3 anchor for lossless coding

Table 2. Bit-rate saving by the proposed IntraMC method with 2 left LCU reference over RCE3 anchor for lossy coding

Table 3. Bit-rate saving by the proposed IntraMC method with 2 left LCU reference over RCE 3.3.3 Method 2 [1] for lossless coding

Table 4. Bit-rate saving by the proposed IntraMC method with 2 left LCU reference over RCE 3.3.3 Method 2 [1] for lossy coding

The proposed method is also applied with 1 left LCU reference and compared with HM RExt-3.0 (RCE3) anchor. Test results for lossless coding and lossy coding are shown in Tables 5 and 6, respectively.

Table 5. Bit-rate saving by the proposed IntraMC method with 1 left LCU reference over RCE3 anchor for lossless coding

Table 6. Bit-rate saving by the proposed IntraMC method with 1 left LCU reference over RCE3 anchor for lossy coding

# Draft Text

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| intra_mc_flag[x0][y0] | ae(v) |
| if(intra_mc_flag[x][y0]) { |  |
| hor_motion_flag[x0][y0] | ae(v) |
| intra_motion[x0][y0] | ae(v) |
| } |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if ( !intra_mc_flag[x0[y0] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| …… |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( (CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] )) ¦¦ PredMode[ x0 ][ y0] = = MODE_INTRA_MC ) |  |
| rqt_root_cbf | ae(v) |
| …… |  |
| } |  |
| } |  |
| } |  |

intra_mc_flag [x0][y0] equals to 1 specifies that the current coding unit is encoded using MODE_INTRA_MC. intra_mc_flag [x0][y0] equals to 1 specifies that the coding unit using other intra or inter mode.

hor_motion_flag [x0][y0] eqauls to 1 specifies that the current coding unit has non-zero horizontal intra motion and vertical intra motion is inferred to be equal to 0. hor_mv_flag [x0][y0] eqauls to 0 specifies that the current coding unit has non-zero vertical intra motion and horizontal intra motion is inferred to be equal to 0.

intra_motion [x0][y0] specifies the absoltue value of either horizontal or vertical luma intra motion, which is specifified by hor_motion_flag[x0][y0], in the unit of integer pel.

#### 9.3.2.2 Initialization process for context variables

Table 4-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | intra_mc_flag | Table 9-xx | 0..2 | 3..5 | 6..8 |

Table 9-xx. Values of variable initValue for ilp_flag ctxIdx

| Initialization variable | intra_mc_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 185 | 185 | 201 | 197 | 197 | 185 | 197 | 197 | 185 |

#### 9.3.3 Binarization

9.3.3.1 General

Table 4-2 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_uint( ) | intra_mc_flag | FL | cMax = 1 |
|  | hor_motion_flag | FL | cMax = 1 |
|  | intra_motion | EG3 | _ |

#### 9.3.4.2 Derivatino process for ctxTable, ctxIdx and bypassFlag

9.3.4.2.1 General

Table 4-3 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_mc_flag | 0,1,2 | na | na | na | na | na |
| hor_motion_flag | bypass | na | na | na | na | na |
| intra_motion | bypass | bypass | bypass | bypass | bypass | bypass |

9.3.4.2.2 Derivation process of ctxInc using left and above syntax elements

Table 4-4 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| intra_mc_flag[ x0 ][ y0 ] | intra_mc_flag [ xNbL ][ yNbL ] | intra_mc_flag [ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |

# References

1. D.-K. Kwon and M. Budagavi, “RCE3: Results of test 3.3 on intra motion compensation,” JCTVC-N0205.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).