| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0207 |
| --- | --- |

| Title: | RCE2: Combination of Mirroring of Coefficients (Test B.3), Significance map context (Test B.1) and RDPCM (Test D.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajitha Weerakkody, Marta Mrak and Matteo Naccari 56 Wood Lane, London, W12 7SB, UK | Tel: Email: | rajitha.weerakkody@bbc.co.uk,marta.mrak@bbc.co.uk matteo.naccari@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This proposal presents the combination of the methods from RCE2 for lossy coding including mirroring the residual for non-transformed coefficients (Test B.3), constant context significance map (from Test B.1) and residual DPCM (RDPCM, from Test D.1) for both intra and inter coded transform units. The results show that the gains achieved in each of the tools above are incremental. Combined gains up to 7.5%, 5.5% and 5.8% are shown in all intra (AI), random access (RA) and low delay (LD-B) configurations respectively (for Screen content RGB 444).

# Introduction

This document presents the combination of a number of coding tools proposed in RCE2, namely: mirroring of coefficients in residual blocks where transform is skipped (Test B.3) [1], along with the constant context significance map (from Test B.1) [2], and residual DPCM (RDPCM) for both intra and inter coded TUs (Test D.1).

The objective of this work is to evaluate any positive or negative interactions between the tools listed above, in a potential co-existence. The combination is tested under the RCE2 common test conditions for lossy coding.

# Implementation

The implementations proposed in Tests B.1, B.3 and D.1 are combined over the HM range extension software Version 3.0 (HM-10.1rext-3.0).

# Results

The methods proposed in this document are implemented in HM10.1_Rext3.0 and simulated under the RCE2 common test conditions [3] for lossy coding.

Table 1. Mirroring with Constant Context and Residual DPCM

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -3.9% | -4.2% | -4.5% | -4.0% | -4.4% | -4.6% | -4.0% | -4.4% | -4.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.2% | 0.1% |
| SC RGB 444 | -7.5% | -7.3% | -7.0% | -7.5% | -7.4% | -7.1% | -6.9% | -6.7% | -6.4% |
| SC YUV 444 | -5.5% | -5.8% | -5.8% | -5.7% | -6.3% | -6.4% | -5.7% | -6.2% | -6.1% |
| RangeExt | -0.2% | -0.1% | -0.3% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.7% | -2.6% | -3.1% | -2.5% | -2.5% | -2.8% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -5.2% | -5.1% | -5.0% | -5.5% | -5.3% | -5.2% |  |  |  |
| SC YUV 444 | -3.7% | -4.1% | -4.0% | -3.7% | -4.3% | -4.3% |  |  |  |
| RangeExt | -0.1% | -0.1% | -0.1% | -0.1% | 0.1% | -0.1% |  |  |  |
| Enc Time[%] | 103% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.6% | -1.5% | -1.3% | -1.5% | -1.6% | -1.7% |  |  |  |
| Class B | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -5.7% | -5.7% | -5.8% | -5.6% | -5.5% | -5.3% |  |  |  |
| SC YUV 444 | -3.1% | -3.2% | -3.4% | -3.1% | -3.5% | -3.6% |  |  |  |
| RangeExt | -0.1% | -0.2% | -0.1% | 0.1% | 0.1% | 0.2% |  |  |  |
| Enc Time[%] | 103% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |

For reference, the results for only residual DPCM (without mirroring and constant context) is reproduced below (from test D.1).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.3% | -2.8% | -2.9% | -2.4% | -2.9% | -3.0% | -2.4% | -2.9% | -3.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -5.9% | -5.8% | -5.8% | -6.0% | -6.0% | -5.8% | -5.5% | -5.4% | -5.1% |
| SC YUV 444 | -4.3% | -4.9% | -4.8% | -4.6% | -5.3% | -5.4% | -4.5% | -5.1% | -5.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  | 93% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.4% | -1.7% | -1.8% | -1.3% | -1.6% | -1.7% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -4.3% | -4.4% | -4.4% | -4.3% | -4.4% | -4.4% |  |  |  |
| SC YUV 444 | -2.9% | -3.5% | -3.5% | -2.9% | -3.7% | -3.8% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -0.8% | -1.0% | -0.7% | -0.9% | -0.9% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -4.7% | -4.8% | -4.9% | -4.4% | -4.5% | -4.3% |  |  |  |
| SC YUV 444 | -2.5% | -2.8% | -2.8% | -2.5% | -2.9% | -3.0% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 106% |  |  | 107% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |

# Conclusions

The results presented in this documents shows that the gains achieved in the mirroring of non-transformed coefficients, with constant context (Test B.1) and the use of RDPCM for both intra and inter (Test D.1) are incremental.

# References

- R. Weerakkody and M.Mrak, “Mirroring of Coefficients for Transform Skipping”, JCTVC-K0294, Shanghai, CN, October 2012.

- T. Tsukuba and T. Yamamoto, “Constant coefficient context for intra transform skipping”, JCTVC-J0069, Stockholm, SE, July 2012.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, JCTVC-M1122, Incheon, KR, April 2013.

# Working Draft Text

The changes to the working draft are as per the proposal documents for tests B.3 (Mirroring of transform skipped coefficients), B.1 (Constant context) and D.1 (RDPCM).

# Patent rights declaration(s)

British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).