| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT 24 Jul – 2 Aug 2013 | Document: JCTVC-N0209 |
| --- | --- |

| Title: | SHVC skipped picture indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Vidyo Xiaoyu Xiu Yong He Yan Ye InterDigital Communications, Inc. | Email: Email: | jill@vidyo.com xiaoyu.xiu@intedigital.com |
| Source: |  |  |  |

_____________________________

# Abstract

It is proposed to add a skip_picture_flag syntax element in the SHVC slice segment header to indicate if a picture is skipped, and to define a normative skipped picture decoding process.  The proposals are similar to SVC’s slice_skip_flag and associated decoding process, but apply to an entire picture rather than a single slice.

# Proposal

It is proposed add a skip_picture_flag syntax element in the SHVC slice segment header, similar in functionality to SVC’s slice_skip_flag.  The slice skip feature in SVC has been found useful in SVC implementations.  It is useful to an encoder that is CPU starved or bitrate starved, as a low CPU/low bitrate method to encode an enhancement layer slice.  It is also useful to a decoder or middle box for error resiliency, by feeding to the decoder a NAL unit containing the syntax for a skipped slice in place of a missing coded slice, as an error concealment strategy.

The proposed skip_picture_flag syntax element indicates that all enhancement layer slices in a picture be skipped. It is only present in enhancement layers ( nuh_layer_id > 0) and when at least one direct reference layer is available  ( NumDirectRefLayers[ nuh_layer_id ] > 0).

- To decode a skipped picture, motion compensation (uni-prediction or bi-prediction) is performed by applying the re-sampled motion field from the closest reference layer to the corresponding temporal reference pictures in the current layer’s DPB. Prediction residual is set to be all zero. If a block in the re-sampled motion field is intra coded, re-sampled texture from the closest reference layer is used. Decoding of a skipped picture makes use of existing modules in SHVC WD2 and single layer HEVC; it does not require any new decoding modules.

## Proposed Syntax

### Proposed slice segment header modification

Slice segment header:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| if( num_extra_slice_header_bits > 0 ) |  |
| discardable_flag | u(1) |
| for( i = 1; i < num_extra_slice_header_bits ; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| skip_picture_flag | u(1) |
| if ( !skip_picture_flag ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 && !skip_picture_flag ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B && !skip_picture_flag ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 ) |  |
| alt_collocated_indication_flag | u(1) |
| if( alt_collocated_indication_flag ) |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

skip_picture_flag equal to 1 specifies that slice_segment_data( ) is not present for the current slice, and the decoding process for skipped picture with nuh_layer_id greater than 0.  skip_picture_flag equal to 0 specifies that slice_segment_data( ) is present for the current slice.  When skip_picture_flag not present, it is inferred to be equal to 0. It is a requirement of bitstream conformance that skip_picture_flag shall be the same for all slices of a coded picture.  When skip_picture_flag is equal to 1, it is a requirement of bitstream conformance that NumMotionPredRefLayers[ nuh_layer_id  ] and  NumSamplePredRefLayers[ nuh_layer_id ] shall be greater than 0.

alt_collocated_indication_flag equal to 0 specifies that a collocated picture for temporal motion vector prediction is indicated by collocated_from_l0_flag, when present, and collocated_ref_idx. alt_collocated_indication_flag equal to 1 specifies that a collocated picture for temporal motion vector prediction is indicated by collocated_ref_layer_idx. When alt_collocated_indication_flag is not present, it is inferred to be equal to skip_picture_flag.

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture. When not present, the value of inter_layer_pred_enabled_flag is inferred to be equal to skip_picture_flag.

It is a requirement of bitstream conformance that the value of alt_collocated_indication_flag shall be the same for all slices of a coded picture.

### Proposed slice segment layer RBSP modification

| slice_segment_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_segment_header( ) |  |
| if (!skip_picture_flag) |  |
| slice_segment_data( ) |  |
| rbsp_slice_segment_trailing_bits( ) |  |
| } |  |

## Proposed decoding process

- G.8.1.1 Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found and Error: Reference source not found specify the following decoding processes using syntax elements in the slice segment layer and above:

–	At the beginning of the decoding process of the first P or B slice of the current picture, subclause Error: Reference source not found is invoked.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause Error: Reference source not found is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

- If skip_picture_flag is equal to 1, the process in subclause G.8.1.5 specifies the decoding process of a skipped picture with nuh_layer_id greater than 0.

- Otherwise, if skip_picture_flag is equal to 0, the processes in subclauses 8.4, 8.5, 8.6, and 8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

- G.8.1.5 Decoding process for skipped picture with nuh_layer_id greater than 0

Input to this process is:

- a decoded reference layer picture rlPic

Output of this process is	 the decoded samples of the skipped picture skipPicSamples consisting of the following:

–	a ( PicWidthInSamplesL ) x ( PicHeightInSamplesL ) array skipPicSampleL of luma samples,

–	a ( PicWidthInSamplesC ) x ( PicHeightInSamplesC ) array skipPicSampleCb of chroma samples of the component Cb,

–	a ( PicWidthInSamplesC) x ( PicHeightInSamplesC ) array skipPicSampleCr of chroma samples of the component Cr.

The following steps are applied to derive the reconstructed samples of the skipped picture when skip_picture_flag is equal to 1. For each luma sample location xPb = 0 ... ( ( PicWidthInSamplesL + 15 ) >> 4 ) − 1 and yPb = 0 … ( ( PicHeightInSamplesL + 15 ) >> 4) − 1,

  - The variables xP and yP are set to ( xPb  << 4 ) and ( yPb  << 4 ), respectively,

  - The variables predMode[xP][yP], refIdxLX[xP][yP], mvLX[xP][yP], RefPicOrderCntLX[xP][yP], and predFlagLX[xP][yP], with X = 0,1, are derived by invoking inter layer motion derivation process specified in G.8.1.4.2.1 with the luma location ( xP, yP ), predModeRL, refIdxLXRL, mvLXRL, RefPicOrderCntLXRL, and predFlagLXRL, with X = 0,1, given as inputs.

  - If both predFlagL0[xP][yP] and predFlagL1[xP][yP] are equal to 0,

    - the sample value of  skipPicSampleL [ x ][ y ], with x = xP…xP+15, y = yP…yP+15, is derived by invoking the luma sample interpolation process in G.8.1.4.1.3 with the luma reference sample array rlPicSampleL and the luma sample location (x, y) given as inputs,

    - the sample value of  skipPicSampleCb [ x ][ y ], with x = xP/2…xP/2+7, y = yP/2…yP/2+7,  is derived by invoking the chroma sample interpolation process in G.8.1.4.1.4 with the chroma reference sample array rlPicSampleCb and the chroma sample location (x, y) given as inputs,

    - the sample value of  skipPicSampleCr [ x ][ y ], with x = xP/2…xP/2+7, y = yP/2…yP/2+7, is derived by invoking the chroma sample interpolation process in G.8.1.4.1.4 with the chroma reference sample array rlPicSampleCr and the chroma sample location (x, y) given as inputs.

  - Otherwise, if either or both of predFlagL0[xP][yP] and predFlagL1[xP][yP] are equal to 1,

    - a 16x16 array predSamplesL of prediction luma samples and two 8x8 arrays predSamplesCb and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, are derived by invoking the decoding process for inter sample prediction as specified in 8.5.3.2 with the luma coding block location ( xP, yP ), the luma prediction block location ( 0, 0 ), the luma coding block size block 16, the luma prediction block width 16, the luma prediction block height 16, the luma motion vectors mvL0[xP][yP] and mvL1[xP][yP], the chroma motion vectors mvL0[xP][yP]/2  and mvL1[xP][yP]/2, the reference indices refIdxL0[xP][yP] and refIdxL1[xP][yP], and the prediction list utilization flags predFlagL0[xP][yP] and predFlagL1[xP][yP] as inputs,

    - The 16x16 array predSamplesL of luma samples and two 8x8 arrays predSamplesCb and predSamplesCr of chroma samples, one for each of the chroma components Cb and Cr, are assigned to the sample values skipPicSampleL[ x ][ y ], with x = xP…xP+15, y = yP…yP+15, skipPicSampleCb [ x ][ y ], with x = xP/2…xP/2+7, y = yP/2…yP/2+7, and skipPicSampleCr [ x ][ y ], with x = xP/2…xP/2+7, y = yP/2…yP/2+7, respectively.

# SVC Background

The related skipped slice syntax in SVC is copied below.

## Slice layer extension RBSP syntax

| slice_layer_extension_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| if( svc_extension_flag ) { |  |  |
| slice_header_in_scalable_extension( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| if( !slice_skip_flag ) |  |  |
| slice_data_in_scalable_extension( ) /* specified in Annex Error: Reference source not found */ | 2 ¦ 3 ¦ 4 |  |
| } else { |  |  |
| slice_header( ) | 2 |  |
| slice_data( ) | 2 ¦ 3 ¦ 4 |  |
| } |  |  |
| rbsp_slice_trailing_bits( ) | 2 |  |
| } |  |  |

## Slice header in scalable extension syntax

| slice_header_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| if( !no_inter_layer_pred_flag ) { |  |  |
| slice_skip_flag | 2 | u(1) |
| if( slice_skip_flag ) |  |  |
| num_mbs_in_slice_minus1 | 2 | ue(v) |
| else { |  |  |
| adaptive_base_mode_flag | 2 | u(1) |
| if( !adaptive_base_mode_flag ) |  |  |
| default_base_mode_flag | 2 | u(1) |
| if( !default_base_mode_flag ) { |  |  |
| adaptive_motion_prediction_flag | 2 | u(1) |
| if( !adaptive_motion_prediction_flag ) |  |  |
| default_motion_prediction_flag | 2 | u(1) |
| } |  |  |
| adaptive_residual_prediction_flag | 2 | u(1) |
| if( !adaptive_residual_prediction_flag ) |  |  |
| default_residual_prediction_flag | 2 | u(1) |
| } |  |  |
| if( adaptive_tcoeff_level_prediction_flag ) |  |  |
| tcoeff_level_prediction_flag | 2 | u(1) |
| } |  |  |
| if( !slice_header_restriction_flag && !slice_skip_flag ) { |  |  |
| scan_idx_start | 2 | u(4) |
| scan_idx_end | 2 | u(4) |
| } |  |  |
| } |  |  |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Vidyo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).