| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0211 |
| --- | --- |

| Title: | On AVC base layer coding structure indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye, Yuwen He 9710 Scranton R-D, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

It is proposed to add an identical GOP structure indicator for the AVC base scalability to indicate if the AVC base layer GOP structure is the same as the GOP structure of the enhancement layers. The benefit of such indicator is to inform the decoder if all the pictures associated with the same output time can be decoded consecutively or not, and further facilitate the decoding initialization and memory allocation.

# Introduction

HEVC specifies the reference picture set (RPS) to indicate the GOP prediction structure, and such RPS is signaled in sequence parameter set (SPS). However, AVC does not have such GOP structure indication and the GOP prediction structure can only be obtained after decoding one GOP of pictures. For AVC base standard scalability application, it would be benefit to indicate if the HEVC enhancement layers and AVC base layer share the same GOP prediction structure or not. If AVC base and HEVC enhancement layers share the same GOP structure, the decoder could decode all the pictures associated with the same output time synchronically and no extra memory are needed. If the GOP structure between AVC base and HEVC enhancement layers are different, the relative decoding initialization and memory allocation may be needed due to different decoding order.

Figure 1 shows an unequal BL and EL GOP length example proposed in [1] where the decoding order of base layer and enhancement layers is different. The decoding order is ..., B4, B2, B1, B3, B8, E8, E4, E2, E1, E3, B6, E6, B5, E5, B7, E7, ... The additional DPB memory use is equal to 2 BL pictures (B1, B3) when compared to using the same GOP length (4) in both BL and EL. In addition, the pictures with the same POC value cannot be decoded consecutively in such scenario.

- Unequal AVC BL and HEVC EL GOP length

To indicate if the GOP structure is identical among all layers to facilitate the decoding initialization , an identical GOP structure indicator is proposed to add in VPS extension.

Table 1 shows the proposed indicator flag signaled in the VPS extension.

- Proposed Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| if (avc_base_layer_flag) |  |
| avc_identical_gop_structure_flag | u(1) |
| splitting_flag | u(1) |
| ... |  |
| } |  |

avc_identical_gop_structure_flag equal to 1 specifies that the GOP structure, including GOP length and prediction structure, of the AVC base is identical to that of the HEVC enhancement layers. avc_identical_gop_structure_flag equal to 0 indicates that the AVC base may have different GOP structure from that of the HEVC enhancement layers. avc_identical_gop_structure_flag is inferred to 1 when it is not present.

# Reference

- JCTVC-L0171, SHVC HLS: support for unequal BL and EL GOP lengths, January 2013

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).