| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0213 |
| --- | --- |

| Title: | On inter-layer resampling buffer indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye 9710 Scranton R-D, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The indicator signaling of two types of ILP, sample and motion, was proposed in JCTVC-M0457 and adopted into SHVC WD2. The motivation is to indicate whether a reference layer is used for sample prediction, motion prediction, or both, and the resampling operation and the corresponding buffer could be allocated in advance. However, for SNR scalability, the resampling operation and the corresponding buffer allocation could be bypassed even though the sample and/or motion prediction are carried out for a reference layer. In this contribution, an extra resampling buffer indicator is proposed to indicate if the corresponding resampling buffer is required or not in advance.

# Introduction

In SHVC WD2, the syntax element direct_dependency_type[i][j] is signaled in VPS extension to indicate if inter-layer sample prediction, motion prediction, or both would be carried out between i-th layer and j-th layer where j-th layer is the direct reference layer of i-th layer. Various sample prediction and motion prediction variables are derived   based on such indicator as specified in SHVC WD2 section F.7.4.3.1.1.

In case the sample and/or motion prediction are enabled between the current layer and a particular reference layer, the corresponding resampling operation (specified in SHVC WD2 G.8.1.4) would be invoked in the decoding process. To signal such inter-layer prediction methodology in VPS extension serves the purpose of capability exchange and allows the codec to allocate the tools and buffer in advance. However, for SNR scalability case, the resampling process is unnecessary since the reference picture samples and/or motion can be used directly without allocating any extra buffers.When either sample or motion prediction is enabled, we propose an additional indicator to indicate if resampling buffer allocation is required or not in advance.

# Resampling buffer indication

Table 1 is the proposed syntax signaling in VPS extension.

- Proposed video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| max_one_active_ref_layer_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) { |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| if (((direct_dependency_type[i][j]+1) & 1) ¦¦ (((direct_dependency_type[i][j]+1) & 2) >> 1)) |  |
| resampling_buffer_enable_flag[i][j] | u(1) |
| } |  |
| } |  |

resampling_buffer_enable_flag[ i ][ j ] equal to 0 specifies that  the resampling process between the i-th layer and the j-th layer may be by-passed and no resampling buffer needs to be allocated. resampling_buffer_enable_flag equal to 1 specifies that a related buffer for the resampling process of picture sample or motion values shall be invoked.  When resampling_buffer_enable_flag is not present, the default value is 0.

The resampling process specified in SHVC WD2 G.8.1.4 can be modified as follows:

–	if resampling_buffer_enable_flag[nuh_layer_id][dRlIdx] is set to 0,

  - rsPicSample is set equal to rlPicSample,

  - When alt_collocated_indication_flag is equal to 1, rsPicMotion is set equal to rlPicMotion.

–	otherwise, rsPic is derived as follows:

  - The picture sample resampling process as specified in subclause G.8.1.4.1. is invoked with the sample values of rlPicSample as input, and with the resampled sample values of  rsPicSample as output.

  - When alt_collocated_indication_flag is equal to 1, the picture motion field resampling process as specified in subclause G.8.1.4.2 is invoked with rlPicMotion as input, and with the  resampled motion field of rsPicMotion as output.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).