| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0214_v3 |
| --- | --- |

| Title: | Non-SCE1: Dynamic range control of intermediate data in re-sampling process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Xiang Li Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to apply the same scheme used in motion compensation interpolation filter to limit the dynamic range of intermediate data of 2D separable interpolation filtering in SHVC resampling process within 16-bit accuracy.

# Problem Statement

2D separable interpolation filter is employed in the SHVC re-sampling process. The filter coefficients are quantised with 6-bit accuracy which is the same as that of motion compensation interpolation filter. In the 2D separable interpolating process of the current SHVC test model, the output of the first interpolating step (horizontal direction) is used directly as the input of the second interpolating step (vertical direction). The intermediate data between the two dimension interpolation steps may be beyond 16 bits when the input signal is more than 8 bits. This will increase the implementation cost of the resampling filter in hardware as well as software solution.

# Proposed method

To address the problems as mentioned above, this contribution proposes to apply the same scheme used in motion compensation interpolation filter to limit the dynamic range of intermediate data of 2D separable interpolation filter in resampling process within 16-bit accuracy. That is to conduct right shift operation on the intermediate data after the first interpolation step. The shift bit is calculated in a same manner as done in motion compensation interpolation filter, which is determined by the bit depth of input signal. When the input signal is 8-bit, the right shift bit is 0, which means the proposed does not change the coding process in this case.

# Experimental results

Two 10 bits test sequences Nebuta and Steam Locomotive are used in the experiment. The two sequences (2560x1600) are down-sampled to (1280x800) size by using the software provided with interdigital which is used to generate SHVC CTC test sequences.

The SHVC common test condition of 2.0x spatial ratio and the above-mentioned test sequnces (100 frames) are used in the expirement. Three softwares are tested, including SHM2.0, SHM2.0 with 32-bits intermediate data (SHM2.0 + 32bits) and the SHM2.0 with proposed fix method (SHM2.0 + 16bits).

Table 1 shows the coding performance with the simulcast used as anchor for all intra configuration. It was found that both bug fixes on SHM2.0 with 32 bits and with proposed method dramatically improve the coding performance over SHM2.0 when the input is 10-bits sequence. The results show that there is serious overflow in up-sampling filter process of SHM2.0 when the input is 10-bits content.

Table 1 Performance comparison of SHM2.0 and bug fix on SHM2.0

| Test methods | BD rate |  |  |
| --- | --- | --- | --- |
|  | Y | Cb | Cr |
| SHM2.0 | -4.8% | -5.2% | -5.7% |
| SHM2.0+32bits | -29.05% | -27.37% | -28.73% |
| SHM2.0+16bits | -29.05% | -27.38% | -28.74% |

Table 2 shows coding performance comparison of SHM2.0 with 32 bits intermediate data buffer and the proposed fix method. The results of SHM2.0 with 32bits intermediate data buffer are used as anchor in the table. It was found that there are very marginal coding performance difference between the proposed fix and the method using 32-bits intermediate buffer.

Table 2 Performance comparison of SHM2.0+32bits and proposed fix

| Test methods | BD rate |  |  |
| --- | --- | --- | --- |
|  | Y | Cb | Cr |
| All Intra | 0.00% | -0.02% | -0.01% |
| Random access | 0.01% | -0.04% | 0.02% |
| Lowdelay-B | 0.02% | -0.09% | 0.01% |

# Specification Statement

Modifications are highlighted in yellow.

Modificatino in “G.8.1.4.1.3 Luma sample interpolation process”

The value of the interpolated luma sample IntLumaSample is derived by applying the following ordered steps:

- The derivation process for reference layer sample location used in resampling as specified in subclause G.6.2 is invoked with cIdx equal to 0 and luma sample location ( xP, yP ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample  as output.

- The variables xRef and xPhase are derived as follows:

xRef     = ( xRef16 >> 4 )	(G-1)
xPhase = ( xRef16 ) % 16	(G-2)

- The variables yRef and yPhase are derived as follows

yRef     = ( yRef16 >> 4 )	(G-3)
yPhase = ( yRef16 ) % 16	(G-4)

- The variables shift1, shift2 and offset are derived as follows:

shift1 = BitDepthY – 8
shift2 = 20 – BitDepthY 
offset =1 << (shift2 – 1).

- The sample value tempArray[ n] with n = 0..7, is derived as follows.

yPosRL = Clip3( 0, RefLayerPicHeightInSamplesL – 1, yRef + n – 1 ) 	(G-5)
refW      = RefLayerPicWidthInSamplesL

tempArray[n] = ( fL[ xPhase, 0 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 3), yPosRL ] +
	fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 2), yPosRL ] +
	fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 1), yPosRL ] +
	fL[ xPhase, 3 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef      ), yPosRL ] +
	fL[ xPhase, 4 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 1), yPosRL ] +	(G-6)
	fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 2), yPosRL ] +
	fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 3), yPosRL ] +
	fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 4), yPosRL ] ) >> shift1

- The interpolated luma sample value intLumaSample is derived as follows.

intLumaSample = ( fL[ yPhase, 0 ] * tempArray [ 0 ] +	
	fL[ yPhase, 1 ] * tempArray [ 1 ] +	
	fL[ yPhase, 2 ] * tempArray [ 2 ] +	
	fL[ yPhase, 3 ] * tempArray [ 3 ] +	
	fL[ yPhase, 4 ] * tempArray [ 4 ] +	(G-7)
	fL[ yPhase, 5 ] * tempArray [ 5 ] +	
	fL[ yPhase, 6 ] * tempArray [ 6 ] +	
	fL[ yPhase, 7 ] * tempArray [ 7 ] + (1<<11)offset ) >> 12shift2

intLumaSample = Clip3( 0, ( 1 << BitDepthY) – 1 , intLumaSample )	(G-8)

Modificatino in “G.8.1.4.1.4 Chroma sample interpolation process”

The value of the interpolated chroma sample value intChromaSample is derived by applying the following ordered steps:

- The derivation process for reference layer sample location in resampling as specified in subclause G.6.2Error: Reference source not found is invoked with cIdx and chroma sample location ( xPC, yPC ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample as output.

- The variables xRef and xPhase are derived by

xRef     = ( xRef16 >> 4 )	(G-9)
xPhase = ( xRef16 ) % 16	(G-10)

- The variables yRef and yPhase are derived by

yRef     = ( yRef16 >> 4 )	(G-11)
yPhase = ( yRef16 ) % 16	(G-12)

- The variables shift1, shift2 and offset are derived as follows:

shift1 = BitDepthC – 8
shift2 = 20 – BitDepthC 
offset =1 << (shift2 – 1).

- The sample value tempArray[ n ] with n = 0 .. 3, is derived as follows.

yPosRL = Clip3( 0 , RefLayerPicHeightInSamplesC – 1, yRef + n – 1 )	(G-13)
refWC   = RefLayerPicWidthInSamplesC	(G-14)

tempArray[n] = ( fC[ xPhase, 0 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef – 1), yPosRL ] +
	fC[ xPhase, 1 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef      ), yPosRL ] +
	fC[ xPhase, 2 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef + 1), yPosRL ] +	(G-15)
	fC[ xPhase, 3 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef + 2), yPosRL ] ) ) >> shift1

- The interpolated chroma sample value intChromaSample is derived as follows.

intChromaSa3mple = (fC[ yPhase, 0 ] * tempArray [ 0 ] +
	fC[ yPhase, 1 ] * tempArray [ 1 ] +
	fC[ yPhase, 2 ] * tempArray [ 2 ] +	(G-16)
	fC[ yPhase, 3 ] * tempArray [ 3 ] + (1<<11)offset ) >> 12shift2

intChromaSample = Clip3( 0, ( 1 << BitDepthC ) – 1 , intChromaSample )	(G-17)

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).