| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0215 |
| --- | --- |

| Title: | On SNR Scalability Indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye 9710 Scranton R-D, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to signal SNR scalability for the needs of re-sampling process, inter-layer filters and certain applications. It would be beneficial to differentiate SNR scalability from spatial scalability in high level syntax so that the encoder and decoder operations can be configured and/or initialized according to the relevant coding tools to be supported.

# Introduction

Scalable video coding supports multiple layers, and each layer could be designed to support spatial scalability, temporal scalability, SNR (Signal-to-Noise Ratio) scalability, or any other type of scalability.

Spatial scalability and SNR scalability are two major scalable cases being evaluated in SHVC common test conditions. For spatial scalability, the video is coded at different resolutions at different layers. For example, the base layer video may have 720p resolution and the enhancement layer may have 1080p resolution. For SNR scalability, the video resolution is the same across multiple layers but different layers may be coded at different quality. For example the base layer may be coded at 33dB, whereas the enhancement layer may be coded at 36dB. In the current SHVC WD 2, a syntax element, scalability_mask, is included in Video Parameter Set (VPS) to differentiate multiview scalability and spatial/SNR scalability (as shown in Table 1).

- Mapping of ScalabiltyId to scalability dimensions

| scalability_mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | Multiview | ViewId |
| 1 | spatial/SNR scalability | DependencyId |
| 2-15 | Reserved |  |

However, in the current SHVC working draft, the scalability_mask syntax does not differentiate spatial and SNR scalability. The spatial scalability and SNR scalability are two different kinds of scalability requiring different codec operations and memory allocation. Some examples are given below:

- The re-sampling process for reference layer picture samples and reference layer motion field are necessary for spatial scalability but they are unnecessary for SNR scalability.

- Some inter-layer filters, such as fixed alternative resampling filter being evaluated in core experiment SCE3, may achieve significant performance gain in SNR scalability scenario but are not applicable to spatial scalability.

- Certain applications may prefer single-loop design (which was supported by SVC, the scalable extension of H.264) for SNR scalability only.

- The sampling grid work currently undergoing in core experiment SCE1 addresses the particular issues related to spatial scalability, not SNR scalability.

Therefore, it would be beneficial to differentiate SNR and spatial scalability in the high level syntax so that the encoder and decoder operations can be configured and/or initialized according to the relevant coding tools to be supported.

Here we propose a couple of options to signal SNR scalability.

# SNR scalability signaling

## Option 1: Scalability dimension

One straightforward way is to assign spatial scalability and SNR scalability separate scalability dimensions. Table 2 is the modified scalability dimension table where spatial and SNR scalability has their distinct values.

- Proposed Mapping of ScalabiltyId to scalability dimensions

| scalability_mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | multiview | ViewId |
| 1 | Spatial scalability | DependencyId |
| 2 | SNR scalability | SnrId |
| 3-15 | Reserved |  |

Besides ViewId and DependencyId specified in SHVC WD 2 section F.7.4.3.1.1, variable SnrId[layer_id_in_nuh[i]] is proposed here  as the SNR identifier of the i-th layer. SnrId can be derived as follows:

for (i = 0; i <= vps_max_layers_minus1; i++) { for( smIdx= 0, j =0; smIdx< 16; smIdx ++ ) if( ( i ! = 0 ) && scalability_mask[ smIdx ] ) ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ] else ScalabilityId[ i ][ smIdx ] = 0 ViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ] DependencyId [ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 1 ] SnrId [ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 2 ] }

## Option 2: SNR scalability flag

Another way to signal the SNR scalability is to add a SNR_scalability_flag in VPS extension for each dependent reference layer. Table 3 shows the proposed syntax element in the VPS extension.

- Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| max_one_active_ref_layer_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) { |  |
| SNR_scalability_flag[i][j] | u(1) |
| direct_dependency_type[ i ][ j ] | u(v) |
| } |  |
| } |  |

SNR_scalability_flag equal to 1 specifies the scalability between layers with nuh_layer_id equal to layer_id_in_nuh[ i ] and nuh_layer_id equal to layer_id_in_nuh[ j ] is SNR scalability. SNR_scalability_flag equal to 0 specifies the scalability between layers with nuh_layer_id equal to layer_id_in_nuh[i] and nuh_layer_id equal to layer_id_in_nuh[j] is not SNR scalability. When SNR_scalability_flag is not present, it is inferred to be equal to 0.

With the proposed SNR_scalability_flag, the resampling process specified in SHVC WD2 G.8.1.4 can be simplified as below:

–	if SNR_scalability_flag[nuh_layer_id][dRlIdx] is set to 1,

  - rsPicSample is set equal to rlPicSample,

  - When alt_collocated_indication_flag is equal to 1, rsPicMotion is set equal to rlPicMotion.

–	otherwise, rsPic is derived as follows:

  - The picture sample resampling process as specified in subclause G.8.1.4.1 is invoked with the sample values of rlPicSample as input, and with the resampled sample values of  rsPicSample as output.

  - When alt_collocated_indication_flag is equal to 1, the picture motion field resampling process as specified in subclause G.8.1.4.2 is invoked with rlPicMotion as input, and with the  resampled motion field of rsPicMotion as output.

## Option 3: SPS extensions

Another alternative is to signal the SNR_scalability_flag in SPS extension instead of in VPS extension. Table 4 depicts an example to signal SNR_scalability_flag in SPS extensions.

- Sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| num_SNR_scalability_flags | ue(v) |
| for (j = 0; j < num_SNR_scalability_flags; j ++) |  |
| SNR_scalability_flag[j] | u(1) |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_left_offset[i] | se(v) |
| scaled_ref_layer_top_offset[i] | se(v) |
| scaled_ref_layer_right_offset[i] | se(v) |
| scaled_ref_layer_bottom_offset[i] | se(v) |
| } |  |
| } |  |

num_SNR_scalability_flags specifies the number of SNR_scalability_flag signaled in SPS extension.

SNR_scalability_flag[j] equal to 1 specifies the scalability between current layer with layer ID equal to nuh_layer_id and its j-th direct reference layer is SNR scalability. SNR_scalability_flag[j] equal to 0 specifies the scalability between current layer and j-th direct reference layer is not SNR scalability. When SNR_scalability_flag is not present, it is inferred to be equal to 0.

In general the value of num_SNR_scalability_flags should be equal to the number of direct reference layers of the current enhancement layer.

In addition, the scaled ref layer offset syntax elements signaled in SPS extension can be used to condition whether the scaled offsets between layers are signaled. Because for SNR scalability, the scaled offsets are inferred to be zero. Table 5 shows an example of such signaling and two syntax elements, num_SNR_scalability_flags and num_scaled_ref_layer_offsets in Table 5 are merged into a single num_ref_layers since the values of two syntax elements are identical.

- Proposed sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| num_ref_layers | ue(v) |
| for( i = 0; i < num_ref_layers; i++) { |  |
| SNR_scalability_flag[j] | u(1) |
| if (SNR_scalability_flag[i] == 0) { |  |
| scaled_ref_layer_left_offset[i] | se(v) |
| scaled_ref_layer_top_offset[i] | se(v) |
| scaled_ref_layer_right_offset[i] | se(v) |
| scaled_ref_layer_bottom_offset[i] | se(v) |
| } |  |
| } |  |
| } |  |

num_ref_layers specifies the number of reference layers in the CVS. The value of num_ref_layers shall be the same as NumDirectRefLayers[nuh_layer_id] derived from VPS extension.

SNR_scalability_flag[j] equal to 1 specifies the scalability between current layer with layer ID equal to nuh_layer_id and its j-th direct reference layer is SNR scalability. SNR_scalability_flag[j] equal to 0 specifies the scalability between current layer and j-th direct reference layer is not SNR scalability. When SNR_scalability_flag is not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).