| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0218 |
| --- | --- |

| Title: | AhG14: On bit-depth scalability support |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution gives preliminary information about performance of scalable system which combines spatial and bit-depth scalability. Comparison was done against single layer encoding of higher resolution and bit-depth layer.    SHM2.0 results for class A in ×2 scalability tests are 11,3% (AI)/ 21,7% (RA) / 32,0%(LBD)/ 30,4%(LDP) BD-rate drop compare to single layer HM10.1 (“main” configuration). Similar test with 10 bits content of class A resolution gives only 0,7% (AI)/ 2,4% (RA) / 9,7 %(LBD)/ 6,0%(LDP) BD-rate drop compare to single layer HM10.1 (“main10” configuration) if spatial and bit-depth scalability are combined. So, noticeable performance improvement can be achieved by combination of spatial and bit-depth scalability.

# Tests description

The scheme of SHm2.0 in case of spatial scalability test for class A is shown in Figure 1(left part of the figure). During HEVC standardization 2 video 10 bits sequences same with class A size 2560x1600 were used for tests:

- NebutaFestival_2560x1600_60_10bit_crop.yuv

- SteamLocomotiveTrain_2560x1600_60_10bit_crop.yuv

SHM2.0 s/w was modified in order to allow different bit-depth for reference and dependent layers. The only changes are:

- bit_depth_luma_minus8, bit_depth_chroma_minus8 need to be signalled for each layer separately;

- the difference in bit-depth is taken into account during re-sampling process (the descaling shift value at the very end of re-sampling interpolation is 12 if both layers have the same bit-depth, it was changed to 10 if base laeyr bit-depth is 8 but enhancement layer bit-depth is 10).

So SHM2.0 s/w changes are minimal and there is no low-level changes.

Mofieid s/w was tested using 8-bits low resolution reference layer video. Reconstructed video was up-sampled with bit-depth conversion to 10 bits and put into the refernce list during enhancement laeyr encoding. Enhancement layer was encoded with additional inter-layer reference frame using “main10” configuration for HM10.1. This is exactly how SHM2.0 operates in so-called RefIdx framework. The only difference both resolution and bit-depth are different for reference and dependent layer,.

Figure 1. Spatial scalability in SHM2.0 (on the left) and combination of spatial and bit-depth scalability (on the right).

# Test results

As comparison point we selected HM10.1 single layer codec. For SHM2.0 testing it is natural to use HM10.1 “main” configuration. For proposed spatial + bit-depth scalability enhancement layer is encoded using higher bit-depth, so natural comparison point is HM10.1 “main10” configuration.

Performance of SHM2.0 compare to HM10.1 single layer is reported in official template released by AhG10. For class A and spatial scalability ×2 tests this performance is summarized in Table 1 (first row).

Result compare to single layer HM10.1 (main10) for modified SHM2.0 s/w with spatial plus bit-depth scalability support are summarized in second row of Table 1. It is easy to notice that is spatial and bit-depth scalability are combined then noticeably smaller drop compare to single later decoding can be achieved by scalable system.

|  | All intra |  |  | Random Access |  |  | LD-B |  |  | LD-P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| SHM2.0 class A | 11,3% | 13,1% | 11,4% | 21,7% | 42,9% | 37,9% | 32,0% | 52,6% | 49,9% | 30,4% | 50,9% | 48,3% |
| +bit depth scalability | 0,7% | -1,7% | -5,4% | 2,4% | 75,4% | 53,4% | 9,7% | 40,9% | 29,2% | 6,04% | 43,6% | 30,7% |

# Conclusion

We suggest the study for combination of spatial and bit-depth scalability. Additionally Chroma format and color gamut scalability can be combined all together. Both they can be integrated into so-called refIdx approach with no low-level change. Merging of re-sampling and bit-depth (Chroma format and color gamut) conversion into the one process will minimize rounding error.

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).