| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0219 |
| --- | --- |

| Title: | Non-SCE1: On arbitrary spatial ratio scalability in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen, Xiang Li, Marta Karczewicz Elena Alshina, Aelxander Alshin K. Ugur | Email: | cjianle@qti.qualcomm.com elena_a.alshina@samsung.com kemal.ugur@nokia.com |
| Source: | Qualcomm Incorporated, Samsung Electronics, Ltd., Nokia. |  |  |

_____________________________

# Abstract

This contribution proposes to support arbitrary spatial ratio between base and enhancement layers. It is asserted that the complexity impact of supporting arbitrary spatial ratio in SHVC is marginal. It is also reported that the 1/8 pixel phase accuracy shows noticed coding performance drop when comparing to 1/16 pixel phase accuracy.

# Proposal

Currently, SHVC only support the cases of spatial ratio equal to 1, 1.5 and 2. This contribution proposes to support arbitrary spatial ratio between base and enhancement layers in SHVC. The reasons are as follows:

- Spatial resolutions of internet video and screen resolutions of tablets / phones are diverse so that supporting scalability between various resolutions would be helpful for SHVC deployment in internet stream and mobile device applications.

- The additional complexity to support arbitrary spatial ratio in SHVC is marginal

  - The reason ESS is in a separate profile in SVC is due to rather complex operations needed as SVC has many low level tools that require accessing BL data.

  - SHVC employs HLS-only solution such that only picture resampling process is to be modified to support arbitrary spatial ratio. It is asserted that no additional complexity is introduced in motion mapping process and the additional complexity of texture resampling process is marginal.

# Specification modification

Modifications are highlighted in yellow.

Modificatino in “G.8.1.4.1.3 Luma sample interpolation process”

Table G-1 – 16-phase luma resampling filter

| phase p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | −1 | 2 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | −1 | 3 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | −1 | 4 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | −1 | 4 | −11 | 52 | 26 | −8 | 3 | −1 |
| 6 | −1 | 4 | −11 | 50 | 28 | −8 | 2 | 0 |
| 7 | −1 | 4 | −11 | 45 | 34 | −10 | 4 | −1 |
| 8 | −1 | 4 | −11 | 40 | 40 | −11 | 4 | −1 |
| 9 | −1 | 4 | −10 | 34 | 45 | −11 | 4 | −1 |
| 10 | 0 | 2 | −8 | 28 | 50 | −11 | 4 | −1 |
| 11 | −1 | 3 | −8 | 26 | 52 | −11 | 4 | −1 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 4 | −1 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 3 | −1 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 2 | −1 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

Modificatino in “G.8.1.4.1.4 Chroma sample interpolation process”

Table G-2 – 16-phase chroma resampling filter

| phase p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | −2 | 62 | 4 | 0 |
| 2 | −2 | 58 | 10 | −2 |
| 3 | −4 | 56 | 14 | −2 |
| 4 | −4 | 54 | 16 | −2 |
| 5 | −6 | 52 | 20 | −2 |
| 6 | −6 | 46 | 28 | −4 |
| 7 | −4 | 42 | 30 | −4 |
| 8 | −4 | 36 | 36 | −4 |
| 9 | −4 | 30 | 42 | −4 |
| 10 | −4 | 28 | 46 | −6 |
| 11 | −2 | 20 | 26 | −6 |
| 12 | −2 | 16 | 54 | −4 |
| 13 | −2 | 14 | 56 | −4 |
| 14 | −2 | 10 | 58 | −2 |
| 15 | 0 | 4 | 62 | −2 |

# Experimental results

The proposal does not change the coding performance in the common test condition. To check the trade-off between coding efficiency and phase accuracy, the 1/8 pixel phase accuracy was tested. In this test, instead of quantizing the phase to 1/16 pixel grid, the filter phase is rounded in accuracy of 1/8 pixel grid

The performance of 1/8 phase accuracy comparing to 1/16 pixel phase accuracy is as follows. The 1/8 pixel phase accuracy shows 1.1% , 0.8% and 0.7% BD rate loss for 1.5x spatial ratio in AI, RA and LD-B configurations, respectively.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).