| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0220 |
| --- | --- |

| Title: | SCE3: Result of test 5.1 on adaptive up-sampling of base layer picture using simplified separable bilateral filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao, Kiran Misra, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 jzhao@sharplabs.com misrak@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This document reports the results for SCE3 Test 5.1 previously proposed in JCTVC-M0215.  JCTVC-M0215 proposed the use of non-linear and content adaptive “bilateral” filters for inter-layer prediction. It integrates a separable bilateral filter directly into the upsampling process. It is asserted that with this design, the upsampling and filtering operation can be performed without additional buffering at decoder, no increase in memory bandwidth or on-chip memory if in block based upsampling mode.  In picture based upsampling mode, memory for an additional inter layer picture is needed. The complexity of the method is asserted to be low – with a worst case of an additional 9 multiplications and 15 additions per pixel.  Results are reported following SCE3 test conditions and show EL+BL rate improvements compared to SHM2.0 anchors of: -0.9% (AI 2x), -0.3% (AI 1.5x), -0.6% (RA 2x), -0.1% (RA 1.5x),  -0.4% (LD-B 2x), 0.0 (LD-B 1.5x), and  -0.9% (LD-P 2x), -0.3 (LD-P 1.5x). The proposed bilateral filtering is not applied to SNR scalability.

# Introduction

The contribution proposes a non-linear and content adaptive upsampling process using SHM-2.0 REF_IDX framework. Here, two versions of the upsampled base layer pictures are added into reference picture list for inter-layer prediction. The first upsampled base layer picture corresponds to the fixed upsampler of SHM-2.0, while the second upsampled base layer picture is generated by combined upsampling and a separable bilateral filter.

Previously, in JCTVC-M0215, this adaptive upsampling with bilateral filter is tested using SHM-1.0 INTRA_BL framework. Here, we report its performance in SHM-2.0 REF_IDX framework. The rest of the proposal is organized as follows: In Section 2, we provide background on the bilateral filter.  Section 3 describes our proposed filtering process that integrates the bilateral and upsampling filters. Section 4 lists the software implementation details. Results are presented in Section 5 followed by complexity analysis in Section 6.

# Bilateral filter

A bilateral filter is a non-linear filter [2].  It reduces the noise in the upsampled base layer picture while preserving edges. We assert that this is beneficial for spatial scalability, as it removes low amplitude and high frequency content that is normally associated with noise, for e.g. due to quantization, loop filtering, while preserving high amplitude and high frequency information corresponding to edges and structure.  This is achieved by replacing the intensity value at each pixel by a weighted average of intensity values from nearby pixels within a window. The weight is based on a Gaussian distribution and relative pixel intensities.

# For a pixel at location (x,y), the bilateral filtered output is then expressed as:

Ox,y=f(a,b,Ia,b,Ix,y)∗Ia,bf(a,b,Ia,b,Ix,y)	(1)

where the summation is performed on pixels within a supporting window, Ia,b denotes the input pixel value at location (a,b), and f(a,b, Ia,b, Ix,y ) is a function that maps the spatial distance and relative pixel intensity information at a location (a,b) to a weight value.

The design of the function f() provides the desirable non-linear and content adaptive properties.  Traditionally, the function consists of two parts – a spatial kernel and a gradient kernel – and is expressed as:

f(a,b,Ia,b,Ix,y)=S(a,b)∗e−(G(Ia,b,Ix,y)22∗σR2)	(2)

where S(a,b) denotes a two-dimensional Gaussian kernel with standard deviation σS, G(a,b,Ix,y) denotes the absolute difference between Ia,b, and Ix,y, and σR is a control parameter. In general, the larger difference between the two pixel values, the lower the weight value. This preserves edges and avoids over smoothing. We advocate using simple differences to calculate the gradient.

# Combined bilateral filter with upsampler

We propose an approximation of the upsampling and bilateral processes with significantly fewer multiplies and additions when compared to the sequential application of an upsampler and traditional two-dimensional bilateral filter. In addition, the approximation does not require additional line buffers. As a first step in the approximation, we embed the bilateral filter into the upsampling framework. This is achieved by considering a separable operation for both the interpolation and content adaptive components. We note that in most applications, a separable bilateral filter does not perform well. For example, it may perform poorly on diagonal structures. However, for this application, we report that embedding the operation into an upsampling process mitigates this trait.

While a separable bilateral filter begins to reduce the number of additions and multiplies in the system, it does not address the need for additional line buffers and latency. To solve this, we remove all dependencies on neighboring pixels that are upsampled, using only integer-pel pixels in the content adaptive process. This allows the bilateral filtering to be done in place in the same pass as the upsampling of a pixel. Thus, our new filter is expressed as

Output(Ai,j)=f(Ai−1,j)∗Ai−1,j+Ai,j+f(Ai+1,j)∗Ai+1,jf(Ai−1,j)+1+f(Ai+1,j)						(3)

Output(bi,j)=f(Ai,j)∗Ai,j+bi,j+f(Ai+1,j)∗Ai+1,jf(Ai,j)+1+f(Ai+1,j)						(4)

where Ai,j is an integer-pel pixel and bi,j denotes a fractional-pel pixel.

A block diagram of the proposed separable upsampling and bilateral filtering is shown in Figure 1. As a key novelty, the calculation of the integer-pel pixels only uses information from other integer-pel locations, and so does not require interpolation of neighboring values. Similarly, the calculation of the fractional-pel pixel values use only the interpolated pixel value at the current location. All other input pixel values are integer-pel. By restricting the calculation to use only integer-pel pixel values (and a co-located fractional-pel value), the need for additional line buffers is removed. The bilateral operation is now a separable operation that is performed in-place with the interpolation. Shaded blocks denote the non-linear and content adaptive components.

Fig. 1. Illustration of 1-D upsampling with bilateral (BLT) filtering.

# Software implementation details

## Inter-layer picture placement

In addition to the default upsampled base layer picture, the bilateral filtered/upsampled base layer picture is also added into the reference picture list.

In our test, the position of the bilateral filtered/upsampled base layer picture is:

- For B SLICE, it is added to the end of LIST1 and replaces the default base layer picture.

- For P_SLICE, is added to the end of LIST0 after the default base layer picture.

Other placement position is also possible. With the REF_IDX framework, no additional syntax is needed.

## Bilateral filtering

For implementation, we realize the function f() with the lookup table [16, 15, 12, 9, 6, 4, 2, 1 0] and with the index to the table defined as min(gradient>>2, 8).  Here, “gradient” defines the difference between two pixel values, “>>” is a right shift operation, and min(x,8) returns the minimum value of x or 8. The support of the bilateral filter is restricted to a 3x1 kernel. The division in Equation (1) is replaced with a reciprocal multiplication, where the reciprocal is stored in a LUT.

## Source organization

In our implementation, we modified upsampleBasePic to generate two upsampled base layers. The first base layer corresponds to using the existing SHM upsampler. The second base layer corresponds to the process proposed in this document. At encoder xCheckRDCostILRUni is called twice at CU level, once with the default upsampled picture and once with the bilateral filtered upsampled picture. Encoder then chooses the best reference picture according to RD decision.

# Experimental setup

## Proposed adaptive upsampling with bilateral filter

We evaluated the proposed method in SHM-2.0. Here, we report results using SCE3 and SHM common test condition [3] [4] relative to SHM-2.0 anchor. Below is a summary BD rate results for test 5.1 with SHM-2.0 anchors. The experimental setup corresponds to the common test condition [4].

Table 1. Result of proposed adaptive upsampling method vs. SHM2.0 Anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1.4% | -0.9% | -1.0% |  |  |  |  |  |  |
| Class B | -0.7% | 0.1% | 0.1% | -0.3% | 0.3% | 0.3% |  |  |  |
| Overall (Test vs Ref) | -0.9% | -0.2% | -0.2% | -0.3% | 0.3% | 0.3% |  |  |  |
| Overall (Test vs single layer) | 11.7% | 14.7% | 14.4% | 10.2% | 10.1% | 9.7% |  |  |  |
| Overall (Ref vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| EL only (Test vs Ref) | -1.2% | -0.4% | -0.4% | 0.7% | 1.4% | 1.4% |  |  |  |
| Enc Time[%] | 109.7% |  |  | 114.5% |  |  |  |  |  |
| Dec Time[%] | 125.5% |  |  | 129.1% |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | 0.0% | -0.5% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | -0.4% | 0.2% | 0.2% | -0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.6% | 0.1% | 0.0% | -0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 18.4% | 33.5% | 32.0% | 16.1% | 29.1% | 29.3% | 14.4% | 32.1% | 34.1% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.8% | 0.0% | -0.2% | 0.7% | 0.9% | 0.9% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98.9% |  |  | 101.5% |  |  | 100.3% |  |  |
| Dec Time[%] | 141.3% |  |  | 138.4% |  |  | 98.3% |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.8% | -0.2% | -0.5% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | -0.3% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.4% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 28.0% | 39.0% | 39.7% | 24.8% | 33.1% | 36.0% | 24.3% | 34.7% | 39.5% |
| Overall (Ref vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| EL only (Test vs Ref) | -0.6% | -0.2% | -0.2% | 0.4% | 0.5% | 0.5% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107.7% |  |  | 101.6% |  |  | 96.8% |  |  |
| Dec Time[%] | 138.8% |  |  | 135.9% |  |  | 98.2% |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | -0.3% | -0.8% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | -0.8% | -0.2% | -0.2% | -0.3% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.9% | -0.2% | -0.3% | -0.3% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 25.5% | 37.8% | 38.7% | 22.5% | 32.6% | 35.6% | 23.4% | 34.6% | 39.4% |
| Overall (Ref vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | -1.2% | -0.5% | -0.6% | 0.9% | 0.9% | 1.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106.0% |  |  | 106.1% |  |  | 99.5% |  |  |
| Dec Time[%] | 140.9% |  |  | 140.5% |  |  | 98.4% |  |  |

## Decoder Speedup  ̶  PU-based on-the-fly upsampling

The decoder timing information shown in Table 1 is from picture based upsampler and bilateral filter. In addition, we also we also tested PU based upsampler. We implemented PU based upsampler and bilateral filter into both the SHM2.0 reference software and our proposed software. Upsampling is done in xPredInterUni at decoder side only when the Inter Layer picture is used as reference.

Firstly, we compared the decoder speedup of SHM-2.0 with PU based upsampler vs. SHM-2.0 with the existing picture based upsampler. This is shown in Table 2. And it shows about 15% to 20% speed up for inter prediction test cases.  In this test, both anchor and tested bitstreams are decoded on the same speed nodes. Next, we compared of our proposed tool with PU based upsampling vs. SHM2.0 with picture based or PU based upsampler. It can be seen that in PU based upsampling, the decoding run time increase of the proposed tool is less than 10% in most cases.

Table 2. Decoder Timing of SHM2.0 with PU based upsampler vs. SHM2.0 with picture based upsampler

| Dec Time[%] | 2x | 1.5x |
| --- | --- | --- |
| AI | 99.3% | 97.9% |
| RA | 85.2% | 82.9% |
| LD-B | 84.4% | 83.8% |
| LD-P | 81.9% | 82.6% |

Table 3 Decoder Timing of Proposed tool with PU based upsampler/bilateral filter

vs. SHM2.0

|  | Proposed vs. SHM2.0 with default picture based upsampler |  | Proposed vs. SHM2.0 with PU based upsampler |  |
| --- | --- | --- | --- | --- |
| Dec Time[%] | 2x | 1.5x | 2x | 1.5x |
| AI | 99.3% | 97.9% | 110.9% | 110.2% |
| RA | 85.2% | 82.9% | 104.8% | 104.3% |
| LD-B | 84.4% | 83.8% | 104.8% | 104.1% |
| LD-P | 81.9% | 82.6% | 107.3% | 109.3% |

# Complexity assessment

We also report the complexity of the method.  As mentioned above, we propose a window size of 3x1 and the use of a reciprocal LUT for performing the division. The operations shown in equation (3), (4) then become:

Output(Ai,j)=(f(Ai−1,j)∗Ai−1,j+Ai,j+f(Ai+1,j)∗Ai+1,j)∗LUT[f(Ai−1,j)+f(Ai+1,j)]		(5)

Output(bi,j)=(f(Ai,j)∗Ai,j+bi,j+f(Ai+1,j)∗Ai+1,j)∗LUT[f(Ai,j)+f(Ai+1,j)]		(6)

Thus, the process needs 3 multiplication, and 5 additions (including 2 to calculate the gradient) to compute an output value.  Since the operation is separable and applied to both luma and chroma, the worst-case complexity is 9 multiplications and 15 additions for each sample.  To be clear, this is in addition to the existing complexity of the upsampler (37 multiplies and 32 additions per sample).

For memory bandwidth, if using PU based upsampling, there is no image bandwidth change. If using picture based upsampling, due to an additional upsampling picture is added into reference picture list, there are 10 to 20% memory increase for spatial scalability cases.

Table 4. Complexity Assessment for PU based upsampling

Table 5. Complexity Assessment for Picture based upsampling

# Conclusion

In this document, a non-linear and content adaptive bilateral filtering process is proposed.  Average bit-rate reductions (BL+EL) of -0.9%, -0.6%, -0.4% and -0.9% are observed for AI, RA, LD-B and LD-P 2X conditions relative to SHM-2.0 anchors for the S-HEVC test conditions respectively. The proposed tool is combined into the upsampling process with very small computation increase, and no memory bandwidth change if using PU based upsampling.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Zhao, K. Misra, A. Segall, “Non-SCE4: Adaptive up-sampling of base layer picture using Simplified Separable bilateral filters,” JCTVC-M0215, Incheon, KR, Apr. 2013

- C. Tomasi and R. Manduchi, “Bilateral filtering for gray and color images,” IEEE International Conference on Computer Vision, pp.839-846, 1998.

- J. Chen, A. Segall et al, “HEVC Scalable Extensions Core Experiment 3 (SCE3): Inter-layer filtering,” JCTVC-M1103, Incheon, KR, Apr. 2013

- X. Li, J. Boyce et al, “Common SHM test conditions and software reference configurations,” JCTVC-M1009, Incheon, KR, Apr. 2013