| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0222 |
| --- | --- |

| Title: | Non-RCE2: Results for combination of methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Liwei Guo, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal presents the coding performance results obtained by combining techniques proposed in RCE2 and variants of them: rotation of residual and constant context for the significance map, Rice parameter initialization and update, RDPCM for inter and lossy, DC mode replacement and transform skip 8×8. The BD-rate gains for lossless are

|  | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -5.1% | -2.0% | -15.6% | -11.5% | -4.1% |
| RA | -6.1% | -0.7% | -15.6% | -11.9% | -3.0% |
| LB | -7.0% | -0.7% | -16.2% | -13.1% | -3.0% |

For lossy and high-tier, reported average intra BD-rates are -15.0 % for SC RGB444, -9.2% for SC YUV444 and -6.2% for class F, for random access are -12.2% for SC RGB444, -7.1% for SC YUV444, and -4.2% for class F and for low delay are -12.6%, -6.0%, and -2.9%, respectively.

# Technical Description

This contribution provides the results of combining several methods proposed in RCE2 [1] or variants of them. The complexity of these methods is relatively low and two are extensions of methods already present in HEVC RExt:

- Rotation of the residual and constant context (JCTVC-N0044)

  - Setting: Maximum size of the rotation of the residual is set to 4

- Replacement of Intra DC mode (JCTVC-N0053)

- Extension of RDPCM to inter and lossy (JCTVC-N0083)

- Rice Parameter Update and Initialization (JCTVC-N0222)

  - Setting: Applied to transform and transform skip & max Rice equal to 5 with fast update

- Extension of transform skip to 8×8 (JCTVC-N0288)

  - Setting: Fast encoder method

# Results

The proposed combination is implemented in HM10.1_Rext3.0 and simulated under the RCE2 common test conditions Error: Reference source not found. The performance is compared to the anchor for lossless and lossy configurations.

Table 1. Performance for lossless

| Bit-rate saving | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  |  | compression ratio |  |  | compression ratio |  |  |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.6 | -5.1% | 31.7 | 34.8 | -6.1% | 49.8 | 55.6 | -7.0% |
| Class B | 2.2 | 2.3 | -2.0% | 2.6 | 2.6 | -0.7% | 2.6 | 2.6 | -0.7% |
| SC RGB 444 | 10.1 | 12.2 | -15.6% | 100.4 | 124.0 | -15.6% | 381.6 | 493.1 | -16.2% |
| SC YUV 444 | 11.4 | 13.4 | -11.5% | 128.9 | 155.0 | -11.9% | 325.6 | 414.9 | -13.1% |
| RangeExt | 2.4 | 2.4 | -4.1% | 2.5 | 2.6 | -3.0% | 2.5 | 2.6 | -3.0% |
| Enc Time[%] | 88% |  |  | 99% |  |  | 94% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  | 96% |  |  |

Table 2. Performance for lossy

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -5.9% | -6.2% | -6.5% | -6.2% | -6.6% | -6.8% | -6.4% | -6.7% | -6.9% |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| SC RGB 444 | -12.8% | -12.7% | -12.6% | -15.0% | -15.0% | -14.8% | -16.8% | -16.9% | -16.7% |
| SC YUV 444 | -8.4% | -8.5% | -8.6% | -9.2% | -9.8% | -9.8% | -10.3% | -10.9% | -10.9% |
| RangeExt | -0.3% | -0.1% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] | 111% |  |  | 110% |  |  | 108% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 97% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4.2% | -4.3% | -4.5% | -4.2% | -4.2% | -4.4% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% | -0.1% |  |  |  |
| SC RGB 444 | -10.3% | -10.3% | -10.3% | -12.2% | -12.2% | -12.1% |  |  |  |
| SC YUV 444 | -6.5% | -6.8% | -6.7% | -7.1% | -7.7% | -7.6% |  |  |  |
| RangeExt | -0.1% | 0.0% | 0.0% | -0.1% | 0.2% | 0.0% |  |  |  |
| Enc Time[%] | 107% |  |  | 109% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.9% | -3.0% | -2.1% | -2.9% | -3.0% | -2.7% |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.1% |  |  |  |
| SC RGB 444 | -10.6% | -10.6% | -10.8% | -12.6% | -12.6% | -12.6% |  |  |  |
| SC YUV 444 | -5.5% | -5.3% | -5.5% | -6.0% | -6.2% | -6.3% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.2% | 0.1% | 0.3% | 0.4% |  |  |  |
| Enc Time[%] | 109% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |

# References

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” JCTVC-M1122, Incheon, KR, April 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).