| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0223_r1 |
| --- | --- |

| Title: | Non-RCE1: In-loop Chroma Enhancement for HEVC Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye, Yuwen He 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858.210.4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an in-loop chroma enhancement filtering scheme to HEVC Range Extensions, which is performed after SAO and right before the reconstructed picture is added into the DPB. It aims at enhancing the chroma planes of a reconstructed picture, which, if used as a reference picture, also improves the accuracy of future MCP for chroma components. Specifically, a reconstructed chroma pixel is enhanced by adding an appropriate offset obtained by applying the chroma enhancement filters, which usually have high-pass characteristics, on the surrounding luma pixels. By doing this, the chroma edges lost during compression are well restored using the high frequency components from the corresponding luma plane. Experimental results based on common test condition JCTVC-L1006 show that the average {Y, U, V} gain over three color formats (i.e., RGB 4:4:4, YUV 4:4:4, YUV 4:2:2) is {0.0%, -3.0%, -4.9%}, {0.0%, -1.4%, -3.2%}, {0.0%, -0.6%, -1.7%}, {0.2%, -6.0%, -8.4%}, {0.0%, -3.6%, -6.8%}, {-0.1%, -4.6%, -6.7%}, and {-0.2%, -2.7%, -5.1%} for AI-MT, AI-HT, AI-SHT, RA-MT, RA-HT, LB-MT, and LB_HT, respectively.

# Introduction

It is observed that with medium to high QPs, the chroma planes of a reconstructed picture, having much more delicate edges than the luma plane due to very small dynamic range, are severely blurred, whereas the luma plane still has fair to good quality. In order to take advantages of the high correlation among the luma and chroma planes, such as content, structure, and motion, and the better quality of the associated luma plane, we propose to enhance the chroma planes of a reconstructed picture using the corresponding information from the luma plane. Specifically, a reconstructed chroma pixel is enhanced by adding an appropriate offset obtained by applying the chroma enhancement filters, which usually have high-pass characteristics, on the surrounding luma pixels. By doing this, the chroma edges lost during compression are well restored using the high frequency components from the corresponding luma plane. The filter coefficients are trained on the encoder side on a slice basis, and transmitted in the slice header.

The original idea of this chroma enhancement filtering scheme was proposed to SHVC to enhance the chroma planes of an ILR picture [1][2], and in this contribution is extended to the HEVC Range Extensions single layer coding as an in-loop chroma enhancement filtering scheme. Experimental results based on common test condition [3] show that the average {Y, U, V} gain over three color formats (i.e., RGB 4:4:4, YUV 4:4:4, YUV 4:2:2) is {0.0%, -3.0%, -4.9%}, {0.0%, -1.4%, -3.2%}, {0.0%, -0.6%, -1.7%}, {0.2%, -6.0%, -8.4%}, {0.0%, -3.6%, -6.8%}, {-0.1%, -4.6%, -6.7%}, and {-0.2%, -2.7%, -5.1%} for AI-MT, AI-HT, AI-SHT, RA-MT, RA-HT, LB-MT, and LB_HT, respectively.

# Proposed In-loop Chroma Enhancement Filtering Scheme

## Proposed Syntax and Semantics

First, a binary flag sps_chroma_enhancement_filter_enabled_flag is added to SPS to indicate whether the proposed tool is used in the current coded video sequence.

The detailed chroma enhancement filters are signaled in slice header, as shown in Table 1, followed by the semantics.

Table 1 Syntax of the proposed in-loop chroma enhancement filtering in slice header

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ...... |  |
| if (sps_chroma_enhancement_filter_enabled_flag) { |  |
| num_coeff_hori_minus1 | ue( ) |
| num_coeff_verti_minus1 | ue( ) |
| cb_enhancement_flag | u(1) |
| if (cb_enhancement_flag) { |  |
| for ( i=0; i<( (num_coeff_hori_minus1+1)×(num_coeff_verti_minus1+1)-1 ); i++ ) |  |
| cb_filter_coeff_ plus8 [i] | u(4) |
| cb_scaling_factor_abs_minus1 | u(10) |
| cb_scaling_factor_sign | u(1) |
| cb_bit_shifting | u(5) |
| } |  |
| cr_enhancement_flag | u(1) |
| if ( cr_enhancement_flag ) { |  |
| for ( i=0; i<( (num_coeff_hori_minus1+1)×(num_coeff_verti_minus1+1)-1 ); i++ ) |  |
| cr_filter_coeff_ plus8 [i] | u(4) |
| cr_scaling_factor_abs_minus1 | u(10) |
| cr_scaling_factor_sign | u(1) |
| cr_bit_shifting | u(5) |
| } |  |
| } |  |
| ...... |  |
| } |  |

num_coeff_hori_minus1 plus one indicates the number of coefficients in the horizontal direction of the chroma enhancement filters.

num_coeff_verti_minus1 plus one indicates the number of coefficients in the vertical direction of the chroma enhancement filters.

cb_enhancement_flag equal to 1 specifies that cb_filter_coeff_ plus8 [i], cb_scaling_factor_abs_minus1, cb_scaling_factor_sign, and cb_bit_shifting are present in the slice header. cb_enhancement_flag equal to 0 specifies that cb_filter_coeff_ plus8 [i], cb_scaling_factor_abs_minus1, cb_scaling_factor_sign, and cb_bit_shifting are not present in the slice header. When not present, cb_enhancement_flag is inferred to be equal to 0.

cb_filter_coeff_ plus8[i] minus 8 specifies the i-th filter coefficient of the high pass filter for the chroma component Cb. The value of cb_filter_coeff_ plus8 [i] shall be in the range of 0 to 15, inclusive.

cb_scaling_factor_abs_minus1 plus 1 and cb_scaling_factor_sign together specify the value of variable CbScalingFactor as follows:

CbScalingFactor = (1 – 2 * cb_scaling_factor_sign) * (cb_scaling_factor_abs_minus1+1)

The value of cb_scaling_factor_abs shall be in the range of 1 to 1023, inclusive.

cb_bit_shifting specifies the number of bits to be right shifted after the scaling process to enhance the chroma component Cb. The value of cb_bit_shifting shall be in the range of 0 to 31, inclusive.

cr_enhancement_flag equal to 1 specifies that cr_filter_coeff_ plus8 [i], cr_scaling_factor_abs_minus1, cr_scaling_factor_sign, and cr_bit_shifting are present in the slice header. cr_enhancement_flag equal to 0 specifies that cr_filter_coeff_ plus8 [i], cr_scaling_factor_abs_minus1, cr_scaling_factor_sign, and cr_bit_shifting are not present in the slice header. When not present, cr_enhancement_flag is inferred to be equal to 0.

cr_filter_coeff_ plus8[i] minus 8 specifies the i-th filter coefficient of the high pass filter for the chroma component Cr. The value of cr_filter_coeff_ plus8 [i] shall be in the range of 0 to 15, inclusive.

cr_scaling_factor_abs_minus1 plus 1 and cr_scaling_factor_sign together specify the value of variable CrScalingFactor as follows:

CrScalingFactor = (1 – 2 * cr_scaling_factor_sign) * (cr_scaling_factor_abs_minus1+1)

The value of cr_scaling_factor_abs shall be in the range of 1 to 1023, inclusive.

cr_bit_shifting specifies the number of bits to be right shifted after the scaling process to enhance the chroma component Cr. The value of cr_bit_shifting shall be in the range of 0 to 31, inclusive.

## Process of Chroma Enhancement

The chroma enhancement filter coefficients are trained on the encoder side using least minimum mean square error (LMMSE) estimator, such that the error energy between the enhanced reconstructed chroma planes and the original chroma planes are minimized. Interested readers are referred to [1] and [2] for more details.

Without loss of generality, only the enhancement for Cb plane of the reconstructed picture is introduced.

Denote the filter taps in the horizontal and vertical directions as TapHor (equal to num_coeff_hori_minus1+1) and TapVer (equal to num_coeff_verti_minus1+1), respectively. To enhance a Cb pixel with position (x,y), high-pass filter cb_filter_coeff_plus8 is first applied to the surrounding TapHor×TapVer luma pixels to generate the intermediate result z(x,y), as below,

z(x,y)=endV

where the four parameters startV, startH, endV, and endH determine the support region of filtering relative to (x,y), and are equal to 1-TapVer+endV, 1-TapHor+endH, TapVer>>1, and TapHor>>1, respectively. (ComponentScaleX, ComponentScaleY) is the color subsampling ratio in the horizontal and vertical directions, and is equal to (1, 1), (2, 1), and (2, 2) for 4:4:4, 4:2:2, and 4:2:0 formats, respectively.

Then, z(x,y) is scaled by CbScalingFactor/2cb_bit_shifting to the normal range, denoted as o(x,y). The integer realization of this scaling step is shown as below.

o(x,y)=Sign(z(x,y)*CbScalingFactor)*((Abs(z(x,y)*CbScalingFactor)+(1<<(cb_bit_shifting-1)))>>cb_bit_shifting)

The enhanced Cb pixel, denoted as Cbenh(x,y), is obtained by Cbenh(x,y) = Cb(x,y) + o(x,y).

# Experimental Results

The proposed in-loop chroma enhancement processing is implemented in HM-10.1+RExt-3.0 and the performance is tested under the common test condition [3]. Two sets of experiments were conducted, where the filter support region TapHor×TapVer is equal to 5×5 and 3×3, respectively. The former has larger overhead and higher complexity, including arithmetic operations and memory access, but better performance, whereas the latter has smaller overhead and lower complexity, but less gain. The summaries of the R-D performances of the two trade-offs are shown in Table 2 and Table 3. The detailed performance for each sequence, configuration, and color formats can be found in the enclosed spreadsheets “JCTVC-L1006_r1_5x5tap.xlsm” and “JCTVC-L1006_r1_3x3tap.xlsm”. Please note that the comparison of the encoding/decoding time is not reliable, because the anchor time is provided by the software coordinator and the tested time is generated from the proponent’s cluster.

Table 2 BD-rate reduction provided by 5×5 filter compared with HM-10.1+RExt-3.0

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | -0.9% | -1.8% | 0.0% | -0.5% | -1.1% | 0.0% | -0.2% | -0.6% |
| YCbCr 4:4:4 | 0.0% | -3.0% | -6.0% | 0.0% | -1.8% | -4.8% | 0.0% | -0.9% | -3.1% |
| YCbCr 4:2:2 | 0.0% | -5.2% | -7.0% | 0.0% | -2.1% | -3.7% | 0.0% | -0.7% | -1.5% |
| Enc Time[%] | 157% |  |  | 152% |  |  | 150% |  |  |
| Dec Time[%] | 118% |  |  | 118% |  |  | 120% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.2% | -4.4% | -7.6% | -0.2% | -2.6% | -5.3% |  |  |  |
| YCbCr 4:4:4 | 0.6% | -4.3% | -8.4% | 0.2% | -3.3% | -8.3% |  |  |  |
| YCbCr 4:2:2 | 0.2% | -9.2% | -9.3% | 0.0% | -4.9% | -6.8% |  |  |  |
| Enc Time[%] | 120% |  |  | 119% |  |  |  |  |  |
| Dec Time[%] | 123% |  |  | 121% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -1.3% | -4.1% | -7.1% | -0.7% | -2.2% | -4.6% |  |  |  |
| YCbCr 4:4:4 | 0.7% | -3.0% | -7.1% | 0.0% | -2.0% | -6.5% |  |  |  |
| YCbCr 4:2:2 | 0.2% | -6.7% | -5.8% | 0.0% | -3.8% | -4.2% |  |  |  |
| Enc Time[%] | 120% |  |  | 120% |  |  |  |  |  |
| Dec Time[%] | 128% |  |  | 128% |  |  |  |  |  |

Table 3 BD-rate reduction provided by 3×3 filter compared with HM-10.1+RExt-3.0

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | -0.8% | -1.3% | 0.0% | -0.5% | -0.9% | 0.0% | -0.2% | -0.5% |
| YCbCr 4:4:4 | 0.0% | -2.0% | -5.2% | 0.0% | -1.2% | -4.3% | 0.0% | -0.7% | -2.9% |
| YCbCr 4:2:2 | 0.0% | -3.6% | -5.5% | 0.0% | -1.4% | -2.7% | 0.0% | -0.5% | -1.0% |
| Enc Time[%] | 121% |  |  | 121% |  |  | 124% |  |  |
| Dec Time[%] | 111% |  |  | 111% |  |  | 116% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.3% | -4.3% | -6.8% | -0.3% | -2.6% | -4.9% |  |  |  |
| YCbCr 4:4:4 | 0.3% | -3.0% | -6.8% | 0.1% | -2.1% | -6.4% |  |  |  |
| YCbCr 4:2:2 | 0.1% | -7.3% | -7.4% | 0.0% | -3.8% | -5.0% |  |  |  |
| Enc Time[%] | 119% |  |  | 119% |  |  |  |  |  |
| Dec Time[%] | 121% |  |  | 120% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -1.1% | -3.5% | -6.0% | -0.7% | -2.0% | -3.9% |  |  |  |
| YCbCr 4:4:4 | 0.4% | -1.8% | -4.8% | 0.0% | -1.2% | -4.4% |  |  |  |
| YCbCr 4:2:2 | 0.1% | -3.6% | -3.3% | 0.0% | -2.3% | -2.4% |  |  |  |
| Enc Time[%] | 121% |  |  | 120% |  |  |  |  |  |
| Dec Time[%] | 120% |  |  | 118% |  |  |  |  |  |

# Conclusion

This contribution proposes an in-loop chroma enhancement filtering scheme to HEVC Range Extensions, which is performed after SAO and right before the reconstructed picture is added into the DPB. It aims at enhancing the chroma planes of a reconstructed picture, which, if used as a reference picture, also improves the accuracy of future MCP for chroma components. Specifically, a reconstructed chroma pixel is enhanced by adding an appropriate offset obtained by applying the chroma enhancement filters, which usually have high-pass characteristics, on the surrounding luma pixels. By doing this, the chroma edges lost during compression are well restored using the high frequency components from the corresponding luma plane. Experimental results based on common test condition [3] show that the average {Y, U, V} gain over three color formats (i.e., RGB 4:4:4, YUV 4:4:4, YUV 4:2:2) is {0.0%, -3.0%, -4.9%}, {0.0%, -1.4%, -3.2%}, {0.0%, -0.6%, -1.7%}, {0.2%, -6.0%, -8.4%}, {0.0%, -3.6%, -6.8%}, {-0.1%, -4.6%, -6.7%}, and {-0.2%, -2.7%, -5.1%} for AI-MT, AI-HT, AI-SHT, RA-MT, RA-HT, LB-MT, and LB_HT, respectively. We suggest further studying this technique in RExt core experiment (RCE).

# Reference

- J. Dong, Y. He, Y. Ye, “Chroma enhancement for ILR picture,” JCTVC-L0059, Geneva, Switzerland, Jan. 2013.

- J. Dong, Y. He, Y. Ye, “SCE4: Results of test 4.2.4 on chroma enhancement for inter layer prediction,” JCTVC-M0183, Incheon, South Korea, April 2013.

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-L1006, Geneva, Switzerland, Jan. 2013.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).