| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0224_r1 |
| --- | --- |

| Title: | SEI message: post filters to enhance the chroma planes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye, Yuwen He 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858.210.4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an SEI message carrying post filter coefficients, in order to enhance the quality of the reconstructed chroma planes. On the decoder side, a reconstructed chroma pixel is enhanced by adding an appropriate offset obtained by applying the received post filters, which usually have high-pass characteristics, on the surrounding luma pixels. By doing this, the chroma edges lost during compression are well restored using the high frequency components from the corresponding luma plane. The proposed SEI message is implemented in HM-10.1 and HEVC RExt. software “HM-10.1+RExt-3.0”. Compared with HM-10.1 anchor and HEVC RExt. anchor, significant chroma PSNR improvement, e.g., up to 1.5 dB, can be observed in a wide range of sequences, including Class A, Class B, screen contest sequences, and YUV sequences with higher bit-depth (10-bit) and color sampling ratios (4:4:4 and 4:2:2).

# Introduction

It is observed that with medium to high QPs, the chroma planes of a reconstructed picture, having much more delicate edges than the luma plane due to very small dynamic range, are severely blurred, whereas the luma plane still has fair to good quality. In order to take advantages of the high correlation among the luma and chroma planes, such as content, structure, and motion, and the better quality of the associated luma plane, we propose to enhance the chroma planes of a reconstructed picture using the corresponding information from the luma plane. Specifically, we propose a new SEI message carrying post filters with high-pass characteristics, which are received and applied to the surrounding luma pixels of the chroma pixel to be enhanced. The output value of high-pass filtering is then added as an offset to the chroma pixel to be enhanced. The filter coefficients are trained on the encoder side, and transmitted in the proposed SEI message on a picture basis.

The original idea of this chroma enhancement scheme was proposed to SHVC to enhance the chroma planes of an ILR picture [1][2], and is extended to the HEVC single layer coding as a post-processing scheme, where the filter coefficients are transmitted as a proposed SEI message. The proposed SEI message is implemented in HM-10.1 and HEVC RExt. software “HM-10.1+RExt-3.0”. Compared with HM-10.1 anchor and HEVC RExt. anchor, significant chroma PSNR improvement, e.g., up to 1.5 dB, can be observed in a wide range of sequences, including Class A, Class B, screen contest sequences, and YUV sequences with higher bit-depth (10-bit) and color sampling ratios (4:4:4 and 4:2:2).

# Proposed SEI Message and Application

## Proposed SEI Message Syntax and Semantics

The proposed SEI message syntax is shown in Table 1, followed by the semantics.

Table 1 Syntax of the proposed SEI message chroma_enhancement_filter ( )

| chroma_enhancement_filter (payloadSize ) { | Descriptor |
| --- | --- |
| chroma_enhancement_filter_enabled_flag | u(1) |
| if (chroma_enhancement_filter_enabled_flag) { |  |
| num_coeff_hori_minus1 | ue( ) |
| num_coeff_verti_minus1 | ue( ) |
| cb_enhancement_flag | u(1) |
| if (cb_enhancement_flag) { |  |
| for ( i=0; i<( (num_coeff_hori_minus1+1)×(num_coeff_verti_minus1+1)-1 ); i++ ) |  |
| cb_filter_coeff_ plus8 [i] | u(4) |
| cb_scaling_factor_abs_minus1 | u(10) |
| cb_scaling_factor_sign | u(1) |
| cb_bit_shifting | u(5) |
| } |  |
| cr_enhancement_flag | u(1) |
| if ( cr_enhancement_flag ) { |  |
| for ( i=0; i<( (num_coeff_hori_minus1+1)×(num_coeff_verti_minus1+1)-1 ); i++ ) |  |
| cr_filter_coeff_ plus8 [i] | u(4) |
| cr_scaling_factor_abs_minus1 | u(10) |
| cr_scaling_factor_sign | u(1) |
| cr_bit_shifting | u(5) |
| } |  |
| } |  |
| } |  |

chroma_enhancement_filter_enabled_flag equal to 1 specifies that chroma enhancement is enabled. chroma_enhancement_filter_enabled_flag equal to 0 specifies that chroma enhancement is disabled. When not present, chroma_enhancement_filter_enabled_flag is inferred to be equal to 0.

num_coeff_hori_minus1 plus one indicates the number of coefficients in the horizontal direction of the chroma enhancement filters.

num_coeff_verti_minus1 plus one indicates the number of coefficients in the vertical direction of the chroma enhancement filters.

cb_enhancement_flag equal to 1 specifies that cb_filter_coeff_ plus8 [i], cb_scaling_factor_abs_minus1, cb_scaling_factor_sign, and cb_bit_shifting are present in the SEI message. cb_enhancement_flag equal to 0 specifies that cb_filter_coeff_ plus8 [i], cb_scaling_factor_abs_minus1, cb_scaling_factor_sign, and cb_bit_shifting are not present in the SEI message. When not present, cb_enhancement_flag is inferred to be equal to 0.

cb_filter_coeff_ plus8[i] minus 8 specifies the i-th filter coefficient of the high pass filter for the chroma component Cb. The value of cb_filter_coeff_ plus8 [i] shall be in the range of 0 to 15, inclusive.

cb_scaling_factor_abs_minus1 plus 1 and cb_scaling_factor_sign together specify the value of variable CbScalingFactor as follows:

CbScalingFactor = (1 – 2 * cb_scaling_factor_sign) * (cb_scaling_factor_abs_minus1+1)

The value of cb_scaling_factor_abs shall be in the range of 1 to 1023, inclusive.

cb_bit_shifting specifies the number of bits to be right shifted after the scaling process to enhance the chroma component Cb. The value of cb_bit_shifting shall be in the range of 0 to 31, inclusive.

cr_enhancement_flag equal to 1 specifies that cr_filter_coeff_ plus8 [i], cr_scaling_factor_abs_minus1, cr_scaling_factor_sign, and cr_bit_shifting are present in the SEI message. cr_enhancement_flag equal to 0 specifies that cr_filter_coeff_ plus8 [i], cr_scaling_factor_abs_minus1, cr_scaling_factor_sign, and cr_bit_shifting are not present in the SEI message. When not present, cr_enhancement_flag is inferred to be equal to 0.

cr_filter_coeff_ plus8[i] minus 8 specifies the i-th filter coefficient of the high pass filter for the chroma component Cr. The value of cr_filter_coeff_ plus8 [i] shall be in the range of 0 to 15, inclusive.

cr_scaling_factor_abs_minus1 plus 1 and cr_scaling_factor_sign together specify the value of variable CrScalingFactor as follows:

CrScalingFactor = (1 – 2 * cr_scaling_factor_sign) * (cr_scaling_factor_abs_minus1+1)

The value of cr_scaling_factor_abs shall be in the range of 1 to 1023, inclusive.

cr_bit_shifting specifies the number of bits to be right shifted after the scaling process to enhance the chroma component Cr. The value of cr_bit_shifting shall be in the range of 0 to 31, inclusive.

## Chroma Enhancement Using the Proposed SEI Message

Without loss of generality, only the enhancement for Cb plane of the reconstructed picture is introduced.

Denote the filter taps in the horizontal and vertical directions as TapHor (equal to num_coeff_hori_minus1+1) and TapVer (equal to num_coeff_verti_minus1+1), respectively. To enhance a Cb pixel with position (x,y), high-pass filter cb_filter_coeff_plus8 is first applied to the surrounding TapHor×TapVer luma pixels to generate the intermediate result z(x,y), as below,

z(x,y)=endV

where the four parameters startV, startH, endV, and endH determine the support region of filtering relative to (x,y), and are equal to 1-TapVer+endV, 1-TapHor+endH, TapVer>>1, and TapHor>>1, respectively. (ComponentScaleX, ComponentScaleY) is the color subsampling ratio in the horizontal and vertical directions, and is equal to (1, 1), (2, 1), and (2, 2) for 4:4:4, 4:2:2, and 4:2:0 formats, respectively.

Then, z(x,y) is scaled by CbScalingFactor/2cb_bit_shifting to the normal range, denoted as o(x,y). The integer realization of this scaling step is shown as below.

o(x,y)=Sign(z(x,y)*CbScalingFactor)*((Abs(z(x,y)*CbScalingFactor)+(1<<(cb_bit_shifting-1)))>>cb_bit_shifting)

The enhanced Cb pixel, denoted as Cbenh(x,y), is obtained by Cbenh(x,y) = Cb(x,y) + o(x,y).

# Experimental Results

The proposed SEI message is implemented in HM-10.1 and HEVC RExt. software “HM-10.1+RExt-3.0” and used to enhance the chroma planes of the reconstructed picture. The performance is tested under the common test conditions [3] and [4]. The experimental results show that the proposed SEI message and the corresponding post-processing increases the chroma PSNR significantly, e.g., up to 1.5 dB, for a wide range of sequences, including Class A, Class B, screen content sequences, and YUV sequences with higher bit-depth (10-bit) and color sampling ratios (4:4:4 and 4:2:2).

Fig. 1 and Fig. 2 show the chroma PSNR gain provided by using the proposed SEI message. As can be seen, for both Cb and Cr planes of SlideEditing and Nebuta, the gain is up to 1.5 dB.

(a)

(b)

Fig. 1 Operational R-D curves of screen content sequence SlideEditing of (a) Cb plane and (b) Cr plane.

(a)

(b)

Fig. 2 Operational R-D curves of Class A 10-bit sequence Nebuta of (a) Cb plane and (b) Cr plane.

Fig. 3 and Fig. 4 show the difference of the chroma planes before and after the proposed chroma enhancement is applied. Fig. 3(c) and Fig. 4(c) are the reconstructed Cr planes coded at QP equal to 32, and their enhanced versions are shown in Fig. 3(d) and Fig. 4(d), respectively. As can be seen, with the chroma enhancement applied, edges are better restored, especially those areas highlighted by black lines.

(a)                                                                                  (b)

(c)                                                                                      (d)

Fig. 3 Cr plane of the first picture in Kimono with QP=32. (a) Original Image; (b) Original Cr plane; (c) reconstructed Cr plane without chroma enhancement; (d) reconstructed Cr plane with chroma enhancement.

(a)                                                                                        (b)

(c)                                                                                     (d)

Fig. 4 Cr plane of the first picture in ParkScene with QP=32. (a) Original Image; (b) Original Cr plane; (c) reconstructed Cr plane without chroma enhancement; (d) reconstructed Cr plane with chroma enhancement.

In order to show that the proposed SEI message can efficiently improve the chroma quality on a great variety of test sequences, and is useful for HEVC and its range extensions, we convert the PSNR gain into BD-rate, the common measure to evaluate the performance of a coding tool, and summarize the results in Table 2 to Table 5.

Table 2 Pseudo BD-rate reduction under Main coding configuration compared with HM-10.1

|  |  | All Intra (Main) |  | Random Access (Main) |  | Low Delay B (Main) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cb | Cr | Cb | Cr | Cb | Cr |
| Class A | Traffic | -5.6% | -7.8% | -12.0% | -11.0% |  |  |
|  | PeopleOnStreet | -11.7% | -2.0% | -15.3% | -3.0% |  |  |
|  | Nebuta | -36.8% | -34.4% | -73.8% | -73.3% |  |  |
|  | SteamLocomotive | -48.1% | -62.3% | -61.5% | -76.9% |  |  |
|  | Average | -25.5% | -26.7% | -40.7% | -41.0% |  |  |
| Class B | Kimono | -6.8% | -12.6% | -8.9% | -17.8% | -8.9% | -19.3% |
|  | ParkScene | -7.7% | -13.0% | -11.2% | -17.3% | -12.4% | -21.0% |
|  | Cactus | -7.2% | -9.4% | -13.9% | -14.4% | -14.2% | -15.1% |
|  | BasketballDrive | -5.9% | -1.9% | -9.8% | -3.3% | -10.0% | -3.4% |
|  | BQTerrace | -8.5% | -5.2% | -19.6% | -11.0% | -19.6% | -13.6% |
|  | Average | -7.2% | -8.4% | -12.7% | -12.8% | -13.0% | -14.5% |

Table 3 Pseudo BD-rate reduction under Main10 coding configuration compared with HM-10.1

|  |  | All Intra (Main10) |  | Random Access (Main10) |  | Low Delay B (Main10) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cb | Cr | Cb | Cr | Cb | Cr |
| Class A | Traffic | -6.1% | -9.3% | -12.6% | -13.0% |  |  |
|  | PeopleOnStreet | -12.8% | -2.9% | -16.3% | -4.1% |  |  |
|  | Nebuta | -36.6% | -33.8% | -74.4% | -73.8% |  |  |
|  | SteamLocomotive | -48.2% | -58.4% | -62.4% | -74.3% |  |  |
|  | Average | -25.9% | -26.1% | -41.4% | -41.3% |  |  |
| Class B | Kimono | -8.8% | -14.5% | -11.4% | -19.9% | -10.9% | -21.1% |
|  | ParkScene | -8.9% | -14.1% | -12.7% | -18.6% | -13.8% | -22.1% |
|  | Cactus | -8.1% | -10.4% | -15.2% | -15.6% | -15.0% | -16.1% |
|  | BasketballDrive | -7.0% | -2.8% | -11.2% | -4.1% | -11.1% | -4.1% |
|  | BQTerrace | -9.6% | -7.1% | -21.9% | -14.9% | -22.9% | -17.9% |
|  | Average | -8.5% | -9.8% | -14.5% | -14.6% | -14.7% | -16.3% |

Table 4 Pseudo BD-rate reduction with 5×5 filter size compared with HEVC RExt.

|  |  | All Intra Main-tier |  | Random Access Main-tier |  | Low delay B Main-tier |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cb | Cr | Cb | Cr | Cb | Cr |
| YUV 4:4:4 | Traffic | -1.80% | -7.60% | -3.00% | -9.40% | -3.20% | -10.60% |
|  | Kimono | -2.70% | -7.30% | -6.20% | -10.90% | -5.40% | -11.80% |
|  | EBULupoCandlelight | -7.80% | -1.90% | -10.10% | -2.70% | -11.50% | -2.60% |
|  | EBURainFruits | -3.60% | -7.90% | -4.30% | -10.60% | -4.20% | -10.60% |
|  | BirdsInCage | -0.80% | -13.20% | -2.30% | -22.80% | -2.10% | -29.50% |
|  | CrowdRun | -2.70% | -3.40% | -5.10% | -5.20% | -5.10% | -5.10% |
|  | Average | -3.23% | -6.88% | -5.17% | -10.27% | -5.25% | -11.70% |
| YUV 4:2:2 | Traffic | -3.30% | -9.90% | -6.00% | -13.40% | -6.00% | -14.80% |
|  | Kimono | -6.90% | -12.20% | -11.50% | -16.70% | -10.80% | -17.50% |
|  | EBUHorse | -3.50% | -2.70% | -6.70% | -4.50% | -8.10% | -5.30% |
|  | EBUWaterRocksClose | -3.30% | -10.00% | -14.10% | -22.10% | -11.50% | -18.60% |
|  | EBUKidsSoccer | -12.80% | -7.80% | -22.20% | -13.80% | -29.50% | -18.80% |
|  | Seeking | -6.40% | -5.60% | -10.00% | -8.40% | -10.40% | -8.60% |
|  | Average | -6.03% | -8.03% | -11.75% | -13.15% | -12.72% | -13.93% |

Table 5 Pseudo BD-rate reduction with 3×3 filter size compared with HEVC RExt.

|  |  | All Intra Main-tier |  | Random Access Main-tier |  | Low delay B Main-tier |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cb | Cr | Cb | Cr | Cb | Cr |
| YUV 4:4:4 | Traffic | -1.30% | -5.70% | -2.20% | -6.80% | -2.30% | -7.70% |
|  | Kimono | -2.00% | -6.10% | -4.40% | -9.20% | -3.80% | -9.60% |
|  | EBULupoCandlelight | -3.60% | -1.50% | -5.00% | -2.20% | -6.30% | -1.90% |
|  | EBURainFruits | -2.90% | -7.90% | -4.00% | -11.10% | -3.70% | -10.90% |
|  | BirdsInCage | -0.70% | -11.20% | -2.00% | -18.20% | -1.70% | -23.60% |
|  | CrowdRun | -2.10% | -2.80% | -4.10% | -4.30% | -4.00% | -4.30% |
|  | Average | -2.10% | -5.87% | -3.62% | -8.63% | -3.63% | -9.67% |
| YUV 4:2:2 | Traffic | -2.40% | -7.10% | -4.10% | -9.50% | -4.40% | -10.60% |
|  | Kimono | -5.00% | -10.00% | -8.30% | -13.50% | -7.70% | -14.00% |
|  | EBUHorse | -2.60% | -2.00% | -5.00% | -3.30% | -6.10% | -3.80% |
|  | EBUWaterRocksClose | -0.10% | -0.40% | -0.10% | -0.30% | -0.20% | -0.50% |
|  | EBUKidsSoccer | -2.60% | -9.00% | -11.30% | -19.80% | -9.10% | -16.60% |
|  | Seeking | -9.30% | -5.80% | -16.20% | -10.10% | -21.80% | -14.10% |
|  | Average | -3.50% | -4.60% | -5.60% | -7.00% | -5.80% | -7.10% |

# Conclusion

This contribution proposes an SEI message carrying post filter coefficients, in order to enhance the quality of the reconstructed chroma planes. On the decoder side, a reconstructed chroma pixel is enhanced by adding an appropriate offset obtained by applying the received post filters, which usually have high-pass characteristics, on the surrounding luma pixels. By doing this, the chroma edges lost during compression are well restored using the high frequency components from the corresponding luma plane. The proposed SEI message is implemented in HM-10.1 and HEVC RExt. software “HM-10.1+RExt-3.0”. Compared with HM-10.1 anchor and HEVC RExt. anchor, significant chroma PSNR improvement, e.g., up to 1.5 dB, can be observed in a wide range of sequences, including Class A, Class B, screen contest sequences, and YUV sequences with higher bit-depth (10-bit) and color sampling ratios (4:4:4 and 4:2:2).  We suggest adopting this SEI message into HEVC.

Reference

- J. Dong, Y. He, Y. Ye, “Chroma enhancement for ILR picture,” JCTVC-L0059, Geneva, Switzerland, Jan. 2013.

- J. Dong, Y. He, Y. Ye, “SCE4: Results of test 4.2.4 on chroma enhancement for inter layer prediction,” JCTVC-M0183, Incheon, South Korea, April 2013.

- F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-L1100, Geneva, Switzerland, Jan. 2013.

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-L1006, Geneva, Switzerland, Jan. 2013.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).