| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0227 |
| --- | --- |

| Title: | RCE1: EXP1/4 The performance of extended chroma mode for non 4:2:0 format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Email: | jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution reports the performance of chroma prediction which predicts chroma samples using linear combination of luma samples for non 4:2:0 formats in the scope of RCE1. It includes two experimental results in RCE1, one of which (Experiment 1) utilized luma samples correspond to chroma samplings and the other (Experiment 2) always utilized luma reference samples in the closes column. RCE1 Experiment 1 shows gains up to 10.3%, 14.2%, 15.2% in average over all RGB and YUV sequences. In the case of Random access, it shows 5.3%~6.8% gains for luma component and upto 10.8%, 11.9% gains for Cb and Cr component. In the case of Lowdelay B, it shows 2.6%~4.6% gains for luma component and 6.2%, 7.2% gains for Cb and Cr component. RCE1 Experiment 4 which utilized luma reference samples used for luma prediction shows gains in average over all RGB and YUV sequences up to 10.3%, 14.1%, 15.1% with respect to Y, Cb and Cr in the case of All Intra. In the case of Random access it shows 5.7%~6.8% gains for luma component and up 10.7% and 11.9% for Cb and Cr components. In Lowdelay B cases, it shows 2.6%~4.6% gains for luma component and up to 6.2% and 7.1% for Cb and Cr components.

# Introduction

For HEVC and HEVC range extension, chroma prediction based on the linear combination of luma samples which was called LM chroma in short was proposed [1-7]. It utilized the relations between chroma samples and corresponding luma samples and estimated the chroma samples by linear combination of the luma samples. Since the chroma samples for the current block are not known for decoder, the parameters for the linear combination are calculated with the neighbouring samples.

In case of 4:2:0 sampling, chroma samples are predicted as as follows.

predC[x,y]=α×(PL[2x,2y]+PL[2x,2y+1])>>1+β	(1),

where PL indicates luma samples.

The derivation of the parameter α and β can be described as follows.

α=R(PL,P'C)R(PL,PL)β=M(P'C)−α×M(PL)	(2),

where P'c indicates reconstructed chroma samples around the target block. And PL indicates the luma samples considering the phase difference between luma and chroma samples, which is derived by

PL[x,y]=(PL[2x,2y]+PL[2x,2y+1])>>1	(3).

Fig. 1 depicts chroma samples and luma samples used for the prediction and the calculation in the case of 4:2:0 sampling. The locations of the samples used for the derivation of α and β are shown as gray circles and the location of luma samples used for the prediction of chroma samples are shown as red circles.

Fig. 1 chroma and luma samples involved in LM chroma prediction

Since chroma prediction is done by TU, these parameters α and β are also derived by each TU with the neighbours around the TU.

# Proposed Method

We propose to use the extended chroma prediction using linear combination of luma samples (LM chroma) for non 4:2:0 sampling. The basic algorithm remains as before regardless of the samplings but the samples involved should be changed.

## Experiment 1 : with luma reference samples correspond to chroma samplings

In experiment 1, left luma samples used for the parameter calculation in different columns according to the chroma samplings. For YUV 4:2:2 format, the luma samples from two-column-left column and closest row are used considering luma-chroma sampling difference. Fig. 2 and Fig. 3 depict the locations of samples involved in the parameter derivation for YUV 4:2:2 format and YUV 4:4:4 or RGB format. Blue circles indicates the location of luma and chroma samples used for the parameter derivation. Since YUV 4:4:4 format and RGB format do not require sampling, they use luma samples in the closest column.

Fig. 2 chroma and luma samples involved in LM chroma prediction for 4:2:2 sampling

Fig. 3 reference samples for the parameter derivation in the case of 4:4:4 sampling or RGB format

## Experiment 4 : with luma reference samples closest to the current block

In experiment 4, luma samples used for the parameter calculation are always in the closest column to the current block. The luma samples are same as those used for the prediction of luma samples in the current block. Fig. 4 and Fig. 5 depict luma samples used for the prediction for 4:2:2 sampling and 4:4:4 sampling. Blue circles indicate reference samples. Even when the locations of chroma samples are 1:2 downsampled positions of luma samples for 4:2:2 format, the column of luma reference samples used for the parameter derivation is the closest column to the current block.

Fig. 4 reference samples for the parameter derivation in the case of 4:2:2 sampling

Fig. 5 reference samples for the parameter derivation in the case of 4:4:4 sampling or RGB format

# Experimental Results

The experimental result is shown below. The proposed method is implemented on top of HM10.1-range-extensions ver.3 provided. The detailed results are described in the accompanied excel files. The anchor of the test is HM10.1-range-extensions ver. 3 and the chroma prediction is used in all of Main, High and Super High Tier configuration as specified in [8]. Also, the results of additional QP set of {QP7, QP12, QP17, QP22} are included. PSNRs in YUV domain are calculated with YUV sequences converted from RGB sequences. The conversion equation is provided by RCE1 coordinator. The encoding time and the decoding time are not reliable since the tests are performed on the linux cluster.

## Experiment 1

Experiment 1 shows gains up to 10.3%, 14.2%, 15.2% with respect to Luma, chroma Cb and Cr in average in the case of All Intra. It shows gains up to 5.7%, 10.8%, 11.9% and 2.6%, 6.2%, 7.2% with respect to Random access and Lowdelay B case.

|  |  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AIMT | RExt | -7.5% | -12.2% | -13.8% | -19.7% | -17.9% | -18.7% | -8.1% | -18.8% | -18.6% |
|  | SC | -13.8% | -16.6% | -16.9% | -25.4% | -24.5% | -24.7% | -16.0% | -25.3% | -25.2% |
|  | Overall | -10.3% | -14.2% | -15.2% | -22.7% | -21.4% | -21.9% | -11.6% | -22.2% | -22.1% |
|  | Enc Time[%] | 115% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |
| AIHT | RExt | -6.2% | -8.6% | -10.9% | -15.4% | -14.4% | -15.0% | -6.6% | -15.0% | -15.0% |
|  | SC | -15.3% | -17.3% | -17.7% | -24.4% | -23.5% | -23.7% | -16.1% | -24.4% | -24.4% |
|  | Overall | -9.9% | -12.1% | -13.6% | -19.9% | -19.0% | -19.4% | -10.5% | -19.7% | -19.7% |
|  | Enc Time[%] | 115% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |
| AIST | RExt | -4.8% | -6.0% | -8.0% | -11.5% | -10.7% | -11.3% | -5.1% | -11.2% | -11.2% |
|  | SC | -14.5% | -16.2% | -16.6% | -22.2% | -21.3% | -21.5% | -15.1% | -22.1% | -22.0% |
|  | Overall | -8.7% | -10.1% | -11.5% | -16.8% | -16.0% | -16.4% | -9.2% | -16.6% | -16.6% |
|  | Enc Time[%] | 117% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |
| AIUT | RExt | -3.7% | -4.3% | -5.7% | -8.8% | -8.2% | -8.5% | -4.0% | -8.6% | -8.6% |
|  | SC | -13.6% | -14.9% | -15.2% | -20.7% | -20.1% | -20.1% | -14.2% | -20.7% | -20.6% |
|  | Overall | -7.7% | -8.6% | -9.5% | -14.8% | -14.2% | -14.3% | -8.1% | -14.6% | -14.6% |
|  | Enc Time[%] | 118% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |
|  |  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| RAMT | RExt | -3.9% | -10.0% | -11.2% | -11.1% | -9.8% | -11.8% | -4.0% | -9.5% | -9.3% |
|  | SC | -8.4% | -12.0% | -13.0% | -20.0% | -19.2% | -19.4% | -13.4% | -19.9% | -19.9% |
|  | Overall | -5.7% | -10.8% | -11.9% | -15.6% | -14.5% | -15.6% | -7.8% | -14.7% | -14.6% |
|  | Enc Time[%] | 113% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 130% |  |  |  |  |  |  |  |  |
| RAHT | RExt | -2.8% | -5.7% | -8.7% | -7.9% | -6.3% | -8.5% | -2.9% | -6.7% | -6.5% |
|  | SC | -12.6% | -15.3% | -15.8% | -20.4% | -19.6% | -19.7% | -13.7% | -20.3% | -20.3% |
|  | Overall | -6.8% | -9.6% | -11.6% | -14.1% | -12.9% | -14.1% | -7.3% | -13.5% | -13.4% |
|  | Enc Time[%] | 116% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 127% |  |  |  |  |  |  |  |  |
| RAST | RExt | -1.8% | -3.0% | -5.8% | -5.0% | -3.9% | -5.3% | -1.9% | -4.3% | -4.2% |
|  | SC | -12.4% | -14.3% | -14.7% | -19.3% | -18.5% | -18.5% | -13.0% | -19.2% | -19.1% |
|  | Overall | -6.1% | -7.6% | -9.4% | -12.2% | -11.2% | -11.9% | -6.4% | -11.7% | -11.7% |
|  | Enc Time[%] | 119% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 124% |  |  |  |  |  |  |  |  |
| RAUT | RExt | -1.3% | -1.8% | -3.5% | -3.4% | -2.6% | -3.4% | -1.4% | -3.1% | -3.1% |
|  | SC | -11.1% | -12.3% | -12.7% | -17.1% | -16.3% | -16.4% | -11.6% | -16.9% | -16.9% |
|  | Overall | -5.3% | -6.1% | -7.2% | -10.3% | -9.4% | -9.9% | -5.5% | -10.0% | -10.0% |
|  | Enc Time[%] | 122% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |
|  |  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| LBMT | RExt | -1.3% | -5.1% | -5.7% | -4.2% | -3.5% | -4.5% | -1.2% | -2.9% | -2.8% |
|  | SC | -4.5% | -7.9% | -9.3% | -15.8% | -14.9% | -15.0% | -10.1% | -15.4% | -15.4% |
|  | Overall | -2.6% | -6.2% | -7.2% | -10.0% | -9.2% | -9.8% | -4.8% | -9.1% | -9.1% |
|  | Enc Time[%] | 113% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 140% |  |  |  |  |  |  |  |  |
| LBHT | RExt | -0.9% | -2.4% | -4.0% | -2.9% | -2.0% | -3.0% | -0.9% | -2.0% | -2.0% |
|  | SC | -9.4% | -11.7% | -12.3% | -16.0% | -15.2% | -15.2% | -10.7% | -15.8% | -15.8% |
|  | Overall | -4.4% | -6.1% | -7.4% | -9.4% | -8.6% | -9.1% | -4.9% | -8.9% | -8.9% |
|  | Enc Time[%] | 116% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 135% |  |  |  |  |  |  |  |  |
| LBST | RExt | -0.7% | -1.1% | -2.5% | -1.9% | -1.2% | -1.9% | -0.7% | -1.5% | -1.5% |
|  | SC | -10.2% | -11.9% | -12.4% | -15.7% | -15.0% | -14.9% | -10.7% | -15.6% | -15.6% |
|  | Overall | -4.5% | -5.5% | -6.5% | -8.8% | -8.1% | -8.4% | -4.8% | -8.5% | -8.5% |
|  | Enc Time[%] | 119% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 131% |  |  |  |  |  |  |  |  |
| LBUT | RExt | -0.7% | -0.9% | -1.8% | -1.9% | -1.1% | -1.6% | -0.8% | -1.7% | -1.7% |
|  | SC | -10.2% | -11.6% | -12.0% | -15.4% | -14.7% | -14.5% | -10.7% | -15.3% | -15.3% |
|  | Overall | -4.6% | -5.2% | -5.9% | -8.7% | -7.9% | -8.1% | -4.8% | -8.5% | -8.5% |
|  | Enc Time[%] | 121% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |

## Experiment 4

Experiment 4 shows gains up to 10.3%, 14.1%, 15.1% with respect to Luma, chroma Cb and Cr in average in the case of All Intra. It shows gains up to 5.7%, 10.7%, 11.9% and 2.6%, 6.2%, 7.1% with respect to Random access and Lowdelay B case.

|  |  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AIMT | RExt | -7.5% | -12.1% | -13.7% | -19.7% | -17.9% | -18.7% | -8.0% | -18.8% | -18.6% |
|  | SC | -13.8% | -16.6% | -16.9% | -25.4% | -24.5% | -24.7% | -16.0% | -25.3% | -25.2% |
|  | Overall | -10.3% | -14.1% | -15.1% | -22.7% | -21.4% | -21.9% | -11.6% | -22.2% | -22.1% |
|  | Enc Time[%] | 115% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |
| AIHT | RExt | -6.1% | -8.5% | -10.8% | -15.4% | -14.4% | -15.0% | -6.6% | -15.0% | -15.0% |
|  | SC | -15.3% | -17.3% | -17.7% | -24.4% | -23.5% | -23.7% | -16.1% | -24.4% | -24.4% |
|  | Overall | -9.9% | -12.1% | -13.6% | -19.9% | -19.0% | -19.4% | -10.5% | -19.7% | -19.7% |
|  | Enc Time[%] | 116% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |
| AIST | RExt | -4.8% | -5.9% | -7.9% | -11.5% | -10.7% | -11.3% | -5.1% | -11.2% | -11.2% |
|  | SC | -14.5% | -16.2% | -16.6% | -22.2% | -21.3% | -21.5% | -15.1% | -22.1% | -22.0% |
|  | Overall | -8.7% | -10.1% | -11.5% | -16.8% | -16.0% | -16.4% | -9.2% | -16.6% | -16.6% |
|  | Enc Time[%] | 117% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |
| AIUT | RExt | -3.7% | -4.3% | -5.7% | -8.8% | -8.2% | -8.5% | -3.9% | -8.6% | -8.6% |
|  | SC | -13.6% | -14.9% | -15.2% | -20.7% | -20.1% | -20.1% | -14.2% | -20.7% | -20.6% |
|  | Overall | -7.7% | -8.6% | -9.5% | -14.8% | -14.2% | -14.3% | -8.1% | -14.6% | -14.6% |
|  | Enc Time[%] | 119% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |
|  |  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| RAMT | RExt | -3.9% | -9.9% | -11.1% | -11.1% | -9.8% | -11.8% | -4.0% | -9.5% | -9.3% |
|  | SC | -8.4% | -12.0% | -13.0% | -20.0% | -19.2% | -19.4% | -13.4% | -19.9% | -19.9% |
|  | Overall | -5.7% | -10.7% | -11.9% | -15.6% | -14.5% | -15.6% | -7.8% | -14.7% | -14.6% |
|  | Enc Time[%] | 113% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 148% |  |  |  |  |  |  |  |  |
| RAHT | RExt | -2.8% | -5.6% | -8.6% | -7.9% | -6.3% | -8.5% | -2.9% | -6.7% | -6.5% |
|  | SC | -12.6% | -15.3% | -15.8% | -20.4% | -19.6% | -19.7% | -13.7% | -20.3% | -20.3% |
|  | Overall | -6.8% | -9.6% | -11.5% | -14.1% | -12.9% | -14.1% | -7.3% | -13.5% | -13.4% |
|  | Enc Time[%] | 116% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 145% |  |  |  |  |  |  |  |  |
| RAST | RExt | -1.8% | -2.9% | -5.8% | -5.0% | -3.9% | -5.3% | -1.9% | -4.3% | -4.2% |
|  | SC | -12.4% | -14.3% | -14.7% | -19.3% | -18.5% | -18.5% | -13.0% | -19.2% | -19.1% |
|  | Overall | -6.1% | -7.5% | -9.4% | -12.2% | -11.2% | -11.9% | -6.4% | -11.7% | -11.7% |
|  | Enc Time[%] | 119% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 139% |  |  |  |  |  |  |  |  |
| RAUT | RExt | -1.3% | -1.8% | -3.5% | -3.4% | -2.6% | -3.4% | -1.4% | -3.1% | -3.1% |
|  | SC | -11.1% | -12.3% | -12.7% | -17.1% | -16.3% | -16.4% | -11.6% | -16.9% | -16.9% |
|  | Overall | -5.3% | -6.1% | -7.2% | -10.3% | -9.4% | -9.9% | -5.5% | -10.0% | -10.0% |
|  | Enc Time[%] | 121% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |
|  |  | Y | U | V | G psnr | B psnr | R psnr | psnrYUV | psnrGBR | psnrGBRm |
| LBMT | RExt | -1.3% | -5.0% | -5.6% | -4.2% | -3.5% | -4.5% | -1.2% | -2.9% | -2.8% |
|  | SC | -4.5% | -7.9% | -9.3% | -15.8% | -14.9% | -15.0% | -10.1% | -15.4% | -15.4% |
|  | Overall | -2.6% | -6.2% | -7.1% | -10.0% | -9.2% | -9.8% | -4.8% | -9.1% | -9.1% |
|  | Enc Time[%] | 113% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 160% |  |  |  |  |  |  |  |  |
| LBHT | RExt | -0.9% | -2.3% | -4.0% | -2.9% | -2.0% | -3.0% | -0.9% | -2.0% | -2.0% |
|  | SC | -9.4% | -11.7% | -12.3% | -16.0% | -15.2% | -15.2% | -10.7% | -15.8% | -15.8% |
|  | Overall | -4.4% | -6.1% | -7.4% | -9.4% | -8.6% | -9.1% | -4.8% | -8.9% | -8.9% |
|  | Enc Time[%] | 116% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 155% |  |  |  |  |  |  |  |  |
| LBST | RExt | -0.7% | -1.1% | -2.4% | -1.9% | -1.2% | -1.9% | -0.7% | -1.5% | -1.5% |
|  | SC | -10.2% | -11.9% | -12.4% | -15.7% | -15.0% | -14.9% | -10.7% | -15.6% | -15.6% |
|  | Overall | -4.5% | -5.5% | -6.5% | -8.8% | -8.1% | -8.4% | -4.7% | -8.5% | -8.5% |
|  | Enc Time[%] | 119% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 149% |  |  |  |  |  |  |  |  |
| LBUT | RExt | -0.7% | -0.9% | -1.7% | -1.9% | -1.1% | -1.6% | -0.8% | -1.7% | -1.7% |
|  | SC | -10.2% | -11.6% | -12.0% | -15.4% | -14.7% | -14.5% | -10.7% | -15.3% | -15.3% |
|  | Overall | -4.6% | -5.2% | -5.9% | -8.7% | -7.9% | -8.1% | -4.8% | -8.5% | -8.5% |
|  | Enc Time[%] | 122% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | #VALUE! |  |  |  |  |  |  |  |  |

# Number of calculation and time delay of proposed chroma prediction mode

## Number of calculation for a 2Nx2N chroma TU

| number of parameter derivation |  | 1 |
| --- | --- | --- |
| L & C | addition | (2N-1)*2 + (2N-1)*2 |
| LL & LC | addition | (2N-1)*2 + (2N-1)*2 |
|  | multiplication | 2N*2 + 2N*2 |
| Etc. | addition | 14 |
|  | multiplication | 9 |
| Total | addition | 16N+6 |
|  | multiplication | 8N+9 |

## Number of computation in a CU

### 64x64 CU with four 32x32 TUs

| number of parameter derivation |  | 4 |
| --- | --- | --- |
| L & C | addition | 4*{(31*2) + (31*2)} |
| LL & LC | addition | 4*{(31*2) + (31*2)} |
|  | multiplication | 4*{(31*2) + (31*2)} |
| Etc. | addition | 4*{14} |
|  | multiplication | 4*{9} |
| Total | addition | 2072 |
|  | multiplication | 1060 |

### 64x64 CU with 256 4x4 TUs

| number of parameter derivation |  | 256 |
| --- | --- | --- |
| L & C | addition | 256*{(3*2) + (3*2)} |
| LL & LC | addition | 256*{(3*2) + (3*2)} |
|  | multiplication | 256*{(3*2) + (3*2)} |
| Etc. | addition | 256*{14} |
|  | multiplication | 256*{9} |
| Total | addition | 17920 |
|  | multiplication | 10496 |

# References

- J. Kim, S. Park, Y. Choi, Y. Jeon, B. Jeon, “New intra chroma prediction using inter-channel correlation”, JCTVC-B021, 2nd JCT-VC Meeting, Geneva, Switzerland, July 2010

- J. Chen, V. Seregin, “Chroma intra prediction by reconstructed luma samples”, JCTVC-C206, 3rd Meeting, Guangzhou, China, Oct. 2010

- J. Chen, V. Seregin, S. Lee, W-J Han, J. Kim, B. Jeon, “CE6.a: Chroma intra prediction by reconstructed luma samples”, JCTVC-D350, 4th Meeting, Daegu, Korea, Jan. 2011

- J. Chen, V. Seregin, W-J Han, J. Kim, B. Jeon, “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, JCTVC-E266, 5th Meeting, Geneva, Switzerland, Mar. 2011

- J. Kim, J. Park, B. Jeon, “AHG7: The performance of extended chroma mode for non 4:2:0 format”, JCTVC-K0253, 11th Meeting, Shanghai, China, Oct. 2012

- J. Kim, B. Jeon, “AHG7: The performance of extended intra chroma prediction for non 4:2:0 format”, JCTVC-L0240, 12th Meeting, Geneva, Switzerland, Jan. 2013

- J. Kim, “RCE1: The performance of extended chroma mode for non 4:2:0 format”, JCTVC-L0240, 13th Meeting, Incheon, Korea, Apr. 2013

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, 12th Meeting, Geneva, Switzerland, Jan. 2013

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).