| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0230 |
| --- | --- |

| Title: | SAO software cleanup and non-normative encoder-only bug-fixes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Yang Tsai, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chiayang.tsai, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a significant software cleanup and three non-normative encoder-only bug- fixes for sample adaptive offset (SAO) on top of HM-11.0 are proposed. In comparison with HM-11.0 plus the three bug-fixes under common test conditions, the new encoder generates exactly the same bitstreams, and the new decoder outputs exactly the same decoded videos. No run time increase is observed. Legacies for picture quadtree SAO, adaptation parameter set (APS), and fine granularity slices are removed, and thus the readability is significantly improved. The number of SAO code lines is reduced by half. It is also reported that the bug-fixes can achieve 0.1% BD-rate saving.

# Proposed SAO software

## Non-normative encoder-only bug-fixes

Three non-normative encoder-only bug-fixes are proposed for SAO. The fixed HM-11.0 software is attached with this contribution: source_HM11_SAO_Fixes.zip. The fixes are described as follows.

- SAO merge flag assignment fix

When slice_sao_luma_flag is equal to 0 and slice_sao_chroma_flag is equal to 1, sao_merge_left_flag and sao_merge_up_flag decided by rate-distortion optimization (RDO) process and those coded into the bitstream are inconsistent. The related fix is specified by the macro, SAO_BUGFIX_LUMA_MERGE_FLAG.

- Rate calculation fix

If slice_sao_luma_flag or slice_sao_chroma_flag is equal to 0, the corresponding SAO block parameters which belong to the same slice are not coded. However, the SAO type index syntax is always coded during the rate calculation in RDO. This bug may affect the SAO mode decision results. The related fix is specified by the macro, SAO_BUGFIX_RDO_RATE.

- Boundary condition fix for edge offset (EO)

To favor coding tree unit (CTU) pipelining encoder hardware design, during estimation of SAO parameters for a CTU, sample lines at the bottom of the CTU are skipped with one exceptional case: only when the CTU is at the bottom boundary of a picture, the sample lines at the bottom of the CTU can be used. However, this exception is missing for 0-degree EO, so the bottom sample lines are always skipped for 0-degree EO. The related fix is specified by the macro, SAO_BUGFIX_EO_ZERO_BOUNDARY.

## New SAO software implementation

Several features were in HM but later removed, for example, SAO picture quadtree, APS, and fine granularity slices. However, a lot of SAO source codes for supporting these legacies remain in HM-11.0. These legacies considerably decrease the software readability and cause lots of redundancies. Therefore, we propose a new SAO implementation based on HM-11.0. The proposed SAO implementation supports all encoder functionalities in HM-11.0 and excludes all redundant codes. Besides, the proposed three non-normative encoder-only bug-fixes are also included. Compared with HM-11.0 SAO, the new implementation has much better code quality and reduces the number of code lines by half. The proposed software implementation can be found in the attachment, source_MediaTek_SAOCleanup.zip.

# Experimental results

The proposed software is implemented on HM-11.0. All the results were simulated according to the common test conditions (CTC) defined in JCTVC-L1100 [1].

## Non-normative encoder-only bug-fixes

Table 1 shows the results of the three non-normative encoder-only bug-fixes based. HM-11.0 is used as the anchor.

## New SAO software implementation

Table 2 shows the results of the proposed SAO software. The anchor is HM-11.0 with the three non-normative encoder-only bug-fixes. The bitstreams generated by the new encoder are exactly the same as the anchor, and the decoded videos output by the new decoder are also exactly the same as the anchor.

## Conformance tests

To further verify the new SAO decoder, we also conducted the conformance tests. There are seven conformance bitstreams for SAO specified in JCTVC-M1004 [2]. Among the seven conformance bitstreams related to SAO, SAO_E, SAO_F, and SAO_G are decodable by HM-11.0, whiles others are actually not conformant to HEVC text specification. Therefore, we tested SAO_E, SAO_F, and SAO_G, and the tests were passed.

# Conclusions

In this contribution, a significant software cleanup for SAO is done on top of HM-11.0. In addition, three non-normative encoder-only bug-fixes are proposed. The new implementation reduces the number of SAO code lines by half and significantly improves the software readability without run time increase.

# Reference

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1100, January, 2013.

- T. Suzuki, G. Sullivan, “HEVC conformance test draft 3,” Document of Joint Collaborative Team on Video Coding, JCTVC-M1004, April, 2013.

Table 1.  Results of non-normative encoder-only bug-fixes (partial results, will be updated)

Table 2.  Results of the new SAO software implementation (partial results, will be updated)

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).