| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0231 |
| --- | --- |

| Title: | AHG 8: Intra mode coding for screen contents |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or | Junghye Min Sunil Lee Chanyul Kim | Email: | jh643.min@samsung.com sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents an intra mode coding method for screen contents. 3 MPMs (Most Probable Mode) are set without referring intra modes of neighbouring prediction units. The proposed method provides performance gains of -0.7% for both screen content sequences with RGB and YUV formats respectively for AI lossless test conditions. For AI lossy test conditions, gains of -1.0 and -0.2 % are observed for screen content sequences with RGB and YUV formats

# Introduction

It has been proved that intra prediction mode coding using most probable modes from neighbouring prediction units are efficient for coding efficiency of intra prediction. But derivation process of most probable modes in the HM-10.1+RExt-3.0 is complicated as follows.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

- Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

- Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..2 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is less than 2 (i.e. equal to INTRA_PLANAR or INTRA_DC), candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = INTRA_PLANAR	(8-1)

candModeList[ 1 ] = INTRA_DC		(8-2)

candModeList[ 2 ] = INTRA_ANGULAR26	(8-3)

  - Otherwise, candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-4)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(8-5)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(8-6)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

  - candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-7)

candModeList[ 1 ] = candIntraPredModeB	(8-8)

  - If neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_PLANAR, candModeList[ 2 ] is set equal to INTRA_PLANAR,

  - Otherwise, if neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_DC, candModeList[ 2 ] is set equal to INTRA_DC,

  - Otherwise, candModeList[ 2 ] is set equal to INTRA_ANGULAR26.

For screen contents coding, real time encoding/decoding is more critical than any other applications. A simplified prediction process is desirable for intra coding of screen contents. As in following figure, one of characteristics of screen contents is correlation of patterns along horizontal and vertical directions.

Figure 1. Example of screen contents

Simple derivation method for most probable intra modes utilizing the characteristics of screen contents is proposed in the next section.

# Proposed method

Without referring to neighbouring prediction units, three most probable modes (candModeList[ x ], x=0,1,2) are set as follows.

candModeList[ 0 ] = INTRA_HOR

candModeList[ 1 ] = INTRA_VER

candModeList[ 2 ] = INTRA_ANGULAR_18

# Experimental results

Proposed methods were implemented on top of HM10.1_RExt3.0 s/w under common test condition described in JCTVC-M1023 [1].  Table 1 shows performance gains for lossless test condition. For all class types, gains are observed in AI configuration. -0.7% gains are observed for both SC RGB and YUV cases.

Table 1 – Test results for lossless case

Table 2 shows performance gains for lossy test condition. -1.0% and -0.2% gains are observed AI main-tier test for screen content sequences.

Table 2 – Test results for lossy case

# Conclusions

This contribution provides a simplified intra mode coding method for screen contents. Derivation process of most probable modes is removed and three MPMs are set to horizontal, vertical and angular_18 mode. The proposed method provides performance gains for screen contents for both lossy and lossless test conditions.

# References

[1] Liwei Guo, HEVC Range Extensions Core Experiment 3 (RCE3): Intra Coding Methods for Screen Content, JCTVC-M1123, 13th Meeting: Incheon, KR, 18–26 Apr. 2013.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).