| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0232 |
| --- | --- |

| Title: | Non-RCE2: Rice parameter update method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or | Junghye Min Sunil Lee Chanyul Kim | Email: | jh643.min@samsung.com sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Two rice parameter update methods for transform skip and transform bypass block are proposed. In the HM-10.1+RExt-3.0, rice parameter is increased by one if absCoeff, absolute value of coefficient level, meets a pre-defined condition. This update process is efficient for transform coefficient coding especially when absCoeffs are small. In the proposed methods, rice parameter is calculated using absCoeff and the previous rice parameter. The proposed method provides gains of -1.4%,-0.7%,-9.1%,-4.0% and -0.5% for classF, classB, SC RGB, SC YUV and RangeExt classes in AI lossless test condition.

# Introduction

Current rice parameter update process was optimized for coefficient coding especially where absCoeffs (absolute value of coefficient levels) are increased as the coefficient position approaches the left top position in the transform units. The rice parameter update method in the HM-10.1+RExt-3.0 is as follows.

if(absCoeff[idx] > 3*(1<<uiGoRiceParam))

{

uiGoRiceParam = min<UInt>(uiGoRiceParam+ 1, 4);

}

.

The rice parameter (uiGoRiceParam) is increased by 1 when absCoeff is larger than 3*(1<<uiGoRiceParam)). The maximum value of rice parameter is 4.

# Proposed method

Two methods for rice parameter update are proposed. Rice parameter is calculated depending the absCoeff(absolute value of coefficient level) and previous rice parameter as follows.

Method 1

uiGoRiceParam = min<UInt>(uiGoRiceParam+ ((absCoeff[idx]+3)>>(uiGoRiceParam+3)), RICE_ MAX);

Method 2

uiGoRiceParam = min<UInt>(uiGoRiceParam+ ((absCoeff[idx]+4)>>(uiGoRiceParam+3)), RICE_ MAX);

RICE_MAX can be set to 4,5, and 6.

# Experimental results

Proposed methods were implemented on top of HM10.1_RExt3.0 s/w under common test condition described in JCTVC-M1022 [1]. Proposed methods were tested for following three test cases.

- Lossless test :

- Lossy test 1: proposed methods applied for transform skip and transform bypass TU

- Lossy test 2: proposed methods applied for all transform unit types

Table 1, 2 and 3 present performances of the method 1 with RICE_ MAX = 6. For lossless tests, performance gains are observed in all class types in AI configurations. Even in Class B sequences which have natural image patterns, -0.7%, -1.1% and -1.1% gains are observed for AI, RA and LD lossless conditions. For SC RGB 444 sequences, bit-rate saving is -9.1% while the maximum gain of -15.1% can be observed for ‘cad_wave_form’ sequence. For lossy cases, performances gains are obtained for high-tier and super-high tier cases. For lossy test2, -2.8% gain is observed for SC RGB sequences in super high-tier all intra condition.

Table 1: lossless test results for method 1 (RICE_ MAX=6)

Table 2: lossy test 1 for method 1 (RICE_ MAX=6)

Table 3: lossy test 2 for method 1 (RICE_ MAX=6)

Table 4, 5 and 6 present performances of the method 2 with RICE_ MAX = 6. For lossless tests, performance gains are observed in all class types and test configurations. For SC RGB 444 sequences, bit-rate saving is -9.2%. For lossy cases, performances gains are obtained for most of test cases. For lossy test2, -3.0% gain is observed for SC RGB sequences in super high-tier all intra condition.

Table 4: lossless test results for method 2 (RICE_ MAX=6)

Table 5: lossy test 1 for method 2 (RICE_ MAX=6)

Table 6: lossy test 2 for method 2 (RICE_ MAX=6)

# Conclusions

This contribution provides rice parameter update methods for transform skip and transforms bypass blocks. Proposed methods provide performance gains for all classes in lossless AI test condition where -9.2% performance gains are observed for SC RGB case. Proposed methods are simpler than previous update process while they provide significant gains.

# References

[1] Rajan Joshi , Peter Amon, Robert Cohen, Sunil Lee, Matteo Naccari, HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks , JCTVC-M1122, 13th Meeting: Incheon, KR, 18–26 Apr. 2013.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Additional test results

Table 7: lossless test results for method 1 (RICE_ MAX=4)

Table 8: lossless test results for method 1 (RICE_ MAX=5)

Table 9: lossy test1 for method 1 (RICE_ MAX=5)

Table 10: lossy test2 for method 1 (RICE_ MAX=5)

Table 11: lossless test results for method 2 (RICE_ MAX=5)

Table 12: lossy test1 for method 2 (RICE_ MAX=5)

Table 13: lossy test2 for method 2 (RICE_ MAX=5)