| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0235 |
| --- | --- |

| Title: | Non-RCE3: base color merging for MBCIM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

Merging of base color in MBCIM in RCE3 is proposed to improve coding efficiency. Experimental results state that compared to MBCIM, there are -1.5% for SC RGB sequences and -0.5% for SC YUV 444 sequences in lossless coding on average; -0.3% for class F sequences at all tiers, -1.6% for SC RGB sequences at all tiers and -0.9% for SC YUV 444 sequences at all tiers in lossy coding on average.

# Introduction

MBCIM is proposed in JCTVC-M0330[1] and evaluated in RCE3[2]. For each CU, if it is encoded as MBCIM mode. MBCIM codes the number of base color and values of base color (Major colors in Figure 1). Then the quantized index for each pixel in the CU is coded. Signaling base color for each MBCIM mode is redundant.

Figure 1 Diagram of JCTVC-M0330

# Proposed solution

There are some correlations between base colors of CUs in a local region. Such correlation can be used to reduce the redundancy of signaling base color. To achieve this, merging flags to the left and above neighboring CU are signaled first. If neighboring CUs are MBCIM modes, merging flags (i.e. color_merge_left_flag and color_merge_up_flag) will be signaled to copy either neighboring base color. If both merge flags are off, base color will be signaled as the original algorithm. There is one dedicated context for the merge flags of all color components in CABAC.  The mechanism is similar to SAO merge flags.

# Simulation results

Proposed tool is implemented based on JCTVC-M0330 (RCE3.2) software released on July 7, 2013 to RCE3 participant list and tested under testing conditions defined in RCE3.

## Lossless coding

The results for lossless coding are summarized in Table 1, where the anchor is HM10.1_RExt3.1. Detailed results are available in excel file accompanying with this contribution. Compared to JCTVC-M0330 as shown in Table 2, proposed solution improves coding efficiency by -1.3%~-1.5% for SC RGB sequences and -0.2%~-0.6% for SC YUV 444 sequences.

Table 1 Summary of proposed solution,  anchor is HM10.1_RExt3.1

Table 2 Summary of results compared to JCTVC-M0330 (RCE3.2), anchor is JCTVC-M0330 (RCE3.2)

## Lossy coding

The results for lossless coding are summarized in Table 3, where the anchor is HM10.1_RExt3.1. Detailed results are available in excel file accompanying with this contribution. Compared to JCTVC-M0330 as shown in Table 2, proposed solution improves coding efficiency by -0.3% for class F sequences at all tiers, -1.6% for SC RGB sequences at all tiers and -0.9% for SC YUV 444 sequences at all tiers.

Table 3 Summary of proposed solution,  anchor is HM10.1_RExt3.1

Table 4 Summary of results compared to JCTVC-M0330 (RCE3.2), anchor is JCTVC-M0330 (RCE3.2)

## Complexity

Encoding and decoding time information is not available in simulation results because simulations are conducted in a heterogeneous cluster. Since the proposed approach is relatively small change to JCTVC-M0330, the complexity changes should be negligible.

# Conclusion

This proposal improves MBCIM in RCE3 by merging base color for left and above neighboring CU. Results show consistent coding gain for both lossless and lossy coding. It is recommended to adopt proposed approach to RExt.

# Reference

- W. Zhu, J. Xu, W. Ding, “Screen Content Coding with Multi-stage Base Color and Index Map Representation,” JCTVC-M0330, Incheon, Korea, April 2013.

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Coding Methods for Screen Content,” JCTVC-M1123, Incheon, Korea, April 2013.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

Text changes are based on text provided in RCE3. All changes are highlighted in yellow.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| MBCIM_mode_flag[x0][y0] | ae(v) |
| if (mbcim_mode_flag) { |  |
| mbcim_coding ( x0, y0, nCbS, nCbS, 0) |  |
| mbcim_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC , 1) |  |
| mbcim_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC, 2) |  |
| } |  |
| else{ |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| …… |  |

| mbcim_coding ( x0, y0, nPbW, nPbH , cIdx){ |  |
| --- | --- |
| If (IsColorMergeLeftAllowed){ |  |
| color_merge_left_flag | ae(v) |
| } |  |
| If(IsColorMergeUpAllowed && ! color_merge_left_flag){ |  |
| color_merge_up_flag | ae(v) |
| } |  |
| If(! color_merge_left_flag && ! color_merge_up_flag){ |  |
| color_size_minus1[x0][y0] | ae(v) |
| for( i = 0; i <=color_size_minus1[x0][y0]; i ++ ) |  |
| color_table[x0][y0][i] | ae(v) |
| } |  |
| for( j = 0; j < nPbW; j = j + 1) |  |
| for( i = 0; i < nPbH ; i = i + 1) |  |
| prev_index_first_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nPbW; j = j + 1) |  |
| for( i = 0; i < nPbH ; i = i + 1) |  |
| if(prev_index_first_pred_flag [ x0 + i ][ y0 + j ] && color_size_minus1 [x0][y0] > 1) |  |
| prev_index_second_pred_flag [ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nPbW; j = j + 1) |  |
| for( i = 0; i < nPbH ; i = i + 1) |  |
| if ( prev_index_first_pred_flag [ x0 + i ][ y0 + j ] && prev_index_second_pred_flag [ x0 + i ][ y0 + j ] && color_size_minus1 [x0][y0] > 2 ) |  |
| rem_index [ x0 + i ][ y0 + j ] | ae(v) |

## Semantics

MBCIM_mode_flag[x0][y0] equal to 1 specifies that the current coding unit is encoded using BCIM mode. palette_mode_flag  equal to 0 specifies the current coding unti is encoded using a mode other than BCIM mode.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. The array indices xL, yL specify the location ( xL, yL ) of the top-left luma sample of the left coding block relative to the top-left luma sample of the picture. The array indices xU, yU specify the location ( xU, yU ) of the top-left luma sample of the above coding block relative to the top-left luma sample of the picture.

The variables IsColorMergeLeftAllowed and IsColorMergeLeftAllowed are defined as follows:

- If availableL is TRUE and  MBCIM_mode_flag[xL][yL]  is equal to 1, IsColorMergeLeftAllowed = 1;

- Otherwise, IsColorMergeLeftAllowed = 0;

- If availableU is TRUE and  MBCIM_mode_flag[xU][yU]  is equal to 1, IsColorMergeUpAllowed = 1;

- Otherwise, IsColorMergeUpAllowed = 0;

color_merge_left_flag equal to 1 specifies that the syntax elements color_size_minus1 and color_table are derived from the corresponding syntax elements of the left coding block. color_merge_left_flag equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the left coding block. When color_merge_left_flag is not present, it is inferred to be equal to 0.

color_merge_up_flag equal to 1 specifies that the syntax elements color_size_minus1 and color_table are derived from the corresponding syntax elements of the above coding block. color_merge_up_flag equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the above coding block. When color_merge_up_flag is not present, it is inferred to be equal to 0.

color_size_minus1[x0][y0] plus 1 specifies the number of elements in the color table. palettecolor_size_minus1[x0][y0] shall be the range of 0 to 15.

When palette_size_minus1 [x0][y0] is not present, it is inferred to be equal to 0.

When color_size_minus1 [x0][y0] is not present, it is derived as follows:

- If color_merge_left_flag is equal to 1, color_size_minus1 [ x0][ y0 ] is set equal to color_size_minus1 [ xL][ yL ].

- Otherwise, if color_merge_up_flag is equal to 1, color_size_minus1 [ x0][ y0 ] is set equal to color_size_minus1 [ xU][ yU ].

color_table [x0][y0][i] specifies the i-th element in the color table.

When color_table [x0][y0] is not present, it is derived as follows:

- If color_merge_left_flag is equal to 1, color_table [ x0][ y0 ] is set equal to color_table [ xL][ yL ].

- Otherwise, if color_merge_up_flag is equal to 1, color_table [ x0][ y0 ] is set equal to color_table [ xU][ yU ].

prev_index_first_pred_flag[ x0 + i ][ y0 + j ] equal to 1 specifies that the current index value x is equal to the first prediction index and the decoded pixel value equals to color_table [x0][y0][x].

prev_index_second_pred_flag[ x0 + i ][ y0 + j ]  equal to 1 specifies that the current index value x is equal to the second prediction index and the decoded pixel value equals to color_table [x0][y0][x].

rem_index [ x0 + i ][ y0 + j ]  equals to the actual index value x when the first and second prediction indexes are incorrect,  and the decoded pixel value equals to color_table [x0][y0][x].