| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0237 |
| --- | --- |

| Title: | HEVC version 1: Level 2.2 for support of WVGA@30fps |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | -858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to create a new level, 2.2, for HEVC version 1, for support of WVGA@30fps.

# Introduction

At the most recent 3GPP SA4 meeting (SA4#74 in July 2013), it was decided to specify the support of HEVC in 3GPP Dynamic Adaptive Streaming over HTTP (3GP-DASH, 3GPP TS 26.247) and 3GPP file format (3GPP TS 26.244). 3GPP is further evaluating the support of HEVC in 3GPP Packet-Switched Streaming (PSS, 3GPP TS 26.234), 3GPP Multimedia Broadcast and Multicast Service (MBMS, 3GPP TS 26.346), 3GPP Multimedia Messaging Service (MMS, 3GPP TS 26.140), and 3GPP Multimedia Telephony Service over IMS (MTSI, 3GPP TS 26.114).

Currently, they are considering specifying the minimum client capability requirement for HEVC decoding, with WVGA(800x480)@30fps and 720p@30fps as two potential candidates. The existing Level 3.1 suits well for 720p@30fps. However WVGA(800x480)@30fps sits roughly in the middle (from the number of samples per second or the maximum bit rate points of view) between Level 2.1 (Q720p(640x360)@30fps) and Level 3 (QHD(960x540)@30fps).

Thus, for potenial 3GPP use, it is proposed to create a new level, 2.2, for support of WVGA(800x480)@30fps, as specified below.

# Proposal

The changes are revision-marked.

  - Tiers and levels

    - General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag equal to 0 is considered to be a lower tier than the tier with general_tier_flag equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this annex:

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0 (i.e. the 0-th access unit).

- Let picture n be the coded picture or the corresponding decoded picture of access unit n.

- Let the variable CpbBrVclFactor be equal to 1000.

- Let the variable CpbBrNalFactor be equal to 1100.

Bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

- PicSizeInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A-1.

- The value of pic_width_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of pic_height_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 shall be less than or equal to MaxDpbSize, which is derived as follows:

if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf, 16 )
else if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, 16 )	(A-1)
else if( PicSizeInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxDpbSize = maxDpbPicBuf

where MaxLumaPs is specified in Table A-1 and maxDpbPicBuf is equal to 6.

- For level 5 and higher levels, the value of CtbSizeY shall be equal to 32 or 64.

- The value of NumPocTotalCurr shall be less than or equal to 8.

- The value of num_tile_columns_minus1 shall be less than MaxTileCols and num_tile_rows_minus1 shall be less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A-1.

- For the VCL HRD parameters, CpbSize[ i ] shall be less than or equal to CpbBrVclFactor * MaxCPB for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxCPB is specified in Table A-1 in units of CpbBrVclFactor bits.

- For the NAL HRD parameters, CpbSize[ i ] shall be less than or equal to CpbBrNalFactor * MaxCPB for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxCPB is specified in Table A-1 in units of CpbBrNalFactor bits.

Table A-1 specifies the limits for each level of each tier.

A tier and level to which the bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc as follows:

–	general_tier_flag equal to 0 indicates conformance to the Main tier, and general_tier_flag equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A-1. general_tier_flag shall be equal to 0 for levels below level 4 (corresponding to the entries in Table A-1 marked with "-").

–	general_level_idc shall be set equal to a value of 30 times the level number specified in Table A-1.

Table A-1 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (1000 bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 2.2 | 425 984 | 4 800 | - | 25 | 2 | 2 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |

Informative subclause A.4.3 shows the effect of these limits on picture rates for several example picture formats.

    - Profile-specific level limits for the Main and Main 10 profiles

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

Bitstreams conforming to the Main or Main 10 profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

- The nominal removal time of access unit n (with n greater than 0) from the CPB, as specified in subclause Error: Reference source not found, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( PicSizeInSamplesY  MaxLumaSr, fR ) for the value of PicSizeInSamplesY of picture n − 1, where MaxLumaSr is the value specified in Table A-2 that applies to picture n − 1.

- The difference between consecutive output times of pictures from the DPB, as specified in subclause Error: Reference source not found, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( PicSizeInSamplesY  MaxLumaSr, fR ) for the value of PicSizeInSamplesY of picture n, where MaxLumaSr is the value specified in Table A-2 for picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

- The removal time of access unit 0 shall satisfy the constraint that the number of slice segments in picture 0 is less than or equal to Min( MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSliceSegmentsPerPicture * PicSizeInSamplesY / MaxLumaPs, MaxSliceSegmentsPerPicture ), for the value of PicSizeInSamplesY of picture 0, where MaxSliceSegmentsPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Table A-1 and Table A-2, respectively, that apply to picture 0.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slice segments in picture n is less than or equal to Min( MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ), MaxSliceSegmentsPerPicture ), where MaxSliceSegmentsPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Table A-1 and Table A-2, respectively, that apply to picture n.

- For the VCL HRD parameters, BitRate[ i ] shall be less than or equal to CpbBrVclFactor * MaxBR for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxBR is specified in Table A-2 in units of CpbBrVclFactor bits/s.

- For the NAL HRD parameters, BitRate[ i ] shall be less than or equal to CpbBrNalFactor * MaxBR for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxBR is specified in Table A-2 in units of CpbBrNalFactor bits/s.

- The sum of the NumBytesInNalUnit variables for access unit 0 shall be less than or equal to 1.5 * ( Max( PicSizeInSamplesY, fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr for the value of PicSizeInSamplesY of picture 0, where MaxLumaSr and MinCr are the values specified in Table A-2 that apply to picture 0.

- The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) shall be less than or equal to 1.5 * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr and MinCr are the values specified in Table A-2 that apply to picture n.

- The removal time of access unit 0 shall satisfy the constraint that the number of tiles in picture 0 is less than or equal to Min( MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * PicSizeInSamplesY / MaxLumaPs, MaxTileCols * MaxTileRows ), for the value of PicSizeInSamplesY of picture 0, where MaxTileCols and MaxTileRows are the values specified in Table A-1 that apply to picture 0.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in picture n is less than or equal to Min( MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ), MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Table A-1 that apply to picture n.

Table A-2 – Tier and level limits for the Main and Main 10 profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (1000 bits/s) |  | Min Compression Ratio MinCr |
| --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |
| 1 | 552 960 | 128 | - | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 |
| 2.2 | 12 779 520 | 4 800 | - | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 |

    - Effect of level limits on picture rate for the Main and Main 10 profiles (informative)

This subclause does not form an integral part of this Specification.

Informative Tables A-3 and A-4 provide examples of maximum picture rates for the Main and Main 10 profiles for various picture formats when MinCbSizeY is equal to 64.

Table A-3 – Maximum picture rates (pictures per second) at level 1 to 4.1 for some example picture sizes
when MinCbSizeY is equal to 64

| Level: |  |  |  | 1 | 2 | 2.1 | 2.2 | 3 | 3.1 | 4 | 4.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max luma picture size (samples): |  |  |  | 36 864 | 122 880 | 245 760 | 425 984 | 552 960 | 983 040 | 2 228 224 | 2 228 224 |
| Max luma sample rate (samples/sec) |  |  |  | 552 960 | 3 686 400 | 7 372 800 | 12 779 520 | 16 588 800 | 33 177 600 | 66 846 720 | 133 693 440 |
| Format nickname | Luma width | Luma height | Luma picture size |  |  |  |  |  |  |  |  |
| SQCIF | 128 | 96 | 16 384 | 33.7 | 225.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QCIF | 176 | 144 | 36 864 | 15.0 | 100.0 | 200.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QVGA | 320 | 240 | 81 920 | - | 45.0 | 90.0 | 156.0 | 202.5 | 300.0 | 300.0 | 300.0 |
| 525 SIF | 352 | 240 | 98 304 | - | 37.5 | 75.0 | 130.0 | 168.7 | 300.0 | 300.0 | 300.0 |
| CIF | 352 | 288 | 122 880 | - | 30.0 | 60.0 | 104.0 | 135.0 | 270.0 | 300.0 | 300.0 |
| 525 HHR | 352 | 480 | 196 608 | - | - | 37.5 | 65.0 | 84.3 | 168.7 | 300.0 | 300.0 |
| 625 HHR | 352 | 576 | 221 184 | - | - | 33.3 | 57.7 | 75.0 | 150.0 | 300.0 | 300.0 |
| Q720p | 640 | 360 | 245 760 | - | - | 30.0 | 52.0 | 67.5 | 135.0 | 272.0 | 300.0 |
| VGA | 640 | 480 | 327 680 | - | - | - | 39.0 | 50.6 | 101.2 | 204.0 | 300.0 |
| 525 4SIF | 704 | 480 | 360 448 | - | - | - | 35.4 | 46.0 | 92.0 | 185.4 | 300.0 |
| 525 SD | 720 | 480 | 393 216 | - | - | - | 32.5 | 42.1 | 84.3 | 170.0 | 300.0 |
| 4CIF | 704 | 576 | 405 504 | - | - | - | 31.5 | 40.9 | 81.8 | 164.8 | 300.0 |
| WVGA | 800 | 480 | 425 984 |  |  |  | 30.0 | 38.9 |  |  |  |
| 625 SD | 720 | 576 | 442 368 | - | - | - |  | 37.5 | 75.0 | 151.1 | 300.0 |
| 480p (16:9) | 864 | 480 | 458 752 | - | - | - |  | 36.1 | 72.3 | 145.7 | 291.4 |
| SVGA | 800 | 600 | 532 480 | - | - | - |  | 31.1 | 62.3 | 125.5 | 251.0 |
| QHD | 960 | 540 | 552 960 | - | - | - |  | 30.0 | 60.0 | 120.8 | 241.7 |
| XGA | 1024 | 768 | 786 432 | - | - | - |  | - | 42.1 | 85.0 | 170.0 |
| 720p HD | 1280 | 720 | 983 040 | - | - | - |  | - | 33.7 | 68.0 | 136.0 |
| 4VGA | 1280 | 960 | 1 228 800 | - | - | - |  | - | - | 54.4 | 108.8 |
| SXGA | 1280 | 1024 | 1 310 720 | - | - | - |  | - | - | 51.0 | 102.0 |
| 525 16SIF | 1408 | 960 | 1 351 680 | - | - | - |  | - | - | 49.4 | 98.9 |
| 16CIF | 1408 | 1152 | 1 622 016 | - | - | - |  | - | - | 41.2 | 82.4 |
| 4SVGA | 1600 | 1200 | 1 945 600 | - | - | - |  | - | - | 34.3 | 68.7 |
| 1080 HD | 1920 | 1080 | 2 088 960 | - | - | - |  | - | - | 32.0 | 64.0 |
| 2Kx1K | 2048 | 1024 | 2 097 152 | - | - | - |  | - | - | 31.8 | 63.7 |
| 2Kx1080 | 2048 | 1080 | 2 228 224 | - | - | - |  | - | - | 30.0 | 60.0 |
| 4XGA | 2048 | 1536 | 3 145 728 | - | - | - |  | - | - | - | - |
| 16VGA | 2560 | 1920 | 4 915 200 | - | - | - |  | - | - | - | - |
| 3616x1536 (2.35:1) | 3616 | 1536 | 5 603 328 | - | - | - |  | - | - | - | - |
| 3672x1536 (2.39:1) | 3680 | 1536 | 5 701 632 | - | - | - |  | - | - | - | - |
| 3840x2160 (4*HD) | 3840 | 2160 | 8 355 840 | - | - | - |  | - | - | - | - |
| 4Kx2K | 4096 | 2048 | 8 388 608 | - | - | - |  | - | - | - | - |
| 4096x2160 | 4096 | 2160 | 8 912 896 | - | - | - |  | - | - | - | - |
| 4096x2304 (16:9) | 4096 | 2304 | 9 437 184 | - | - | - |  | - | - | - | - |
| 7680x4320 | 7680 | 4320 | 33 423 360 | - | - | - |  | - | - | - | - |
| 8192x4096 | 8192 | 4096 | 33 554 432 | - | - | - |  | - | - | - | - |
| 8192x4320 | 8192 | 4320 | 35 651 584 | - | - | - |  | - | - | - | - |

Table A-4 – Maximum picture rates (pictures per second) at level 5 to 6.2 for some example picture sizes
when MinCbSizeY is equal to 64

| Level: |  |  |  | 5 | 5.1 | 5.2 | 6 | 6.1 | 6.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max luma picture size (samples): |  |  |  | 8 912 896 | 8 912 896 | 8 912 896 | 35 651 584 | 35 651 584 | 35 651 584 |
| Max luma sample rate (samples/sec) |  |  |  | 267 386 880 | 534 773 760 | 1 069 547 520 | 1 069 547 520 | 2 139 095 040 | 4 278 190 080 |
| Format nickname | Luma width | Luma height | Luma picture size |  |  |  |  |  |  |
| SQCIF | 128 | 96 | 16 384 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QCIF | 176 | 144 | 36 864 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QVGA | 320 | 240 | 81 920 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 525 SIF | 352 | 240 | 98 304 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| CIF | 352 | 288 | 122 880 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 525 HHR | 352 | 480 | 196 608 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 625 HHR | 352 | 576 | 221 184 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| Q720p | 640 | 360 | 245 760 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| VGA | 640 | 480 | 327 680 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 525 4SIF | 704 | 480 | 360 448 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 525 SD | 720 | 480 | 393 216 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 4CIF | 704 | 576 | 405 504 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| WVGA | 800 | 480 | 425 984 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 625 SD | 720 | 576 | 442 368 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 480p (16:9) | 864 | 480 | 458 752 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| SVGA | 800 | 600 | 532 480 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| QHD | 960 | 540 | 552 960 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| XGA | 1024 | 768 | 786 432 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 720p HD | 1280 | 720 | 983 040 | 272.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 4VGA | 1280 | 960 | 1 228 800 | 217.6 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| SXGA | 1280 | 1024 | 1 310 720 | 204.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 525 16SIF | 1408 | 960 | 1 351 680 | 197.8 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 16CIF | 1408 | 1152 | 1 622 016 | 164.8 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 4SVGA | 1600 | 1200 | 1 945 600 | 137.4 | 274.8 | 300.0 | 300.0 | 300.0 | 300.0 |
| 1080 HD | 1920 | 1080 | 2 088 960 | 128.0 | 256.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 2Kx1K | 2048 | 1024 | 2 097 152 | 127.5 | 255.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 2Kx1080 | 2048 | 1080 | 2 228 224 | 120.0 | 240.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 4XGA | 2048 | 1536 | 3 145 728 | 85.0 | 170.0 | 300.0 | 300.0 | 300.0 | 300.0 |
| 16VGA | 2560 | 1920 | 4 915 200 | 54.4 | 108.8 | 217.6 | 217.6 | 300.0 | 300.0 |
| 3616x1536 (2.35:1) | 3616 | 1536 | 5 603 328 | 47.7 | 95.4 | 190.8 | 190.8 | 300.0 | 300.0 |
| 3672x1536 (2.39:1) | 3680 | 1536 | 5 701 632 | 46.8 | 93.7 | 187.5 | 187.5 | 300.0 | 300.0 |
| 3840x2160 (4*HD) | 3840 | 2160 | 8 355 840 | 32.0 | 64.0 | 128.0 | 256.0 | 300.0 | 300.0 |
| 4Kx2K | 4096 | 2048 | 8 388 608 | 31.8 | 63.7 | 127.5 | 127.5 | 255.0 | 300.0 |
| 4096x2160 | 4096 | 2160 | 8 912 896 | 30.0 | 60.0 | 120.0 | 120.0 | 240.0 | 300.0 |
| 4096x2304 (16:9) | 4096 | 2304 | 9 437 184 | - | - | - | 113.3 | 226.6 | 300.0 |
| 4096x3072 | 4096 | 3072 | 12 582 912 | - | - | - | 85.0 | 170.0 | 300.0 |
| 7680x4320 | 7680 | 4320 | 33 423 360 | - | - | - | 32.0 | 64.0 | 128.0 |
| 8192x4096 | 8192 | 4096 | 33 554 432 | - | - | - | 31.8 | 63.7 | 127.5 |
| 8192x4320 | 8192 | 4320 | 35 651 584 | - | - | - | 30.0 | 60.0 | 120.0 |

The following should be noted in regard to the examples shown in Tables A-3 and A-4:

–	This Specification is a variable-picture-size specification. The specific listed picture sizes are illustrative examples only.

–	The example luma picture sizes were computed by rounding up the luma width and luma height to multiples of 64 before computing the product of these quantities, to reflect the potential use of MinCbSizeY equal to 64 for these picture sizes, as pic_width_in_luma_samples and pic_height_in_luma_samples are each required to be a multiple of MinCbSizeY. For some illustrated values of luma width and luma height, a somewhat higher number of pictures per second can be supported when MinCbSizeY is less than 64.

–	As used in the examples, "525" refers to typical use for environments using 525 analogue scan lines (of which approximately 480 lines contain the visible picture region), and "625" refers to environments using 625 analogue scan lines (of which approximately 576 lines contain the visible picture region).

–	XGA is also known as (aka) XVGA, 4SVGA aka UXGA, 16XGA aka 4Kx3K, CIF aka 625 SIF, 625 HHR aka 2CIF aka half 625 D-1, aka half 625 ITU-R BT.601, 525 SD aka 525 D-1 aka 525 ITU-R BT.601, 625 SD aka 625 D-1 aka 625 ITU-R BT.601.

# Patent rights declaration(s)

Qualcomm Incorporated has no current or pending patent rights relating to the technology described in this contribution.