| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0238 WG11 Number: m29874 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0082 |
| --- | --- |

| Title: | On Source Representation Information Signaling in VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes syntax and semantics for signaling bit-depth, spatial resolution, color chromaticity and color format related information regarding layers in VPS extension.  It is asserted that this is useful for session negotiation.

# Introduction

This document proposes syntax and semantics for signaling source representation information including - bit-depth, spatial resolution, color chromaticity and color format related information regarding layers in VPS extension. Currently this source representation information is signalled in SPS in SHVC [1] and MV-HEVC[2] working drafts .

It is asserted that it is useful for session negotiation that a client can select layers to decode based on the knowledge regarding bit-depth, spatial resolution, color chromaticity and color format of the layers and the client device capability. Thus this document proposes to signal this information in VPS.

The upcoming deployment of UHDTV devices and content will use a different color gamut than legacy devices.  Specifically, HD uses the ITU-R BT.709 recommendation [1], while UHDTV will use the ITU-R BT.2020 recommendation [2].  A key difference between these systems is that the color gamut of UHDTV is significantly larger than HD. Also common consumer applications using BT.709 have an 8-bit sample depth while BT.2020 only defines 10-bit and 12-bit sample depths. Thus it is asserted that signalling information about bit-depth and colour chromaticity in VPS is important in enabling determination of UHDTV layers which use ITU-R BT.2020 during session negotiation.

# Proposal

We propose syntax and semantics for signaling source representation information about layers in VPS.

Proposed changes are highlighted below compared to JCTVC-M1008 [1].

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| … | u(1) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| source_rep_info( i ) |  |
| … |  |

| source_rep_info( id ){ |  |
| --- | --- |
| chroma_format_layer_idc[ id ] | u(2) |
| if( chroma_format_idc[ id ] = = 3 ) |  |
| separate_colour_plane_layer_flag[ id ] | u(1) |
| pic_width_in_luma_samples_layer[ id ] | u(16) |
| pic_height_in_luma_samples_layer[ id ] | u(16) |
| bit_depth_layer_luma_minus8[ id ] | u(3) |
| bit_depth_layer_luma_minus8[ id ] | u(3) |
| layer_color_gamut[ id ] | u(1) |
| } |  |

chroma_format_layer_idc[ id ]  specifies the chroma sampling relative to the luma sampling for the layer with index id as specified in subclause Error: Reference source not found. The value of chroma_format_layer_idc[ id ] shall be in the range of 0 to 3, inclusive.

separate_colour_plane_layer_flag[ id ]  equal to 1 specifies that the three colour components of the 4:4:4 chroma format are coded separately for the layer with index id. separate_colour_plane_layer_flag[ id ]  equal to 0 specifies that the colour components are not coded separately for the layer with index id. When separate_colour_plane_layer_flag[ id ] is not present, it is inferred to be equal to 0. When separate_colour_plane_layer_flag[ id ] is equal to 1, the coded picture for the layer with index id consists of three separate components, each of which consists of coded samples of one colour plane (Y, Cb, or Cr) and uses the monochrome coding syntax.

pic_width_in_luma_samples_layer[ id ]  specifies the width of each decoded picture in units of luma samples for the layer with index id. pic_width_in_luma_samples_layer[ id ] shall not be equal to 0.

pic_height_in_luma_samples_layer[ id ]  specifies the height of each decoded picture in units of luma samples for the layer with index id. pic_height_in_luma_samples_layer[ id ] shall not be equal to 0.

bit_depth_layer_luma_minus8[ id ] + 8 specifies the bit depth of the samples of the luma array for the layer with index id, as specified by

BitDepthLY [ id ]     = 8 + bit_depth_layer_luma_minus8[ id ]

bit_depth_layer_luma_minus8[ id ] shall be in the range of 0 to 6, inclusive.

bit_depth_layer_chroma_minus8[ id ] + 8 specifies the bit depth of the samples of the chroma arrays for the layer with index id, as specified by

BitDepthLC  [ id ]    = 8 + bit_depth_layer_chroma_minus8[ id ]

bit_depth_layer_chroma_minus8[ id ] shall be in the range of 0 to 6, inclusive.

layer_color_gamut[ id ]  equal to 1 specifies that the chromaticity coordinates of the source primaries for the layer with index id are defined as per Rec. ITU-R BT.2020. layer_color_gamut[ id ] equal to 0 specifies that the chromaticity coordinates of the source primaries for the layer with index id are defined as per Rec. ITU-R BT.709.

# Conclusion

This document proposes syntax and semantics for signaling bit-depth, spatial resolution, color chromaticity and color format related information regarding layers in VPS extension.  It is asserted that this is useful for session negotiation.

We propose to adopt the proposal in the working draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 1,” JCTVC-M1008, Incheon, May 2013

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 4 (ISO/IEC 23008-2:201x/PDAM2),” JCTVC-D1004, Incheon, May 2013.