| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0239r1 |
| --- | --- |

| Title: | SCE2: Result of Test 1.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +33(0)299876800 christophe.gisquet@crf.canon.fr +81 50 3750 2741 Kazushi.Sato@jp.sony.com +1-408-352-4086 jun.xu@am.sony.com |
| Source: | Canon, Sony Corp. / Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports result of SCE2: Test 1.1.

# Introduction

Combinations of proposals on motion vector coding and motion data compression are tested under SCE2. Test 1.1 is defined as the combination of the following proposals:

JCTVC-M0112:

In the case that motion data of the base layer is compressed by 4:1, the collocated position of the base layer is calculated as

xRL  = ( ( xRL + R ) >> S ) << S

yRL  = ( ( yRL + R ) >> S ) << S

where (R=4, S=4) when tested in SHM1.0 (16x16 motion compression).

Other values of R and S may be more relevant when M0112 proposal is combined with the other proposal from this SCE. Therefore different values of R and S (e.g. R=2 and S=3 for 8x8 compression) will be evaluated for each of the tested combinations.

JCTVC-M0141:

2-stage motion data compression as follows: first BL motion data is compressed by 2:1 after encoding/decoding of base layer and then by 2:1 after encoding/decoding of the enhancement layer.

# Simulation

## Simulation Condition

SHM-2.0 is used as anchor. On top of anchor proposed methods have been implemented by the proponents.

The followings are macro settings of the test in “Typedef.h”:

For JCTVC-M0112:

#define M0112_POSITION_ROUNDING          1

For JCTVC-M0141:

#define M0141_MOTION_COMP                1

#if M0141_MOTION_COMP

#define HALF_COMP                        1

#endif

{RA,LB,LP}_conditions are tested with Class A&B sequences.

## Simulation Result

Summary of the result is shown in Table 2.2.1. Complete result is shown in JCTVC-N0239.xls.

The authors would like to thank Sharp for crosschecking [5].

Table 2.2.1

# Conclusion

This document reports result of SCE2: Test 1.1. Gain by -0.4%, -0.1%, 0.0% -0.4%, -0.1%, 0.0%, -0.3% -0.1% and 0.0% is obtained with RA_2x, RA_1.5x, RA_SNR, LB_2x, LB_1.5x, LB_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively.

It is recommended that the proposed methods be considered for adoption.

# References

[1] “Non-SCE5: Position derivation for using Base Layer motion in SHVC”, JCTVC-M0112, April 2013

[2] “SCE5: Result of SCE5.2.2”, JCTVC-M0141, April 2013

[3] “Common SHM test conditions and software reference configurations”, JCTVC-M1009, April 2013

[4] “HEVC Scalable Extensions Core Experiment 2 (SCE2): Combination of inter-layer syntax prediction and motion data compression”, JCTVC-M1102, April 2013

[5] “SCE2 Cross check report of test 1.1”, JCTVC-N0257, July 2013

# Patent rights declaration(s)

Canon may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. / Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).