| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0240r1 |
| --- | --- |

| Title: | SCE2: Crosscheck Result of Test 1.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document provides the crosscheck result of SCE2: Test 1.2.

# Introduction

Combinations of proposals on motion vector coding and motion data compression are tested under SCE2. Test 1.1 is defined as the combination of the following proposals:

JCTVC-M0112:

In the case that motion data of the base layer is compressed by 4:1, the collocated position of the base layer is calculated as

xRL  = ( ( xRL + R ) >> S ) << S

yRL  = ( ( yRL + R ) >> S ) << S

where (R=4, S=4) when tested in SHM1.0 (16x16 motion compression).

Other values of R and S may be more relevant when M0112 proposal is combined with the other proposal from this SCE. Therefore different values of R and S (e.g. R=2 and S=3 for 8x8 compression) will be evaluated for each of the tested combinations.

JCTVC-M0258:

For the motion field mapped from the base layer the motion information is replaced with motion information from a candidate enhancement layer motion field, if the candidate enhancement layer motion information is valid and does not reference its own base layer. In particular, the motion information includes - prediction mode, motion vector, reference picture POC, reference picture used as long term status.

The replacement process is controlled at the sequence and slice level using syntax elements sps_override_mfm_flag and el_collocated_enabled_flag respectively. The candidate enhancement layer motion field is identified using slice level syntax elements el_collocated_from_flag and el_collocated_ref_idx.

# Simulation

## Simulation Condition

SHM-2.0 is used as anchor. On top of anchor proposed methods have been implemented by the proponents, which have been successfully verified by the author.

The followings are macro settings of the test in “Typedef.h”:

For JCTVC-M0112:

#define M0112_POSITION_ROUNDING          1

For JCTVC-M0258:

#define SHARP_M0258_CE2_5_3_1            1

{RA,LB,LP}_conditions are tested with Class A&B sequences.

## Simulation Result

Summary of the result is shown in Table 2.2.1. Complete result is shown in JCTVC-N0240.xls.

The result matches the one provided by the proponent [5].

Table 2.2.1

# Conclusion

The result of SCE2: Test 1.2 is verified. It is recommended that the proposed methods be considered for adoption.

# References

[1] “Non-SCE5: Position derivation for using Base Layer motion in SHVC”, JCTVC-M0112, April 2013

[2] “Modified Motion Vector Signalling and Prediction Under Reference Index Based SHVC”, JCTVC-M0258, April 2013

[3] “Common SHM test conditions and software reference configurations”, JCTVC-M1009, April 2013

[4] “HEVC Scalable Extensions Core Experiment 2 (SCE2): Combination of inter-layer syntax prediction and motion data compression”, JCTVC-M1102, April 2013

[5] “SCE2: Results on combination test 1.2”, JCTVC-M0302, July 2013

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.