| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0241r2 |
| --- | --- |

| Title: | SCE2: Result of Test 1.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan Jun Xu 1730 N. First Street San Jose, CA 95112, USA Kiran Misra 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +33(0)299876800 christophe.gisquet@crf.canon.fr +81 50 3750 2741 Kazushi.Sato@jp.sony.com +1-408-352-4086 jun.xu@am.sony.com +1 360 817 8451 misrak@sharplabs.com |
| Source: | Canon, Sony Corp. / Sony Electronics Inc., SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document reports the result of SCE2: Test 1.4.

# Introduction

Combinations of proposals on motion vector coding and motion data compression are tested under SCE2 [5]. Test 1.4 is defined as the combination of the following proposals:

JCTVC-M0112 [1]:

In the case that motion data of the base layer is compressed by 2:1, the collocated position of the base layer is calculated as

xRL  = ( ( xRL + R ) >> S ) << S

yRL  = ( ( yRL + R ) >> S ) << S

where (R=-2, S=3).

The values of R and S depend on the memory compression used, and are therefore different from Test 2.1.

JCTVC-M0141 [2]:

2-stage motion data compression as follows: first BL motion data is compressed by 2:1 after encoding/decoding of base layer and then by 2:1 after encoding/decoding of the enhancement layer.

JCTVC-M0258 [3]:

For the motion field mapped from the base layer the motion information is replaced with motion information from a candidate enhancement layer motion field, if the candidate enhancement layer motion information is valid and does not reference its own base layer. In particular, the motion information includes - prediction mode, motion vector, reference picture POC, reference picture used as long term status.

The replacement process is controlled at the sequence and slice level using syntax elements sps_override_mfm_flag and el_collocated_enabled_flag respectively. The candidate enhancement layer motion field is identified using slice level syntax elements el_collocated_from_flag and el_collocated_ref_idx.

# Simulation

## Simulation Condition

SHM-2.0 is used as anchor. On top of anchor proposed methods have been implemented by the proponents.

The followings are macro settings of the test in “TypeDef.h”:

For JCTVC-M0112:

#define M0112_POSITION_ROUNDING          1

For JCTVC-M0141:

#define M0141_MOTION_COMP                1

#if M0141_MOTION_COMP

#define HALF_COMP                        1

#endif

For JCTVC-M0258:

#define SHARP_M0258_CE2_5_3_1            1

{RA,LB,LP} conditions [4] are tested with Class A&B sequences.

## Simulation Result

Summary of the result is shown in Table 2.2.1. Complete result is shown in JCTVC-N0241.xls.

The authors would like to thank LG for crosschecking [6].

Table 2.2.1

# Conclusion

This document reports result of SCE2: Test 1.4. Gain by -0.8%, -0.3%, 0.2%, -0.4%, -0.1%, 0.0%, -0.3%, -0.1% and 0.0% is obtained with RA_2x, RA_1.5x, RA_SNR, LB_2x, LB_1.5x, LB_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively.

It is recommended that the proposed methods be considered for adoption.

# References

[1] “Non-SCE5: Position derivation for using Base Layer motion in SHVC”, JCTVC-M0112, April 2013

[2] “SCE5: Result of SCE5.2.2”, JCTVC-M0141, April 2013

[3] “Modified Motion Vector Signalling and Prediction Under Reference Index Based SHVC”, JCTVC-M0258, April 2013

[4] “Common SHM test conditions and software reference configurations”, JCTVC-M1009, April 2013

[5] “HEVC Scalable Extensions Core Experiment 2 (SCE2): Combination of inter-layer syntax prediction and motion data compression”, JCTVC-M1102, April 2013

[6] “Crosscheck of SCE 2 test 1.4”, JCTVC-N0300, July 2013

# Patent rights declaration(s)

Canon may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp./Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SHARP Laboratories of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).