| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0246 |
| --- | --- |

| Title: | AHG5: Modified SAO for range extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, chihming.fu, yuwen.huang,shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This proposal attempts to improve the coding efficiency of the existing SAO coding tool for HEVC range extension applications. The proposed method includes a new set of syntax elements such that the SAO coding tool can be effectively adapted to different sample bit depths, coding bitrates and sampling formats while making no significant modifications to the existing SAO method in the current HEVC. The experimental results on RExt sequences under the common test conditions reportedly show overall luma BD-rate savings as 0.2%, 0.5%, and 0.7% for AI-Main, RA-Main, and LB-Main, respectively.

# Introduction

In the HEVC standard, the sample-adaptive offset (SAO) processing is adopted to reduce the distortion of reconstructed pictures. The SAO tool supports two kinds of sample classification methods: band offset (BO) and edge offset (EO). The concept of EO is to classify the reconstructed pixels into categories by comparing the current sample with its neighboring samples along the direction identified by the EO classes. The category index, cat_idx, for the current sample “c” is determined by

cat_idx = sign(c-c1) + sign(c-c-1)+ 2                                                  (1)

sign(x) = 1;x>00;x=0−1;x<0                                                         (2)

In Equ. (1) and Equ. (2), “c1” and “c-1” are the neighboring samples corresponding to a given EO class. Fig. 1 shows the four EO classes with selections of neighboring samples from different orientations. An offset value is derived for all samples in each category. Four offset values, respectively corresponding to category indices 1 – 4, are coded in one coding tree block in HEVC.

Fig. 1.  Four EO classes in HEVC

In the HEVC standard, the syntax elements sao_offset_sign and sao_offset_abs respectively represent the sign and scaled magnitude of an offset value, SaoOffsetVal, given by

SaoOffsetVal =	
		offsetSign * sao_offset_abs <<  ( bitDepth − Min( bitDepth, 10 ) )               (3)

In Equ. (3), bitDepth is the bit depth for the current color component, offsetSign is equal to -1 when sao_offset_sign is equal to 1 and equal to 1 otherwise. The syntax sao_offset_abs is entropy coded using truncated Rice (TR) binarization process with the parameters given by

cMax = ( 1  <<  ( Min( bitDepth, 10 ) − 5 ) ) − 1, cRiceParam = 0                       (4)

# Proposed method

The proposed method attempts to improve the coding efficiency of the existing SAO coding tool for application to different video formats and configurations. The new method includes three new syntax elements, sao_sign_thre, sao_bit_shift, and sao_eo_offset_max, in the slice header.

The syntax element sao_sign_thre is utilized in the modified scheme for sample classification for EO. The category index, cat_idx, for the current pixel “c” in the modified method is determined by

cat_idx = sao_sign(c-c1) + sao_sign(c-c-1) + 2                                         (5)

sao_ sign(x) = 1;x>=saoSignThre0;|x|<saoSignThre−1;x<=−saoSignThre                                       (6)

In Equ (6), variable saoSignThre is specified by the syntax element sao_sign_thre. It should be noted that the new classification method is equivalent to the current HEVC method defined by Equ. (1) and Equ. (2) when the decision threshod saoSignThre is fixed to be equal to 1. However, it should be noted that the effective “zero interval” for decision function in Equ. (2) in the current HEVC is decreasing with the increasing bitdepth without having the decision threshod scaled with the bitdepth.

The syntax element sao_bit_shift is utilized in the modified scheme for representing the offset value SaoOffsetVal, given by

SaoOffsetVal =	
		offsetSign * sao_offset_abs <<  saoBitShift                                 (7)

In Equ. (7), variable saoBitShift is specified by the syntax element sao_bit_shift, which is coded for luma and chroma components, separately, in each slice.

The new syntax element sao_offset_max is utilized for representing the offset value. In the modified method, the syntax element sao_offset_abs is entropy coded using truncated Rice (TR) binarization process with the parameters given by

cMax = saoOffsetMax, cRiceParam = 0                                           (8)

In Equ. (8), variable saoOffsetMax is set to the current cMax parameter specified by Equ. (4) for BO and is set to the syntax value of sao_eo_offset_max for EO. The syntax sao_eo_offset_max is separately coded for each color component and for each EO offset index in a slice.

# Experimental results

The proposed algorithm has been evaluated according to the common test conditions defined in [1]. The BD-rate results are summarized in Table 1.

Table 1.  BD-rate results of the proposed algorithm (short length for now, will be updated)

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U |  | V |  |
| RGB 4:4:4 | -0.3% | -0.2% | -0.3% | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% |  | -0.1% |  |
| YCbCr 4:4:4 | -0.3% | -0.3% | -0.4% | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% |  | -0.2% |  |
| YCbCr 4:2:2 | -0.1% | -0.6% | -0.9% | -0.1% | -0.2% | -0.4% | -0.1% | -0.1% |  | -0.2% |  |
| Overall | -0.2% | -0.3% | -0.5% | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% |  | -0.2% |  |
|  | -0.2% | -0.3% | -0.5% | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% |  | -0.2% |  |
| Enc Time[%] | 106% |  |  | 105% |  |  | 104% |  |  |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |  |  |
| RGB 4:4:4 | -0.7% | -1.3% | -0.8% | -0.7% | -0.9% | -0.6% |  |  |  |  |  |
| YCbCr 4:4:4 | -0.4% | -2.6% | -0.3% | -0.6% | -1.6% | -0.1% |  |  |  |  |  |
| YCbCr 4:2:2 | -0.3% | -1.3% | -1.3% | -0.3% | -1.1% | -0.6% |  |  |  |  |  |
| Overall | -0.5% | -1.7% | -0.8% | -0.5% | -1.2% | -0.4% |  |  |  |  |  |
|  | -0.5% | -2.8% | -0.8% | -0.5% | 4.0% | -0.4% |  |  |  |  |  |
| Enc Time[%] | 102% |  |  | 102% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |  |  |
| RGB 4:4:4 | -0.9% | -1.2% | -1.1% | -0.7% | -0.8% | -0.7% |  |  |  |  |  |
| YCbCr 4:4:4 | -0.8% | -2.2% | -0.9% | -0.8% | -1.5% | -0.7% |  |  |  |  |  |
| YCbCr 4:2:2 | -0.4% | -1.5% | -1.4% | -0.3% | -1.0% | -0.8% |  |  |  |  |  |
| Overall | -0.7% | -1.6% | -1.1% | -0.6% | -1.1% | -0.7% |  |  |  |  |  |
|  | -0.7% | -3.3% | -1.1% | -0.6% | -0.2% | -0.7% |  |  |  |  |  |
| Enc Time[%] | 102% |  |  | 102% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |  |  |

# References

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1006, January, 2013.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).