The proposed modifications are based on Range Extensions text specification: Draft 3 (JCTVC-M1005). The inserted parts are marked in yellow.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ….. |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| if (sample_adaptive_offset_enabled_flag && general_profile_idc == 4) { |  |
| adaptive_sao_params_active_flag | u(1) |
| if (adaptive_sao_params_active_flag) { |  |
| adaptive_sao_bit_shift_active_flag | u(1) |
| adaptive_sao_sign_thre _active_flag | u(1) |
| adaptive_sao_eo_offset_max _active_flag | u(1) |
| } |  |
| } |  |
| pcm_enabled_flag | u(1) |
| ……. |  |
| } |  |

#### 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ……. |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType > 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_sao_luma_flag && adaptive_sao_bit_shift_active_flag ) |  |
| sao_luma_bit_shift | ue(v) |
| if( slice_sao_chroma_flag && adaptive_sao_bit_shift_active_flag ) |  |
| sao_chroma_bit_shift | ue(v) |
| if( ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag) ) |  |
| { |  |
| if (adaptive_ sao_sign_thre_active_flag) |  |
| sao_sign_thre | ue(v) |
| if(adaptive_ sao_eo_offset_max_active_flag) |  |
| default_sao_eo_max_offset_flag | u(1) |
| } |  |
| if( slice_sao_luma_flag && ! default_sao_eo_max_offset_flag ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_eo_offset_max[0][i] | ue(v) |
| if ( slice_sao_chroma_flag && ! default_sao_eo_max_offset_flag ) |  |
| for ( cIdx=1; cIdx<3; cIdx++ ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_eo_offset_max[cIdx][i] | ue(v) |
| ……. |  |
| } |  |