| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0247 |
| --- | --- |

| Title: | RCE3: Results of Test 3.1 on Palette Mode for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of RCE3 test 3.1 on palette coding for screen content.

# Introduction

The coding of screen contents plays an important role in applications like remote desktop control and on-line collaboration. Compared to natural video content, screen content tends to have much fewer distinct pixel values (discrete tone) and much sharper object edges. A palette-based coding method (JCTVC-M0323) [1] was presented in Incheon meeting and was included as test 3.1 in RCE3 for Vienna meeting [2]. This contribution reports the evaluation results.

# Proposed method

A palette is a set of representative pixel values in the current CU. For locations with pixel values in the palette, only a palette index needs to be transmitted (instead of the pixel value). Furthermore, run coding is used when a number of consecutive locations have the same pixel value (“run mode”) or when a number of consecutive locations have the same value as their top neighboring pixels (“copy above mode”). The following is a more detail step-by-step explanation of palette-based coding.

For each CU, a flag palette_mode_flag is signaled to indicate if the current CU is encoded as palette mode. If the flag equals to 1, the following process is performed.

- Transmission of the palette: a flag palette_pred_flag is first transmitted to indicate if the current CU copies the palette from its left CU. If the flag equals to 0, the palette is transmitted for the CU. Each color component has its own palette transmitted. The maximum value of the palette size is set to 4.

- Transmission of the CU pixel values: the locations in the CU are encoded in raster scan order, and there can be 3 different modes.

  - “Copy above mode”: for each location, a flag copy_above_flag is first transmitted to indicate if it switches to “copy above” mode (copy_above_flag = 1) or not (copy_above_flag = 0). In “copy above” mode, a syntax copy_run (e.g., copy_run = N) is transmitted to indicate for the following N positions, the pixel values (or palette indices) are copied from the same location in the above row (and thus no extra information need to be transmitted).

  - “Run mode”: if it is not “copy above” mode, a flag palette_flag is transmitted to indicate if it switches to “run mode” (palette_flag = 1) or “pixel mode” (palette_flag = 0). In “run mode”, palette_index is first signaled followed by palette_run (say, palette_run = M). No further information needs to be transmitted as these M locations have the same pixel value as represented by palette_index.

  - “Pixel mode”: In “pixel mode”, the pixel sample value is transmitted.

Lossy coding:

Palette mode can be applied to lossless coding and lossy coding. In lossy coding, an error tolerance d is introduced. When the difference between the pixel value and a palette element, P, is smaller than d, the pixel is encoded with the palette index. Element P is the reconstructed value at the decoder.

# Experimental results

The proposed method is implemented on HM-10.1+RExt-3.0. Test conditions specified in [2] are used. Encoding time and decoding time is measured on a homogenous cluster.

## Lossless results

Table 1  Results of proposed method for lossless coding

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.3% | 31.7 | 32.1 | -0.6% | 49.8 | 50.0 | -0.2% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 15.1 | -24.9% | 100.4 | 151.2 | -20.0% | 381.6 | 584.6 | -18.0% |
| SC YUV 444 | 11.4 | 14.0 | -11.6% | 128.9 | 157.7 | -10.2% | 325.6 | 426.7 | -9.4% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 113% |  |  | 105% |  |  | 104% |  |  |
| Dec Time[%] | 83% |  |  | 98% |  |  | 98% |  |  |

## Lossy results

Table 2 Results of proposed method for lossy coding

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.4% | -0.3% | -0.2% | -0.8% | -0.7% | -0.5% | -1.6% | -1.3% | -1.1% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -14.4% | -14.7% | -14.8% | -18.9% | -19.0% | -19.1% | -24.6% | -25.3% | -25.4% |
| SC YUV 444 | -3.7% | -3.4% | -3.6% | -5.9% | -5.3% | -5.9% | -9.3% | -8.8% | -9.6% |
| RangeExt | 0.3% | 0.3% | 0.4% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 110% |  |  | 109% |  |  | 108% |  |  |
| Dec Time[%] | 93% |  |  | 90% |  |  | 87% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.5% | -0.1% | -0.7% | -0.8% | -0.4% |  |  |  |
| Class B | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -13.1% | -13.3% | -13.4% | -17.2% | -17.2% | -17.5% |  |  |  |
| SC YUV 444 | -3.5% | -3.2% | -3.6% | -5.5% | -5.1% | -5.8% |  |  |  |
| RangeExt | 0.2% | 0.4% | 0.4% | 0.2% | 0.2% | 0.3% |  |  |  |
| Enc Time[%] | 104% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | -0.6% | 0.3% | -0.1% | -0.7% | 0.0% |  |  |  |
| Class B | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -9.1% | -9.1% | -9.2% | -13.7% | -13.7% | -14.0% |  |  |  |
| SC YUV 444 | -2.6% | -2.5% | -2.8% | -4.4% | -4.0% | -4.7% |  |  |  |
| RangeExt | 0.3% | 0.2% | 0.4% | 0.2% | 0.1% | 0.2% |  |  |  |
| Enc Time[%] | 103% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |  |  |  |

# Complexity Analysis

## Throughput

In “pixel mode”, the pixel reconstruction value is the received sample value. In “run mode”, the pixel reconstruction value equals to the value of palette [index]. In “copy above mode”, at the parsing stage the position (y, x) can be set to be palette[index] (“run mode”) or sample value (“pixel mode”).  In both cases a pixel reconstruction is independent of its adjacent pixels and thus all the samples in an intra CU could be encoded and decoded in parallel.

## Number of operations

Palette-based coding does not involve any inter-pixel prediction. In parsing stage, locations with “copy above mode” can be converted to either “pixel mode” or “run mode”. In “pixel mode”, the reconstruction value is the parsed sample value and thus no extra operation is needed. In “run mode”, the reconstruction value is palette[index] where palette and index are obtained at the parsing stage and only one LUT operation (the table/palette size is 4) is needed for decoding.

## Line buffers

The reconstruction of the pixel values do not depend on values of previous CU, so no line buffers of the reconstructed picture are necessary. Similarly, the palette parameters do not depend on the parameters of the above CU.

# Conclusions

This contribution describes RCE3 test 3.1 on palette-based coding for screen content and reports the results under RCE3 common test conditions.

# Draft texts

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if (cu_transquant_bypass_flag) |  |
| palette_mode_flag[x0][y0] | ae(v) |
| if (palette_mode_flag) { |  |
| palette_pred_flag[x0][y0] | ae(v) |
| palette_coding ( x0, y0, nCbS, nCbS, 0, palette_pred_flag) |  |
| palette_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC , 1) |  |
| palette_coding ( x0, y0, nCbS/ SubWidthC , nCbS/ SubHeightC, 2) |  |
| } |  |
| else |  |
| { |  |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| …… |  |
| } |  |

| palette_coding ( x0, y0, nPbW, nPbH , cIdx){ |  |
| --- | --- |
| if (!palette_pred_flag[x0][y0]){ |  |
| palette_size_minus1[x0][y0] | ae(v) |
| for( i = 0; i <=palette_size_minus1[x0][y0]; i ++ ) |  |
| palette[x0][y0][i] | ae(v) |
| } |  |
| else{ |  |
| palette_size_minus1[x0][y0] = x0? (palette_mode_flag[x0-1][y0]? palette_size_minus1[x0-1][y0]: 0 ) : 0 |  |
| for(i = 0; i <= palette_size_minus1 [x0][y0]; i ++ ) |  |
| palette[x0][y0] [i]= x0?palette[x0-1][y0] [i]:0 |  |
| } |  |
| scanPos = 0 |  |
| while (scanPos < nPbW * nPbH) { |  |
| copy_above_flag | ae(v) |
| if (copy_above_flag){ |  |
| copy_run |  |
| run = copy_run |  |
| } |  |
| else{ |  |
| palette_flag |  |
| if (palette_flag) { |  |
| if (palette_size > 1) { |  |
| palette_index |  |
| } | ae(v) |
| palette_run |  |
| run = palette_run |  |
| } |  |
| else{ |  |
| sample_value |  |
| xC = scanPos% nPbW |  |
| yC = scanPos/ nPbW |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = sample_value |  |
| scanPos++ |  |
| } |  |
| If (copy_above_flag ¦¦ palette_flag){ |  |
| runPos= 0 |  |
| while (runPos <= run){ |  |
| xC = scanPos% nPbW |  |
| yC = scanPos/ nPbW |  |
| if (palette_flag){ |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = palette[x0][y0] [paletteIdx] |  |
| } |  |
| else{ |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC-1 ] |  |
| } |  |
| runPos ++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Semantics

palette_mode_flag[x0][ y0] equal to 1 specifies that the current coding unit is encoded using palette mode. palette_mode_flag  equal to 0 specifies the current coding unti is encoded using a mode other than palette mode.

When palette_mode_flag is not present, it is inferred to be equal to 0.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

palette_pred_flag[x0][y0] equal to 1 specifies for the current CU, palette_size_minus1 [x0][y0] and palette[x0][y0][i], i = 0.. palette_size_minus1[x0][y0], are not parsed. palette_pred_flag[x0][y0] equal to 0 specifies that they are not skipped.

When palette_pred_flag[x0][y0] is not present, it is inferred to be equal to 0.

palette_size_minus1[x0][y0] plus 1 specifies the number of elements in the palette. palette_size_minus1[x0][y0] shall be the range of 0 to 3.

When palette_size_minus1 [x0][y0] is not present, it is inferred to be equal to 0.

palette[x0][y0][i] specifies the i-th element in the palette.

When palette[x0][y0][i] is not present, it is inferred to be equal to 0.

copy_above_flag equal to 1 specifies that the decoding is switched to “copy from above mode” where the decoded pixel value is equal to the pixel at the same location in the above row. copy_above_flag equal to 0 specifies that the decoding is switched to either “run mode” or “pixel mode”.

When copy_above_flag is not present, it is inferred to be equal to 0.

palette_flag equal to 1 specifies that the decoding is swtiched to “run mode” where the decoded pixel value is equal to palette[palette_index]. The palette_flag equal to 0 specifies that the decoding is switched to “pixel mode” where the decoded pixel value is the parsed sample value.

When palette_flag is not present, it is inferred to be equal to 0.

palette_index represents an index in the palette.

When palette_index is not present, it is inferred to be equal to 0.

palette_run represents the number of consecutive locations with the same palette entry.

sample_value represents a coded sample value. The number of bits used to represent each of these samples is BitDepthY for cIdx = 0 and BitDepthC for cIdx = 1 or 2.

## Decoding process:

No extra decoding process is needed as the reconstructed pixel values equal to TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ].

# References

- L. Guo, M. Karczewicz, and J. Sole. “Palette Mode for Screen Content Coding”, JCTVC-M0323, Incheon, Korea, Apr. 2013.

- L. Guo, M. Karczewicz, and J. Sole. “RCE3: Results of Test 3.1 on Palette Mode for Screen Content Coding”, JCTVC-L0XXX, Vienna, Austria, Jul.-Aug. 2013.

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE 3): Intra Coding Methods for Screen Content”, JCTVC-M1123, Incheon, Korea, Apr. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).